3GPP TSG RAN WG1 #105-e                                              R1-2104518

e-Meeting, May 19th –May 27th, 2021

Agenda Item:	8.4.3

Source:	CATT

Title:	HARQ operation enhancement for NTN

Document for:	Discussion and Decision

Introduction

For the enhancements on NTN HARQ, there were some discussions on enhancement on HARQ process ID indication, Disabling/enabling HARQ feedback, transmission enhancement and HARQ feedback enhancement etc. Moreover, a few agreements on HARQ operation were concluded as follows [1] in last RAN1 meeting:

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement:

For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

In this contribution, we further analyze remaining technical issues of NTN HARQ operation and provide our views on these issues.

Discussion

  - Enhanced HARQ process ID indication

Since the maximal supported HARQ process number is up to 32, there is one remaining issue needed to consider, this is, how to configure HARQ ID more than 16 in the DCI. For DCI 0-1/1-1, several possible solutions have been discussed at RAN1#103e. In the following paragraphs, some further comparisons are made.

  - Alt 1: Using slot index as additional HARQ bit

If additional 1 bit is placed on MSB, then even slot can only accommodate the first 16 processes, and odd slot can only accommodate the last 16 processes. Nevertheless, if the additional 1 bit is placed on the LSB, the odd slot is mapping to the odd process, and the even slot is mapping to the even process. By comparison, using slot index as HARQ ID LSB seems more flexible, since it can enable asynchronous HARQ process configuration.

  - Alt 2: Reusing one bit from other DCI bit field

For reusing one bit from other bit field, one simple approach is to re-interpret the second data block indication since NTN scneario will not support more than one stream transmission. Other possible way is to reuse 1 bit of RV field, e.g, only RV0 and RV3 are used, but it will degrade the performance in certain degree.

- Alt 3: Extending the HARQ process ID field up to 5 bits

If extending the HARQ process ID field to 5 bits, the size of DCI of NTN will be different to that of TN, so it should define a new rule to faciliate DCI blind detection of NTN UE, for example, using RRC signalling to inform UE working in NTN mode, then NTN UE would have specific DCI size. As a result, NTN UE would require new DCI detecion behavior.

- Additional HARQ bit can be taken from second block DCI field if only one layer tansmission supported in NTN.

- Consider to use DCI field of second block as additional HARQ bit indication to support 32 HARQ processes in DCI 0-1/1-1.

With respect to the non-fallback DCI 0-2/1-2, this feature is defined for URLLC type UE which are not a feature in NTN, and there is no need to support it in NTN.

For DCI 0-0/1-0, since 32 HARQ processes are used for high throughput, the scheduling of which can be carried out with non-fallback DCI 0_1/1_1. Therefore, in NTN case, 16 processes are sufficient for fallback DCI 0_0/1_0.

- 16 HARQ processes for DCI 0-0 /1-0 can be supported.

- No HARQ enhancement for DCI 0-2/1-2 is needed.

It was discussed whether to determine the maximum number of processes based on UE capabilities at RAN1#103e[2]. Regardless whether the UE capability report is supported, UE expects that the network can configure a maximum of 16 processes in the fallback case. In non-fallback case, the network will configure the maximum process according to the reported capability if UE capability reporting is supported, otherwise, a maximum of 32 HARQ processes per cell is supported by the UE.

- 32 processes can be supported based on UE capabilities.

  - Enhancement on HARQ-ACK codebook

In term of HARQ-ACK codebook, one new issue is that if one HARQ process is not configured with ACK/NACK feedback, whether to redesign the HARQ-ACK reported mechanism or format. There are two main different types of Codebook determination mechanism, called Type 1 and Type 2 in the NR Rel-15. In the last meeting, there was a heated discussion about whether and how to enhance these codebooks.

Type 1 is semi-static HARQ-ACK codebook, where the UE determines a set of occasions for candidate PDSCH receptions on which the UE can transmit corresponding HARQ-ACK information in a PUCCH/PUSCH in slot n based on a set of values K1, possible PDSCH transmission occasion, and TDD UL-DL-configuration. The size of the semi-statice HARQ-ACK codebook is fixed based on configuration and should not be change dynamically.

Based on last meeting discussion, a proposal is made as follows for type 1 codebook feedback:

  - if only disabled HARQ process are used for scheduling in occasions, no HARQ-ACK feedback is expected at UE side. Otherwise, legacy behavior is assumed.

The proposal is only applicable to the case that DCI decoding at UE side is successful in all  occasions, which is contrary to the principle that Type-1 codebook does not depend on DCI detection. The original intention of the type1 codebook is to avoid inconsistent understanding between the gNB and the UE due to the missed DCI detection. Therefore, one simpler approach for minimizing specification impact is to insert ACK or NACK depending on the decoding results of the PDSCHs, no matter it is the PDSCH with or without HARQ-ACK feedback disabling. With this method, the network can be aware of the transmisson quality so that it can adjust the transmission parameters, e.g. aggregation factor, MCS, etc.The example is as shown Figure 1.

Figure 1 Type1 codebook with disabling and enabling HARQ-ACK feedback

- No enhancement is needed for type 1 HARQ-ACK codebook.

Type 2 codebook is a dynamic HARQ-ACK codebook, where UE determines the HARQ-ACK codebook based on the K1 value, C-DAI and T-DAI in the DCI format1_1 and 1_0, PDCCH monitoring occasions across configured cells, and CBG transmission information. Type2 codebool is designed to save the overhead, so we should make some enhancements on dynamic HARQ-ACK codebook with disabling/enabling HARQ feedback for NTN case. Regarding the enhancement on Type 2 HARQ codebook, it has been agreed that only PDSCH with feedback-enabled HARQ processes should be included in HARQ-ACK codebook to reduce codebook size. As for the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes, several candidate options were proposed at RAN1#103e [2] as follows:

    - Alt-1: C-DAI and T-DAI count only PDSCH with feedback-enabled HARQ processes.

      - In the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes)

      - In the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes), despite they are not incremented.

    - Alt-2: C-DAI and T-DAI count only PDSCH with feedback-enabled HARQ processes.

      - In the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes)

      - In the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are given a reserved value.

    - Alt-3: HARQ-ACK codebook only includes HARQ-ACK of enabled PDSCH.

      - C-DAI/T-DAI value in DCI scheduling enabled PDSCH is counted for enabled PDSCH

      - C-DAI/T-DAI value in DCI scheduling disabled PDSCH is counted for disabled PDSCH.

The difference between Alt-1 and Alt-2 is the DAI value of PDSCH with feedback-disabled HARQ processes. Compared with Alt-2, Alt-1 is more reliable when the UE misses a certain DCI. The example of Alt-1 is as shown In the Figure 2. For example, if the DCI of the feedback enabled process is missed by the UE in monitor occasion m=3, the UE generates 5 HARQ-ACK information bits according to the DAI filed of the last two DCIs. When this happens in Alt-2, UE would determine the total number of HARQ-ACK bits for the dynamic codebook as 4.This would lead to inconsistent understanding between the gNB and the UE. Regarding Alt 3, it can provide additional information on whether DCI for disabled HARQ process is missed by a UE. However, it makes no sense for scheduling because of no HARQ-ACK feedback for disabled HARQ process. So we prefer to support Alt-1, in which the counter DAI and total DAI for a PDSCH with a feedback disabled HARQ process are the same as the last PDSCH with a feedback enabled HARQ process. Furthermore, a UE can simply ignore counter DAI from a PDCCH that is associated with a feedback disabled HARQ process. If total DAI is present, the last one indicating the sum of all the scheduled PDCCHs is associated with feedback enabled HARQ process.

Moreover, when the PDSCHs with feedback-disabled HARQ processes is scheduled before the first PDSCH with feedback-enabled in monitor occasion m=0, the the C-DAI and T-DAI value in DCI filed of the PDSCHs with feedback-disabled HARQ processes should be all set to ‘00’.

Figure 2 Type2 codebook with disabling and enabling HARQ-ACK feedback

- Type 2 HARQ-ACK codebook can be optimized, and the counter DAI and total DAI for a PDSCH with a feedback disabled HARQ process are the same as the previous PDSCH with a feedback enabled HARQ process.

Regarding SPS case, if activation and release are not received correctly by UE, it will cause data transmission mess and signal interference in a long period of time. In order to ensure the reliability, it is necessary to report feedback for the first SPS PDSCH reception and the release. Therefore,UE expects that only one HARQ process with feedback for DL scheduling is configured for the scheduling of SPS PDSCH via RRC signalling.

- For SPS case, the HARQ-ACK feedback for activation and release command can be enabled.

Although it has been agreed that the Type-3 is still applicable for licensed spectrum in RAN1#103e, and it is still an optional UE feature and its performance is always worse than Type-2 which is a mandatory UE feature. There is no need to support and enhance it in NTN.

In conlusion, for type 1 HARQ-ACK codebook, it seems no reason to introduce new design. With same overhead, no need to restrict ACK/NACK feedback. For type 2 HARQ-ACK codebook, we may make some enhancements on type 2 HARQ-ACK codebook from saving overhead perspective. For type 3 HARQ-ACK codebook, it is no need for NTN.

- Type 3 HARQ-ACK codebook is not needed in NTN case.

  - Restriction on HARQ feedback disabling

According to agreement above on HARQ feedback disabling at RAN1#102e[3], Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling. Some companies proposed that MAC CE action timing can be defined based on K1 value indicated in the DCI scheduling the HARQ-feedback disabled process similar to the HARQ-feedback enabled case in last meeting. But in this way, the gNB does not know whether UE has successfully decoded the PDSCH with MAC CE packets which depend on HARQ-ACK, and would lead to inconsistency between gNB and UE in understanding whether MAC CE command is effective or not, obviously, it is untolerable. UE expects that any PDSCH carrying a MAC CE command, whose activation/deactivation time is coupled to the transmission time of the associated HARQ-ACK, is scheduled via a HARQ process with HARQ feedback enabled.

- UE expects that at least one HARQ process with feedback is configured for the scheduling of MAC-CE.

  - Enhancement on the transmission

Since satellite communication sometimes works in harsh weather conditions, such as heavy rain, some proposals for transmission enhancement were raised in the last meeting. A few candidates are listed based on last meeting discussion:

  - Enhancements on aggregated transmission (including repetition)

  - Enhancements on MCS (including CQI report)

For slot aggregation, it is also a NR Rel-15 feature, which is used for enhancement on transmission of downlink and uplink, then it is nature to use it as the baseline to improve the reliability. And we do see the benefit of having larger aggregation/repetition factor.

- Slot-aggregation factor can be extended to 16 for very low SINR case.

Considering the rain and cloud attenuation in NTN channel, UE will suffer from very low SINR. It is better to provide more robustness in specification defining. Then one simple way is to increase repeated transmission times. More specifically, it should allow to configure larger slot-aggregation/repetition factor. So far, NR only defines 8 repetitions at most, so it is expected to support more 8 repetitions, e.g 16 or 32. Thus, only one RRC parameter is changed, no additional specification impact.

Considering time selectivity of NTN channel, the time interleaved slot aggregation scheme can be considered. The motivation is that NTN system will have residual doppler shift in most of time, so the time interleaved slot aggregation will achieve the diversity gain. One detailed illustration is shown in the Figure 3. Two slot-aggregations are assoicated with different HARQ processes, and transmission redundant verison is interlocked. For the time gap between two repeated transmissions, RRC signalling can be used to indicate it.

Figure 3 time interleaved slot aggreation transmission.

- Support time interleaved slot aggregation to improve transmission reliability.

For MCS enhancement, we need to analyze it on top of URLLC feature with the BLER targets at 10% and 0.001% two MCS tables. In addition, in most of cases, NTN channel is similar to AWGN channel due to strong LOS presence. So small fluctuation of SINR will have significantly impact to CQI estimation, but larger feedback delay in NTN scenario will eat the gain of finer CQI reporting since it is impossible to get very accurate CQI. Hence, we think the motivation to enhance the MCS/CQI is not so strong.

- There is no need for MCS enhancement.

  - PDSCH/ PUSCH scheduling restriction

Regarding the PDSCH scheduling restriction for HARQ disabling, there is one agreement based on last meeting discussion.

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

The X = Tproc,1 is enough as the gap between two PDSCH with same HARQ process. There is no additional UE processing burden and it does not need to be extended to K1 which is related to gNB scheduling.

- The minimum time gap between two neighbouring disabled PDSCHs with same HARQ ID should be X = T_proc,1.

For the PUSCH scheduling, it has been agreed that “enabling”/”disabling” HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism at RAN2#112e[4]. But as defined in 38.214[5]:

The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

The specification means that the scheduling of another PUSCH of a given HARQ process should be later than the end of the transmission of another PUSCH of that HARQ process.This restriction is contrary to the conclusion of RAN2 and needs to be removed.

- There is no need to follow the legacy scheduling restriction of PUSCH in NTN.

Conclusion

In the previous sections, we analyzed potential issues of HARQ impact due to longer propogation delay in NTN. We raised a set of observations and solutions for HARQ operation enhancement:

Observation 1:Additional HARQ bit can be taken from second block DCI field if only one layer tansmission supported in NTN.

- Consider to use DCI field of second block as additional HARQ bit indication to support 32 HARQ processes in DCI 0-1/1-1.

- 16 HARQ processes for DCI 0-0 /1-0 can be supported.

- No HARQ enhancement for DCI 0-2/1-2 is needed.

- 32 processes can be supported based on UE capabilities.

- No enhancement is needed for type 1 HARQ-ACK codebook.

- Type 2 HARQ-ACK codebook can be optimized, and the counter DAI and total DAI for a PDSCH with a feedback disabled HARQ process are the same as the previous PDSCH with a feedback enabled HARQ process.

- For SPS case, the HARQ-ACK feedback for activation and release command can be enabled.

- Type 3 HARQ-ACK codebook is not needed in NTN case.

- UE expects that at least one HARQ process with feedback is configured for the scheduling of MAC-CE.

- Slot aggregation factor can be extended to 16 for very low SINR case.

- Support time interleaved slot aggregation to improve transmission reliability.

- There is no need for MCS enhancement.

- The minimum time gap between two neighboring disabled PDSCHs with same HARQ ID should be X = T_proc,1.

- There is no need to follow the legacy scheduling restriction of PUSCH in NTN.

References

- RAN1 #104-e Chairman notes

- RAN1 #103-e Chairman notes

- RAN1 #102-e Chairman notes

- RAN2 #112-e Chairman notes

- 3GPP TS 38.214: "NR; Physical layer procedures for data"