3GPP TSG RAN WG1 #105-e		R1-2104617

e-Meeting, May 10th – 27th, 2021

Source: 	CMCC

Title:	Discussion on reduced number of Rx branches

Agenda item:	8.6.1.2

Document for:	Discussion & Decision

- Introduction

During RAN#91e, the WID for RedCap was updated, with the objective related to reduced number of Rx branches as following [1],

- Reduced minimum number of Rx branches:

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

  - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

  - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

In RAN1#104-e, the following agreements for reduced minimum number of Rx branches of RedCap UEs were made [2],

Agreements:

- For reduced minimum number of Rx branches in FR1 and FR2 frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports:

  - FFS: need for solutions to reduced PDCCH blocking

  - FFS: need for reporting of UE antenna related information to gNB (e.g., # of panels, polarization, etc.)

  - Information related to the reduction of the number of antenna branches is assumed to be known at the gNB (either implicitly or explicitly, to be FFS)

In RAN1#104b-e, the following Rx branches number related agreements are made[3],

Agreements:

- At least using UE capability report according the existing framework to indicate (implicitly or explicitly) the number of Rx branches

- FFS: whether/how to support earlier indication of Redcap UEs with # Rx branches by Msg1 and/or Msg3, and MsgA

  - FFS: Network configurability of early indication of the number of Rx branches via SIB1, if supported

Agreements:

- Reuse at least the existing DCI formats 0_x/1_x (including Rel-16 DCI format 0_2/1_2) applicable to Redcap devices as a starting point.

  - FFS Whether and how potential modification on fields of existing DCI formats is considered to reduce PDCCH block issue, if any.

  - FFS: Which DCI formats are mandatory for the RedCap UEs to support.

In this contribution, earlier indication of Redcap UEs with # Rx branches and schemes to reduce PDCCH blocking are discussed.

- Discussion on earlier indication of Redcap UEs with # Rx branches

As described in the WID, a means shall be specified by which the gNB can know the number of Rx branches of the UE. During last RAN1 meeting, using UE capability report had been agreed for indication of Rx branches. This is useful since early identification is configurable by gNB, and when the network decides that there is no need to distinguish RedCap devices from the initial access, all the initial procedures for non-RedCap devices should be reused, including the capability report framework, therefore, using UE capability report according the existing framework to indicate (implicitly or explicitly) the number of Rx branches is the baseline scheme.

Then whether/how to support earlier indication of Redcap UEs with # Rx branches by Msg1 and/or Msg3 is discussed in this section.

Discussion on whether to support earlier indication of Redcap UEs with # Rx branches

The motivation of early indication is discussed from two aspects, one is scheduling strategy and the other one is access control.

- Motivation from scheduling strategy

For uplink scheduling, the uplink performance has no relation with the number of Rx branches, so there is no motivation to identify number of Rx branches in uplink.

For downlink performance, the received SINR quality is highly related to the number of Rx branches, so discussion is needed for downlink.

According to the WID description,

- For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

For RedCap devices with 2Rx, the downlink performance is the same as non-RedCap devices. Therefore, for these frequency bands, discussing the motivation of the early indication of RedCap devices equals to discuss the motivation of early indication of 1Rx and 2Rx devices.

For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, RedCap devices may support 1Rx or 2Rx, early indication of 1Rx and 2Rx is a further optimization for early identification of RedCap devices.

Although the above two frequency bands have different antenna configurations, identification in the number of antennas brings similar performance gains, so whether to support early indication of # Rx branches can be discussed together.

For downlink scheduling, when 1Rx has poor coverage than 2Rx, there are two possible options for gNB to scheduling RedCap devices,

- Option 1: 1Rx and 2Rx devices are not distinguished between each other, and gNB makes common scheduling for both 1Rx and 2Rx.

- Option 2: 1Rx and 2Rx devices are distinguished by Msg.1/Msg.3, and gNB schedules them separately.

When the number of RedCap devices is small, option 1 is better. For example, 50 non-RedCap UEs and 5 RedCap UEs, gNB can configure smaller number of separate PRACH resources for RedCap devices, no separation between 1Rx and 2Rx. In this case, there is only small amount of resource waste due to conservative scheduling for both 1Rx and 2Rx during the initial access.

When the number of RedCap devices is large, and gNB has the knowledge of minimum Rx number for the deployment, option 1 can still be efficient. For example, for connected industries use cases, it is very likely that gNB knows the minimum number of Rx for the sensors, for example, 1Rx or 2Rx only, then gNB can make decision for the suitable scheduling schemes.

Only when the number of RedCap devices is large, and both 1Rx and 2Rx device coexist in the same network, option 2 will be a better choice. And the additional benefit compared to capability report of Rx number comes from link adaption for RedCap UEs after early identification and before capability report of Rx number, which may be a short time duration.

Observation 1: The benefit of early indication of Redcap UEs with # Rx branches depends on deployment scenarios and gNB’s knowledge of UEs’ Rx number.

- Motivation from access control

Besides more resource consumption during initial access, 1Rx will consume more dedicated resources after RRC connection due to its lower SINR than 2Rx UEs with same location. Therefore, larger number of 1Rx UEs may cause high PRB usage and high blocking rate. Access control after early identification of 1Rx devices provides the network more flexibility to avoid network congestion.

In our companion contribution [4], different schemes for network access control have been discussed, among them, the network can indicate that whether 1Rx or 2Rx RedCap UEs can access the network in SIB1. However, access control during initial access procedure can provide a finer soft control than the hard control broadcasted in SIB1. Specifically, the access control broadcasted in SIB1 cannot vary fast, it can configure all the 1Rx UEs or even a fractional number of 1Rx UE cannot access the network. While during the initial access procedure, gNB can adopt dynamic access control for RedCap UEs with different Rx branches, by less RAR, or RRC rejection for some of the RedCap UEs.

Observation 2: With early indication of # Rx branches, the network can enable flexible and finer access control for RedCap UEs.

Based on analysis from the above two aspects, early indication of # Rx branches is beneficial for resource utilization and access control, so we are open to introduce early indication of # Rx branches by Msg1 and/or Msg3.

Proposal 1: Early indication of # Rx branches can be optionally supported.

Discussion on how to support earlier indication of Redcap UEs with # Rx branches

Earlier indication of Redcap UEs with # Rx branches is a further optimization scheme for early identification of RedCap devices, so it is proposed the configurations of both identifications are aligned, including where to configure them, and the identification option, e.g. Msg.1 and or Msg.3.

Proposal 2: The configuration of early indication of # Rx branches and early indication of RedCap devices should be aligned, including where to configure the indication and the indication option.

SIB1 can be used to configure whether early indication of 1Rx or 2Rx are enabled or not, it can also configure whether Msg.1 or Msg.3 is used for indication. When Msg.1 is configured, separate PRACH resources will be allocated to RedCap UEs with different Rx branches. However, PRACH partition is a widely discussed or used scheme for distinguish between different kinds of procedures, or UEs, for example, 2-step RA has use separate PRACH resource, which will segment the PRACH resources and reduce the normal UEs PRACH capacity, the practical usage may be limited. So when early identification is configured, we prefer that both Msg.1 and Msg.3 are supported for early indication.

Proposal 3: SIB1 is used to configure the early indication.

Proposal 4: Both Msg.1 and Msg.3 are supported for early indication.

- Discussion on solution to reduce PDCCH blocking

During RAN1#104e, PDCCH blocking issue was raised considering that larger aggregation levels may be used for RedCap devices due to reduced Rx number and smaller CORESET bandwidth.

According to agreements in RAN1#104b, existing DCI format will be applicable to Redcap devices as a starting point, we think examination of the DCI content for RedCap should be done based on all the reduced capabilities, some DCI field bit length are naturally reduced. We analyse the potential modification of DCI in our companion contribution[5]. And in this section, we focus on other solutions.

Separate BWP is an efficient way to reduce PDCCH blocking. For separate initial DL BWP, the Msg.2 and Msg.4 PDCCH can be offloaded to a different BWP from the non-RedCap devices, avoiding crowed CORESET#0 utilization. For separate dedicated BWP, it is naturally support for connected UEs, where different UEs can be configured with BWPs non overlapping with each other, the PDCCH blocking probability can be greatly reduced.

Proposal 5: Separate (initial) DL BWP can be used to reduce PDCCH blocking.

However, when the bandwidth of carrier is limited, configure non overlapped resource is difficult, then other solution can be further studied.

Considering the two application scenarios of reduced capability UEs, such as industrial wireless sensors and video surveillance. There are some similar characteristics for these two scenarios.

- The data traffic may be same or similar among different UEs in the scenario, e.g., the specific industrial wireless sensors which measure and report specific data with a periodicity.

- The channel environment is not changed so fast, e.g., industrial wireless sensors in the same workshop of factory, video surveillance on the lamp-post beside the roads.

These two characteristics make similar scheduling parameters among different UEs, such as the SPS or grant free periodicity. In traditional SPS or UL grant Type 2 procedure, UE-specific DCI is used as SPS or UL grant Type 2 activation signalling considering the different SPS or UL grant Type 2 configurations among UEs. However, in the above reduced capability UEs scenarios, the data traffic are the same or similar among different UEs and the SPS or UL grant Type 2 configuration may be the same as well. In addition, considering the number of reduced capability UEs are huge in one network, if the SPS or UL grant Type 2 activation is per-UE as usual, and with larger ALs, the PDCCH overhead of network may be large, and PDCCH blocking probability will be increased. Therefore, in these scenarios where the data traffic are the same or similar among UEs and the channel environment are not changed so fast, one DCI can activate more than one UE’s SPS or UL grant Type 2 to reduce network signalling overhead. And one DCI can also be used to scheduling multiple TBs for the same UE.

Proposal 6: Multi-UE scheduling can be further studied to reduce PDCCH blocking, e.g., simultaneously activation for multiple UEs’ SPS or UL grant Type 2 transmission.

Proposal 7: Multi-TB scheduling can be further studied to reduce PDCCH blocking.

- Conclusions

In this contribution, considerations on the solutions to reduce PDCCH blocking, and UE number of Rx branches related system information indication and identification are discussed, the following proposals are made,

Observation 1: The benefit of early indication of Redcap UEs with # Rx branches depends on deployment scenarios and gNB’s knowledge of UEs’ Rx number.

Observation 2: With early indication of # Rx branches, the network can enable flexible and finer access control for RedCap UEs.

Proposal 1: Early indication of # Rx branches can optionally supported.

Proposal 2: The configuration of early indication of # Rx branches and early indication of RedCap devices should be aligned, including where to configure the indication and the indication option.

Proposal 3: SIB1 is used to configure the early indication.

Proposal 4: Both Msg.1 and Msg.3 are supported for early indication.

Proposal 5: Separate (initial) DL BWP can be used to reduce PDCCH blocking.

Proposal 6: Multi-UE scheduling can be further studied to reduce PDCCH blocking, e.g., simultaneously activation for multiple UEs’ SPS or UL grant Type 2 transmission.

Proposal 7: Multi-TB scheduling can be further studied to reduce PDCCH blocking.

- References

- RP-210918, Revised WID on support of reduced capability NR devices, Nokia, Ericsson, 3GPP TSG RAN Meeting #91e, Electronic Meeting, 22nd – 26th March, 2021.

- Draft Report of 3GPP TSG RAN WG1 #104-e v0.3.0, 25th January - 5th February 2021.

- Draft_Minutes_report_RAN1#104bis-e, v0.2.0, 12th - 20th April 2021.

- R1-2104620, Discussion on higher layer support of RedCap UE, RAN 1 #105-e, May 10th – 27th, 2021.

- R1-2104621, Discussion on potential modification of existing DCI formats, RAN 1 #105-e, May 10th – 27th, 2021.