3GPP TSG RAN WG1 #105-e		R1-2104630

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.11.1.1

Source: 	CMCC

Title: 	Discussion on resource allocation for power saving

Document for:	Discussion and Decision

Introduction

In RAN#90-e meeting, the revised work item of NR sidelink enhancements was approved [1], in which objectives on resource allocation for power saving was updated as follows:

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

In RAN1#104-bis-e meeting, issues on periodic-based partial sensing and random resource selection were discussed. In this contribution, we continually discuss the enhancements on the partial sensing and the random resource selection mechanism.

Partial sensing

For the partial sensing mechanism in Rel-14 LTE-V, UE only performs sensing in a subset of subframes relevant for resource allocation, instead of the entire sensing window for full sensing mechanism. If partial sensing is configured, the following steps are used, as shown in Figure 1:

- Step 1: UE determines by its implementation a set of subframes which consists of at least  subframes (the orange subframes in Figure.1) within the time interval ;

- Step 2: If a subframe  is included in the set of subframes in Step 1, UE shall monitor any subframe (like the blue subframes in Figure.1) if k-th bit of the high layer parameter gapCandidateSensing is set to 1;

- Step 3: UE excludes any candidate single subframe resource from the selection window via pre-defined rules which are the same procedure with resource selection with full sensing.

Figure 1: Illustration of Rel-14 LTE-V partial sensing.

The resource allocation for power saving in NR sidelink takes the Rel-14 LTE-V partial sensing as a starting point, however, LTE-V’s partial sensing is designed particularly for periodic traffic and P-UE does not need to perform reception. Potential issues such as various resource reservation periodic, the aperiodic traffic, and diverse use cases can be identified if NR sidelink adopts the LTE-V partial sensing without any modifications. In RAN1#104-e meeting, a basic framework for the periodic-based partial sensing based on the LTE-V partial sensing scheme was agreed. In addition, the contiguous partial sensing mechanism was introduced with the intention to solve the issues caused by aperiodic reservations. In the following, we provide our views on the remaining issues of the two partial sensing mechanisms.

Periodic-based partial sensing

Determination of Preserve and k values

In LTE-V, it mainly focuses on the periodic traffic model with the reservation period to be a multiple of 100ms, therefore, a 1000ms sensing window is divided into 10 pieces with a sensing step of 100ms. The step size 100ms can work well since other period values are multiple of 100ms, that is, whenever a candidate resource in the selection window is reserved by other UEs, the P-UE can recognize the resource reservation and exclude the resources via pre-defined rules. However, NR sidelink identifies extra periodic traffic models, i.e. {(1…99), 100, 200, 300, 400, 500, 600, 700, 900, 1000} ms reservation period and up to 16 reservation period values can be (pre-)configured per resource pool. If other UE reserves resources with reservation period of 20ms (i.e. 40 slots), P-UE would not be able to identify this reservation if the fixed step size 100ms is applied, since P-UE does not perform sensing at slot .

To match the features of NR sidelink traffic models, the step size of sensing needs be re-considered, and the high layer parameter gapCandidateSensing should be enhanced accordingly. In RAN1#104-e meeting, several alternatives to determine Preserve and k valus were agreed for further discussion. In RAN1#104-b-e meeting, the alternatives were further down scoped as follows:

Agreements: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics - FFS whether/how to restrict the set of values - For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible) - Alt 1: Option 1 as in RAN1#104-e - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1 - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc. - Note: companies are encouraged to provide more evaluations

In the following, we discuss the pros and cons of listed alternatives of Preserve and k:

- On Preserve

  - Alt 1: By setting Preserve corresponds to all values from the configured reservation period list, it is the most straightforward way to identify all the reserved resources of enhanced reservation periods. However, it lost the configuration flexibility and provides no advantages in case that a resource pool supports a reservation period that is a multiple integer of the others.

  - Alt 2: Alt 2 of Preserve being a subset of all configured reservation period seems more reasonable when a reservation period of a RP is the integer multiple of the others. In such a case, setting only one Preserve for monitoring is enough. To be more specific, the selected Preserve can be the common divisor among the reservation periods to ensure that the UE can identify each resource reservation and exclude the corresponding candidate single slot resource regardless of reservation period of other UEs. Note that Alt.2 supports up to the full set of values from the configured reservation periods, which includes Alt. 1, Alt. 2 is more preferred.

- On k values: During the discussion, proponent of Alt. 1 argued that the rule of sensing only the most recent reservations in LTE-V and R16 NR-V should be followed, as the SCI only reserves resources for the next period, therefore, sensing the most recent occasion provides the most valid reservation information. In addition, it was mentioned that it is beneficial to balance the power consumption and performance. There is no necessary to support k>1 at the cost of more power consumption. However, we think that the k value should be set flexibly, and should allow the UE to monitor more than one occasions for each reservation period to enhance the detection performance. Note that the current Alt. 2 leaves the room to always include Alt. 1, therefore, Alt. 2 is more preferred.

Proposal 1: In a resource pool (pre-)configured with at least partial sensing, if a UE performs periodic-based partial sensing, the UE monitors slots of periodic sensing occasions ty-k×PreserveSL, where

- A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

- k is (pre-)configured, including multiple values

Another issue is the reference timing of the sensing occasion, whether the sensing is before the resource (re)selection trigger or before the first slot of the set of Y candidate slots subject to processing time restriction. In partial sensing mechanism in LTE-V, the UE determines the Y candidate subframe up to its implementation and shall monitor any subframe  if k-th bit of the high layer parameter gapCandidateSensing is set to 1. From the perspective of UE sensing, there is no sensing window in LTE-V partial sensing, the sensing occasion is determined by the selected Y subframes and the bitmap indication configured by higher layer. In NR-V, extra traffic models including the short reservation period traffic with reservation period of 1~99ms, and the aperiodic transmissions, it should be beneficial to extend the sensing occasion before the first slot of the set of Y candidate slots, which is the first potential slot for transmission.

Proposal 2: Support sensing before the first slot of the set of Y candidate slots subject to processing time restriction.

Conditions/cases to perform periodic-based partial sensing

In RAN1#104-e meeting, the periodic-based partial sensing was agreed taking the LTE-V partial sensing mechanism as the baseline. In LTE-V, the partial sensing scheme mainly targets the periodic traffic types; however, considering the aperiodic transmissions in NR-V, companies shared diverse views on the conditions/cases and timings for which the periodic-based partial sensing is performed by UE. In RAN1#104-b-e meeting, the following agreement were made:

Agreement: - When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing - Partial sensing configured by higher layer in the UE

During the discussion, there is no common understanding regarding the following issues:

- whether the periodic-based partial sensing is only performed when the UE has periodic transmissions;

- whether the periodic-based partial sensing is only performed when there is actual packet to be sent;

- how to deal with the case when there have no sufficient sensing results;

First, in our view, the condition of UE performing periodic-based partial sensing is irrelevant to its traffic type. In other words, the periodic-based partial sensing is to exclude the periodic reservations from other UEs in a resource pool. If the resource pool allows both periodic and aperiodic transmissions, the Tx UE can still perform periodic-based partial sensing when it has aperiodic transmissions.

Conclusion 1: The periodic-based partial sensing is performed by the Tx UE to exclude the periodic reservations from other UEs in a resource pool, and is irrelevant to the traffic type of the Tx UE.

One argument would be that the arrival time of the aperiodic traffic is unpredictable; therefore, the UE has no idea how to determine the Y candidate slots to obtain appropriate sensing results. We think that the higher layer in the UE is able to determine the timing (i.e., slot n) to trigger the lower layer to do the resource (re)selection in advance according to the assistance/prediction information of the traffic model, based on which the Tx UE can determine the Y candidate slots to perform periodic-based partial sensing. When the actual aperiodic packet arrives in slot n’, if the selected Y candidate slots can meet the remaining PDB of the packet, then the periodic-based sensing results can be used for the aperiodic transmissions; otherwise, it leads to another issue, which is how to deal with the case when there have no sufficient results. In our view, it can be up to UE implementation to handle this issue. If the remaining PDB of the packet is not urgent, the Tx UE can to perform contiguous partial sensing; if the priority level of the packet is high, the Tx UE can use random resource selection, which is able to pre-emp the reserved resources of other sensing-based UEs.

Proposal 3: It is up to UE implementation to handle the case where the UE has insufficient sensing results.

Determination of Y candidate slots

As discussed in section 2, in NR sideink, the design principle of partial sensing mechanism should be that the UE determines by its implementation a set of slots which consists at least Y slots according to a pre-defined resource selection pattern. In RAN1#104-e meeting, the enhancements on the determination of Y candidate slots in LTE-V was discussed and the following agreement was made:

Agreements: In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where - FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE - The resource selection window is [n+T1, n+T2] - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4] - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB - T2-T1 ≤ (pre-)configured threshold - A minimum value for Y is (pre-)configured from a range of values, FFS details - FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX) - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX - Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

In the last meeting, the following conclusion was made on the definition of T1 and T2 of the resource selection window:

Conclusion: - In periodic-based partial sensing, - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

We will further discuss the remaining bullets regarding the determination of Y candidate slots:

- Minimum value of Y: In LTE-V partial sensing, a UE determines by its implementation of Y candidate subframes, and Y shall be greater than or equal to the high layer parameter minNumCandidateSF. However, it is possible that the determined set of slots may not meet the requirement of remaining PDB, or the number of determined set of slots that satisfy the PDB may be limited. In such a case, the data transmission is highly probable to be interfered and jammed, and the transmission reliability is decreased. Moreover, in LTE-V, the high layer parameter minNumCandidateSF is fixed regardless of the data priority and which resource pool the transmission would take place. Note that since the (pre-)configured parameter minNumCandidateSF represents the level of power consumption and number of retransmissions; therefore, it can be considered that the parameter minNumCandidateSF is (pre-)configured parameter for each priority value and per resource pool.

Proposal 4: In NR sidelink partial sensing, UE shall determine by its implementation a set of slots which consists at least Y slots within selection window and Y shall be greater than or equal to a higher layer parameter minMumCandidateSF, where minMumCandidateSF is (pre-)configured for each priority value and per resource pool.

- Tx-Rx alignment: In addition, the typical use case of Rel-14 LTE-V partial sensing is V2P, where the pedestrian UEs identifies the need of power saving. In such a case, only the transmitter (P-UE) uses partial sensing to perform resource selection for power saving and the receiver (V-UE) would perform reception via full sensing mechanism. In R17 sidelink, solutions for power saving are targeted not only P-UEs, but also UEs in public safety and commercial use cases (e.g., smart home, smart factory, etc.), in these scenarios, both transmitters and receivers identify the motivation for power saving. Hence, enhancements for sensing pattern alignment should be studied. In the above agreement, one remaining issue is that whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL DRX. In our views, to confine the resource selection window within a set of periodic set of resources are mainly used for Tx-Rx alignment when both transmitters and receivers needs power saving, and the concept is basically same as the SL DRX. Therefore, we would like to discuss this issue under the introduction of SL DRX and its potential impact on PHY layer resource selection.

Observation 1: The intention to confine the resource selection window within a set of periodic set of resources is for Tx-Rx alignment when both transmitters and receivers have the need for power saving. The concept is basically the same as the SL DRX.

Proposal 5: Considering public safety and commercial use cases where both transmitters and receivers have the requirement of power saving, enhancements for sensing pattern alignment should be studied.

Other potential enhancements

In NR sidelink mode-2, if the number of candidate single-slot resources remaining in the set SA after resource exclusion procedure is smaller than X% of the total number of candidate resources, the RSRP threshold will be repeatedly increased with step of 3dB until X% of single-slot resources remain. If partial sensing mechanism is configured, since the determined set of slots is only a subset of the resource selection window and the determination is totally by UE implementation, the determined set of slots may be not enough or suitable to meet the requirement of reliability. To be specific, when the determined set of slots is slightly greater than the higher layer parameter minNumCandidateSF or if the selected Y subframes are just right highly occupied and interfered by other UEs, the less and unsuitable candidate single-slot resources may result in unlimited increase of RSRP threshold, which leads to severe transmission interference and further impact the transmission reliability.

Proposal 6: Partial sensing mechanism in NR sidelink needs to take into account the unlimited increasing of RSRP threshold.

One possible solution is to set an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold. When the upper limit or the maximum value is reached, instead of continue increasing RSRP threshold, UE can increase the number of determined set of slots resulting in more candidate single-slot resources. With more candidate resources, the interference can be decreased and reliability is increased.

Proposal 7: In NR sidelink partial sensing, an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold can be configured. When the upper limit or the maximum value is reached, UE increases the number of determined set of slots.

Contiguous partial sensing

LTE-V partial sensing mainly designed to deal with periodic transmissions without considering aperiodic traffic, which would degrade the sensing performance if LTE-V’s partial sensing mechanism is directly applied to NR sidelink. Take Figure 2 as an example, if other UE dynamically reserves resources just before slot n (the green resources in Figure 2), the reserved resources would not be detected by UE configured with partial sensing, hence it may not exclude the corresponding candidate resource.

Figure 2: Illustration of impact of aperiodic traffic or dynamic reservations

In RAN1#104-e meeting, the following agreement was reached with the intention to cope with the impact of aperiodic transmissions in a resource pool:

Agreements: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

In a resource pool (pre-)configured with at least partial sensing, when resource (re-)selection is triggered in slot n, a UE is enabled to support contiguous partial sensing, where it monitors slots between [n+TA, n+TB] and identifies the candidate slots based on all available sensing results. The values of TA, and TB are still FFS, of which the possible candidates can be zero, positive or negative. In addition, whether the slot n in the contiguous sensing window can be replaced by indices of Y candidate slots is left for further discussion.

In our view, the location and the range of the contiguous sensing window is dependent on different conditions and cases:

- In case that a resource pool allows both periodic and aperiodic transmissions, and a UE performs periodic-based partial sensing. To take into consideration of resource reservations by aperiodic traffic, one possible solution is to add an additional contiguous sensing window before the candidate single slot resources. That is, after UE determines a set of slots which consists of at least Y slots within the selection window, UE monitors all resources within time period [, ], if a slot  is included in the set of slots determined above. The selection of TA needs to accommodate the maximum time of dynamic reservation, and TB is subject to T_proc,0. In such a case, the slot n in the contiguous sensing window should be replaced by indices of Y candidate slots, and the values of TA and TB are negative. In this way, UE can identify the dynamic reservations caused by both periodic and aperiodic traffic and achieve better performance of resource selection.

Observation 2: In case that a resource pool allows both periodic and aperiodic transmission, and a UE performs periodic-based partial sensing can additionally performs contiguous partial sensing to take into consideration of resource reservations by aperiodic traffic.

Proposal 8: For the purpose of resource (re-)selection, a UE monitors slots between [n+TA, n+TB], where n is replaced by the indices of Y candidate slots  , and values of TA, TB are negative. The UE performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results by monitoring slots .

- In case that periodic reservations are disabled in a resource pool so that the UE does not perform periodic-based partial sensing, or a resource (re-)selection is triggered in slot n with an urgent remaining PDB, where the Y candidate slots cannot meet the requirement of the remaining PDB. To solve this issue, the UE can monitor contiguous sensing window between [n+TA, n+TB] and identifies the candidate slots. In such a case, the value of TA and TB can be zero and positive, when both TA and TB are zero, the UE performs random resource selection.

Observation 3: In case that periodic reservations are disabled in a resource pool so that the UE does not perform periodic-based partial sensing, or a resource (re-)selection is triggered in slot n with an urgent remaining PDB, where the Y candidate slots cannot meet the requirement of the remaining PDB, the UE can perform contiguous partial sensing.

Proposal 9: For the purpose of resource (re-)selection, a UE monitors slots between [n+TA, n+TB], and values of TA, TB are zero and positive.

Random resource selection

In the RAN1#103-e meeting, it was agreed that a SL mode 2 Tx pool can be (pre-)configured to enable different resource selection schemes, including full sensing only, partial sensing only, random selection only, and any combinations thereof. In the last meeting, solutions on how to solve the potential collision of random selected resources with low priority with partial/full sensing resources in a mix RA resource pool were discussed. Various of options were proposed and due to limited time budget, no consensus was made during the last meeting.

In our view, one possible way is to indicate resource selection schemes via SCI and when transmitter performs resource selection, it determines whether to exclude a candidate single-slot resource based on separately configured RSRP thresholds for different resource selection schemes. Alternatively, if random resource selection is identified via SCI decoding and overlapping is observed between the reserved resource and candidate resource, UE is expected to exclude the corresponding candidate resource without RSRP comparison. When the number of remaining candidate single-slot resources after exclusion procedure is less than X% of total number of candidate resources, the increased RSRP threshold value can be different among resource selection schemes. Moreover, when UE reporting candidate resource set to higher layer for resource selection, additional information such as whether the candidate resource overlaps with resources reserved by UE using random resource selection can be reported together, which may be beneficial for MAC layer to perform resource selection.

Proposal 10: RAN 1 discusses collision control mechanism among different resource selection schemes and the following alternatives can be considered:

- Different RSRP thresholds or increased RSRP threshold value is (pre-)configured for different resource selection scheme;

- UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection.

Conclusions

In this contribution, we discuss on the resource allocation for power saving, and the following observations and proposals are provided:

Observation 1: The intention to confine the resource selection window within a set of periodic set of resources is for Tx-Rx alignment when both transmitters and receivers have the need for power saving. The concept is basically the same as the SL DRX.

Observation 2: In case that a resource pool allows both periodic and aperiodic transmission, and a UE performs periodic-based partial sensing can additionally performs contiguous partial sensing to take into consideration of resource reservations by aperiodic traffic.

Observation 3: In case that periodic reservations are disabled in a resource pool so that the UE does not perform periodic-based partial sensing, or a resource (re-)selection is triggered in slot n with an urgent remaining PDB, where the Y candidate slots cannot meet the requirement of the remaining PDB, the UE can perform contiguous partial sensing.

Conclusion 1: The periodic-based partial sensing is performed by the Tx UE to exclude the periodic reservations from other UEs in a resource pool, and is irrelevant to the traffic type of the Tx UE.

Proposal 1: In a resource pool (pre-)configured with at least partial sensing, if a UE performs periodic-based partial sensing, the UE monitors slots of periodic sensing occasions ty-k×PreserveSL, where

- A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

- k is (pre-)configured, including multiple values

Proposal 2: Support sensing before the first slot of the set of Y candidate slots subject to processing time restriction.

Proposal 3: It is up to UE implementation to handle the case where the UE has insufficient sensing results.

Proposal 4: In NR sidelink partial sensing, UE shall determine by its implementation a set of slots which consists at least Y slots within selection window and Y shall be greater than or equal to a higher layer parameter minMumCandidateSF, where minMumCandidateSF is (pre-)configured for each priority value and per resource pool.

Proposal 5: Considering public safety and commercial use cases where both transmitters and receivers have the requirement of power saving, enhancements for sensing pattern alignment should be studied.

Proposal 6: Partial sensing mechanism in NR sidelink needs to take into account the unlimited increasing of RSRP threshold.

Proposal 7: In NR sidelink partial sensing, an upper limit of the number of RSRP threshold increments or the maximum value of increased RSRP threshold can be configured. When the upper limit or the maximum value is reached, UE increases the number of determined set of slots.

Proposal 8: For the purpose of resource (re-)selection, a UE monitors slots between [n+TA, n+TB], where n is replaced by the indices of Y candidate slots  , and values of TA, TB are negative. The UE performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results by monitoring slots .

Proposal 9: For the purpose of resource (re-)selection, a UE monitors slots between [n+TA, n+TB], and values of TA, TB are zero and positive.

Proposal 10: RAN 1 discusses collision control mechanism among different resource selection schemes and the following alternatives can be considered:

- Different RSRP thresholds or increased RSRP threshold value is (pre-)configured for different resource selection scheme;

- UE reports whether one candidate resource overlaps with resources reserved by random resource selection UE to higher layer for further resource selection.

References

- RP-202846, WID revision: NR sidelink enhancement, LG Electronics, RAN#90-e meeting.