3GPP TSG RAN WG1 Meeting #105-e                                     R1-2104881

e-Meeting, 19th May – 27th May, 2021

Source:               TCL Communication

Title:                    Discussion on reduced maximum UE bandwidth

Agenda item:      8.6.1.1

Document for:    Discussion and Decision

- Introduction

In the RAN1#104b-e meeting, the following agreements were achieved for reduced maximum bandwidth.

Working assumption: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs. - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS). Working assumption: After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2) Agreement: - During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Agreement: - After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e: - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Working assumption: A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

In 3GPP TSG RAN WG1#104-e meeting, the following agreements about RACH occasions are achieved:

Agreements: - Study further how to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, with the following options: - Option 1: Proper RF-retuning for RedCap - Option 2: Separate initial UL BWP(s) for RedCap UEs - Option 3: gNB configuration (e.g., restrictions on existing PRACH configurations, or FDM-ed ROs, or always restricting the initial UL BWP to within RedCap UE bandwidth) - Option 4: Dedicated PRACH configurations (e.g., ROs) for RedCap UEs - Other options are not precluded

This contribution discusses the issues caused by reduced maximum bandwidth.

- Discussion

At previous meetings, it was agreed that Redcap UEs can share the same initial DL/UL BWP (derived based on MIB/SIB)  with non-Redcap UE at least when the initial DL/UL BWP is no wider than the Redcap UE bandwidth. The initial BWP, usually with a small bandwidth is used to perform initial access process. All DL common signals and channels before RRC connection establishment, e.g. SSB, RMSI, paging, RAR, MSG4 and RRC signaling, are transmitted through initial DL BWP. As the number of RedCap UEs in the network is large, the load of initial DL BWP would be very heavy with the camped Redcap UEs. Since the heavy load would cause the performance degradation for legacy UEs, the Redcap UEs would switch to a dedicated BWP as soon as possible after the initial access.

Proposal 1: Redcap UEs should switch to a dedicated BWP as soon as possible after the initial access.

Figure 1: BWP switch timeline

The dedicated BWP as a target BWP is selected by gNB. If its channel quality is good, coverage compensation is not required; if it is configured with SSB, re-switching to the initial BWP is unnecessary for synchronization. Therefore, when selecting the target BWP, interrelated factors need to be considered to simplify the behavior of gNB or UE..

Proposal 2: Interrelated factors should be considered for the selection of the target BWP.

For Redcap UEs with limited maximum bandwidth (20 MHz in FR1 and 100 MHz in FR2), if they are configured with 8-FDMed Rach occasions, the required BW is greater than the UE bandwidth. A RACH occasion associated with the best SSB can fall outside the UE bandwidth.

During the initial access, it is beneficial if a gNB knows which beam is the strongest for UE receiving.  One SSB corresponds to one beam, therefore the SSB RSRP is used to determine which beam is strongest. Each SSB is associated with one or more Rach Occasions, and once a SSB is selected, the ROs that can be selected is determined. In NR specifications, a UE is provided a SSB RSRP threshold for selecting a proper SSB and the number of SSBs per RACH occasion by higher layer.

Figure 2 shows how to select SSB and RO. It can be seen that the SSB RSRP threshold is the only factor in selecting the SSB. If more than one SSB’s RSRP are greater than the RSRP threshold, the SSB may be selected randomly among them. If more than one ROs associated with the selected SSB, select one associated RO randomly with equal probability; otherwise select the associated RO.

Figure 2 Steps of the existing random access procedure

For option#1 and option#2 agreed on the topic of RO in RAN1#104e, they imply that step 1 and step 2 of fig.2 are unchanged, but the frequency retuning is required between step 2 and step 3.  The UE may need to retune back to the original frequency after transmitting preamble. As we know, the frequency retuning requires transition time. This complicates the initial access procedure and spends more time, thus may increase access latency. The configurations in option#3 and option#4 can avoid the selected RO outside the UE bandwidth, but cause inflexibility or fragmentation of PUSCH resources.

For the current mechanism, the selection of SSB is based only on the SSB rsrp threshold, and UE may select an improper SSB (e.g. SSB3 in Figure 3a/3b) and the associated RO.  If the frequency position of the associated RO is also taken as a factor when selecting SSB, SSB3 will be excluded and SSB2 will be selected, and no other optimizations are need. Therefore, optimizing the selection of SSB (step 1) may be a feasible option.

Figure 3a One SSB per RO

Figure 3b Multi-SSB per RO

Observation 1：According to the current specification, UE may select an improper SSB-RO pair even with a proper SSB-RO pair.

Proposal 3: Whether the associated RO is within the UE bandwidth is a consideration for SSB selection.

For one SSB is associated more than one RO, under the mechanism with random equal probability for selecting RO, UE also may select an improper RO(RO#8 in Figure 4) even with a proper RO (RO#7 in Figure 4).

Figure 4 Multi-RO per SSB

Observation 2：According to the mechanism with random equal probability, UE may select an improper RO even with a proper RO.

Proposal 4: Whether the associated RO is within the UE bandwidth is a consideration for RO selection.

- Conclusion

In this contribution the following proposals and observations have been made:

Proposal 1: Redcap UEs should switch to a dedicated BWP as soon as possible after the initial access.

Proposal 2: Interrelated factors should be considered for the selection of the target BWP.

Observation 1：According to the current specification, UE may select an improper SSB-RO pair even with a proper SSB-RO pair.

Proposal 3: Whether the associated RO is within the UE bandwidth is a consideration for SSB selection.

Observation 2：According to the mechanism with random equal probability, UE may select an improper RO even with a proper RO.

Proposal 4: Whether the associated RO is within the UE bandwidth is a consideration for RO selection.

References

[1] Meeting report, “Chairman’s Notes RAN1#104b-e”

[2] Meeting report, “Chairman’s Notes RAN1#104-e”

[3] TS 38.321, “Medium Access Control (MAC) protocol specification”