3GPP TSG RAN WG1 #105-e			R1-2105173

e-Meeting, May 10th – 27th, 2021

Agenda Item	8.6.2

Source: 	Sony

Title:	UE Identification for RedCap devices

Document for:	Discussion & Decision

# Introduction

The work item will discuss how to specify higher layer support of RedCap devices. Identifying the UE type is part of the following objective from WID [1].

- Specify higher layer support of enhancements listed above [RAN2, RAN1]. Details are to be refined at RAN#91e taking the outcome of the RedCap SI into account, and work on this objective shall start after RAN#91e: - Specify definition of RedCap UE type(s) including set(s) of L1 capabilities for RedCap UE identification and for constraining the use of those RedCap L1 capabilities only for RedCap UEs, and preventing RedCap UEs from using capabilities not intended for RedCap UEs including at least carrier aggregation, dual connectivity and wider bandwidths. - Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks and allow operators to restrict their access if desired. - Specify necessary updates of UE capabilities (38.306) and RRC parameters (38.331).

During the study item phase some observations regarding UE identification for the following options were made (Option 4 was deprioritized) [2].

- Option 1: During Msg1 transmission - E.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning - Option 2: During Msg3 transmission - Option 3: Post Msg4 acknowledgment. - E.g., during Msg5 transmission or part of UE capability reporting - Option 4: During MsgA transmission - Subject to support of 2-step RACH procedure

This document discusses and covers the topic of UE identification for RedCap devices based on the observations from the study item report and some extra analysis [2].

# Discussion

## UE Identification

The WID [1] lists three RedCap use cases, “Industrial wireless sensors, Video surveillance and Wearables". Additionally, the WID objective defines complexity reduction features impacting how the UE operates e.g. operating bands (at least FR1/FR2), the maximum bandwidth, number of Rx branches, MIMO layers, modulation order, duplex operations, coverage recovery / enhancement etc. Addressing all uses cases with one RedCap UE type might be difficult. Therefore defining more UE types than just differentiating between RedCap UEs and non-RedCap UEs could be necessary.

Proposal 1: 	UE identification should support identification of multiple UE types and not only RedCap or non-RedCap UEs

The RedCap study item report [2] shows several options for UE identification. Options 1, 2 and 3 are listed with pros and cons (option 4 is down prioritized). It is observed that all options have their pros and cons and none is perfect in all aspects. The pros and cons from the study item report are condensed in a decision matrix in Table 1. The “empty” cells from the report (where nothing was directly noted in study item report [2]) are filled in with how we interpret the pros and cons list to make the table more complete.

Table 1 Decision matrix based on the Options and pros & cons from RAN1 study item report [2], Green = Pros, Orange= Cons.

| KPI | Option 1 (Msg 1) | Option 2 (Msg 3) | Option 3 (Msg 5) |
| --- | --- | --- | --- |
| Processing times between RedCap / non-RedCap UEs | Enables efficient handling of different UE type with minimum processing times | Cannot facilitate scheduling with separate minimum timing relationships. => Increased initial access latency of non-redcap UE | Cannot facilitate scheduling with separate minimum timing relationships. => Increased initial access latency of non-redcap UE |
| Coverage recovery for broadcast PDCCH and/or Msg2 PDSCH, and/or Msg3 PUSCH | Enables coverage recovery and link adaptations | Cannot facilitate additional coverage recovery or separate link adaptation => increased system OH for initial access in the initial DL and UL BWPs. | Cannot facilitate additional coverage recovery or separate link adaptation => increased system OH for initial access in the initial DL and UL BWPs. |
| Coverage recovery and link adapt. for PDSCH and associated PDCCH and PUCCH (for Msg4, and scheduling of Msg5. | Enables coverage recovery and link adaptations | Enables post Msg 3 coverage recovery and link adaptations | Cannot facilitate additional coverage recovery or separate link adaptation => increased system OH for initial access in the initial DL and UL BWPs. |
| Separate initial BWP + Address congestion | Supports both a separate initial BWP and thereby addresses congestion | Not supported - congestion restricted to the mandatory required BW for Red Cap UEs in the band/FR. | Not supported - congestion restricted to the mandatory required BW for RedCap UEs in the band/FR. |
| PRACH user capacity | With PRACH separation of preambles, impacting both RedCap and non-RedCap. Impact depends on numbers of device type(s)/sub-types/capabilities preamble partitioning schemes | No PRACH capacity impact | No PRACH capacity impact |
| UL OH from PRACH separation or UL BWP | For separation of PRACH resources, impacting both RedCap and non-RedCap UEs and for complexity in configuration and maintenance of multiple initial UL BWP for the gNB, for the option of configuring separate initial UL BWPs. | For conf. and maintenance of multiple initial UL BWP in gNB | No impact since separate initial UL BWP not supported |
| Nbr of sub-types/capabilities | Limited number of types | Multiple nbr of types only if extended number of bits | Not really a restriction in number of types |
| SIB signaling OH | More SIB signaling OH compared to other options. | Limited additional SIB signalling overhead | Limited additional SIB signalling overhead |
| Standard impact | Highest impact to RAN1 and RAN2 specifications | Impact if multiple types are to be supported. Limited impact to RAN1 specifications if only the spare bit in Msg3 payload is utilized. | Impact to RAN2 specifications |
| Freq hopping of Msg3 / PUCCH (Msg 4, Msg5) | Can address hopping | Cannot address the issue where Msg3 is scheduled with a bandwidth/hopping range larger than the maximum RedCap UE bandwidth in the UL initial BWP. | Cannot address the issue where Msg3 or PUCCH in response to Msg4 or Msg5 is scheduled with a bandwidth/hopping range larger than the maximum RedCap UE bandwidth in the UL initial BWP. |
| Msg 3 spare bits | No spare bit used | Last one used if only RedCap/non-Redcap is identified. (Extended bits needed if multiple UE types should be indicated) | No spare bit used |

UE identification done as early as possible i.e. as part of Msg1 is beneficial in several aspects but, there also are drawbacks, such as limitations in e.g. the number of supported UE types and degradation of PRACH user capacity. Therefore, if Msg1 is used for UE Identification, the number of UE types identified by Msg1 should be restricted to the minimum needed for supporting an efficient initial access (avoid degrading the non-RedCap performance). Similar, if Msg3 is used for identification of multiple UE types, a higher number of Msg3 bits are needed.

Observation 1: 	Neither of Option 1, Option 2 nor Option 3 is attractive as stand-alone solutions if multiple UE types are supported.

This leads into thinking about combining the options into a multi-step option. Additionally, in the RedCap study item report [2], it is mentioned that at least the maximum supported UE BW is included in the set of L1 capabilities for RedCap early identification. There is also a note saying that it does not preclude the case where early identification only indicates if it is a RedCap type or any specific type of a multiple redcap types. We see that those kinds of basic UE types could be identified with a first identification phase providing good enough functionality for initial access.  E.g. combining the earliest identification of Option 1 with the latest and most flexible Option 3. By letting Msg1 identify an initial UE type and set pre-agreed (between UE and NW) or initial/default parameters for the identified UE type to be used during initial access. Then when Msg5 (or the capability signalling after Msg5) is received, the UE type and its parameters is refined/updated. The pros and cons of such a solution are listed in Table 2. It is shown that most of the drawbacks are gone but at the cost of some overhead and complexity.

Table 2 - Decision matrix of combination of using Option 1 together with Option 3, Green = Pros, Orange= Cons, Yellow = in between

| KPI | Option 1 + Option 3 |
| --- | --- |
| Different proc. times between RedCap / non-RedCap UEs | Same as for Option 1. It enables efficient handling of different UE minimum processing times |
| Coverage recovery for broadcast PDCCH and/or Msg2 PDSCH, and/or Msg3 PUSCH | Same as for Option 1. Enables coverage recovery and link adaptations |
| Coverage recovery for PDSCH (and associated PDCCH and PUCCH) for Msg4, and scheduling of Msg5. | Same as for option 1. Enables coverage recovery and link adaptations |
| Address congestion (if congestion may occur) in the initial UL BWP | Same as for option 1. Supports a separate initial BWP and thereby avoid address congestion |
| PRACH user capacity | Similar as option 1 but fewer types need to be supported by PRACH since option 3 is also there. Gives less impact on PRACH user capacity. |
| UL OH from PRACH separation or UL BWP | Same as option 1. For separation of PRACH resources, impacting both RedCap and non-RedCap UEs and for complexity in configuration and maintenance of multiple initial UL BWP for the gNB, for the option of configuring separate initial UL BWPs. |
| Nbr of sub-types/capabilities | Same as option 3 – no real restriction in types. Type signalled partly by PRACH and partly by Msg5. |
| SIB signaling OH | Same as option 1. More SIB signaling OH compared to other options. |
| Standard impact | As option 1 + option 3 |
| Freq hopping of Msg3 / PUCCH (Msg 4, Msg5) | Same as option 1. Can address the hopping issue |
| Msg 3 spare bits | Same as option 1 and 3. No extension of Msg3 bits |

Observation 2: 	With a combination of options more UE types can be supported at the same time as there are less restrictions to both non-redcap and redcap UEs.

An initial identification of the UE type and its parameters will enable many of the system features (as discussed in Table 2), to be supported. It will enable e.g. coverage recovery for the latter parts of initial access and address congestion with the use of a separate BWP. Then by utilizing a second phase of updating the UE type, the full set of parameters for complete UE functionality may be refined covering e.g. the different use case specific requirements listed in the WID [1] or any sub-versions thereof. In the end the pros and cons of the combined  method in Table 2 look much more appealing than the stand-alone options of Table 1.

Proposal 2: 	Let an initial UE type be identified by a first identification (e.g. PRACH/Msg1) and let later signalling (e.g. Msg5 or capability signalling) refine/change the UE type and its parameters.

# Conclusions

This document has condensed and discussed the UE Identification pros and cons from the RedCap Study Item report [2] and has made the following observations and proposals.

Observation 1: 	Neither of Option 1, Option 2 nor Option 3 is attractive as stand-alone solutions if multiple UE types are supported.

Observation 2: 	With a combination of options more UE types can be supported at the same time as there are less restrictions to both non-redcap and redcap UEs.

Proposal 1: 	UE identification should support identification of multiple UE types and not only RedCap or non-RedCap UEs

Proposal 2: 	Let an initial UE type be identified by a first identification (e.g. PRACH/Msg1) and let later signalling (e.g. Msg5 or capability signalling) refine/change the UE type and its parameters.

# References

- RP-202933. “New WID on support of reduced capability NR devices.”

- 3GPP TR 38.875 V17.0.0. “Study on support of reduced capability NR devices.”