3GPP TSG RAN WG1 #105-e	R1- 2105187

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.3.3

Source: 	Quectel, Langbo

Title: 	Discussion on Intra-UE Multiplexing/Prioritization

Document for:	Discussion and Decision

1   Introduction

At RAN1 #104bis-e, intensive discussions for intra-UE multiplexing were mainly focused on the coding schemes [1]. Many other aspects are highly dependent on the outcome of coding scheme decision. In this contribution, aspects on intra-UE UCI multiplexing are further discussed based on the latest progress.

2   Multiplexing HARQ-ACK on PUCCH

2.1 Channel coding

The following agreements were made at RAN1 #104bis-e:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs.

- FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s).

- (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N.

  - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

  - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2.

The pros and cons respectively using separate coding and joint coding were extensively discussed when the total number of LP and HP HARQ-ACK bits is larger than 2 at RAN1#104bis-e [1]. Separate coding was eventually agreed. In order to not increase the number of channel encoders, CSI dropping was also discussed and a working assumption was made. In our view, the limitation of the number of channel encoders is very important for not increasing UE implementation complexity, so we support the spirit of the working assumption. In a parallel agenda item dedicated for CSI enhancements, the support of aperiodic CSI on PUCCH is still open. If it is supported, this kind of aperiodic CSI is quite likely to be of the same priority as HARQ-ACK or even regarded as a part of HARQ-ACK feedback. In this case, whether the aperiodic CSI on PUCCH, if supported, would be dropped or not needs to be clarified for this working assumption.

Proposal 1: The working assumption is confirmed with clarification: Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N if CSI is for a report quantity as defined in Rel-15/16.

Coding schemes for the HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s) are still open. In current specifications, repetition coding for 1 bit UCI or simplex coding for 2-bit UCI is supported for UCI on PUSCH. It is natural to reuse the repetition coding for 1 bit HP HARQ-ACK or 1 bit LP HARQ-ACK and reuse the simplex coding for 2-bit HP HARQ-ACK or 2-bit LP HARQ-ACK when the total number of LP and HP HARQ-ACK bits is larger than 2.

Proposal 2: Repetition coding or simplex coding is supported for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s) when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

When the number of encoded bits is smaller than the modulation order for repetition coding or simplex coding, placeholder bits are inserted to maximize the Euclidean distance. The placeholder bits are not scrambled by the scrambling bit sequence to avoid distorting the constellation, so as to maintain the maximized Euclidean distance. As repetition coding or simplex coding is only supported by UCI on PUSCH in current specifications, the scrambling design for PUCCH does not take the placeholder bits in to account. In order to support repetition coding and simplex coding on PUCCH, the scrambling design for PUCCH needs to consider the placeholder bits. Specifically, as the maximum modulation order of PUCCH is 2, only placeholder bits “y” exist for PUCCH.

Proposal 3: Placeholder bits “y” are introduced for the scrambling of PUCCH.

2.2 Rate matching

In existing design for PF2 and PF3/PF4, a total number of bits for rate matching  is firstly calculated according to the number of available REs for UCI transmission and spreading factors for a given modulation scheme. When two separate encoders are applied, a number of bits for the first part of UCI is calculated according to the number of information bits and a configured maximum PUCCH coding rate . The remaining bits are used for the second part of UCI. One open issue for PUCCH rate matching supporting multiplexing of HP HARQ-ACK and LP HARQ-ACK is whether to introduce two separate coding rate configurations respectively for HP HARQ-ACK and LP HARQ-ACK. In our view, a single coding rate configuration is sufficient to control the effective coding rates of both HP HARQ-ACK and LP HARQ-ACK by reusing the existing rate matching mechanism, i.e.,  and - are used respectively for HP HARQ-ACK and LP HARQ-ACK rate matching, as maximal two encoders are expected by a PUCCH with UCI multiplexing of different priorities.

Proposal 4: Rate matching output sequence lengths respectively for HP HARQ-ACK and LP HARQ-ACK are computed based on a single coding rate  configured for HP HARQ-ACK when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

Another issue is whether the existing (i.e., Rel-15 ) configuration can be reused for the HP HARQ-ACK and LP HARQ-ACK multiplexing.  In existing PUCCH design, is configured per PF per PUCCH configuration and requirements of the value range or the granularity of the coding rate configuration for the HP and LP multiplexing compared to same priority multiplexing cases may be different (e.g., finer granularity may be required to carefully control the effective coding rate of HP HARQ-ACK). As a result, a dedicated configuration for HP HARQ-ACK and LP HARQ-ACK rate matching may be needed when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

Proposal 5: A configuration dedicated for HP HARQ-ACK and LP HARQ-ACK rate matching is supported when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

2.3 PUCCH resource determination

At RAN1 #104-e, the following agreements were achieved:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17,

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2.

- FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK.

- FFS in case the total number of LP and HP HARQ-ACK bits is 2.

- FFS details

It is still open whether a dedicated PUCCH resource could be configured for HP-LP HARQ-ACK multiplexing. In our view, compared to sharing the multiplexing PUCCH resource with HP UCI only PUCCH resource, dedicated configuration of PUCCH resources for HP HARQ-ACK and LP HARQ-ACK multiplexing provide a possibility to avoid the ambiguity between gNB and UE on the multiplexing caused by mis-detection of LP DCI. In addition, dedicated PUCCH resource could provide more configuration flexibility in terms of independent maxPayloadSize setting. This could allow gNB to further optimize the coding rate and resource allocation for HP HARQ-ACK and LP HARQ-ACK multiplexing. And also, forward compatibility is expected by dedicated PUCCH resource configuration considering more complicated multiplexing of UCIs in future. The configuration of dedicated multiplexing PUCCH resources could be optional and shared HP HARQ-ACK PUCCH resources are used when the configuration is absent.

Proposal 6: Optional configuration of a set or subset of PUCCH resources dedicated for multiplexing of UCIs with different priorities is supported.

2.4 Multiplexing Enabling/Disabling

It is still undecided how to enable/disable HARQ-ACK multiplexing for different priorities on PUCCH. Two options were discussed at the last e-meeting. These two options are:

- Option 1: DCI indication;

- Option 2: RRC configuration

In our view, these two options are not mutually exclusive. The DCI indication could be used on top of RRC configuration. When the DCI indication is unavailable or absent (e.g., fallback DCI or SPS PDSCH), the RRC configuration is followed. The DCI indication can allow the network to dynamically and timely control the multiplexing considering different conditions and requirements. It can also avoid defining further implicit restrictions to ensure the reliability and latency of the HP HARQ-ACK when multiplexing is enabled by RRC configuration. The exact DCI design could be for further study.

Proposal 7: Dynamic enabling/disabling by DCI for HARQ-ACK multiplexing on PUCCH with different priorities is supported on top of RRC configuration.

3   Multiplexing HARQ-ACK on PUSCH

3.1 Channel coding

The following agreements were made at RAN1 #104bis-e:

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs.

- It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

In Rel-15/Rel-16, at most three encoding chains are used for UCI multiplexing on PUSCH, i.e., one encoding chain for HARQ-ACK, one encoding chain for CSI part I and one encoding chain for CSI part II. Similarly to separate coding for HP HARQ-ACK and LP HARQ-ACK on PUCCH, CSI part II could be dropped when HP HARQ-ACK and LP HARQ-ACK are multiplexed on a PUSCH, to not exceed the number of encoding chains supported in Rel-15/16.

Proposal 8: CSI part II (if exists) is dropped when HP HARQ-ACK and LP HARQ-ACK are multiplexed on a PUSCH.

3.2 Rate matching

It has been agreed in RAN1 #102-e to support separate configurations of beta-offset values for multiplexing with different priority combinations. For a given PUSCH, which beta-offset value(s) should be used is determined by the priority of a multiplexed UCI. It is also beneficial to separately configure scaling factor (i.e., parameter “alpha”) for multiplexing with different priority combinations from the perspectives of HP PUSCH protection and gNB configuration flexibility.

Proposal 9: Separate configuration of scaling factors (“alpha”) is supported for UCI-PUSCH multiplexing with different priority combinations.

3.3 Resource mapping

It was agreed at RAN1 #102e to support multiplexing an LP HARQ-ACK in a HP PUSCH and a HP HARQ-ACK in an LP PUSCH in Rel-17. For the case of multiplexing an HP HARQ-ACK in a LP PUSCH, when the HP HARQ-ACK is associated with a later HP DCI while the LP PUSCH is scheduled by an earlier DCI, as shown in Figure 1, the UE may not be able to re-execute the rate matching and resource mapping by taking into account the HP HARQ-ACK multiplexing as the LP PUSCH (which may also include LP UCI) has already been prepared. Puncturing could be one possible solution for this problem. Alternatively, the UE could expect that the multiplexing of HP HARQ-ACK in an LP PUSCH is supported only when a certain timeline criteria between the earlier LP DCI and the later HP DCI  and/or a certain timeline criteria between the later HP DCI and the LP PUSCH are met.

Figure 1． multiplexing of a HP HARQ-ACK associated to a later DCI and an LP PUSCH scheduled by an earlier DCI

Proposal 10:  When multiplexing of a HP HARQ-ACK associated to a later DCI and a LP PUSCH scheduled by an earlier DCI is enabled, two possible solutions (or a hybrid of the two solutions) could be considered:

- The HP HARQ-ACK is punctured on the LP PUSCH;

- The multiplexing is expected to be applied only when certain timeline criterion are met.

3.4 Multiplexing Enabling/Disabling

For the same reason as HP HARQ-ACK and LP HARQ-ACK multiplexing on PUCCH, it is our preference to also support dynamic enabling/disabling by DCI for UCI-PUSCH multiplexing with different priorities. Whether the existing field (e.g., Beta_offset field) is reused or re-interpreted, or a new field dedicated for this purpose could be for further study.

Proposal 11: Dynamic enabling/disabling by DCI for UCI-PUSCH multiplexing with different priorities is supported on top of RRC configuration.

4  Multiplexing of HARQ-ACK and SR

A number of candidate solutions were proposed by companies and the following agreements were made at RAN1#104-e:

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

- Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  - Opt.1a: The UE does not transmit negative SR.

  - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

  - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

  - FFS: whether with power boost to transmit multiplexed payload or not.

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

- Opt.3: No enhancement over Rel-16.

- Other options not excluded.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Agreements:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):

- Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  - Opt.1a: The UE does not transmit negative SR.

  - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.

  - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource

  - FFS: whether with power boost to transmit multiplexed payload or not.

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  - Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  - FFS on conditions of multiplexing.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.5: No enhancement over Rel-16.

- Other options not excluded.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Agreements:

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):

- Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.

  - Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

  - Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource

- Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.

  - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.

  - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

  - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

  - Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.

- Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.

- Opt.4: No enhancement over Rel-16.

- Other options not excluded.

- FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

According to the discussions and agreements at the RAN1#104-e and RAN1#104bis-e, three cases get interest from most companies. The three cases are:

- Case 1: a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0;

- Case 2: a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1;

- Case 3: a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0.

In general, for all these three case, the performance (including both reliability and latency) of HP positive SR should be sufficiently guaranteed whenever any of LP HARQ-ACK bits is multiplexed.

For Case 1, the HP SR may enjoy different configurations (e.g., a different power control parameter or a frequency selective PUCCH resource) from the LP HARQ-ACK when no multiplexing exists. This kind of superiority should be maintained when multiplexing with LP HARQ-ACK is applied. Therefore, it is preferred that the HP positive SR and LP HARQ-ACK are multiplexed and transmitted on the SR resource. For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. By this way, the SR detection performance may be different from HP SR only transmission as increased cyclic shift values are required to convey HARQ-ACK. However, the performance impact is expected to be marginal and could be potentially compensated by gNB implementation based on PRB switching. And also, appropriate power setting could be employed by gNB to ensure the HP positive SR performance. The cyclic shift applied for the multiplexed HP positive SR and LP HARQ-ACK can follow Rel-15 specification.

Proposal 12: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, Opt.1b (i.e., The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource, and the UE transmits only HARQ-ACK on the HARQ-ACK resource for negative SR) is supported.

For Case 2, based on the same reason as Case 1, the HP SR resource should be used for the HP positive SR transmission. It is possible to multiplex the HARQ-ACK bits into the SR resource. PF1 is used mainly for coverage reason. When the PF1 HARQ-ACK bit(s) is multiplexed into a PF0 PUCCH, the HARQ-ACK performance may not be ensured, which certainly lower the interest on supporting this kind of multiplexing.

Proposal 13: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, Opt.4 (i.e., for positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource) is supported.

A similar behavior as Case 1 could be applied for Case 3. The open issue is whether enhancements need be specified to further protect the reliability of HP SR when the SR is positive. As mentioned above for Case 1,  gNB could compensate the performance of HP SR detection by implementation (e.g., power boosting and PRB detection based on PRB switching) if the multiplexing could degrade HP SR reliability. And also, payload size based power control is supported by PF1 which can further alleviate the impact to HP SR reliability. Therefore, no further enhancement is needed for Case 3 compared to Case 1.

Proposal 14: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, Opt 3 (i.e., for positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource) is supported.

5  Conclusions

In this contribution, several aspects on the multiplexing of UCIs with different priorities are discussed. The following proposals are drawn based on the discussions:

Proposal 1: The working assumption is confirmed with clarification: Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N if CSI is for a report quantity as defined in Rel-15/16.

Proposal 2: Repetition coding or simplex coding is supported for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s) when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

Proposal 3: Placeholder bits “y” are introduced for the scrambling of PUCCH.

Proposal 4: Rate matching output sequence lengths respectively for HP HARQ-ACK and LP HARQ-ACK are computed based on a single coding rate  configured for HP HARQ-ACK when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

Proposal 5: A configuration dedicated for HP HARQ-ACK and LP HARQ-ACK rate matching is supported when the total number of HP or LP HARQ-ACK bits multiplexed on the PUCCH is larger than 2.

Proposal 6: Optional configuration of a set or subset of PUCCH resources dedicated for multiplexing of UCIs with different priorities is supported.

Proposal 7: Dynamic enabling/disabling by DCI for HARQ-ACK multiplexing on PUCCH with different priorities is supported on top of RRC configuration.

Proposal 8: CSI part II (if exists) is dropped when HP HARQ-ACK and LP HARQ-ACK are multiplexed on a PUSCH.

Proposal 9: Separate configuration of scaling factors (“alpha”) is supported for UCI-PUSCH multiplexing with different priority combinations.

Proposal 10:  When multiplexing of a HP HARQ-ACK associated to a later DCI and a LP PUSCH scheduled by an earlier DCI is enabled, two possible solutions (or a hybrid of the two solutions) could be considered:

- The HP HARQ-ACK is punctured on the LP PUSCH;

- The multiplexing is expected to be applied only when certain timeline criterion are met.

Proposal 11: Dynamic enabling/disabling by DCI for UCI-PUSCH multiplexing with different priorities is supported on top of RRC configuration.

Proposal 12: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, Opt.1b (i.e., The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource, and the UE transmits only HARQ-ACK on the HARQ-ACK resource for negative SR) is supported.

Proposal 13: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, Opt.4 (i.e., for positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource) is supported.

Proposal 14: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, Opt 3 (i.e., for positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource) is supported.

6  References

[1]    R1-2103857, Summary#1 of R17 intra-UE Multiplexing/Prioritization, Moderator (OPPO), RAN1#104bis-e, Apr, 2021.