3GPP TSG RAN WG1 Meeting #105-e  			    			            R1- 2105228

e-Meeting, May 10th – 27th, 2021

# Source:	ETRI

# Title:		Discussion on resource allocation for power saving

# Agenda item:	8.11.1.1

# Document for:	Discussion

Introduction

In the last meeting, the following conclusion and agreements have been made for resource allocation for power saving [1]:

Conclusion: - In periodic-based partial sensing, - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2. Agreements: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics - FFS whether/how to restrict the set of values - For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible) - Alt 1: Option 1 as in RAN1#104-e - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1 - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc. - Note: companies are encouraged to provide more evaluations Agreement: - When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing - Partial sensing configured by higher layer in the UE

Based on the above conclusion and agreements, we introduce our views on resource allocation for power saving in this contribution.

Sidelink power saving

Random resource selection and partial sensing

In a resource pool (pre-)configured with at least partial sensing, there are at least two types of partial sensing considering periodic and aperiodic transmissions. One is periodic-based partial sensing, and the other is contiguous partial sensing. In case of periodic-based partial sensing for periodic transmission, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window as in LTE. Regarding the resource selection window [n+T1, n+T2], it was agreed that the definition of T1 and T2 are the same as in Rel-16 NR V2X and there is no further study. Regarding a periodicity value(s) from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList), it is proposed to consider a subset of values from the configured set sl-ResourceReservePeriodList which is larger than a certain threshold value. In Rel-16 NR V2X, there is a broad range of periodicities for periodic reservation, i.e. {(1,…,99), 100, 200, 300, 400, 500, 600, 700, 900, 1000} ms, and up to 16 periodic reservation values can be (pre-)configured per resource pool. Considering periodic transmission with small value(s) of periodicity such as 1 or 2 ms, there is no big difference between periodic transmission with short periodicity and continuous transmission. Therefore, partial sensing for such a periodic transmission with short periodicity could be covered by contiguous partial sensing which will be mentioned later. When only the most recent sensing occasion (i.e., k = 1) is considered, the sensing window with short periodicity(s) may fall within Y slots. However, if a subset of periodicity values which are larger than a certain threshold value is considered for periodic-based partial sensing, only the most recent sensing occasion within sensing window for a given reservation periodicity configured would be sufficient. Therefore, regarding the periodic-based partial sensing, the followings are proposed:

Proposal 1: For the periodic-based partial sensing,

- It is proposed to consider a subset of values from the configured set sl-ResourceReservePeriodList which are larger than a certain threshold value. FFS for the threshold value, e.g., 100 ms.

- It is also proposed to consider only the most recent sensing occasion, i.e., k = 1 for a given reservation periodicity

In case of contiguous partial sensing, it is proposed for aperiodic transmission. It is also applicable for periodic transmission with short periodicity as mentioned in the above. Therefore, the sensing window [n+TA, n+TB] subject to triggering resource selection in slot n can be varied according to different purposes. For example, for aperiodic transmission, both TA and TB would be positive values as shown in Figure 1.

Figure 1. Contiguous partial sensing for aperiodic transmission

For periodic transmission with short periodicity, TA or both TA and TB would be negative values, and it could be better to replace n in the definition of the sensing window by the first slot of Y candidate slots in order to keep consistency for sensing procedure of periodic transmission as shown in Figure 2. If a periodicity for periodic transmission is too short (e.g., 1 or 2 ms), a sensing window could be configured after selection triggering slot n.

Figure 2. Contiguous partial sensing for periodic transmission with short periodicity

In case of random selection, both TA and TB would be equal to zero. Therefore, regarding the contiguous partial sensing, the followings are proposed:

Proposal 2: For the contiguous partial sensing,

- The sensing window [n+TA and n+TB] can be configured variously for different purposes e.g., both TA and TB are positive values for aperiodic transmission, either or both of TA and TB are negative values for periodic transmission with short periodicity, both TA and TB are equal to zero for random selection.

- In case of periodic transmission with short periodicity, it is proposed to replace n in the definition of the sensing window by the first slot of Y candidate slots.

Reception capability of power saving UEs

In RAN1#104-e meeting, in addition to Type-A and Type-D, Type-B was newly introduced. Type-B does not have any reception capability except for PSFCH and S-SSB reception. Since Type-B UE cannot perform sensing operation same as Type-A UE, it is defined for random resource selection. However, the reception capability of Type-B UE can be utilized to improve power saving and reliability. Randomly selected resources will cause resource collision with higher probability than sensing-based selected resources. In case of periodic transmission, the collision might be occurred continuously with other UE’s periodic resource reservation. Since the increasing number of retransmissions due to continuous collision can cause power consumption and deteriorate reliability. In order to avoid such a continuous collision, the limited reception capability can be utilized. For example, with PSFCH reception capability, Type-B UE can count the number of NACKs and if the number of NACKs is higher than a certain threshold, Type-B UE can re-select resource(s) randomly.

Proposal 3: It is proposed to study how to utilize the limited reception capability of Type-B for random resource selection.

Coexistence of power saving RA scheme(s)

In LTE sidelink, for random resource selection and partial sensing UEs, a specific resource pool is configured, and the resource pool is separated from other resource pool(s) for full sensing UEs. However, in Rel-17 NR, considering efficient resource utilization and various use cases, it was agreed that a sidelink mode 2 TX resource pool can be (pre-)configured to support coexistence between power saving UEs and full sensing UEs. In order to avoid potential collision between power saving UEs and full sensing UEs, it is necessary to study potential solutions. For example, full sensing UEs can avoid the resources for power saving UEs by configuring different priorities between them i.e., higher priority for power saving UEs than one for full sensing UEs, or an indication for power saving UEs can be included in 1st or 2nd SCI to avoid resource confliction. By detecting higher priority or indication for power saving UEs, full sensing UEs can avoid the usage of the resources for power saving UEs.

Proposal 4: It is proposed to study how to support coexistence between power saving UEs and full sensing UEs in a same resource pool.

Sidelink DRX impacts

Regarding sidelink DRX, if SL DRX wake-up timing between UEs is aligned, it will be beneficial for power saving since the UEs can communicate each other during periodic wake-up time. Even though it seems to be suitable for periodic traffic, it can be extended to aperiodic traffic with additional mechanisms for aperiodic traffic (e.g., wake-up signal). In case of unicast communication, PC5-RRC connection procedure can be considered one of candidate methods to exchange the information on SL DRX configuration. However, in case of groupcast and broadcast communications, another approach/procedure for the delivery of the information on SL DRX should be studied. In addition to data transmission and reception including retransmission subject to SL DRX, it is also needed to study other aspects such as CSI reporting, PSFCH reporting, etc. For example, if a UE receives SL data in SL DRX on-duration time and the corresponding PSFCH is transmitted in SL DRX off-duration time, a proper UE’s behavior should be defined. With respect to CSI reporting, it can be expected a similar situation. Therefore, it is proposed to study potential impacts of SL DRX on power saving.

Proposal 5: It is proposed to study potential impacts of SL DRX on power saving.

Additional enhancements

In addition to random resource selection and partial sensing, additional enhancements for power saving can be considered. First of all, the concept of partial sensing can be extended in different domain (e.g., frequency) rather than time domain. In Uu link, BWP adaptation has been used for power saving via BWP switching. Supporting narrow BWP (and/or reduced resource pool) operation with BWP switching could be one of solutions. For example, in power saving mode, a UE can operate in narrow BWP in order to reduce power consumption for monitoring. When a large amount of data transmission/reception is needed, the UE can switch its BWP into larger one. In order to support BWP switching, it may need to support configuration of multiple SL BWPs, cross-BWP switching, cross-slot switching, SCI only transmission in single subchannel and so on. SL resource pool switching can be also considered instead of SL BWP switching. Since it may cause a heavy specification workload, it should be studied carefully. Even though the main objective of inter-UE coordination is for enhanced reliability and reduced latency in resource allocation mode 2, the inter-UE coordination can be also used for power saving. For example, if a UE selects resource(s) for its own transmission within a white list which is provided as a coordination message, the power consumption for resource selection can be reduced. Other enhancements such as reduced number of blind decoding/estimations, simplified sensing with 1st stage SCI only, etc. could be also studied.

Proposal 6: Additional enhancements besides random resource selection and partial sensing for power saving should be also discussed.

Summary

In this contribution, we made the following proposals for resource allocation for sidelink power saving.

Proposal 1: For the periodic-based partial sensing,

- It is proposed to consider a subset of values from the configured set sl-ResourceReservePeriodList which are larger than a certain threshold value. FFS for the threshold value, e.g., 100 ms.

- It is also proposed to consider only the most recent sensing occasion, i.e., k = 1 for a given reservation periodicity

Proposal 2: For the contiguous partial sensing,

- The sensing window [n+TA and n+TB] can be configured variously for different purposes e.g., both TA and TB are positive values for aperiodic transmission, either or both of TA and TB are negative values for periodic transmission with short periodicity, both TA and TB are equal to zero for random selection.

- In case of periodic transmission with short periodicity, it is proposed to replace n in the definition of the sensing window by the first slot of Y candidate slots.

Proposal 3: It is proposed to study how to utilize the limited reception capability of Type-B for random resource selection.

Proposal 4: It is proposed to study how to support coexistence between power saving UEs and full sensing UEs in a same resource pool.

Proposal 5: It is proposed to study potential impacts of SL DRX on power saving.

Proposal 6: Additional enhancements besides random resource selection and partial sensing for power saving should be also discussed.

References

- Chairman’s notes, RAN1#104bis-e, April 2021.