3GPP TSG RAN WG1 #105-e	R1-2105348

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.15.4

Source: 	Samsung

Title: 	On enhancements on HARQ

Document for:	Discussion and Decision

Introduction

A new study WID was agreed in RAN #86 meeting [1], and RAN1 related part is started from this RAN1 meeting. One objective is to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items related to RAN1:

- Aspects related to random access procedure/signals [RAN1, RAN2]

- Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

- Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

- Aspects related to HARQ operation [RAN2, RAN1]

In this contribution, we discuss possible enhancements on HARQ and give some proposals based on the discussion.

Enhancements on HARQ-ACK feedback disabling

One enhancement on HARQ discussed in NR NTN is to disable HARQ feedback, and it can also be applicable for IOT over NTN. Disabling of HARQ feedback wouldn’t cause UE complexity, and can save UE power consumption which is very beneficial for IOT device. And, the specification impact is acceptable except introducing a higher layer configuration for disabling of HARQ feedback. As agreed in RAN2 for NR NTN, disabling of HARQ feedback can be configured per HARQ process via UE specific RRC signaling signaling.

Proposal 1: Disabling of HARQ feedback should be supported as NR NTN.

Proposal 2: HARQ feedback can be enabled/disabled per HARQ process via UE specific RRC signaling as NR NTN.

If HARQ feedback can be enabled/disabled per HARQ process per UE, one further question is whether to support disabling of HARQ feedback for all the HARQ processes. If it is supported, UE power consumption can be reduced as far as possible and data rate can be improved for all the HARQ processes. However, due to no any HARQ feedback, gNB may not know whether the transmission parameter is appropriate for the current situation or not. A new UCI feedback for reporting DL transmission disruption and/or requesting DL scheduling changes may be required. Of course, the new UCI feedback should have less frequency than HARQ feedback from power consumption perspective, e.g., it can be reported once for a long time.

Proposal 3: Whether to support disabling of HARQ feedback for all the HARQ processes should be discussed.

Enhancements on UE assistance information reporting

In another aspect, a UE is fully aware of the buffer situation in the DL HARQ procedure. If a UE observes the buffer for HARQ operation is full or almost full, it would be beneficial to trigger the HARQ disabling from the UE side, although the final decision of HARQ disabling/enabling is configured from the network side. For example, a UE can report HARQ related information in the RRC parameter UEAssistanceInformation to request for HARQ disabling/enabling, and the HARQ related information could be an explicit indication of HARQ disabling/enabling request, or a buffer condition for HARQ operation for gNB to consider disabling/enabling HARQ. For another example, a UE can report HARQ related information in the RRC parameter UEAssistanceInformation to request for adapting the number of HARQ processes, in an explicit way by requesting the number or an implicit way by reporting the buffer condition. Alternatively, the UE buffer size could be signaled as part of UE capability, then gNB could handle contiguous data transmission, if necessary, by estimating the UE’s soft buffer status.

Proposal 4: UE assistance information for HARQ can be supported.

Conclusions

In this contribution, we discussed the enhancements on HARQ, including enhancements on HARQ-ACK feedback disabling, enhancements on number of HARQ processes, and enhancements on UE assistance information reporting. Our proposals are summarized below:

Proposal 1: Disabling of HARQ feedback should be supported as NR NTN.

Proposal 2: HARQ feedback can be enabled/disabled per HARQ process via UE specific RRC signaling as NR NTN.

Proposal 3: Whether to support disabling of HARQ feedback for all the HARQ processes should be discussed.

Proposal 4: UE assistance information for HARQ can be supported.

- References

[1] RP-193235, New study WID on NB-IoT/eMTC support for NTN, MediaTek Inc.

[2] Final Report for RAN1#102-e Meeting