3GPP TSG RAN WG1 #105-e    			       	 R1-2105482

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.5.1

Source: 	LG Electronics

Title: 	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays

Document for:   Discussion and decision

Introduction

This contribution discusses Rel-17 NR positioning enhancements specifically in mitigating UE Rx/Tx and gNB Rx/Tx timing delays.

Time window for measurement

For timing based positioning measurement, LMF can request UE to measure DL positioning reference signal through multiple methods. And then, UE can report measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference) to LMF in a single measurement report. gNB/TRP also can report multiple measurement instances in a single measurement to LMF. Currently, however, there is no requirement to enforce the DL and UL measurements are associated with the same set of DL PRS and UL SRS resources and the same timestamp. Considering it, a following is agreed in the previous meeting [1].

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

Assuming that propagation time of UL and DL is the same, the UE/network could extract the synchronization time difference included in RSTD measurement by using round trip time measurement. Based on Figure 1, the relation of the round trip time, RSTD, and the time synchronization term can be described as follows:

RTT1-RTT2×0.5=RSTD12+δ12,				(1)

where RTT1/ RTT2 denotes the round trip time between the UE and gNB#1/gNB#2 respectively, and RSTD12 denotes the RSTD measurement regarding gNB#1 and gNB#2 when there is no synchronization error, and δ12 represents the time difference between gNB#1 and gNB#2.

In consideration of the defined timing measurement, the above equation could be described as follows:

(UE RX-Tx)1+(gNB RX-Tx)1- UE RX-Tx2+gNB RX-Tx2 ×0.5=RSTD12+δ12   (2)

Where,

- (UE RX-Tx)k: UE Rx-TX time difference of the UE and gNB#k

- (gNB RX-Tx)j: gNB Rx-Tx time difference of the UE and gNB#j

- RSTDkj: RSTD measurement with respect to gNB#k and gNB#j

Figure 1 An illustrative example of time synchronization error extraction by using DL-TDOA and Multi-RTT

From (2), if the gNB Rx-TX time difference measurement is sent to the UE, the UE also can estimate network time synchronization difference.

Observation #1:

- Time synchronization errors between gNBs/TRPs can be compensated from either network or UE by using signal measurements based on SRS and PRS.

In consideration of the measurement acquisition behaviour of the UE and gNB, however, for the purpose of accurate acquisition of network time synchronization difference, the time duration of the measurement acquisition should be considered. Considering that a specific timing measurement is calculated by averaging multiple measurements, the above (2) could be expressed as follows:

EUE RX-Tx1T1,UE+ EgNB RX-Tx1T1,gNB-EUE RX-Tx2T2,UE-EgNB RX-Tx2T2,gNB ×0.5=ERSTD12T3+δ12,

- T1,UE and T2,UE respectively denote the UE’s measurement averaging window for the UE Rx-Tx time different for gNB#1 and gNB#2,

- T1,gNB and T2,gNB respectively denote the measurement averaging window of gNB#1 and gNB#2 for the gNB Rx-Tx time different with regard to the UE.

- T3 denotes the UE’s measurement averaging window for RSTD.

The measurement averaging windows described above are generally up to implementations for both of the UE and the gNB. In our view, however, if those windows are up to the implementations, it is difficult to extract the time synchronization difference term. For example, a UE can recognize its movement speed, so the UE can report the UE Rx-Tx time difference measurement based on the most recent measurement samples, but the gNB can assume the UE which is semi-static, and then it may calculate the average value of the gNB Rx-Tx time difference measurement acquired over a relatively long time duration for the improved measurement accuracy. It leads to an inaccurate RTT measurement at the LMF. Thus, both of the UE and the gNB should follow an aligned rule to determine a measurement for reporting of the Rx-Tx time difference and RSTD.

Furthermore, Tx/Rx beam direction of the UE/gNB need to be considered. The UE’s reception beam for PRS measurement and the transmission beam of the gNB should be optimized for timing measurement and those beams should be used for SRS transmission and reception.

Proposal #1:

- Support enhancements for introducing measurement acquisition rule on UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement

  - E.g., the same time window for measurement averaging of UE Rx-Tx and gNB Rx-Tx

- Support enhancements for introducing measurement acquisition rule on RSTD measurement and UE/gNB Rx-Tx time difference

  - E.g., the same time window for measurement averaging of RSTD and UE/gNB Rx-Tx

As another option, single shot based measurement can be considered. For example, AP PRS with AP SRS could be considered for the UE/gNB Rx-Tx time difference and RSTD measurements.

Actually, if the network time synchronization error matters, the UE's location can be estimated by using multi-RTT technique, which is not affected by time synchronization problem. Then, why do we need estimations of the time synchronization error by using Multi-RTT and DL-TDOA? For this fundamental question, we have the following answers.

Firstly, from the perspective of a single UE capable of Multi-RTT, estimation of the time synchronization error between different TRPs would not be quite effective for positioning. However, by applying Multi-RTT and DL-TDOA techniques to a lot of UEs where they are located in the different area/region, LMF can obtain the time synchronization difference between many TRPs/gNBs. Based on those information, the LMF can compensate network time synchronization difference of RSTD measurements for many TRPs/gNBs.

- In consideration of the (substantial) coverage difference between DL PRS and UL SRS (for positioning), the number of TRPs/gNBs a UE can receive the PRS would be greater than the number of TRPs/gNBs the UE can transmit the SRS. Thus, from the perspective of a single UE, it is highly limited to estimate time synchronization difference for many different TRPs. However, it would be different through the many UEs,

Secondly, in consideration of the UEs which are not supporting Multi-RTT technique, the time synchronization difference information obtained by other UEs could be effectively used to mitigate time synchronization difference included in the RSTD measurements

Therefore, even if the strict rule for measurement acquisition time is essentially required to mitigate network synchronization time error, our proposal can work well.

Conclusion

In this contribution, we have discussed Rel-17 NR positioning enhancements especially for the mitigation of UE Rx/Tx and gNB Rx/Tx timing delays.

Time window for measurement

Observation #1:

- Time synchronization errors between gNBs/TRPs can be compensated from either network or UE by using signal measurements based on SRS and PRS.

Proposal #1:

- Support enhancements for introducing measurement acquisition rule on UE Rx-Tx time difference measurement and gNB Rx-Tx time difference measurement

  - E.g., the same time window for measurement averaging of UE Rx-Tx and gNB Rx-Tx

- Support enhancements for introducing measurement acquisition rule on RSTD measurement and UE/gNB Rx-Tx time difference

  - E.g., the same time window for measurement averaging of RSTD and UE/gNB Rx-Tx

Reference

- 3GPP RAN1, Chairman’s Notes RAN1#104b-e meeting.