3GPP TSG RAN WG1 Meeting #105-e	R1-2105528

E-meeting, May 10th-27th, 2021

Agenda Item:	8.7.3

Source:	Huawei, HiSilicon

Title:	Other considerations on power saving in Rel-17

Document for:	Discussion and Decision

Introduction

In this contribution, we share our view on the power saving related work on power saving WI and REDCAP SI. Furthermore, the RAN4 lead topic of RLM and BFD relaxation is also discussed from RAN1 perspective.

Power saving for REDCAP UEs

In both Rel-17 power saving enhancements WI and REDCAP SI, power saving is one of the objectives. The objective related with power saving in REDCAP SI is as following:

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]: - Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1]. - Extended DRX for RRC Inactive and/or Idle [RAN2] - RRM relaxation for stationary devices [RAN2]

However, we need to emphasize that Rel-16 has already introduced power saving mechanisms including WUS indication, cross-slot scheduling based power saving, and BWP-based maximum MIMO layer(s) adaptation. The following Rel-16 features can be utilized by the REDCAP UE:

Table 1 Analysis for R16 power saving techniques

| Power saving techniques | Can be utilized by RedCap UEs | Detailed analysis |
| --- | --- | --- |
| PDCCH based wake-up indication | YES | - When a RedCap UE is in CONNECTED mode, it can be configured with C-DRX to save power. Wake-up indication bit before the ON duration of C-DRX cycle can be used to skip PDCCH monitoring in the C-DRX cycle to save power. - Dormancy indication is not needed considering RedCap UE are not likely to support CA. |
| Cross-slot scheduling | YES | - Cross-slot scheduling based power saving can provide the power saving gain by avoiding unnecessary DL OFDM symbol buffering and by relaxing PDCCH processing. - Cross-slot scheduling may introduce little latency increment. However, RedCap UEs with power saving requirements are usually not latency tolerant. |
| BWP based maximum MIMO layer adaptation | YES | - For 2Rx UE, gNB can switch UE to a BWP with maximum MIMO layer of 1 for power saving purpose. In this case, UE can utilize one single Rx chain for power saving. |
| SCell dormancy like behaviour | NO | - Dormancy like behaviour is a scheme to reduce PDCCH monitoring on SCell(s). - Due to the relatively low data rate requirement, there is no need to support CA in RedCap, thus dormancy like behaviour is not needed by RedCap UEs. |
| RRM relaxation for neighbour cell | YES | - RRM relaxation is already part of the RedCap SID, thus the RRM relaxation schemes in Rel-16 should be utilized as baseline for RedCap. |
| UE assistance information | YES | In Rel-16, the following assistance information are specified: - RRC Release Request, to avoid waiting for RRC release message from gNB for a long time - Preferred C-DRX configuration, to make C-DRX configuration more suitable - Preferred max aggregated Bandwidth, to make BWP parameter more suitable - Preferred max number of MIMO layers, to make MIMO parameter more suitable for UE’s traffic - Preferred K0min/K2min value, to make cross-slot scheduling parameter more suitable It is useful to let UE report necessary information to gNB, thus the above assistance information can be utilized for RedCap UE. |

Observation 1: The following Rel-16 power saving mechanism can be utilized by RedCap UEs for power saving with potential modifications, if needed:

- PDCCH based wake-up indication

- Cross-slot scheduling

- maximum MIMO layer adaptation

- RRM relaxation for neighbor cell

- UE assistance information

Furthermore, in RAN#91, the WID of RedCap was updated and a note was added to clarify that power saving enhancement solutions specified in the UE Power Saving Enhancements WI (NR_UE_pow_sav_enh) shall be assumed to be available also to RedCap UEs by default.[1]

Regarding the connected mode UE, extension of DCI based power saving adaptation, including PDCCH skipping, monitoring periodicity adaptation, shall be specified in power saving WI. The Rel-17 connected mode extension of DCI based power saving is also expected to be utilized by REDCAP UE.

Observation 2: The paging enhancement, assistance TRS for IDLE/INACTIVE mode UE and PDCCH adaptation specified in Rel-17 power saving WI need to take RedCap UEs into consideration to follow the note in RedCap WID.

RLM and BFD relaxation

In RAN4 #98bis, it was concluded that RLM/BFD relaxation is feasible for some cases with the following note [2]:

- UE is allowed but not mandatory to perform relaxed RLM/BFD measurements when the relaxation criteria is met in above feasible scenarios.

In the next step, the details will be further discussed, including the criteria of cell quality for stepping in and stepping out measurement relaxation, whether low mobility is used as a criteria, how to determine the scaling factor, and etc. However, all the above remaining issues has no RAN1 impact. So there is no need to have any discussion in RAN1.

Conclusions

In this contribution, we share our view on the power saving of RedCap. The RAN4 lead topic of RLM and BFD relaxation is also discussed from RAN1 perspective. Based on the analysis, we have the following observations and proposals:

Observation 1: The following Rel-16 power saving mechanism can be utilized by RedCap UEs for power saving with potential modifications, if needed:

- PDCCH based wake-up indication

- Cross-slot scheduling

- maximum MIMO layer adaptation

- RRM relaxation for neighbour cell

- UE assistance information

Observation 2: The paging enhancement, assistance TRS for IDLE/INACTIVE mode UE and PDCCH adaptation specified in Rel-17 power saving WI need to take RedCap UEs into consideration to follow the note in RedCap WID.

- References

RP-210918, Revised WID on support of reduced capability NR devices, Nokia, Ericsson.

R4-2105797, WF on NR UE Power Saving Enhancements, MediaTek, vivo.