3GPP TSG RAN WG1 #105-e	R1-2105643

e-Meeting, May 10th – May 27th, 2021

Source:	Sharp

Title:	PUCCH coverage enhancement

Agenda Item:	8.8.2

Document for:	Discussion and Decision

Background

In this paper, we provide views on PUCCH coverage enhancement.

Dynamic PUCCH repetition factor indication

At RAN1#104-e meeting, the following agreement was made in AI 8.8.2 for dynamic PUCCH repetition factor indication [1].

Agreements: Down select from the following two options to support dynamic PUCCH repetition factor indication. - Option 1 (without DCI enhancement): Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI. - FFS details, e.g., via reusing the “PUCCH resource indicator” field (without increase # bits of it), starting CCE index (when applicable) of DCI, by PDCCH aggregation level, etc. - FFS: RRC signaling enhancement details - Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI - e.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication - FFS whether there is a need for RRC update

The main difference between Option 1 and Option 2 is whether DCI enhancement is required or not. Since Option 1 does not change DCI size, it’s applicable to fallback DCI and does not bring about coverage degradation of PDCCH. Although Option 2 is full flexible, it does not enhance PUCCH triggered by fallback DCI or brings about coverage degradation of PDCCH with fallback DCI. To avoid these issues, we support Option 1.

Proposal 1: Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.

As a detail of Option 1, if a new repetition parameter is received, it overrides the Rel-15 repetition factor configured by nrofSlots as NPUCCHrepeat. Namely, in Option 1, the new parameter which overrides the Rel-15 repetition factor configured by nrofSlots should be configured per PUCCH-resource.

Proposal 2: A new parameter which overrides Rel-15 repetition factor configured by nrofSlots is configured per PUCCH-resource.

In Option 1, since the repetition factor is configured per PUCCH-resource, its flexibility is enhanced if a selectable number of PUCCH resources in a PUCCH resource is increased. For this reason, we can additionally support to reuse PRI field and starting CCE index of DCI to enhance Option 1. In Rel-15/16, one of PUCCH resources in first PUCCH resource set is indicated by using both the PRI field and the starting CCE index. This indication is applicable to non-first PUCCH resource set, depending on the number of PDCCH candidates for an aggregation level.

Proposal 3: The PUCCH resource indication by using both PRI field and starting CCE index can be additionally supported for non-first PUCCH resource set.

DMRS bundling across PUCCH repetitions

At RAN1#104-e meeting, the following agreement was made in AI 8.8.2 for DMRS bundling across PUCCH repetitions [1].

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling. - FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16. - Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible Agreements: Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration. - FFS: the configuration is per UE or per PUCCH resource. - FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling - FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

  - Time domain window without frequency hopping

At RAN1#104bis-e meeting, the following agreement was made in AI 8.8.1.3 for joint channel estimation for PUSCH [2].

Agreements: - For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements. - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window - FFS the units the time domain window (e.g. repetitions, slots, and/or symbols) - FFS : association between the potential use case(s) and units of the time window - FFS: single or multiple time domain windows - FFS: relation with UE capability - FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms - FFS whether or not to further consider impacting of timing advance

Necessity of a time domain window was discussed in AI 8.8.1.3 for joint channel estimation for PUSCH of RAN1#104bis-e meeting. Pros and cons of whether or not to specify the time domain window were summarized below [3]:

| Specify a time domain window | Pros | - Make sure that the condition for joint channel estimation is met during the time window. - If DMRS transmissions for joint channel estimation over long time require the processing load, the time window specified per UE can reduce the UE complexity. - Help the UE to know when and how long it should maintain power/phase continuity. - Facilitate the optimization of DMRS for JCE. The DMRS location/granularity in different slots within one time domain window can be jointly designed. - Facilitate the design of inter-slot frequency hopping with inter-slot bundling. |
| --- | --- | --- |
|  | Cons | - More standardization efforts. |
| NOT specify a time domain window | Pros | - Less standardization efforts. |
|  | Cons | - Misunderstanding between gNB and UE may occur in some cases. - It may be difficult to expect improvement in channel estimation performance even if the gNB performs joint channel estimation since there is no expected behaviour of UE which makes UE to operate arbitrarily. - UE may try to keep power consistency and phase continuity as much as possible for all PUSCH transmissions, while gNB may try to jointly estimate channel based on all DMRS symbols over PUSCH transmissions. - If the UE must maintain the phase continuity across scheduled PUSCH transmissions for a very long period, the UE may have to forgo events such as uplink tracking loop, UE calibration, antenna virtualization etc., leading to possible performance degradation. - More UE power consumption. |

These pros and cons are applicable to DMRS bundling for PUCCH. Therefore, for DMRS bundling, it is effective to use a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH transmissions subject to power consistency and phase continuity requirements.

Proposal 4: For DMRS bundling, a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH transmissions subject to power consistency and phase continuity requirements should be adopted.

The time domain window is needed to restrict UE implementation for pathloss update and precoding change for DMRS bundling and to maintain phase continuity and power consistency. Additionally, RAN4 is still discussing whether a UE capability in terms of time length potentially impacts the time domain window and thus the time domain window should consider the UE capability. For example, in FDD, a repetition factor may be larger than the UE capability and all PUCCH repetitions cannot be bundled. Therefore, for at least FDD operation, the time domain window size should be limited by the UE capability if the UE capability is smaller than the repetition factor.

Observation 1: For at least FDD operation, the time domain window size should be limited by the UE capability if the UE capability is smaller than the repetition factor.

As a detail of time domain window, we discuss 2 alternatives for the time domain window configuration as following:

- Alt. 1: Time domain window configuration is not provided by the gNB. The UE determines the location and the size of the time domain window based on at least a value, which is reported via capability reporting.

- Alt. 2: Time domain window configuration is provided by the gNB (either by DCI format or RRC signaling). The size of time domain window should not exceed the size reported by UE via capability reporting.

For comparison of Alt. 1 and Alt. 2, Figure 1 shows an example of the time domain window when the UE capability in terms of time length is smaller than the repetition factor in FDD. In Alt. 1, the UE can determine the time domain window size of Window design 1 based on the UE capability but Window design 2 cannot be realized because the time domain window size of Window design 2 is not aligned with the UE capability. Meanwhile, in Alt. 1, the gNB can flexibly adjust the time domain window size for both Window design 1 and Window design 2.

Figure 1: An example of the time domain window when the UE capability in terms of time length is smaller than the repetition factor in FDD, the UE capability is 6 slots, and the repetition factor is 8.

In Window design 1, the first time domain window size equals to the UE capability and the second time domain window is aligned with remaining slots (i.e., the repetition factor – the UE capability). In Window design 2, both first and second time domain windows have the same size to achieve the same DMRS bundling gain, where the time domain window size does not exceed the UE capability.

Which is better, Window design 1 or Window design 2, for coverage performance depends on channel condition. For example, Window design 1 can maximize SINR after channel equalization in one time domain window and Window design 2 can equivalently improve the SINR for each time domain window. For this reason, we support Alt. 2, which can flexibly adjust the time domain window size by considering the repetition factor and the UE capability.

Proposal 5: We support Alt. 2. Time domain window configuration is provided by the gNB (either by DCI format or RRC signaling). The size of time domain window should not exceed the size reported by UE via capability reporting.

Observation 2: Alt. 1 can flexibly adjust the time domain window size by considering the repetition factor and the UE capability.

  - Time domain window with frequency hopping

At RAN1#104bis-e meeting, the following agreement was made in AI 8.8.1.3 for joint channel estimation for PUSCH [2].

Agreements: - For inter-slot frequency hopping with inter-slot bundling, down select on the following two options: - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size. - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size. - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined. - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD. - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

We discuss Option 1 and Option 2 of the agreement in AI 8.8.1.3 for joint channel estimation for PUSCH of RAN1#104b-e meeting for PUCCH repetition with DMRS bundling.

Option 1: (The bundle size = the time domain window size)

- The bundle size (hopping interval) can be determined at the same time as the time domain window.

- The time domain window is determined by Alt. 1 or Alt. 2 in Section 3.1.

- The bundle size should not exceed the UE capability.

Option 2: (The bundle size ≠ the time domain window size)

- The bundle size (hopping interval) can be determined from the time domain window size or separately from the time domain window size.

- The time domain window is determined by Alt. 1 or Alt. 2 in Section 3.1.

- The time domain window should not exceed the UE capability.

For comparison of Option 1 and Option 2, Figure 3 shows examples of the relation between the time domain windows and the bundles. In Figure 3, we use a new hopping pattern, not the legacy hopping pattern of Rel-15/16, because the legacy hopping pattern performs hopping per slot (i.e., the bundle size is always 1) and DMRS bundling cannot be applied to two consecutive UL slots as shown in Figure 2.

Figure 2: Legacy hopping pattern of Rel-15/16 when repetition factor is 8 and FDD operation.

Figure 3: Examples of the relation between the time domain windows and the bundles when repetition factor is 8 and FDD operation.

In Figure 3, a hopping pattern which indicates whether each available slot for PUCCH repetition is associated with a first hop or second hop should be provided by gNB. Namely, the bundle size should be provided by gNB. This is because both DMRS bundling gain and frequency diversity gain are effective for coverage enhancement and the bundle size bringing a good balance between these gains should be flexibly determined.

Observation 3: The bundle size should be provided by gNB. This is because both DMRS bundling gain and frequency diversity gain are effective for coverage enhancement and the bundle size bringing a good balance between these gains should be flexibly determined.

In both Option 1 and Option 2, the time domain window size should not exceed the UE capability. In Option 2, if the bundle size is larger than the UE capability, the time domain window can be smaller than the bundle size. However, according to Observation 3, since the bundle size can be configured by gNB, it is not effective for coverage enhancement that the time domain window whose size is smaller than the bundle size is provided. Additionally, in Option 2, if the time domain window size is larger than the bundle size, phase continuity and power consistency requirements are not met and an additional issue occurs (e.g., Is the UE expected to maintain phase continuity during a time domain window sticking out?). For these reasons, we support Option 1.

Proposal 6: The bundle size should equal to the time domain window size.

For a hopping pattern, which indicates one of first and second hops at every single bundle, there are two alternatives as the following:

- Alt. 2-1: Rel-15/16 alternating hopping pattern which switches hops alternatively at every single bundle (i.e., first hop → second hop → first hop).

- Alt. 2-2: New hopping pattern (e.g., first hop → second hop → second hop).

For TDD operation with a specific TDD pattern (e.g., DDDSUDDSUU), Alt. 2-1 reduces frequency diversity gain as shown in Figure 4. Alt. 2-2 maximizes frequency diversity gain (i.e., each hop has the same number of slots) in addition to DMRS bundling gain. Therefore, to maximize frequency diversity gain, support new hopping pattern other than alternating hopping pattern of Rel-15/16.

Proposal 7: To maximize frequency diversity gain, support new hopping pattern other than Rel-15/16 alternating hopping pattern.

Figure 4: Hopping patterns for PUCCH repetition with DMRS bundling when a repetition factor is 4 and TDD configuration is DDDSUDDSUU.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.

Proposal 2: A new parameter which overrides Rel-15 repetition factor configured by nrofSlots is configured per PUCCH-resource.

Proposal 3: The PUCCH resource indication by using both PRI field and starting CCE index can be additionally supported for non-first PUCCH resource set.

Proposal 4: For DMRS bundling, a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH transmissions subject to power consistency and phase continuity requirements should be adopted.

Proposal 5: We support Alt. 2. Time domain window configuration is provided by the gNB (either by DCI format or RRC signaling). The size of time domain window should not exceed the size reported by UE via capability reporting.

Proposal 6: The bundle size should equal to the time domain window size.

Proposal 7: To maximize frequency diversity gain, support new hopping pattern other than Rel-15/16 alternating hopping pattern.

Observation 1: For at least FDD operation, the time domain window size should be limited by the UE capability if the UE capability is smaller than the repetition factor.

Observation 2: Alt. 1 can flexibly adjust the time domain window size by considering the repetition factor and the UE capability.

Observation 3: The bundle size should be provided by gNB. This is because both DMRS bundling gain and frequency diversity gain are effective for coverage enhancement and the bundle size bringing a good balance between these gains should be flexibly determined.

References

- RAN1 chairman’ note at RAN1#104 e-meeting, February 2021

- RAN1 chairman’s note, RAN1#104bis e-meeting, April 2021

- [104b-e-NR-R17-CovEnh-02] Summary of email discussion on joint channel estimation for PUSCH at RAN1#104b-e meeting, April 2021