3GPP TSG RAN WG1 #105-e			R1-2105710

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on extension to DCI-based power saving adaptation

Agenda Item:	8.7.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#104-e meeting, the techniques for extension to DCI-based power saving adaptation were discussed and RAN1 made following agreements [1].

Agreements: - Strive for a common design for DCI based PDCCH monitoring adaptation in active time for an active BWP to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration. - Details FFS Agreements: - Further study whether and how to minimize the impact to data scheduling for new transmissions and retransmissions. - FFS details - Further study the application delay for PDCCH adaptation indication Agreements: For DCI based PDCCH skipping in active time for an active BWP (if supported), the following can be further considered, - Explicit indication of PDCCH adaptation - Scheduling DCI - Format 1_1 - Format 0_1 - Format 0_2/1_2 - Non-scheduling DCI - Format 2_6 in active time - Format 2_0 - Format 1_1 (SCell dormancy case 2) - additional indication mechanism - By reusing Rel-16 SCell dormancy indication when CA is configured, FFS details - By reusing Rel-16 cross-slot scheduling indication when R16 cross-slot scheduling is configured, FFS detailds - DCI dynamically indicates a duration/periodic interval for skipping - FFS: how to indicate the duration/period interval, e.g., number of slots or skipping current DRX - PDCCH skipping for a duration indicated by minimum scheduling offset - Others are not precluded Agreements - For DCI based SSSG switching in active time for an active BWP (if supported), the following can be further considered, - Explicit indication of PDCCH adaptation - Scheduling DCI based - Format 1_1, - Format 0_1, - Format 0_2/1_2 - Format 1_0 - Non-scheduling DCI supported by vivo, Samsung - Format 2_6 in active time - Format 2_0 - Format 1_0 - Format 1_1 (SCell dormancy case 2) - additional indication mechanism - By reusing Rel-16 SCell dormancy indication when CA is configured, FFS details - By associating Rel-16 cross-slot scheduling indication when R16 cross-slot scheduling is configured, FFS detailds - DCI dynamically indicates a duration period for the switched SSSG, UE switch back to previous/default SSSG after duration endstimer expried - Timer-based SSSG switching, including RRC configured a timer, UE switch back after timer expired. - SSSG activation/deactivation - FFS: Implicit SSSG switching - SSSG switching triggered by SR - SSSG switching triggered by RACH - Default SSSG that a UE monitors when coming out of DRX to monitor an ON duration. - FFS: whether/how to support SSSG switching for multiple groups of cell(s). - FFS: whether/how to support SSSG switching in active time with DCP outside active time - FFS: whether / how to support more than 2 SSSGs, - FFS: number of SSSGs - FFS: a search space set group to emulate PDCCH skipping - Others are not precluded Agreements: - The following alternatives can be considered for DCI based PDCCH monitoring adaptation in active time for an active BWP for power saving - Alt 1: Enhancement of Rel-16 SSSG switching to support PDCCH monitoring adaptation including skipping for a duration - Alt 2a: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and SSSG switching - Alt 2b: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and PDCCH monitoring periodicity adaptation - Others not precluded

In this contribution, we discuss on the extension to Rel-16 DCI-based power saving adaptation.

- Discussion

  - Common design for both SSSG switching and PDCCH skipping for a duration

It was agreed to strive for common design including the functionalities of both SSSG switching and PDCCH skipping for a duration. SSSG switching and PDCCH skipping for a duration have the similar functionality in terms of PDCCH monitoring reduction. For SSSG switching, it can be realized by the adaptation of PDCCH monitoring periodicity along with SSSG switching. One benefit of SSSG switching is that UE can perform PDCCH skipping multiple times without additional DCI indication, and can stop PDCCH skipping based on the timer or data arrival. It can provide power saving gain for a long term with small DCI overhead. On the other hand, one benefit of PDCCH skipping for a duration is that the skipping duration can be flexibly indicated along with DCI indication. In this case, gNB can flexibly determine the skipping duration based on scheduling condition and so on.

Considering the above benefits of the schemes, following options with the benefits can be considered for the common design for both SSSG switching and PDCCH skipping for a duration.

- Option 1: DCI indicating SSSG switching can indicate PDCCH monitoring periodicity for the target SSSG, i.e., PDCCH skipping duration.

- Option 2: DCI indicating PDCCH skipping for a duration can indicate the number of PDCCH skipping.

  - UE performs PDCCH monitoring between PDCCH skipping durations

Observation 1: The benefit of SSSG switching is that UE can perform PDCCH skipping multiple times without additional DCI indication, and can stop PDCCH skipping based on the timer or data arrival. It can provide power saving gain for a long term with small DCI overhead.

Observation 2: The benefit of PDCCH skipping for a duration is that the skipping duration can be flexibly indicated along with DCI indication. gNB can flexibly determine the skipping duration based on scheduling condition and so on.

Proposal 1: Following options should be considered for the common design for both SSSG switching and PDCCH skipping for a duration.

- Option 1: DCI indicating SSSG switching can indicate PDCCH monitoring periodicity for the target SSSG, i.e., PDCCH skipping duration.

- Option 2: DCI indicating PDCCH skipping for a duration can indicate the number of PDCCH skipping.

  - UE performs PDCCH monitoring between PDCCH skipping durations

Option 1 and 2 can provide the similar functionality in terms of PDCCH monitoring reduction. However, Option 1, i.e., SSSG switching based approach, can also provide the adaptation of PDCCH monitoring configuration other than PDCCH monitoring periodicity. Therefore, option 1 should be supported so that more power saving gain can be obtained.

Proposal 2: SSSG switching enhanced for the licensed bands should be supported.

- DCI indicating SSSG switching can indicate PDCCH monitoring periodicity for the target SSSG, i.e., PDCCH skipping duration.

  - Enhancements on search space set group switching

Although explicit indication only by DCI format 2_0 is possible in the Rel-16 search space set group switching, it does not make sense in licensed bands. At least regular scheduling DCI, i.e., DCI format 0_1 and 1_1 should be able to indicate search space set group switching for licensed bands.

Proposal 3: DCI format 0_1 and 1_1 can indicate SSSG switching.

  - PDCCH skipping along with cross-slot scheduling

In Rel-16, a minimum scheduling offset of K0/K2 was introduced for enhancement on cross-slot scheduling, and the minimum scheduling offset can be indicated by DCI from RRC configured candidate values. When the minimum scheduling offset is indicated by DCI, the actual scheduling offset K0/K2 equal to or larger than the corresponding minimum scheduling offset can be indicated by DCI from RRC configured candidate values. Cross-slot scheduling provides power saving gain by ensuring the sufficient time for PDCCH processing before starting the subsequent PDSCH reception. Because of this, after PDCCH reception UE can sleep without starting PDSCH reception/buffering. Also, in case of cross-slot scheduling, the UE power consumption can be reduced by relaxing the PDCCH processing based on the value of the minimum scheduling offset. However, the typical PDCCH monitoring periodicity seems every slot, and then the benefit of cross-slot scheduling would diminish since the UE needs to wake up to monitor next PDCCH occasion within the duration between PDCCH and PDSCH, and the parallel PDCCH processing may occur if the time for the relaxed PDCCH processing time is more than 1 slot. Basically, cross-slot scheduling would be applied on the condition that power saving is prioritized, e.g., low traffic amount for the UE. Also, even when there is burst traffic for the UE, the data scheduling for the UE might occur once in several slots depending on the number of UEs within the cell. In such cases, PDCCH monitoring in every slot might not be reasonable. Thus, PDCCH skipping along with cross-slot scheduling should be considered so that the benefit of cross-slot scheduling can be maximized.

Observation 3: PDCCH skipping along with cross-slot scheduling can maximize the benefit of cross-slot scheduling.

As mentioned above, assuming the UE monitors PDCCH in slot n, when the UE tries to monitor PDCCH between slot n and slot (n + the value of the applicable minimum scheduling offset), the UE sleep will be disturbed and the UE cannot relax PDCCH processing sufficiently. Thus, PDCCH skipping for the duration of the applicable minimum scheduling offset from PDCCH monitoring occasion should be supported.

Proposal 4: PDCCH skipping for the duration of the applicable minimum scheduling offset from PDCCH monitoring occasion should be supported.

Figure 1: PDCCH skipping along with cross-slot scheduling

- Conclusion

In this contribution, the extension to Rel-16 DCI-based power saving adaptation was discussed. Based on the discussion, the following observations and proposals were made:

Observation 1: The benefit of SSSG switching is that UE can perform PDCCH skipping multiple times without additional DCI indication, and can stop PDCCH skipping based on the timer or data arrival. It can provide power saving gain for a long term with small DCI overhead.

Observation 2: The benefit of PDCCH skipping for a duration is that the skipping duration can be flexibly indicated along with DCI indication. gNB can flexibly determine the skipping duration based on scheduling condition and so on.

Proposal 1: Following options should be considered for the common design for both SSSG switching and PDCCH skipping for a duration.

- Option 1: DCI indicating SSSG switching can indicate PDCCH monitoring periodicity for the target SSSG, i.e., PDCCH skipping duration.

- Option 2: DCI indicating PDCCH skipping for a duration can indicate the number of PDCCH skipping.

  - UE performs PDCCH monitoring between PDCCH skipping durations

Proposal 2: SSSG switching enhanced for the licensed bands should be supported.

- DCI indicating SSSG switching can indicate PDCCH monitoring periodicity for the target SSSG, i.e., PDCCH skipping duration.

Proposal 3: DCI format 0_1 and 1_1 can indicate SSSG switching.

Observation 3: PDCCH skipping along with cross-slot scheduling can maximize the benefit of cross-slot scheduling.

Proposal 4: PDCCH skipping for the duration of the applicable minimum scheduling offset from PDCCH monitoring occasion should be supported.

References

- 3GPP RAN1#104-e, Chairman’s note, February 2020.