3GPP TSG RAN WG1 #105-e	                      	  R1-2105845

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.11.1.1

Source: 	ASUSTeK

Title:  	Discussion on partial sensing and SL DRX impact

Document for:	Discussion and Decision

- Introduction

In previous RAN1 meetings [1][2], there are agreement about partial sensing, quoted as below:

Agreements:

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics

      - FFS whether/how to restrict the set of values

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

        - Alt 1: Option 1 as in RAN1#104-e

        - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

          - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

        - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

      - Note: companies are encouraged to provide more evaluations

Agreements:

- In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option:

  - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable).

    - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.)

    - FFS whether n can be replaced by e.g., index of some of Y candidate slots

  - FFS condition(s) in which contiguous partial sensing is performed by UE

  - FFS interaction with SL-DRX, if any

  - FFS interaction with periodic-based partial sensing, if any

In this contribution, we discuss (contiguous) partial sensing and physical layer impact induced by SL DRX.

- Discussion

In RAN1 #104-e meeting, it is agreed that for performing periodic-based partial sensing, UE will monitor slots of one periodic sensing occasions, which are according to configured set of possible resource reservation periods in the resource pool. There are options 1~2 on Preserve used for determining the periodic sensing occasion. To avoid possible resource collision, it is ideal that Preserve corresponds to all values from the configured reservation period set. Although NR V2X allows possible reservation periods as any of {1:99, 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000}ms, it seems unnecessary to configure diverse reservation periods in a resource pool. For instance, one resource pool may allow reservation period associated with at most two or three common divisor, e.g. {5, 10, 20},{16, 32, 64},{100, 200, 500}, in order to address on different QoS requirements. Reasonable reservation period configuration in a resource pool is compatible to option 1.

Furthermore, contiguous partial sensing is agreed in RAN1 #104-e meeting, such that UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB. In this case, sensing result of some smaller reservation periods can be acquired via contiguous partial sensing as well. Thus, the periodic sensing occasion for partial sensing can consider a subset of configured reservation periods, which is configured set of reservation periods excluding reservation periods covered by contiguous partial sensing. Accordingly, the set of Y candidate slots within a resource selection window can be determined based on the subset and with details up to UE implementation.

Proposal 1:  For Preserve used for determining the periodic sensing occasion, option 1 is adopted 
(all configured reservation periods).

Proposal 2:  If UE performs both periodic-based partial sensing and contiguous partial sensing, periodic sensing occasion can consider a subset of configured reservation periods, which excludes reservation periods covered by contiguous partial sensing.

The motivation of introducing contiguous partial sensing is for supporting aperiodic sidelink data in NR V2X. Since it is beneficial to acquire recent resource reservation information when TX UE selects resources for its aperiodic sidelink data. Moreover, it can also avoid possible collision with other UEs’ aperiodic sidelink data. From these aspects, contiguous partial sensing can be performed no matter TX UE selects resources for its aperiodic sidelink data or periodic sidelink data.

Considering one SCI can schedule sidelink resources up to 31 slots in advance, contiguous partial sensing region [n+TA, n+TB] may be set as 31 slot lengths in default, e.g. TB - TA = 31. Shorter region can be FFS for urgent sidelink data. Moreover, if UE has monitored SCI continuously before resource (re-)selection triggered in slot n, it is possible to set TA as negative value since the continuous sensing result before slot n is usable as well. The case may occur because of other early triggered resource (re-)selection or SL DRX on-duration time.

Proposal 3:  For contiguous partial sensing, TA can be set to negative value if UE has monitored SCI continuously before resource (re-)selection triggered in slot n.

As described in WID Justification, SL DRX is RAN 2 objective supported for power saving. Following the concept of Uu DRX, in case of enabling SL DRX, the UE may monitor PSCCH during SL DRX active time and not monitor PSCCH during SL non-active time. Because of the SL non-active time, SL DRX is able to reduce power consumption for receiver aspect of the UE. However, if any PSCCH/PSSCH for the UE occurs during the SL non-active time, the UE will loss such PSCCH/PSSCH. For unicast and groupcast, the paired UE will need to retransmit the PSCCH/PSSCH. It will induce impact on resource efficiency, reliability and latency degradation, and even power consumption of the paired UE. For broadcast, the UE may loss other vehicles’ message without chance to request retransmission.

Observation 1:  SL DRX may induce UE loss some PSCCH/PSSCH reception in all cast type.

Although SL DRX is handled by RAN2, there seems some physical layer impact. To avoid such PSCCH/PSSCH reception loss, the paired UE needs to know SL DRX active time of the UE. During resource selection procedure for transmitting PSCCH/PSSCH to the UE, the paired UE should ensure selected sidelink resources being within SL DRX active time of the UE. One alternative is that RAN1 excludes sidelink resources within SL non-active time, and another alternative is that RAN2 selects sidelink resources within SL DRX active time. In order to guarantee enough candidate resource set for selection, e.g. 20%, the former alternative is preferred from our aspect.

Moreover, from SL DRX aspect, the UE monitors PSCCH in SL DRX active time, while the UE monitors PSCCH in partial sensing duration from partial sensing aspect. If SL DRX and partial sensing are designed independently, the power saving gain may become limited. Thus, the interwork of SL DRX and partial sensing needs further study to have coordinated PSCCH monitor time.

Proposal 4:  SL DRX active time of intended RX UE is considered for mode 2 resource selection.

- Conclusion

In this contribution, we have following observation and proposals about SL DRX impact on physical layer:

Proposal 1:  For Preserve used for determining the periodic sensing occasion, option 1 is adopted 
(all configured reservation periods).

Proposal 2:  If UE performs both periodic-based partial sensing and contiguous partial sensing, periodic sensing occasion can consider a subset of configured reservation periods, which excludes reservation periods covered by contiguous partial sensing.

Proposal 3:  For contiguous partial sensing, TA can be set to negative value if UE has monitored SCI continuously before resource (re-)selection triggered in slot n.

Observation 1:  SL DRX may induce RX UE loss some PSCCH/PSSCH reception in all cast type.

Proposal 4:  SL DRX active time of intended RX UE is considered for mode 2 resource selection.

- Reference

- Report of 3GPP TSG RAN1#104-e meeting

- Report of 3GPP TSG RAN1#104bis-e meeting