3GPP TSG RAN WG1 #105-e	R1-2105904

e-Meeting, May 19th – May 27th, 2021

Agenda item:		8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	PUCCH coverage enhancements

Document for:		Discussion and Decision

# Introduction

The following can be noted from the work item description (WID) for Rel-17 coverage enhancement [1]:

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

As described in the WID, two main features are introduced for PUCCH coverage enhancements in Rel-17: the support of dynamic PUCCH repetition factor indication and the support of DM-RS bundling (or joint channel estimation) across PUCCH repetitions. This document discusses issues and potential solutions for these two features.

# Discussion

## Dynamic PUCCH repetition factor indication

In RAN1 #104-e, the following was agreed regarding the dynamic indication of the PUCCH repetition factor:

Agreements: Down select from the following two options to support dynamic PUCCH repetition factor indication. ​ - Option 1 (without DCI enhancement): Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. PUCCH repetition factor is implicitly indicated by DCI.​ - FFS details, e.g., via reusing the “PUCCH resource indicator” field (without increase # bits of it), starting CCE index (when applicable) of DCI, by PDCCH aggregation level, etc.​ - FFS: RRC signaling enhancement details​ - Option 2 (with DCI enhancement): PUCCH repetition factor is explicitly indicated by DCI​ - e.g., introduce a new field or increase the number of bits of an existing field (e.g., PRI) in DCI for PUCCH repetition factor indication​ - FFS whether there is a need for RRC update

In Rel-16 specifications and for PUCCH formats 1, 3, or 4, the number of slots for repetitions of a PUCCH transmission can be configured by gNB to UE by higher layer parameter nrofSlots. This means that after RRC configuration, a UE will always repeat the PUCCH transmission by the same number of slots as defined in nrofSlots, regardless of the actual channel conditions.

The current Rel-17 WI is however targeting enhancements to this type of behavior by introducing a dynamic indication of the PUCCH repetition factor. The way this would finally be indicated to the UE is still under discussion and two options remain open. Both options rely on enhancements of the RRC signaling for configuration of the PUCCH repetition factor per PUCCH resource. A new field in the configuration of PUCCH resource would need to be added to indicate the number of repetitions associated with that same PUCCH resource, at least for PUCCH resources supporting repetitions (format 1, 3 or 4). This also means that in order to have enough PUCCH resources to assign for different channel conditions with different number of repetitions, the size of the resource sets may need to be extended at least for a number of bits larger than 2 (pucch-ResourceSetId > 0). Indeed, the maximum number of PUCCH resources in the second, third and fourth PUCCH resource sets (pucch-ResourceSetId > 0) is currently limited to 8.

A relevant implication of increasing the size of the resource sets is that the indication of a specific PUCCH resource becomes more complicated. Indeed, the PUCCH resource indicator (PRI) field in the DCI is currently limited to three bits, and indication of PUCCH resources in a PUCCH resource set larger than 8 may require an explicit or implicit extension.

Option 1 follows the approach currently used for indication of PUCCH resources belonging to PUCCH resource set with ID 0 (which can be configured with up to 32 PUCCH resources) and relies on utilization of some parameters of the PDCCH carrying the scheduling DCI. Although this approach is already employed for the first resource set (pucch-ResourceSetId = 0), it is our understanding that it would limit the flexibility of the PDCCH parameters (CCE index, number of CCE) that carries the scheduling DCI, for DCI format 1_0 or DCI format 1_1.

Option 2 follows instead a more explicit approach for the indication of the PUCCH repetition factor and, as such, offers more flexibility since the PUCCH repetition factor is not bundled to the parameters of the PDCCH carrying the scheduling DCI​. This additional flexibility comes at the cost of larger number of bits in the DCI, but we do not expect it to impair the decoding performance of the DCI since the number of additional bits is small compared to the total size of the DCI.

In addition, for option 2, two main approaches could be followed for indication of the PUCCH repetition factor in the DCI:

- Alt-1: the current PRI field is extended to a number of bits larger than 3.

- Alt-2: a new field is introduced in the DCI and the dynamic repetition factor applies to the resource indicated by the PRI.

In our view, Alt-1 is the one providing the optimal trade-off between flexibility and overhead in the DCI. Indeed, addition of only one bit in the PRI enables the network to flexibly configure up to 16 PUCCH resources with different repetition factors, which may be tailored to cover a set of conditions the UE may be subject to. For example, the network could configure four groups composed of similar four PUCCH resources characterized by the same time-frequency allocation but different repetition factors (e.g. 2 repetitions for the first group, 4 repetitions for the second group, 8 repetitions for the third group, and 16 repetitions for the fourth group).

Proposal 1. Extend the current PRI field to a number of bits larger than 3.

## DMRS bundling across PUCCH repetitions

In RAN1 #104-e, the following agreements on DMRS bundling across PUCCH repetitions were made:

Agreements: Subject to the prerequisites of DMRS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DMRS bundling via RRC configuration. ​ ​ - FFS: the configuration is per UE or per PUCCH resource. ​ - FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling​ - FFS: necessity of additional signaling/configuration of DMRS bundling duration/window and associated size

Although it was agreed that DMRS bundling across PUCCH repetitions is supported via RRC configuration, many points were left open for further discussion.

The configuration is per UE or per PUCCH resource

Based on the discussion in RAN1 #104-e, two options are left open for configuration of DMRS bundling: per UE configuration or per PUCCH resource configuration.

The idea behind configuring DMRS bundling per PUCCH resource is for the network to be able to dynamically activate or deactivate the DMRS bundling feature by selecting a specific PUCCH resource through the PRI field in the scheduling DCI. This additional flexibility is however not entirely justified in our view since we see no disadvantage for a UE to have the DMRS bundling constantly activated each time PUCCH repetitions are scheduled. It could be argued that such bundling may only bring negligible gains in case of high SNR conditions. In this regard, it should be noted that the UE is not expected to be scheduled with PUCCH repetitions in such high SNR conditions, thus the issue would not exist.

For this reason, we believe that we can save the additional burden on the configuration of the single PUCCH resources and that configuration of DMRS bundling per UE is sufficient. Such configuration would finally be based on UE capability (Rel-17 UE).

Proposal 2. The configuration of DMRS bundling is per UE and subject to UE capability.

Whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DMRS bundling

The need for dynamic activation of DMRS bundling is still under discussion in RAN1, and it is strictly related to the discussion on dynamic indication of repetition factor and whether the DMRS bundling configuration should be per UE or per PUCCH resource.

As also described in the previous paragraph, we do not see the need for dynamic activation or deactivation of PUCCH repetitions with DMRS bundling. There is, in our view, no disadvantage in keeping DMRS bundling always activated and exploit it every time gNB selects a PUCCH resource with repetitions. In addition, and if necessary, gNB is always able to break the phase continuity condition and avoid excessive impact to UE throughput and performance. For example, if a UE has heavy DL/UL traffic, the network can always schedule the UE to receive/transmit in between PUCCH repetitions, implicitly deactivating the phase continuity (DMRS bundling) condition.

Proposal 3. Do not introduce additional dynamic signalling to enable/disable PUCCH repetitions with DMRS bundling.

Necessity of additional signalling/configuration of DMRS bundling duration/window and associated size

In RAN1 #104-bis-e, it was agreed to specify a time-domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements. However, it is still unclear whether the configuration of the time-domain window and duration is necessary.

Similar discussion is happening in the context of DMRS bundling across PUCCH repetitions, where introduction of the indication of a DMRS bundling window and its associated size is still under discussion. In our view, there is no benefit in configuration or signalling of a DMRS bundling window and the UE could directly assume that DMRS bundling occurs across all configured repetitions or at least across a number of repetitions defined by UE capability, if any. This is not a problem even for high speed UEs with DMRS bundling configured, since the gNB receiver can always adapt its channel estimator behaviour based on dynamic UE channel conditions (e.g., dynamic UE speed) as long as UE guarantees phase and power continuity across the indicated repetitions.

Proposal 4. No additional semi-static/dynamic signalling is introduced for configuring DMRS bundling window and associated size.

In the case of non-back-to-back PUCCH repetitions (including PUCCH repetitions in non-contiguous slots), there may be cases in which a PDCCH monitoring occasion, or a configured DL/UL channel, occurs in between the repetition time interval and for which a UE may not be able to keep phase continuity across all the scheduled repetitions.

An example of this situation is shown in Figure 1, where a DL reception occasion or UL transmission with different settings than scheduled PUCCH repetitions occurs in between the 2nd and 3rd PUCCH repetition. In this case, and if the network does not instruct the UE to operate otherwise, the UE will need to break phase continuity to receive (resp. transmit) the DL (resp. UL with different settings) channel, which implicitly defines two different DMRS bundling windows with generally different durations. We can conclude that also in this case, signalling or configuring the DMRS bundling window(s) and its associated size does not seem necessary.

Observation 1. Additional signalling or configuration of DMRS bundling window and associated size is not necessary also in the case of non-back-to-back PUCCH repetitions in which a PDCCH monitoring occasion, or a configured DL/UL channel, occurs within the repetition time interval.

Figure 1. Illustration of a DL reception occasion (or other UL transmission with different settings) scheduled in between PUCCH repetitions with DMRS bundling.

## Enhancements to inter-slot frequency hopping pattern for PUCCH repetitions with DM-RS bundling

Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling. ​ - FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16.​ - Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible

According to Rel-16 specification, if the UE is configured to perform frequency hopping for PUCCH transmissions across different slots:

- the UE performs frequency hopping per slot

- the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in slots with even number and starting from the second PRB, provided by secondHopPRB, in slots with odd number. The slot indicated to the UE for the first PUCCH transmission has number 0 and each subsequent slot until the UE transmits the PUCCH in NPUCCHrepeat slots is counted regardless of whether or not the UE transmits the PUCCH in the slot

- the UE does not expect to be configured to perform frequency hopping for a PUCCH transmission within a slot

Frequency hopping behavior needs to be modified in case DMRS bundling is configured. Indeed, based on the reply LS from RAN4 [2], UE is able to keep phase consistency only if the RB allocation in terms of length and frequency position is not changed, and intra-slot and inter-slot frequency hopping is not enabled within a repetition bundle.

One possible direction for this enhancement is to group PUCCH repetitions in two or more adjacent DMRS bundling windows and the hopping is performed across such windows, which results in “inter-window” frequency hopping as illustrated in Figure 2 for the case of two adjacent DMRS bundling windows. Also, in this case, the DMRS bundling window does not need to be explicitly configured or signaled to the UE but can be specified as half, or in general a fraction, of the number of repetitions.

Observation 2. The DMRS bundling window does not need to be explicitly configured or signaled in case of inter-window frequency hopping.

Figure 2. Illustration of inter-window frequency hopping.

However, in case a DL reception/monitoring or an UL transmission with different settings than the PUCCH repetitions occurs in between the repetitions, the phase continuity cannot be maintained throughout the whole duration of the DMRS bundling window. This is especially relevant for TDD deployments. For example, Figure 3 illustrates a similar scenario as in Figure 2, where a DL reception (e.g. configured PDSCH) in between repetition 2 (rep2) and repetition 3 (rep3) occurs. In this case, joint channel estimation can only be applied across rep1 and rep2, or rep3 and rep4 in the first frequency hop. The situation is even worse when the configured PDSCH comes in between rep3 and rep4, which leaves rep4 as an “orphan repetition” and joint channel estimation cannot be applied on it. Such a situation would degrade the performance of the inter-window frequency hopping method.

Figure 3. Limitation of inter-window frequency hopping when there is a downlink reception (or uplink transmission with different settings) in between two repetitions.

To compensate for such loss, one approach would be to let the UE switch frequency hop whenever it is expected to monitor a DL signal or transmit an UL signal with different settings than the scheduled PUCCH repetitions. In this case, the DMRS bundling window (of length 4 slots in the example) is not considered anymore by the UE for the definition of the hopping pattern, as in the case of inter-window frequency hopping. In other words, the DMRS bundling window could be seen as the maximum duration in time within which UE is expected to maintain phase continuity and power consistency and transmit on the same frequency hop, unless an implicit frequency hopping trigger is received during the window duration, in the form of DL reception occasion or UL transmission with different settings that the UE is expected to monitor or transmit. An example of this approach is shown in Figure 4. The UE jumps to the second frequency hop right after monitoring/receiving the DL transmission or upon transmitting an UL channel with different settings, which improves the reliability of channel estimation of the second hop.

Figure 4. An example of frequency hopping mode for joint channel estimation.

From the above discussion, we propose the following:

Proposal 5. For inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation:

- RAN1 to specify at least the following frequency hopping approach:

  - UE switches frequency hop for the repetitions after a DL reception occasion that the UE is expected/configured to monitor/receive or after an UL transmission with different settings (e.g., in antenna port, occupied PRBs and UL power) than the PUCCH repetitions.

# Conclusion

In this contribution, we discussed issues and potential solutions for dynamic PUCCH repetition factor indication and DM-RS bundling across PUCCH repetitions.

We made the following observations and proposals:

Observation 1. Additional signalling or configuration of DMRS bundling window and associated size is not necessary also in the case of non-back-to-back PUCCH repetitions in which a PDCCH monitoring occasion, or a configured DL/UL channel, occurs within the repetition time interval.

Observation 2. The DMRS bundling window does not need to be explicitly configured or signaled in case of inter-window frequency hopping.

Proposal 1. Extend the current PRI field to a number of bits larger than 3.

Proposal 2. The configuration of DMRS bundling is per UE and subject to UE capability.

Proposal 3. Do not introduce additional dynamic signalling to enable/disable PUCCH repetitions with DMRS bundling.

Proposal 4. No additional semi-static/dynamic signalling is introduced for configuring DMRS bundling window and associated size.

Proposal 5. For inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation:

- RAN1 to specify at least the following frequency hopping approach:

  - UE switches frequency hop for the repetitions after a DL reception occasion that the UE is expected/configured to monitor/receive or after an UL transmission with different settings (e.g., in antenna port, occupied PRBs and UL power) than the PUCCH repetitions.

# References

- RP-202928	New WID on NR coverage enhancements, China Telecom (moderator), December 2020.

- R4-2103393 Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), February 2021.