3GPP TSG-RAN WG1 Meeting #105-e	R1-2106115

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4

Source:	Ericsson

Title:	Discussion on how to reply to SA2 LS on new 5QI for NTN

Document for:	Discussion

# 1	Introduction

SA2 sent an LS (R1-2104155) to RAN1 about a new 5QI for GEO satellite access:

SA2 has discussed the topic of QoS for 5G satellite access and has agreed to introduce a new 5QI for best effort traffic with the intention to be able to accommodate the worst-case Packet Delay Budget for GEO.

The 5QI is available in the latest version of TS 23.501, v17.0.0, and copied below for your convenience:

| 10 | Non-GBR | 90 | 832ms (NOTE 13) (NOTE 17) | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- |

NOTE 13:	A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface

NOTE 17:	The worst case one way propagation delay for GEO satellite is expected to be ~270ms, ,~ 21 ms for LEO at 1200km, and 13 ms for LEO at 600km. The UL scheduling delay that needs to be added is also typically 1 RTD e.g. ~540ms for GEO, ~42ms for LEO at 1200km, and ~26 ms for LEO at 600km. Based on that, the 5G-AN Packet delay budget is not applicable for 5QIs that require 5G-AN PDB lower than the sum of these values when the specific types of satellite access are used (see TS 38.300 [27]). 5QI-<New Value> can accommodate the worst case PDB for GEO satellite type.

SA2 would like to verify with RAN1 and RAN2 whether the selected PDB value, resulting in a AN PDB of 812 ms, is reasonable for use with GEO satellite access.

Several companies provide discussion papers on how to reply to this SA2 LS.

- R1-2104726        Discussion on SA2 LS on new 5QI for NTN             Ericsson

- R1-2104774        Discussion on LS on PDB for new 5Q        OPPO

- R1-2105199        Discussion on LS on PDB for new 5QI       ZTE

- R1-2105930        Discusion on PDB for new 5QI     Huawei, HiSilicon

# Discussion

For the newly proposed 5QI, the following observations can be made:

- A PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload.

- 1.5 RTT can only cover one transmission with acknowledgement.

Delivering packets without retransmission(s) with a PER of 10-6 might appear challenging at a first glance. That said, according to TS 23.501, Section 5.7.3.4, for Non-GBR, the requirement is that 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB.

The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, for a Delay-critical GBR resource type, packets delayed more than the PDB are added to the PER and can be discarded or delivered depending on local decision.

In other words, the Non-GBR requirement is not stringent. One way to satisfy the new 5QI may go as follows.

- Configure 1% BLER at the PHY/MAC layer, then 99% of the packets would be correctly delivered in one shot, which is within the PDB of 832 ms.

  - For ease of discussion, a packet is assumed to be the same as a transport block.

- Meanwhile, these 99% of the packets are error free and thus meet the PER of 10-6.

- In conclusion, the 99% of the packets delivered error free within 832 ms meet the requirement that 98% of the packets should not exceed the PDB.

Based on the above discussion, an initial proposal is made as follows. Companies are encouraged to provide views on the proposal.

Initial proposal 1 (Moderator):

RAN1 to reply to SA2 that the selected PDB value, resulting in a AN PDB of 812 ms, is reasonable for use with GEO satellite access.

| Company | Comments |
| --- | --- |
| APT | Support. From RAN1 perspective, without (blind) retransmission, the initial transmisison can achieve 1% BLER by repetition and MCS configurations. If considering processing and scheduling delay, a AN PDB can be within 600ms as shown in R1-2101748. |
| ZTE | Support. Agree with moderator’s interpretation on the 5QI. As discussed in our contrbution, this value can be satisfied with proper setting on the scheduling. |
| Nokia, Nokia Shanghai Bell | Do not agree with moderator analysis. To obtain a PER of 10-6 delivered to higher layers we would need to accomodate either multiple HARQ retransmissions or allow for recovery on RLC layer. Further, for DL direction, it would not be sufficient to rely on HARQ-ACK information, as the NACK-to-ACK error rate would influence the minimum acheivable PER. Hence, in our understanding a much higher PDB threshold would be needed (in the order of 3.5 to 5.5 RTD). And in the moderator analysis the PER is targeted at 1%, which is contrary to the PER requirements from the 5QI, and would therefore not be fulfilling the requirements. |
| Nokia CATT | Not support. Regarding the moderator statement, - In conclusion, the 99% of the packets delivered error free within 832 ms meet the requirement that 98% of the packets should not exceed the PDB. If using this explanation, the 10^-6 PER target is equivalent as 10^-2 PER target, because at least 99% packet can be error free in the first transmission if set the BLER target 10^-2 in PHY. In the end, it will lead to the PER target is meaningless. |
| Ericsson | The key is how to understand the 10^-6 PER target: - Interpreation 1: only the 98% of the packets delivered with the PDB are considered for checking if the 10^-6 PER target is met or not. - Interpretation 2: all the packets (delivered within the PDB and exceeding the PDB) are considered for checking if the 10^-6 PER target is met or not. If Interpretation 1 is the right understanding, we agree with Moderator’s analysis. If Interpretation 2 is the right understanding, it is not reasonable to meet the 5QI with a PER target of 10-6 and a PDB of 812 ms, and we propose to suggest that SA2 consider increasing the PDB and/or increasing the PER target for the 5QI used in GEO satellite access with transparent payload. |
| Huawei, HiSilicon | Support. Our understanding is that even though the retransmission delay has not been considered for the PDB value of 832m, it may be okay due to the following reasons - There will be different ways to guarantee the performance target, e.g. disable HARQ and reply on repetitions. - According to the 5G QoS characteristic of PDB, the PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. Therefore, a retransmission exceeding the PDB can still help to reduce the PER. - For non-GBR traffic, the requirement is that 98 percent of the packets should not experience a delay exceeding the 5QI's PDB in uncongested scenarios. This means that in such scenarios 98% of the packet can be guaranteed within the PDB budget with 10^-6 PER. Regarding the LS, we feel that it may be good to also inform SA2 about the above RAN1 understanding so that a cross TSG check can be done. |
| CMCC | Support. Accrording to TS 23.501 Clause 5.7.3.4, For GBR QoS Flows using the Delay-critical resource type, a packet delayed more than PDB is counted as lost. The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. For a certain 5QI the value of the PDB is the same in UL and DL. In the case of 3GPP access, the PDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). For GBR QoS Flows using the Delay-critical resource type, a packet delayed more than PDB is counted as lost if the data burst is not exceeding the MDBV within the period of PDB and the QoS Flow is not exceeding the GFBR. For GBR QoS Flows with GBR resource type not exceeding GFBR, 98 percent of the packets shall not experience a delay exceeding the 5QI's PDB.. In our understanding, the new 5QI for GEO satellite access is not a Delay-critical GBR, thus, the PER target of 10^(-6) is not required to be fulfilled within PDB restriction. Nevertheless, the PER target of 10^(-6) may require multiple HARQ retransmission, which imposes a challenge to the application of HARQ-feedback disable technique. |
| LG | According to the TS 23.501 captured above, PER interpretation may depend on service/resource type/scenarios. Thus, if Non-GBR requirement is considered in GEO satellite access, we agree with initial proposal 1. |
| Xiaomi | We don’t agree. Based on the definition of PER and PDB in SA: The Packet Error Rate (PER) defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. The newly proposed 5QI gives both PER and PDB requirement, and thus both requirements need to be met. Echo Ericsson’s interpretation on the PER • Interpretation 1: only the 98% of the packets delivered with the PDB are considered for checking if the 10^-6 PER target is met or not. • Interpretation 2: all the packets (delivered within the PDB and exceeding the PDB) are considered for checking if the 10^-6 PER target is met or not. Our understanding is interpretation 2, all the packets needs to be considered, otherwise, the definition of PER is meaningless. Following the moderator’s analysis: - Configure 1% BLER at the PHY/MAC layer, then 99% of the packets would be correctly delivered in one shot, which is within the PDB of 832 ms. - For ease of discussion, a packet is assumed to be the same as a transport block. In this example, the PDB requirement is satisfied while the PER requirement is still not met. Thus, our suggestion is to inform SA2 to relax the PDR or PER requirement. |
| OPPO | Not support. TS 23.501, Section 5.7.3.4 Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER.. According to “an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER“, it seems an error packet within the AN PDB of 812 ms is not counted in the PER requirement as long as this packet is waiting for retransmission. So we agree that the PER requirement within the PDB for Non-GBR seems not stringent. But we don’t agree with “Configure 1% BLER at the PHY/MAC layer, then 99% of the packets would be correctly delivered in one shot, which is within the PDB of 832 ms“ for the following reasons: - Firstly, as discussed in our contribution, configuring 10% target BLER for initial transmission is normaly used in NR and should be the baseline for NR-NTN. If we assume other target BLER, e.g., 1% as baseline, or if we assume aggregation/repetition should always be configured in NR-NTN, we’d better have a common understanding on that first, which we think may have some spec efforts, e.g., introducing a new CQI table targeting for 1% BLER. - Secondly, from TS 23.501, although it says 98 percent of the Non-GBR packets should not experience a delay exceeding the 5QI's PDB in uncongested scenarios, it also says the Non-GBR packets should be prepared to experience congestion-related packet drops and delays. Which means in some cases, the percentage of packets fulfill the PER within the PDB is not required. Based on the above discussion, we don’t think special physical layer configuration or enhancements should not be assumed for achieving this PER requirement within the PDB. We prefer to have a clarification on the 5QI requirement for Non-GBR resource type or to achieve a common understanding in RAN1 before we reply to SA2 that the selected PDB value is reasonable. |
| Qualcomm | Suggest to reply with answers based on different interpretations of SA2 requirements: - If 98% packets are correctly deliverded within 832 ms, RAN1 belives the new 5QI - If the packets deleiverd with 832 ms are required to meet 1e-6 PER, RAN1 believes the new 5QI may not be reasonable and recommend SA2 to consider increasing the PER or PDB. We can further recommend values such as 1e-3 832 ms or 1e-6 2000 ms |

# Summary and further discussions over reflector

| Company | Comments |
| --- | --- |
| Moderator | Thanks very much for providing your inputs in the first round of discussion (up to v009). By reading the comments, I have a feeling that we would need to first align our understanding about the 5QI. For this, I would like to trigger direct email discussion to be more efficient towards the deadline 05/24. Meanwhile, I would also encourage you to check with your SA colleagues about the definition of the 5QI (I’ve had extensive discussions with my SA colleagues 😊). Below I try to describe my understanding, incorporating several elements of your inputs in the first round of discussion. - Event E1: packets delivered by more than 1 transmission with feedback (these packets would exceed PDB) - According to TS 23.501, the PDB for Non-GBR resource type denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. - In my view, not counting a delayed packet towards the PER means that it is not added to the PER for the sole reason of being “expired”. In other words, they will be counted towards PER, but they can accommodate multiple transmissions because they don’t need to meet the PDB bound. - So for these packets, they can be delivered with multiple transmissions to make the errors sufficiently small to meet 10^(-6) PER. - Event E2: packets delivered by 1 transmission with feedback - Event E2-1: TB delivered successfully and ACK successfully - In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. In congested scenarios, we don’t need to guarantee this because flows should be prepared to experience congestion-related packet drops and delays - Event E2-2: TB delivered successfully but ACK-TO-NAK - This would lead to retransmissions exceeding PDB, and thus packets will be counted as part of Event E1 above. - Event E2-3: TB delivered unsuccessfully and NAK successfully - This would lead to retransmissions exceeding PDB, and thus packets will be counted as part of Event E1 above. - Event E2-4: TB delivered unsuccessfully and NAK-TO-ACK - This can be caught by RLC AM, and would lead to retransmissions exceeding PDB, and thus packets will be counted as part of Event E1 above. In short, the new 5QI is really “soft”: - In uncongested scenario - Packets delivered successfully within PDB are correct packets. We need to guarantee there are 98% of such packets. - Packet delivered exceeding the PDB can be either correct or incorrect. But it’s necessary to make the percentage of incorrect packets here small enough to reach 10^(-6) PER, which can be achieved with several retransmissions. - In congested scenario - The requirement is even looser, because the flows should be prepared to experience congestion-related packet drops and delays. TB delivered successfully with 1 transmission with feedback; ACK successfully for the 1 transmission; Percentage of TBs delivered within PDB; Percentage of TBs delivered exceeding PDB; Comments. BLER = 1%; 99%; 99%; 99% * 99% = 98.01% delivered successfully; 1.99%. They require several retransmissions to reach 10^(-6) PER; 1% BLER is not a baseline BLER setting in NR, but can be achieved by implementation. BLER = 10%; 90%; 99%; 90% * 99% = 89.01% delivered successfully; 11.99%. They require several retransmissions to reach 10^(-6) PER; 10% BLER is a baseline BLER setting in NR, which is not sufficient to meet the new 5QI. |
| OPPO | Thank you very much for the nice summary. We would like to share our understandings on the 5QI requirements: - In congested scenario - The flows should be prepared to experience congestion-related packet drops and delays. The requirement is very loose since the packet can be dropped. - In uncongested scenario - 98% of the packets should be delivered and should reach the 10^(-6) PER within the PDB. - Although we didn’t find explicit description that the incorrect packets exceeding the PDB (e.g., 2% of the packets) should also reach the 10^(-6) PER by retransmissions, we tend to agree with moderator’s view considering that the spec says the PDB denotes a "soft upper bound" while the PER defines an upper bound. Based on that, we think the main discussion point would be whether it is reasonable to require 98% of the packets be delivered to reach a 10^(-6) PER with only one-time transmission in uncongested scenario for NTN GEO. Currently there are two suggestions from physical layer point of view: - Suggestion 1: Keep the new 5QI requirement proposed by SA2. The gNB can properly set the scheduling restrictions, e.g., by choosing lower MCS, setting lower target BLER, configuring slot aggregation, etc. to meet this requirement. - Suggestion 2: Relax the new 5QI requirement proposed by SA2, e.g., increasing the PDB and/or increasing the PER target. Our preference is suggestion 2, as we feel the scheduling restrictions should not be a mandatory implementation at gNB side. Besides, even though gNB may have some operations to enhance the reliability of transmissions, this requirement is still challenge since only initial transmission is considered. But if all other companies agree with suggestion 1, we can live with suggestion 1 also. |
| Nokia | Thanks for the discussion so far.Our understanding is that Xingqin is using DL direction in the below example – is that the correct understanding? If this is the case, we have the following considerations: It is true that under ideal conditions it may be possible to fulfill the 5QI as outlined below. However, - It is assumed that all PDCCH will be decoded correctly by the UE, where we would normally expect some error rate for the control channel. - It is assumed that gNB has an understanding of the accurate CQI in order to schedule the UE with such a strict target BLER of 1%. - In order for the gNB to have accurate CQI, we would rely on regular CQI reports à UE is being drained from battery on a constant rate. - It is assumed that UE transmit power is under perfect control since we can reach 1% error rate on the ACK transmission - It is assumed that the UE transmitter is not under transmit power limitation/saturation such that no power is lost. And in reality at least one of the assumptions will not be met, and then the 5QI will not be met, soft requirements or not. Further, even that these “soft upper bounds” may not consider packets exceeding the PDB as “lost”, we still have a strong concern that the “delay tail” that we need for delivering a PER may become extremely long. We are aware that this delay for reaching the extremely low PER target will exist anyway, but we would be more leaning towards having higher PDB setting in the first place as outlined in our first response. On top of this, it would also be better if the target PER would be less strict as well, such that it is possible to deliver according to requirements in the first place. One possible way forward could be for RAN1 to create the LS response such that it outlines two scenarios; - the idealistic scenario as outlined, and - the more realistic scenario which will be achievable under normal operating conditions (where the PDB is higher, and where the PER is potentially also higher). Then we would leave the decision of how to set/define/interpret the actual values (ideal or realistic) to SA2. |
| Moderator | Thanks Zuomin and Frank for the elaborate emails. I think these discussions much help the group align mutual understanding. I also feel now we’re in a better position to draft a reply LS to SA2. Here is a first attempt on the draft text for the reply LS to SA2, based on my assessment of the discussion status. Your further comments over reflector are most welcome. --------- Draft text of reply LS to SA2 --------- RAN1 would like to thank SA2 for sending their LS on PDB for the new 5QI. RAN1 notices that the new 5QI has a PDB of 812 ms and a PER of 10-6 for QoS flow of type Non-GBR. A PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload. RAN1 understands that services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, there are different views in RAN1 whether the new 5QI is reasonable. Some companies hold the view that it is possible to satisfy the new 5QI with proper network configuration and implementation. Some companies hold the view that the new 5QI can only be satisfied in ideal conditions and thus recommend that SA2 consider higher PDB and/or higher PER for this new 5QI. --------- End of draft text of reply LS to SA2 --------- |
| Nokia | Thanks a lot for drafting the text. According to my understanding the third paragraph is still too optimistic as it only points to uncongested scenarios. My suggested updates to this paragraph are as follows: RAN1 understands that services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios (that is, single-UE per slot and line-of-sight conditions), 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. It should be noted that operating under these conditions will cause severe drop in network spectral efficiency due to conservative link adaptation. Further, it should be noted that in order to reach PER of 10-6 for these conditions, significantly longer delay for retransmissions is needed. The PDB for Non-GBR denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. |
| Moderator | Thanks Frank for the comments and proposed edits. Note that the 3rd paragraph is almost a copy and paste from TS 23.501 to indicate RAN1’s awareness. So I would prefer to leave 3rd paragraph intact. Instead, I can incorporate your edits in the 4th paragraph, which reflects companies’ views. Services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, for a Delay-critical GBR resource type, packets delayed more than the PDB are added to the PER and can be discarded or delivered depending on local decision. --------- Draft text of reply LS to SA2 --------- RAN1 would like to thank SA2 for sending their LS on PDB for the new 5QI. RAN1 notices that the new 5QI has a PDB of 812 ms and a PER of 10-6 for QoS flow of type Non-GBR. A PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload. RAN1 understands that services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, there are different views in RAN1 whether the new 5QI is reasonable. Some companies hold the view that it is possible to satisfy the new 5QI with proper network configuration and implementation. Some companies hold the view that satisfying the new 5QI may cause severe drop in network spectral efficiency due to conservative link adaptation and significantly longer delay for retransmissions to reach PER of 10-6, and thus the new 5QI can only be satisfied in ideal conditions and recommend that SA2 consider higher PDB and/or higher PER for this new 5QI. --------- End of draft text of reply LS to SA2 --------- |
| LG | Thanks Xingqin for moderating LS and discussion. For reflecting Nokia’s comment, we prefer updated version by Xingqin. We have one minor comment on this reply LS. Actually, SA2 provides PDB of 832ms that is AN PDB of 812ms + CN PDB of 20ms. So, we would like to clarify and our suggestion on the second bullet is RAN1 notices that the new 5QI has a AN PDB of 812 ms and a PER of 10-6 for QoS flow of type Non-GBR. A AN PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload. |
| Moderator | Thanks Haewook for the careful check. Your proposed edit is incorporated below: --------- Draft text of reply LS to SA2 --------- RAN1 would like to thank SA2 for sending their LS on PDB for the new 5QI. RAN1 notices that the new 5QI has a AN PDB of 812 ms and a PER of 10-6 for QoS flow of type Non-GBR. A AN PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload. RAN1 understands that services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, there are different views in RAN1 whether the new 5QI is reasonable. Some companies hold the view that it is possible to satisfy the new 5QI with proper network configuration and implementation. Some companies hold the view that satisfying the new 5QI may cause severe drop in network spectral efficiency due to conservative link adaptation and significantly longer delay for retransmissions to reach PER of 10-6, and thus the new 5QI can only be satisfied in ideal conditions and recommend that SA2 consider higher PDB and/or higher PER for this new 5QI. --------- End of draft text of reply LS to SA2 --------- |
| Nokia | Thanks a lot for the efforts to capture our concerns. In the red addition below, I think it would reflect reality better if we use the term “will” instead of “may”. If we use super-conservative link adaptation, the spectral efficiency will drop – simple as that. If we do not capture it as “will cause severe drop”, the associated uncertainty of “may cause severe drop” can cause some mis-reading of the statement (since “may” can be read as “may or may not”). |
| Moderator | Thanks Frank for the further comment, which I adopted below. --------- Draft text of reply LS to SA2 --------- RAN1 would like to thank SA2 for sending their LS on PDB for the new 5QI. RAN1 notices that the new 5QI has a AN PDB of 812 ms and a PER of 10-6 for QoS flow of type Non-GBR. A AN PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload. RAN1 understands that services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, there are different views in RAN1 whether the new 5QI is reasonable. Some companies hold the view that it is possible to satisfy the new 5QI with proper network configuration and implementation. Some companies hold the view that satisfying the new 5QI will cause severe drop in network spectral efficiency due to conservative link adaptation and significantly longer delay for retransmissions to reach PER of 10-6, and thus the new 5QI can only be satisfied in ideal conditions and recommend that SA2 consider higher PDB and/or higher PER for this new 5QI. --------- End of draft text of reply LS to SA2 --------- |
| Qualcomm | Thank you for the draft LS. We have talked with our SA2 colleagues and it appears that the previous input from RAN1 may have been misunderstood. Basically, if RAN1 says RTT is X and PER is Y then SA2 assumes that Y can be met with a delay X, even though the RAN1 thinking may have been that Y can be met with a delay that is perhaps n , (n>1) times RTT. We would like to encourage companies to consider clarifying the above to SA2. This may be even more important than the particulars of the current values for NTN. The clarification could be made by indicating there is a RAN1 consensus that the values should be reconsidered and a certain multiple of RTT should be considered for the delay for the nominal cases. |
| Moderator | Thanks Peter for the comment. I’m fine with such a clarification and added a sentence in the end. --------- Draft text of reply LS to SA2 --------- RAN1 would like to thank SA2 for sending their LS on PDB for the new 5QI. RAN1 notices that the new 5QI has a AN PDB of 812 ms and a PER of 10-6 for QoS flow of type Non-GBR. A AN PDB of 812 ms is about 1.5 RTT of the maximum round trip delay in GEO satellite access with transparent payload. RAN1 understands that services using Non-GBR QoS Flows should be prepared to experience congestion-related packet drops and delays. In uncongested scenarios, 98 percent of the packets should not experience a delay exceeding the 5QI's PDB. The PDB for Non-GBR denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded and is not added to the PER. However, there are different views in RAN1 whether the new 5QI is reasonable. Some companies hold the view that it is possible to satisfy the new 5QI with proper network configuration and implementation. Some companies hold the view that satisfying the new 5QI will cause severe drop in network spectral efficiency due to conservative link adaptation and significantly longer delay for retransmissions to reach PER of 10-6, and thus the new 5QI can only be satisfied in ideal conditions and recommend that SA2 consider higher PDB and/or higher PER for this new 5QI. In general, RAN1 holds the view that, if RTT is X and PER is Y, where Y is as small as 10-6, Y can be met with a delay that is multiple times of the RTT. --------- End of draft text of reply LS to SA2 --------- |

# Outcome

The draft reply LS (R1-2106174) was endorsed.

The tdoc number of the final reply LS is R1-2106331.