3GPP TSG-RAN WG1 Meeting #105-e 	R1-2106130

# e-Meeting, May 10th – May 27th, 2021

Title:	Draft LS on the physical layer aspects for small data transmission

Response to:

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Moderator (ZTE) [RAN1]

To:	RAN2

Contact Person:

Name:	Li Tian

E-mail Address:	tian.li150@zte.com.cn

Attachments:	None

1. Overall Description:

TA validation

RAN1 has further discussed how to define the SSB subset for RSRP based TA validation. It is concluded that the SSB subset is determined at least based on a configured absolute RSRP threshold. However, there remains some FFS issues for details as below.

- The SSB subset for RSRP based TA validation is determined at least based on a configured absolute RSRP threshold. - FFS the SSB subset which could be - within a set of SSBs configured per CG configuration - or within a set of SSBs configured for all CG configurations - or within a set of all SSBs actually transmitted as indicated in SIB1 - or highest N SSBs that are measured to derive the subset for a UE across all CG configurations

Association between SSBs and CG resources for CG-SDT

RAN1 has further discussed the association between SSBs and CG resources for CG-SDT, and the following agreements have been achieved. RAN1 will continue working on the details of the association.

In RAN1#104b-e - It is RAN1’s common understanding that the CG configuration mechanism in licensed band can be reused for CG-SDT in principle. - CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling. - FFS how to define an SSB-to-PUSCH resource mapping within the CG configuration. - FFS specific changes to the CG configuration to support the additional SSB-to-PUSCH mapping, if any. In RAN1#105-e - The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined. - The ordering of the SSB and CG PUSCH resources are to be captured in RAN1 spec. - A PUSCH resource refers to a transmission occasion and a DMRS resource used for PUSCH transmission - The ordering of the SSB can reuse from the SSB-to-RO mapping - The ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible - FFS determination of mapping ratio and association period, e.g., explicitly signaled or implicitly derived - FFS any limitation on the combination of the parameters for CG resources

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully requests that RAN2 takes the above into account.

4. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #106-e 	16 Aug. – 27 Aug. 2021	Electronic Meeting

TSG-RAN WG1 Meeting #106-bis-e 	11 Oct. – 19 Oct.  2021		Electronic Meeting