3GPP TSG RAN WG1 #105-e	                                                                 R1- 210xxxx

e-Meeting, May 10th – 27th, 2021

Agenda Item:		7.1

Source:		Moderator (Samsung)

Title:		Summary on editorial spec changes of [105-e-NR-7.1CRs-14]

Document for:		Discussion and Decision

Introduction

Base on Mr. Chairman’s guidance, the corresponding editorial CRs are handled under in this email thread.

Issue#1, Issue#4, Issue#8, Issue#13, Issue#21, Issue#27 R1-2104317 Correction on multi-slot PUSCH transmission ZTE R1-2104810 Draft CR on PUCCH repetition OPPO R1-2105284 Correction on power control for TS 38.213 Samsung R1-2105455 Draft 38.214 CR on reference correction for PDSCH EPRE to CSIRS EPRE assumption vivo R1-2105926 Correction on the MR-DC Uplink Power Control in 38.213 Huawei, HiSilicon R1-2104645 Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different DL and UL SCSs Qualcomm Incorporated [105-e-NR-7.1CRs-14] Discussion on editorial spec changes for recommendation to the editors (Issues #1, #4, #8, #13, #21, #27) – Sungjin (Samsung) by May 21

This document is to collect companies’ inputs and draw potential TP(s) as recommendations for the editors (no CRs) for the issue being agreeable in RAN1 group (including potential Rel-16 shadow CR/TP for some issues).

Discussion

Issue#1 (Rel-15): R1-2104317, Correction on multi-slot PUSCH transmission, ZTE [1]

Background & Proposed TP for TS 38.214

| Reason for change: | For multi-slot PUSCH transmission, one PUSCH transmission in a slot should be omitted in the following cases. - The PUSCH overlaps in one or more slots with PUCCH repetition. This is specified in Clause 9.2.6 in TS38.213. - The PUSCH collides with symbols that are not valid for PUSCH transmission as specified in Clause 11.1 in TS38.213. However, only the conditions in Clause 11.1 of TS38.213 is referred for multi-slot PUSCH transmission in TS38.214. |
| --- | --- |
| Summary of change: | Add Clause 9.2.6 for the conditions of omitting one PUSCH in a slot of a multi-slot PUSCH transmission. |
| Consequences if not approved: | Unclear UE behavior for multi-slot PUSCH transmission. |

TP 1: {38.214: 6.1.2.1 Resource allocation in time domain} for Rel-15

6.1.2.1 Resource allocation in time domain When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission. When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table which is defined by the higher layer configured pusch-TimeDomainAllocationList in pusch-Config. The indexed row defines the start and length indicator SLIV, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where are the corresponding list entries of the higher layer parameter reportSlotOffsetList in CSI-ReportConfig for the triggered CSI Reporting Settings and is the (m+1)th entry of . - The slot where the UE shall transmit the PUSCH is determined by K2 as where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and and are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and - The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row: if then else where, and - The PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row. The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations Table 6.1.2.1-1: Valid S and L combinations PUSCH mapping type; Normal cyclic prefix; Extended cyclic prefix. S; L; S+L; S; L; S+L. Type A; 0; {4,…,14}; {4,…,14}; 0; {4,…,12}; {4,…,12}. Type B; {0,…,13}; {1,…,14}; {1,…,14}; {0,…, 11}; {1,…,12}; {1,…,12}. When transmitting PUSCH scheduled by DCI format 0_1 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, if the UE is configured with pusch-AggregationFactor, the same symbol allocation is applied across the pusch-AggregationFactor consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the pusch-AggregationFactor consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pusch-AggregationFactor -1, is determined according to table 6.1.2.1-2. Table 6.1.2.1-2: Redundancy version when pusch-AggregationFactor is present rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion. n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3. A PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9.2.6 and Clause 11.1 of [6, TS38.213].

Companies’ input

Please kindly provide your views about the proposed TP of Issue#1 in the table below.

Question 1-1: Do you agree the proposed TP of Issue#1?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - The text in 38.214 is about when a slot in a multi-slot PUSCH is sent and when it is dropped. The clause 9.2.6 is about multi-slot PUCCH transmission and it does not discuss dropping of a slot or slots out of multi-slot PUSCH. The reference doesn’t seem refer to any functionality in 9.2.6 that would be relevant. |
| - QC | - We are fine with adopting the CR. A minor comment is the row of affected clause in the CR cover page is filled with an incorrect clause number. |
| - vivo | - Although we understand the intention of the proposed TP, it seems the current spec is clear according to the following text in 9.2.6 of 38.213. So we don’t think the TP is necessary. If a UE would transmit a PUCCH over a first number of slots and the UE would transmit a PUSCH with repetition Type A over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in Clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots. |
| - Samsung | - Agree with CR for alignment between 213 and 214. |
| - Huawei, HiSilicon | - No. There is nothing wrong with current specification. The UE behaviors with respective to multi-slot PUSCH transmissions are clearly described different sections without any inconsistency. |
| - CATT | - We share the same view with vivo and Huawei. We do not see the need of the CR. |
| - ZTE | - Support the CR. - @Nokia, In Rel-15 specification, Clause 9.2.6 in TS38.213 clearly states the PUSCH is a multi-slot PUSCH, and the PUSCH in the overlapping slots should be dropped. For Rel-16 specification, PUSCH repetition type A or type B is explicitly used as quoted by vivo. If a UE would transmit a PUCCH over a first number of slots and the UE would transmit a PUSCH over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in Clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots. - @ vivo, CATT and Huawei, The issue is TS 38214 only specifies part of the conditions and it causes inconsistency among specifications, which could cause potential ambiguity. As this is for editor CR, it should be ok to make the spec clearer. |
| - Ericsson | - Not needed. Our understanding of the statement in the CR is to address that PUSCH repetition is intended for consecutive slots where DL slots for example are excluded, hence reference to 11.1 is used. - The cancelation due to overlapping with PUCCH repetition in 9.2.6 is a different issue and it is applied on top, for example for the PUSCHs that are repeated (2 PUSCHs in the above example)." |
| - ZTE | - @ Ericsson Our understanding is the two conditions in Clause 11.1 and 9.2.6 are independent. For example, the conditions in Clause 11.1 doesn’t need to apply in FDD case while we need to consider the conditions in Clause 9.2.6. So, it is necessary to capture all conditions otherwise confusion may be caused. As this is for editor CR, it should be ok to make the spec clearer. |

Question 1-2: Do you think that proposed TP of Issue#1 can be Rel-16 shadow TP?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - A Rel-16 (Cat A) shadow CR is needed if we agree on a Rel-15 (Cat F) correction CR. If we don’t agree to a Rel-15 correction then we can discuss if we introduce the change as a Rel-16 correction only. If there is a case for adding the reference to 9.2.6 (which seems unclear), then it would be better to introduce it to Rel-15 with a corresponding Rel-16 (Cat A) shadow so that it doesn’t look like this is due to some Rel-16 feature that was added. |
| - QC | - We are OK with adopting the Rel-16 shadow TP |
| - Vivo | - Not necessary if not agreed for Rel-15. |
| - Samsung | - Not necessary regardless of whether this TP is agreed for Rel-15 or not. This is because 214 already include Clause 9 as follows. - For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213]. |
| - Huawei, HiSilicon | - No. Agree with Samsung. |
| - CATT | - No. Agree with Samsung. |
| - ZTE | - Support to adopt the Rel-16 shadow TP. - @Samsung, Huawei and CATT, It should be a normal practice that the Clause to be referred to should be precise enough. Clause 9 is only for the beginning of this Clause which intends for PUSCH dropping due to collision with channels of different priorities (e.g., high priority PUCCH) introduced in Rel-16. Similar to other conditions Clause 11.1 and Clause 11.2A (instead of using Clause 11), a more precise Clause is necessary to avoid any potential ambiguity. |

Summary

Based on the discussion so far, it can be summarized (only considering original proposal without considering Rel-16 shadow TP, CAT#A) for Issue#1 as follows.

| Support | Qualcomm, Samsung, ZTE |
| --- | --- |
| Not support | Nokia, vivo, Huawei/HiSilicon, CATT, Ericsson |

Issue#4 (Rel-15): R1-2104810, Draft CR on PUCCH repetition, OPPO [2]

Background & Proposed TP for TS 38.213

| Reason for change: | This draft CR is submitted for discussion per decision in RAN1 #104b-e. In the current 38.213, a UE operating in unpaired spectrum determines the resources for PUCCH repetition based on “UL symbol, as described in Clause 11.1” and “flexible symbols”. Although “UL symbol, as described in Clause 11.1” refers to the symbol that is semi-statically configured as UL, “flexible symbol” can ambiguously mean either the symbol that is semi-statically configured as flexible or the symbol left as flexible after the dynamic SFI indication is taken into account. This CR intends to solve this ambiguity by clarifying that the “flexible symbol” refers to symbol that is semi-statically configured as flexible, due to following reasons: 1). If “flexible symbol” in the concerned spec text is interpreted as the one based on dynamic SFI, the spec text may unreasonably exclude the symbol that is semi-statically configured as flexible and later dynamically assigned to UL symbol by SFI. 2). There was a RAN1 AH_1801 agreement as following. Agreement: - The ‘unknown’ symbols in semi-static DL/UL assignment can be used for long PUCCH transmission over multiple slots when a UE receives a grant to transmit the long PUCCH |
| --- | --- |
| - |  |
| Summary of change: | Clarify the “flexible symbol” in PUCCH resource determination for PUCCH repetition is based on semi-static configuration. |
| Consequences if not approved: | There is an ambiguity in specification as well as a potential mismatch between UE implementation and gNB implementation. |

TP 2: {38.213: 9.2.6	PUCCH repetition procedure} for Rel-15

<Unchanged part omitted> 9.2.6 PUCCH repetition procedure <Unchanged part omitted> For unpaired spectrum, the UE determines the slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 and having - an UL symbol, as described in Clause 11.1, or flexible symbol, as described in Clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols,as described in Clause 11.1, or flexible symbols, as described in Clause 11.1, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 <Unchanged part omitted>

Companies’ input

Please kindly provide your views about the proposed TP of Issue#4 in the table below.

Question 2-1: Do you agree the proposed TP of Issue#4?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - It is true that the “or flexible symbol” should be before the reference to 11.1, but adding the reference again after the “flexible symbol” is not resolving any ambiguity that might arise. Even though this is technically correct it makes the sentence heavier and not any clearer. We do not support this change even though do agree that it is technically correct. |
| - QC | - We are fine with adopting the CR |
| - Vivo | - Fine with the update. |
| - Samsung | - Fine with the update. For addressing concern from Nokia, can we try to just move “as described in Clause 11.1” after “or flexible symbol” as follow? - an UL symbol, as described in Clause 11.1, or flexible symbol, as described in Clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols,as described in Clause 11.1, or flexible symbols, as described in Clause 11.1, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 |
| - Huawei, HiSilicon | - We agree there may be some issues with the current specification but the proposed change does not really address the problem. The reason is that the semi-statically configured flexible symbols may turn out to be unsalable for PUCCH. As an example, for high-layer configured PUCCH transmissions, if a UE is configured to monitor DCI format 2_0 and the flexible symbols is not indicated as UL, PUCCH cannot be transmitted. In general, PUCCH can be transmitted in semi-static configured UL symbols, flexible symbols that are indicated as UL by DCI format 2_0, flexible symbols that cannot be cancelled by DL DCI or DCI format 2_0 due to timeline constraints. Overall, our current preference is to have a more general description and refer to Section 11. - an UL symbol, as described in Clause 11.1, or flexible symbol, as described in Clause 11, that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols,as described in Clause 11.1, or flexible symbols, as described in Clause 11, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 |
| - OPPO | - Fine with both the proposal TP and Samsung’s version |
| - CATT | - We agree with the intention of the CR but we think that “as described in Clause 11.1” is not clear enough. - We would prefer to reword as follows. For unpaired spectrum, the UE determines the slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 and having - an UL symbol, as described in Clause 11.1, or flexible symbol, indicated by tdd-UL-DL-ConfigurationCommon or by TDD-UL-DL-ConfigurationDedicated, that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols, as described in Clause 11.1, or flexible symbols, indicated by tdd-UL-DL-ConfigurationCommon or by TDD-UL-DL-ConfigurationDedicated, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 |
| - ZTE | - Fine with the updated from Samsung |
| - Ericsson | - We have a hard time seeing that there would be an ambiguity here. A smart reader would understand where to look for definition of flexible symbols. |
| - OPPO | - Given all feedback from the companies, it is our understanding that the suggestion from CATT addresses the concerns from Samsung, Huawei and Nokia (since now “UL symbol” and “flexible symbol” do not have duplicated reference clause so the new text is not “heavy”); it should be also ok to all other companies that are fine with the original proposal. - @Ericsson: the current spec text has a fairly 50-50 chance on two different interpretations regarding to whether the “flexible symbol” should be the one semi-statically configured or the one dynamically indicated, and the spec text should make it clear instead of relying on how the reader decides. - We make some minor modifications to CATT’s editing as below: - -------- For unpaired spectrum, the UE determines the slots for a PUCCH transmission starting from a slot indicated to the UE as described in Clause 9.2.3 and having - an UL symbol, as described in Clause 11.1, or flexible symbol, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, that is not SS/PBCH block symbol provided by startingSymbolIndex in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 as a first symbol, and - consecutive UL symbols, as described in Clause 11.1, or flexible symbols, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols in PUCCH-format1, or in PUCCH-format3, or in PUCCH-format4 |

Question 2-2: Do you think that proposed TP of Issue#4 can be Rel-16 shadow TP?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - If introduced, we could make it a Rel-16 correction (Cat F) rather than a Rel-15 correction with a Rel-16 shadow (Cat A). |
| - QC | - OK with a Rel-16 shadow TP |
| - Vivo | - Ok to introduce the shadow CR. |
| - Samsung | - Fine with a Rel-16 shadow TP. |
| - Huawei, HiSilicon | - This can discussed after the TP for Rel-15 can be agreed. |
| - OPPO | - Yes |
| - CATT | - Fine to have a Rel-16 shadow CR. |
| - ZTE | - OK for Rel-16 shadow TP. |
| - Ericsson | - Having a shadow CR for a later release for a change in an earlier release is 3GPP procedure so we do not understand why this is up for discussion expect for the cases where the change is not relevant for the later release. |

Summary

Based on the discussion so far, it can be summarized (only considering original proposal without considering Rel-16 shadow TP, CAT#A) for Issue#4 as follows.

| Support | Qualcomm, vivo, Samsung, OPPO |
| --- | --- |
| Support (Huawei’s TP) | Huawei |
| Support (Samsung’s TP) | Samsung, OPPO, ZTE |
| Support (CATT’s TP) | CATT |
| Not support | Nokia, Ericsson |

Issue#8 (Rel-15): R1-2105284, Correction on power control for TS 38.213, Samsung [3]

Background & Proposed TP for TS 38.213

| Reason for change: | Reference to the DL BWP for the pathloss calculation is inconsistent in Clauses 7.1.1, 7.2.1, and 7.3.1. While in Clauses 7.1.1 and 7.3.1, there is no reference to the DL BWP index, such reference is included in Clause 7.2.1. Even though due to the UL/DL BWP pairing the reference is not incorrect, the discrepancy can be confusing and the descriptions should be consistent. |
| --- | --- |
| Summary of change: | Remove the notation of ‘b’ for the DL BWP in Clause 7.2.1 as it is only used for the UL BWP throughout Clauses 7.1.1, 7.2.1, and 7.3.1. |
| Consequences if not approved: | Inconsistent descriptions for a same operation which may cause ambiguity. |

TP 3: {38.213: 7.2.1	UE behaviour} for Rel-15

< Unchanged part is omitted > 7.2.1 UE behaviour If a UE transmits a PUCCH on active UL BWP of carrier in the primary cell using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power in PUCCH transmission occasion as [dBm] where < Unchanged part is omitted > - is a downlink pathloss estimate in dB calculated by the UE using RS resource index as described in Clause 7.1.1 for the active DL BWP of carrier of the primary cell as described in Clause 12 - If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB < Unchanged part is omitted >

Companies’ input

Please kindly provide your views about the proposed TP of Issue#8 in the table below.

Question 3-1: Do you agree the proposed TP of Issue#8?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - OK to introduce this in editor’s alignment CR, but it is an unnecessary editorial correction years after freezing the spec and as such we should procedurally reject the CR. |
| - QC | - No. The spec is clear. No need to remove b. We don’t see how the spec could be interpret wrong. - We have two questions as below. Maybe proponents of the CR can provide answers to help us see the motivation of the CR. - Do you observe any issue/error in the field which is caused by unclarity of the spec due to this “b”? - If the answer is yes to above question, what are the multiple interpretations of the spec? |
| - Vivo | - Even without this TP, it is clearly that the notation of ‘b’ is for the UL BWP for PUCCH transmission, and ‘qd’ is reference to the DL BWP. - If majority companies are fine with this CR, we suggest to change the consequence part as below: - ‘Inconsistent descriptions for pathloss calculation for PUCCH and PUSCH/SRS’ - or ‘Redundant descriptions for pathloss calculation for PUCCH’. |
| - Samsung | - Fine with the CR. On the QC’s comments, the motivation of this CR is to remove potential ambiguity in future although we haven’t identified the issue so far. |
| - Huawei, HiSilicon | - No. The current specification works fine even without the change. |
| - ZTE | - We slightly prefer to support this CR for making spec more readable. |
| - Ericsson | - The specification is clear. Any future ambiguity could be handled at that point. |

Question 3-2: Do you think that proposed TP of Issue#8 can be Rel-16 shadow TP?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - Prefer this as a Rel-16 Cat F rather than Rel-15 Cat F + Rel-16 Cat A. |
| - QC | - No. Spec is clear. No need to adopt the CR. |
| - vivo | - Not necessary if alignment in Rel-15 is not agreed. |
| - Samsung | - Agree if Rel-15 CR is agreed. |
| - Huawei, HiSilicon | - No. |
| - ZTE | - Support. |

Summary

Based on the discussion so far, it can be summarized (only considering original proposal without considering Rel-16 shadow TP, CAT#A) for Issue#8 as follows.

| Support | Samsung, ZTE |
| --- | --- |
| Not support | Nokia, Qualcomm, vivo, Huawei, Ericsson |

Issue#13 (Rel-15): R1-2105455, Draft 38.214 CR on reference correction for PDSCH EPRE to CSIRS EPRE assumption, vivo [4]

Background & Proposed TP for TS 38.214

| Reason for change: | The UE shall assume the ratio of PDSCH EPRE to CSI-RS EPRE for the purpose of deriving the CQI index based on RRC parameter powerControlOffset, and if also configured, for deriving PMI and RI. However, current description in 38.214 section 5.2.2.5 refers to 38.214 section 4.1, which does not provide the description of the ratio betweeen PDSCH EPRE to CSI-RS EPRE, but rather provide the ratio between CSI-RS EPRE and SSB EPRE based on RRC parameter powerControlOffsetSS. Such description in 38.214 may cause unnecessary mis-interpretation. This has already been corrected in Rel-16 specifcaiton but not corrected in Rel-15 specifcation. |
| --- | --- |
| Summary of change: | The referred section for assumed EPRE ratio is corrected to section 5.2.2.3.1. |
| Consequences if not approved: | UE behavior is unnecessarily mis-interpreted. |

TP 4: {38.214: 5.2.2.5 CSI reference resource definition} for Rel-15

5.2.2.5 CSI reference resource definition < Unchanged parts are omitted > If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Subclause 5.2.2.3.1. - Assume no REs allocated for NZP CSI-RS and ZP CSI-RS. - Assume the same number of front loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig. - Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition. - Assume the PDSCH symbols are not containing DM-RS. - Assume PRB bundling size of 2 PRBs. - The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i) where is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211], is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Subclause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Subclause 5.2.1.4.2.The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 5.2.2.3.1. < Unchanged parts are omitted >

Companies’ input

Please kindly provide your views about the proposed TP of Issue#13 in the table below.

Question 4-1: Do you agree the proposed TP of Issue#13?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - OK with the change, but as this is already corrected in Rel-16, |
| - QC | - We are fine with the CR |
| - vivo | - OK. |
| - Samsung | - Agree |
| - Huawei, HiSilicon | - Agree |
| - OPPO | - Fine with the TP |
| - ZTE | - Support |
| - Ericsson | - Fine with the change |

Question 4-2: Do you think that proposed TP of Issue#13 can be Rel-16 shadow TP?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - We’d be OK to take this in from Rel-15 onwards, but did we already once agree to take this from Rel-16 onwards only as this is already corrected in Rel-16 spec? |
| - QC | - We are fine with a Rel-16 shadow TP. |
| - vivo | - The Rel-16 part is already corrected by the editor. |
| - Samsung | - No need. It is already covered in Rel-16. |
| - Huawei | - No need. It is already fixed in Rel-16. |
| - OPPO | - Agree with Nokia/vivo/Samsung/huawei |
| - ZTE | - Shadow TP is unnecessary due to the fact that Rel-16 has been corrected already. |
| - Ericsson | - As stated in the cover sheet no Rel-16 CR is needed |

Summary

Based on the discussion so far, it can be summarized (only considering original proposal without considering Rel-16 shadow TP, CAT#A) for Issue#13 as follows.

| Support | Nokia, Qualcomm, vivo, Samsung, Huawei, OPPO, ZTE, Ericsson |
| --- | --- |
| Not support |  |

From moderator perspective, this issue can be agreeable. However, regarding Rel-16 shadow TP (CAT#A) is not needed as other companies have commented above.

Issue#21 (Rel-15): R1-2105926, Correction on the MR-DC Uplink Power Control in 38.213, Huawei, HiSilicon [5]

Background & Proposed TP for TS 38.213

| Reason for change: | In NE-DC, RRC parameter tdm-PatternConfigNE-DC was agreed to used as the TDM pattern RRC parameter in CR R2-2006349, while tdm-PatternConfig is referring to the Rel-15 TDM pattern of EN-DC, which is independent of the aforementioned RRC parameter for NE-DC. tdm-PatternConfigNE-DC This field is used when power control or IMD issues require single UL transmission in NE-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].. tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].. But in the latest version of TS 38.213, tdm-PatternConfig-r15 is still used for NE-DC, which is clearly not in line with TS 36.331. If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and - if the UE transmission(s) in slot of the MCG in FR1 overlap in time with UE transmission(s) in subframe of the SCG, and - if in any portion of slot of the MCG, the UE reduces transmission power in any portion of slot of the MCG so that in all portions of slot , where and are the linear values of the total UE transmission powers in slot of the MCG in FR1 and in subframe of the SCG, respectively. - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]).. In the last RAN1 meeting, a Rel-16 CR (R1-2104019) including a fix to the above issue for Rel-16 has been agreed. The same fix should also be needed for Rel-15. |
| --- | --- |
| Summary of change: | Correct the RRC parameter for TDM pattern used in uplink powr control of NE-DC |
| Consequences if not approved: | Misalignment between the RRC configuration reference in the specification for NE-DC uplink power control. |

TP 5: {38.213: 7.6.1A NE-DC} for Rel-15

<Unchanged parts are omitted> 7.6.1A NE-DC If a UE is configured with a MCG using NR radio access and with a SCG using E-UTRA radio access, the UE is configured a maximum power for transmissions in FR1 on the MCG by p-NR-FR1 and a maximum power for transmissions on the SCG by p-MaxEUTRA. The UE determines transmission power for the MCG in FR1 as described Clauses 7.1 through 7.5 using as the maximum transmission power for . The UE determines transmission power for the MCG in FR2 as described Clauses 7.1 through 7.5. If the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG, the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power. If at least one symbol of slot of the MCG that is indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated overlaps with subframe of the SCG - for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power otherwise - the UE determines a transmission power for the SCG as described in [13, TS 36.213] without considering as the maximum transmission power If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and - if the UE transmission(s) in slot of the MCG in FR1 overlap in time with UE transmission(s) in subframe of the SCG, and - if in any portion of slot of the MCG, the UE reduces transmission power in any portion of slot of the MCG so that in all portions of slot , where and are the linear values of the total UE transmission powers in slot of the MCG in FR1 and in subframe of the SCG, respectively. - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]). <Unchanged parts are omitted>

Companies’ input

Please kindly provide your views about the proposed TP of Issue#21 in the table below.

Question 5-1: Do you agree the proposed TP of Issue#21?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - Agree with the TP |
| - QC | - We are fine with CR. |
| - vivo | - Fine with this RRC parameter alignment TP. |
| - Samsung | - Agree. |
| - Huawei, HiSilicon | - Agree |
| - OPPO | - Fine with the TP |
| - ZTE | - Agree |
| - Ericsson |  |

Question 5-2: Do you think that proposed TP of Issue#21 can be Rel-16 shadow TP?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - This change should be taken in from Rel-15 onwards. |
| - QC | - We are OK with the Rel-16 shadow TP. |
| - Vivo | - Ok. |
| - Samsung | - Agree. |
| - Huawei | - Agree |
| - OPPO | - Yes |
| - ZTE | - Agree |
| - Ericsson | - Again, this is 3GPP procedure |

Summary

Based on the discussion so far, it can be summarized (only considering original proposal without considering Rel-16 shadow TP, CAT#A) for Issue#21 as follows.

| Support | Nokia, Qualcomm, vivo, Samsung, Huawei/HiSilcon, OPPO, ZTE, Ericsson |
| --- | --- |
| Not support |  |

From moderator perspective, this issue can be agreeable. Also, regarding Rel-16 shadow TP(CAT#A), it can be agreeable.

Issue#27 (Rel-16): R1-2104645, Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different DL and UL SCSs, Qualcomm Incorporated [6]

Background & Proposed TP for TS 38.213

| Reason for change: | - At the RAN1#104-e meeting, a Rel-16 CR (R1-2101927) on Type-1 HARQ-ACK for PDSCH repetition with different SCSs in DL and UL was approved (note: a corresponding Rel-15 CR was not approved). R1-2101927 includes following TP: If the UE is provided pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from DL slot n-NPDSCHrepeat+1nD-NPDSCHrepeat+1 to DL slot nnD, if NPDSCHrepeat is provided by pdsch-AggregationFactor or pdsch-AggregationFactor-r16 [6, TS 38.214], or - from DL slot n-repetitionNumber+1 nD-repetitionNumber+1 to DL slot nnD, if the time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber, or - in DL slot nnD, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where n is a UL slot overlapping with the end of the PDSCH reception in DL slot nD and k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.. - However, the current description of the corresponding part of TS38.213 Section 9.1.2 is following. If the UE is provided pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from DL slot nD-NPDSCHrepeat+1+1 to DL slot nD, if NPDSCHrepeat is provided by pdsch-AggregationFactor or pdsch-AggregationFactor-r16 [6, TS 38.214], or - from DL slot nD-repetitionNumber+1 to DL slot nD, if the time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber, or - in DL slot nD, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where n is a UL slot overlapping with the end of the PDSCH reception in DL slot nD and k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.. - The above yellow part must be an error and should be corrected. |
| --- | --- |
| Summary of change: | The redundant “+1” highlighted above, which must be an error, is removed. |
| Consequences if not approved: | Type-1 HARQ codebook is not porperly constructed in case of PDSCH repetition on a carrier with different numberlogy than a carrier with PUCCH transmisisons carrying the corresponding HARQ-ACK. |

TP 6: {38.213: 9.1.2	Type-1 HARQ-ACK codebook determination} for Rel-16

9.1.2 Type-1 HARQ-ACK codebook determination This Clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format. If a UE is not provided pdsch-HARQ-ACK-OneShotFeedback, the UE does not expect to receive a PDSCH scheduled by a DCI format that the UE detects in any PDCCH monitoring occasion and includes a PDSCH-to-HARQ_feedback timing indicator field providing an inapplicable value from dl-DataToUL-ACK-r16. If the UE is provided pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config or pdsch-AggregationFactor in PDSCH-Config; otherwise NPDSCHrepeat,max=1. The UE reports HARQ-ACK information for a PDSCH reception - from DL slot nD-NPDSCHrepeat+1+1 to DL slot nD, if NPDSCHrepeat is provided by pdsch-AggregationFactor or pdsch-AggregationFactor-r16 [6, TS 38.214], or - from DL slot nD-repetitionNumber+1 to DL slot nD, if the time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber, or - in DL slot nD, otherwise only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where n is a UL slot overlapping with the end of the PDSCH reception in DL slot nD and k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK. <unchanged text omitted>

Companies’ input

Please kindly provide your views about the proposed TP of Issue#27 in the table below.

Question 6-1: Do you agree the proposed TP of Issue#27?

- If no, please provide the reasons and your suggestions, if any.

| - Company | - Comment |
| --- | --- |
| - Nokia | - Agree with the TP |
| - QC | - We agree with the TP |
| - vivo | - Support. |
| - Samsung | - Agree. |
| - Huawei, HiSilicon | - This TP can easily be covered by the discussion on Issue #13. |
| - OPPO | - Fine with the TP |
| - CATT | - Agree. |
| - ZTE | - Support |
| - Ericsson | - OK |

Summary

Based on the discussion so far, it can be summarized for Issue#27 as follows.

| Support | Nokia, Qualcomm, vivo, Samsung, Huawei/HiSilcon, OPPO, ZTE, CATT, Ericsson |
| --- | --- |
| Not support |  |

From moderator perspective, this issue can be agreeable.

Summary

TP 4: {38.214: 5.2.2.5 CSI reference resource definition} for Rel-15

5.2.2.5 CSI reference resource definition < Unchanged parts are omitted > If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Subclause 5.2.2.3.1. - Assume no REs allocated for NZP CSI-RS and ZP CSI-RS. - Assume the same number of front loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig. - Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition. - Assume the PDSCH symbols are not containing DM-RS. - Assume PRB bundling size of 2 PRBs. - The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i) where is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211], is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Subclause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Subclause 5.2.1.4.2.The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 5.2.2.3.1. < Unchanged parts are omitted >

TP 5: {38.213: 7.6.1A NE-DC} for Rel-15

<Unchanged parts are omitted> 7.6.1A NE-DC If a UE is configured with a MCG using NR radio access and with a SCG using E-UTRA radio access, the UE is configured a maximum power for transmissions in FR1 on the MCG by p-NR-FR1 and a maximum power for transmissions on the SCG by p-MaxEUTRA. The UE determines transmission power for the MCG in FR1 as described Clauses 7.1 through 7.5 using as the maximum transmission power for . The UE determines transmission power for the MCG in FR2 as described Clauses 7.1 through 7.5. If the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG, the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power. If at least one symbol of slot of the MCG that is indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated overlaps with subframe of the SCG - for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power otherwise - the UE determines a transmission power for the SCG as described in [13, TS 36.213] without considering as the maximum transmission power If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and - if the UE transmission(s) in slot of the MCG in FR1 overlap in time with UE transmission(s) in subframe of the SCG, and - if in any portion of slot of the MCG, the UE reduces transmission power in any portion of slot of the MCG so that in all portions of slot , where and are the linear values of the total UE transmission powers in slot of the MCG in FR1 and in subframe of the SCG, respectively. - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]). <Unchanged parts are omitted>

TP 5’: {38.213: 7.6.1A NE-DC} for Rel-16

<Unchanged parts are omitted> 7.6.1A NE-DC If a UE is configured with an MCG using NR radio access and with a SCG using E-UTRA radio access, the UE is configured a maximum power for transmissions in FR1 on the MCG by p-NR-FR1 and a maximum power for transmissions on the SCG by p-MaxEUTRA. The UE determines transmission power for the MCG in FR1 as described Clauses 7.1 through 7.5 using as the maximum transmission power for . The UE determines transmission power for the MCG in FR2 as described Clauses 7.1 through 7.5. If the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG, the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power. If at least one symbol of slot of the MCG that is indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated overlaps with subframe of the SCG - for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using as the maximum transmission power otherwise - the UE determines a transmission power for the SCG as described in [13, TS 36.213] without considering as the maximum transmission power If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and - if the UE transmission(s) in slot of the MCG in FR1 overlap in time with UE transmission(s) in subframe of the SCG, and - if in any portion of slot of the MCG, the UE reduces transmission power in any portion of slot of the MCG so that in all portions of slot , where and are the linear values of the total UE transmission powers in slot of the MCG in FR1 and in subframe of the SCG, respectively. - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]). <Unchanged parts are omitted>

Appendix

[1] R1-2104317, Correction on multi-slot PUSCH transmission, ZTE

[2] R1-2104810, Draft CR on PUCCH repetition, OPPO

[3] R1-2105284, Correction on power control for TS 38.213, Samsung

[4] R1-2105455, Draft 38.214 CR on reference correction for PDSCH EPRE to CSIRS EPRE assumption, vivo

[5] R1-2105926, Correction on the MR-DC Uplink Power Control in 38.213, Huawei, HiSilicon

[6] R1-2104645, Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different DL and UL SCSs, Qualcomm Incorporated