3GPP TSG RAN WG1 Meeting #105-e	R1-2106194

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon (Moderator)

Title:	Summary of CSI enhancements for MTRP and FDD (Round 3 and NWM)

Document for:	Discussion and Decision

# Summary of NWM

# Agreements

R1-2106071	Summary of CSI enhancements for MTRP and FDD (Round 1)	Huawei, HiSilicon (Moderator)

Agreement

For Rel-17 port selection codebook, the maximal value of CSI-RS port number P as Pmax is 32.

Conclusion

At least for rank 1, no further restriction or condition is applied for polarization-common based free-selection and combinatorial coefficient based port selection for W1.

Working Assumption

At least for rank 1, FD bases used for Wf quantization are limited within a single window with size N configured to the UE whereas FD bases in the window must be consecutive from an orthogonal DFT matrix, i.e. Alt 1

- FFS: Further dependence/restriction, e.g. conditioned on N3 or the number of CSI-RS ports, can be applied to above design. If does, how to support a non-consecutive FD bases used for Wf quantization

- FFS: Whether to introduce thresholds for N3 and/or P

Agreement

A polarization-specific bitmap for indication non-zero coefficients should be supported for W2.

Agreement

For the quantization of W2 coefficient, reusing following Rel-16 quantization mechanism for Rank1 at least:

- Two polarization-specific reference amplitudes:

  - for the polarization associated with the strongest coefficient, the reference amplitude is not reported

  - for the other polarization, reference amplitude is quantized to 4 bits

    - The alphabet is{1, 1/2)^(1/4), (1/4)^(1/4), (1/8)^(1/4), …, (1/2^14)^(1/4), [Reserved]} (-1.5dB step size)

- For coefficients other than the strongest coefficient

  - differential amplitude is calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

    - The alphabet is {1, 1/sqrt(2), 1/2, 1/(2*sqrt(2)), 1/4, 1/(4*sqrt(2)), 1/8, 1/(8*sqrt(2))} (-3dB step size)

  - phase is quantized to 16PSK

- For the reserved state for reference amplitude, down-select one Alt

  - Alt 1: it is kept to be reserved

  - Alt 2: it is replaced as (1/2)^(15/4)

  - Alt 3: it is replaced as (1/2)^(3/8)

Note: whether/how SCI is supported for R17 codebook will be discussed separately

Agreement

Whether a NZP CSI-RS resource can be referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis:

- It is feasible in both FR1 and FR2 but subject to UE capability for FR2. If a UE supports and the sharing is also enabled by gNB, two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses, otherwise they cannot.

Conclusion

Whether to support interference measurement based on NZP CSI-RS outside the CMR pair configured for NCJT measurement hypothesis, in addition to CSI-IM

Alt 2: No, it is not supported

Email Approval

Agreement

For a CSI-RS resource set with Ks NZP CSI-RS resources configured for CMR and N NZP CSI-RS resource pairs configured for NCJT measurement hypotheses, study following default value of Ks, max,

- Alt 1: Ks, max = 4

- Alt 2: Ks, max = 2

- Alt 3: Ks, max = 4 for FR2, and Ks, max = 2 for FR1

- Note that default value means the minimal supported value for Ks,max in UE capability reporting, if UE support this feature.

Agreement

For CSI measurement associated with a CSI-ReportConfig for NC-JT, study whether/how to support following dynamic updating on, e.g. by MAC-CE

- Alt 1: CMR pairs for NCJT measurement hypotheses

- Alt 2: CMRs for Single-TRP measurement hypotheses

- Alt 3: TCI states in CMRs

- Alt 4: the number of single-TRP CSIs (i.e. X=0/1/2) in a NCJT CSI report

Conclusion:

There is no consensus to go with either of the following options in RAN1 #105e:

- Option 1: Confirm the Working Assumption from RAN1 103e

- Option 2: The UE can be expected to report one RI, one PMI, one LI and one CQI per TRP, up to 2 TRPs, for Multi-DCI based NCJT

Agreement

For Rel-17 Multi-TRP CSI enhancement, companies are encouraged to study following potential specification impact:

- CRI codepoint mapping order with CMRs and CMR pairs

- Whether/how to configure RI restriction/CBSR configuration for NCJT CSI measurement

- Whether/how to enhance the CSI updating rule to address CPU overbooking

- Whether/how to introduce new CSI computation delay requirement for NCJT CSI calculation

- Whether/how to support wideband CSI report

Agreement

At least for rank 1 and for Mv>1, Minit for the single window with size N is fixed to be 0

Conclusion

For PS codebook enhancements utilizing DL/UL reciprocity of angle and/or delay, there is no consensus of further enhancement for CSI-RS configurations associated with Rel-17 PS codebook.

R1-2106072	Summary of CSI enhancements for MTRP and FDD (Round 2)	Huawei, HiSilicon (Moderator)

Agreement

At least for rank 1, candidate values of K1 for port selection matrix W1 in NP*K1 are {2, 4, 8, 12, 16, 24, 32}.

- Note: for polarization-common based free-selection, it means to select the same L=K1/2 ports out of P/2 ports for both polarizations

Agreement

Further reduction for possible parameter combinations among codebook parameters of Rel-17 port selection codebook, e.g. {K1, Mv, Beta}, will be discussed jointly once candidate values are determined

- based on trade-off among UPT performance, feedback overhead, and complexity

- based on all supported ranks

- Limit total number of parameter combinations comparable to Rel-16 eType II

- Exact parameters (e.g. with 2 or 3 parameters) within each combination are FFS

- Other parameterizations of codebook parameter (e.g. alpha with K1= Alpha*# of CSI-RS ports and Alpha <=1) are not excluded

Agreement

At least for rank 1 and 2, for the compression coefficient Beta for non-zero coefficients of W2, values of Beta are {[1/4], 1/2, 3/4, 1}

- Note: [1/4] means that 1/4 is also a candidate value for the discussion on reduction of parameter combinations, but has a lower priority compared to other beta values

Agreement

For Wf in CN3*Mv, Mv=2 is supported for R17 PS codebook

- FFS: whether further dependence/restriction, i.e. conditioned on the number of CSI-RS ports, can be applied to Mv=2

- FFS: Whether Mv=4 can be supported for # of CSI-RS ports, e.g. 4 or 8

Agreement

Whether a NZP CSI-RS resource m can be referred by two CMR pairs (m, a) and (m, b) configured for NCJT measurement hypotheses

- Alt 1: It is feasible for FR1 but not for FR2.

Agreement

A CSI-IM resource is configured to be associated with either a CMR for Single-TRP measurement hypothesis or a CMR pair for NCJT measurement hypothesis:

- One-to-one mapping between M+N CSI-IM resources versus M NZP CSI-RS resources for single-TRP measurement hypothesis and N NZP CSI-RS resource pairs for NCJT measurement hypothesis configured in a CSI-RS resource set.

  - FFS the value/definition of M

Note: it is possible to configure the same value of CSI-IM resource ID for both NCJT and Single-TRP measurement hypotheses in FR1 and FR2, subject to QCL-Type D consistency between measurement hypotheses of the shared CMR in FR2

# Summary of CSI enhancement for FDD

Proposal 6:  At least for rank 1 and 2 and Mv > 1, for relationship between N and Mv, study and down-select one Alternative from following in RAN1 106

- Alt 1: N= Mv always, no UE reporting of Wf

- Alt 2-1: N >= Mv, Wf  is layer-common and reported by UE for N>Mv.

- Alt 2-2: N >= Mv, Wf is layer-specific and reported by UE for N>Mv.

Note: Wf is layer-common for N=Mv

Note: For all alternatives, a layer-common window/set of size N is configured.

Proposal 9: For Rel-17 port selection codebook, study following Alternatives and down-select in RAN1 106e:

- Alt 1: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length N3. Wf as an all-one vector of length 1 is not needed

- Alt 2: Wf OFF and Wf ON with Mv=1 are same, and Wf is an all-one vector of length 1, i.e., a scalar. Wf as an all-one vector of length N3 is not needed.

- Alt 3: Keep both Wf OFF and Wf ON with Mv=1.

  - If PMI format is SB, Wf  is an all-one vector of length N3

    - Informative note: this case is considered as “Wf ON with Mv=1” in the agreement in RAN1 104e

  - If PMI format is WB, Wf is an all-one vector of length 1, i.e., a scalar

    - Informative note: this case is considered as “Wf OFF” in the agreement in RAN1 104e

- Note: N3 = NCQISubband*R.

- FFS: the case when no SB size is configured.

Proposal 11: Study whether/how the bitmap for indicating non-zero coefficients for W2 can be absent for CSI reporting

- FFS: applicable conditions of being absent, .e.g. Mv=1 and Beta =1 for rank 1 or higher ranks

- FFS: additional impact for reporting mechanism when/how the bitmap is absent

- Note: The principle of UE determining the real number of NZC (same as Rel-15 and Rel-16) is unchanged in Rel-17

- based on trade-off among UPT performance, feedback overhead and complexity

Proposal 13: Study following alternatives for reporting the strongest coefficient indication (SCI) for Rel-17 port selection codebook in W2

- Alt 0 : Reporting of the position, [il*, fl*], of the strongest coefficient of layer l using ceil(log2(K0)) bits, where K0=Beta*K1*Mv

- Alt 1-1: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits

- Alt 1-2: Reporting of the position, [il*, fl*], of the strongest coefficient of layer l, using ceil(log2(K1*Mv)) or ceil(log2(K1))+ceil(log2(Mv)) bits, and shifting of the strongest coefficient to position fl*=0

- Alt 2: shifting the strongest coefficient to fl* = 0, and using ceil(log2(N)) bits to indicate the shift quantity for l-th layer. The strongest coefficient is indicated by il*, using ceil (log2 (K1)) for l-th layer.

- Alt 3: SCI is not needed so that the SCI in R16 codebook is replaced with a strongest polarization indicator (1 bit)

- Considering further impact of UCI design for above Alternatives

# Summary of CSI enhancement for MTRP

Proposal 16-2: For CSI measurement associated with a CSI-ReportConfig for NC-JT, down-select one or more Alts in RAN1#106-e:

- Alt 2: additional RRC signalling is needed to configure M (M≤ Ks) CMRs from the CSI-RS resource set for CMR for Single-TRP measurement hypotheses

  - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. Additional RRC signaling may select {#0,#3} (if sharing is allowed), or {#1, #3} (if not allowed), or select any from the set for single-TRP measurement hypotheses.

- Alt 3: For CMRs configured in the CSI-RS resource set, support RRC signalling to enable/disable single-TRP measurement hypothesis using CMR configured within CMR pairs for NCJT measurement hypothesis

  - Example: For a given set of {{#0, #1}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypothesis. If gNB enables the sharing, {#0, #1, #2, #3} are for single-TRP measurement. If gNB disable the sharing, {#1, #3} are for single-TRP measurement hypotheses.

- Alt 4: CMR sharing between single-TRP measurement hypothesis and NCJT measurement hypothesis is realized by configuring the same value of CMR ID for single-TRP CMR and NCJT CMR pair.

  - Example: When the UE supports sharing, for a given set of {{#0, #0}, {#2, #3}} with N=1, {#0, #2} are for NCJT measurement hypotheses, the rest {#0, #3} are for single-TRP measurement hypotheses. The CMRs for STRP can be updated by re-configuring the CSI resource set.

Note that above examples are only for the purpose of illustrating/discussing Alternatives.

Proposal 21: For Option 1 CSI reporting associated with NCJT and X single-TRP measurement hypotheses, study whether to support following PMI/RI sharing mechanisms between NCJT CSI and single-TRP CSI(s):

- Enabling/Disabling PMI, RI sharing via higher-layer configuration

- Dynamic indication of PMI, RI sharing in the CSI report

- FFS: other details

- FFS: applicable conditions/restrictions of CMR sharing among Single-TRP and NCJT hypotheses, if above PMI/RI sharing mechanism can be applied

Proposal 22: For a CSI report setting with Option 1 and X=1 or 2, For a CSI reporting associated with NCJT and single-TRP measurement hypotheses, study prioritizing CSI associated with reported CSI hypotheses within a CSI Reporting Setting

- FFS potential impact for UCI payload generation

- FFS whether/how to update CSI priority formula, and additional specification impact due to updated formula

- FFS whether/how to update CSI omission rules for Part 2 CSI based on prioritized CSI

- FFS: whether the X+1 CSI hypotheses per CSI Reporting Setting are mapped to a single CSI report or X+1 CSI reports

- Companies are encouraged to discuss and justify purposes of prioritizing CSI associated with reported CSI hypotheses.