3GPP TSG RAN WG1 #106-e                                                       R1-2106703

E-Meeting, August 16th – 27th, 2021

Agenda Item:	8.4.3

Source:	Spreadtrum Communications

Title:	Discussion on enhancements on HARQ for NTN

Document for:	Discussion and decision

Introduction

In RAN#86, Rel-17 Work Item on Solutions for NR to support non-terrestrial networks was approved with the following RAN1 objectives

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified:

- Timing relationship enhancements[RAN1,RAN2]

- Enhancements on UL time and frequency synchronization [RAN1,RAN2]

- HARQ

  - Number of HARQ process [RAN1]

  - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2]

In this paper, we discuss our views on enhancements on HARQ.

Enhancement on HARQ process number

In RAN1#105e, the follow agreement on HARQ process ID indication has been achieved [1].

Agreement: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2 Agreement: For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion.

The fundamental NR HARQ design is asynchronous and thus flexible, relaxed from early LTE HARQ design. For Option 1 and Option 1-a, the determination of HARQ process ID will be coupled with the index of slot(s)/SFN(s) carrying the corresponding transmission/scheduling and division of time domain resource. It will bring some restrictions for HARQ process scheduling, which violates the HARQ design principle in NR.

For Option 2, there is less specification impact since the same field size can be maintained even if the number of HARQ processes is increased to 32. In NTN, some bit fields are useless, which can be re-interpreted for HARQ process ID indication, e.g., DCI indication on second data block.

For Option 3, it is straightforward and has no restriction on the legacy fields in DCI formats. If extending the HARQ process ID field to 5 bits, the size of DCI of NTN will be different to that of TN.

In our view, a unified solution should be applied for DCI formats 0-1/1-1 and DCI 0-2/1-2. Considering extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2 has been agreed, extend the HARQ process ID field up to 5 bits for DCI 0-1/1-1 should also be supported.

Proposal 1: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-1/1-1 should also be supported.

HARQ codebook enhancement

- Type-1 HARQ codebook

For Type-1 HARQ codebook, since type-1 HARQ-ACK codebook is semi-static, UE does not need to reduce the HARQ-ACK codebook size for HARQ processes with disabled HARQ feedback. Furthermore, due to the potential misdetection of DCI, there is potential ambiguity at gNB side to interpret the feedback. In our view, no enhancement is needed for the Type-1 codebook.

Proposal 2: No enhancement is needed for the Type-1 codebook.

- Type-2 HARQ codebook

In RAN1#105_e, for Type-2 HARQ codebook in NTN, reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes is agreed [1].

Agreement: For Type-2 HARQ codebook in NTN, - For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes - FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI

In NR, whether the SPS PDSCH activation is received correctly is indicated implicitly by HARQ feedback for SPS PDSCH. gNB cannot know whether the activation is received based on HARQ feedback if all HARQ process for SPS PDSCH are feedback disabled. Therefore, UE should feedback for the SPS PDSCH activation to ensure the reliability of SPS PDSCH transmission. PDCCH indicating SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled should be included in counting of C-DAI and T-DAI.

Proposal 3: PDCCH indicating SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled should be included in counting of C-DAI and T-DAI.

- Type-3 HARQ codebook

Type-3 HARQ codebook is already supported for licensed spectrum. We first need to clarify the use case Type-3 HARQ codebook in NTN scenario. In our view, if Type-3 HARQ codebook is supported in NTN, reduce codebook size with HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes should be considered.

Proposal 4: Enhancement for Type-3 HARQ codebook should be considered, if it is supported in NTN.

Conclusion

As summary, we have the following proposals.

Proposal 1: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-1/1-1 should also be supported.

Proposal 2: No enhancement is needed for the Type-1 codebook.

Proposal 3: PDCCH indicating SPS PDSCH activation when the related SPS PDSCH is HARQ feedback disabled should be included in counting of C-DAI and T-DAI.

Proposal 4: Enhancement for Type-3 HARQ codebook should be considered, if it is supported in NTN.

Reference

- 3GPP RAN1#105_e Chairman’s Notes, May 10th – 27th, 2021.