3GPP TSG RAN WG1 Meeting #106-e	R1-2107101

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.2.4

Source:	Futurewei

Title:	Views on Beam management for beyond B52.6 GHz

Document for:	Discussion and decision

# Introduction

In RAN#90-e meeting, the Rel. 17 NR_ext_to_71GHz WID [1] was approved.  We summarize the progress made on the beam management agenda item in the previous two meetings that are relevant to our contribution. In particular,  the following agreements were made in RAN1 #104-e:

Agreement:

- Further study the following:

- For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL.

Agreement:

- Study whether/how to introduce a beam switching gap between signals/channels

  - FFS: condition to apply including potential UE capability definition

  - Study should account for inputs from RAN4

Subsequently, the following agreements were made in RAN1#104b-e meeting.

Agreement:

Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH

Agreement:

For timeDurationForQCL, beamSwitchTiming and beamReportTiming,

- Following candidate values of FR2 are reused for 120 kHz:

  - timeDurationForQCL: 14 and 28 symbols

  - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols

  - beamReportTiming: 14, 28 and 56 symbols

- For 480 kHz

  - Support at least the candidate values for 120 kHz scaled by 4x

  - FFS: Support for additional candidate value(s)

- For 960 kHz

  - Support at least the candidate values for 120 kHz scaled by 8x

  - FFS: Support for additional candidate values(s)

- FFS: UE capability signaling details

- Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16).

Agreement:

For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI

- FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

# UE Beam Switch Capability

Each UE can have a different beam switching capability. A conservative value considered for analog beam switch duration discussed in TR 38.817-01 is of the order of 100ns. Using this conservative value, we can determine that a beam switch cannot be accomplished within the CP length of 73ns in SCS of 960 kHz. Moreover, upon adding delay spread, timing errors and other impairments it may even be infeasible to accommodate beam switch duration within the 146ns CP length for SCS of 480 kHz. The following reply was received from RAN4 on beam switch timing [2].

We note from Table-1 that even for gNB switching a value of 59ns has been tentatively agreed. This implies that adjacent symbol beam switching is very likely to be infeasible for 960 kHz SCS and possibly even for  480 kHz SCS with the latter likely to be true for at least a class of UEs. We note that hitherto beam switch timing was not an issue and indeed there are several situations in FR2-1 where such beam switching over consecutive OFDM symbols is required to be performed by the UE. Some specific instances are the following: (i) when CSI-RS associated with different beams (distinct QCL-D assumptions) are assigned to the same user over adjacent OFDM symbols or  (ii) when a PDCCH monitoring occasion assigned to a UE is immediately followed by a PDSCH symbol scheduled for that UE, with their associated receive beam directions being different.

A safe solution to satisfy beam switch delay constraint for the frequency range 52.6-to-71 GHz (termed FR2-2) is for the gNB to ensure that no UE will need to perform beam switching over adjacent OFDM symbols in both 480 kHz and 960 kHz SCS. In other words, purely via scheduling gNB ensures there is always a symbol gap from a UE’s perspective whenever a beam switch would otherwise be needed in each of these SCS (for example in the aforementioned instances). This gap is then regarded as a symbol by that UE on which nothing intended for it would be transmitted by the gNB.

However, this solution precludes taking advantage of UE implementations that allow for faster beam switch. Our preference is for some signaling to indicate UE beam switch capability, which would then allow gNB to decide whether or not to provision a symbol gap (to accommodate beam switch) in a UE specific manner. Clearly if this capability is signaled then a UE should not be expected to perform a beam switch at a rate exceeding its signaled capability. Nonetheless, it is also important to specify UE behavior in the case a gNB schedules adjacent symbol beam switching exceeding that UE’s capability. For instance, in such a scenario one possible UE behavior  could be to prioritize decoding its scheduled PDCCH symbols.

This behavior will indeed be an extension of  existing Rel.16 behavior from clause 5.1.5 of TS 38.214 summarized below where UE behavior for overlapping PDSCH and PDCCH with distinct QCL-D is specified:

“Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-	the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).”

“Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-	the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).”

More generally, in scenarios entailing adjacent symbol beam switch, a UE incapable of such switch can be mandated to prioritize receiving one type of transmissions it is supposed to receive over the other. Furthermore, the gNB should assume that a UE is incapable of adjacent symbol beam switching in case that UE does not signal it in its capability. Based on the aforementioned examples, a natural precedence relation is to mandate the  UE to prioritize symbols corresponding to PDCCH monitoring occasions and switch on preceding symbols and possibly subsequent symbols whenever a switch (and symbol gap) is needed.

Proposal 1: For both 480 kHz and 960 kHz SCS, UE is not expected to be able receive downlink data or control channel or reference signals with different QCL-TypeD properties on adjacent symbols within a slot if that violates its signaled beam switch capability or if this capability is not signaled.

Proposal 2: Precedence relations must be defined to allow a UE incapable of adjacent symbol beam switch, to determine which symbol(s), if any, to switch on for all instances entailing  adjacent symbol beam switch.

# Timing Associated with Beam-based Operation in new SCS

We proceed to present our proposals on remaining issues pertaining to timing related parameters in NR_ext_to_71GHz (FR2-2).  We first recall the definitions of these parameters.

TimeDurationForQCL: The IE “timeDurationForQCL” defines the minimum number of OFDM symbols required by a UE to perform PDCCH reception, which at-least includes retrieval of spatial QCL information received in the DCI and applying that spatial QCL information on phase shifters in order to prepare spatial receive filters for receiving scheduled PDSCH.

BeamSwitchTiming: The IE “beamSwitchTiming” indicates  the UE’s capability on the minimum number of OFDM symbols it requires between the end of the last symbol of the PDCCH that contains the DCI which triggers aperiodic CSI-RS and the beginning of the first symbol of the CSI-RS transmission.

BeamReportTiming: The IE “beamReportTiming” indicates the minimal number of OFDM symbols required between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report. The UE provides this capability for the band number for which the report is provided (where the measurement is performed) and it includes this field for each supported sub-carrier spacing.

Recall that in FR2-1 beam switching gap can be safely absorbed in the CP length. Moreover, as described in the previous section, whenever there is a conflict between spatial beams to be used by a UE for receiving PDSCH and PDCCH (due to overlapping symbol(s)), precedence in FR2-1 is given to receiving PDCCH.  Next, since beam switching gap is a new issue in FR 2-2 it needs to be accounted for. As we advocated previously, we can keep same order of precedence in FR2-2 as well by prioritizing PDCCH symbols more.

With this understanding, the “minimum” duration (or number of symbols) present in all three timing related parameters’ definitions stated above, needs further clarification in light of the possible beam switching symbol gap that might need to be provisioned for. In particular, in each case the indicated capability will not apply if the UE is further mandated to use a distinct spatial beam and receive (or transmit)  the immediately preceding symbol.

Applying this qualification to the case of TimeDurationforQCL we have that:

“UE is not expected to receive the OFDM symbol immediately preceding the symbol with scheduling offset equal to TimeDurationforQCL with a different receive spatial beam  than the one to be used for the latter, if such consecutive symbol beam switch exceeds its beam switch capability”.

An analogous qualification applies to each one of the other two parameters.

For BeamSwitchTiming: UE is not expected to receive the OFDM symbol immediately preceding the symbol(s) containing the triggered CSI-RS with a different receive spatial beam  than the one to be used for the latter, if such consecutive symbol beam switch exceeds its beam switch capability.

For Beam BeamReportTiming: UE is not expected to transmit the symbol preceding the one with scheduling offset equal to BeamReportTiming with a different transmit beam  than the one to be used for the latter, if such consecutive symbol beam switch exceeds its beam switch capability.

In all instances if the scheduling violates the underlying assumption, then switching will be done by the UE  as per defined precedence relations.

Proposal 3: The timing related parameter capability indicated by a UE for a SCS on which it cannot perform adjacent symbol beam switch is only guaranteed whenever to meet that capability it is not required to perform an adjacent symbol beam switch.

We briefly present our views on the UE supported value for parameter maxNumberRxTxBeamSwitchDL (which defines the number of Tx and Rx beam changes UE can perform within a slot across all configured serving cells). In FR2-1 the UE shall report one value from  list {4,7,14} per each subcarrier spacing. Our preference is to have smaller values added to the list in order to cater for FR2-2 UEs that would leverage similar hardware capability FR2-1. We prefer to support {1, 2, 4,  7, [14]}. Here {1,2} are added to facilitate simpler implementation where we note that the minimal value of 1 is needed to receive PDCCH and PDSCH on different beams within a slot. The remaining values are retained from FR2-1 where feasibility of 14  is clearly only for UE capable of adjacent symbol beam switch.

Proposal 4: For maxNumberRxTxBeamSwitchDL we propose the set {1, 2, 4,  7, [14]}.

# QCL Assumptions for Scheduling Multiple-PDSCH using a single DCI

In Fig. 1 we have depicted multi-slot PDSCH scheduling with a single DCI. Note here that the UE can apply the TCI state indicated in the triggering DCI for all PDSCH slots with offsets no less than  timeDurationForQCL. However, rules for default beam selection need to be defined at-least for all PDSCH slots with offsets less than timeDurationForQCL.   Such a default selection rule will be particularly beneficial for URLLC traffic since the latter can be scheduled without waiting to ensure that the PDSCH slots carrying such traffic have offsets at-least as much as the timeDurationForQCL.

Under many default beam selection rules, the PDSCH slots (if any) that have offsets smaller than “timeDurationForQCL” can be received by the UE with   different RX beam(s) than the RX beam used to process those whose offsets are no less than “timeDurationForQCL”.

In the previous RAN1#104b-e meeting although no agreement on this issue could be achieved, there was progress towards agreeing on the following:

- For multi-PDSCHs scheduled by a single DCI with a single TCI state,

  - Case 1: PDSCH scheduling offset for all PDSCHs ≥ timeDurationForQCL

    - Case 1-1: tci-PresentInDCI enabled

      - Single QCL assumption based on the indicated single TCI state is applied for all scheduled PDSCHs

    - Case 1-2: tci-PresentInDCI disabled

      - QCL assumption of the single DCI scheduled multi-PDSCHs is applied for all scheduled PDSCHs

  - Case 2: PDSCH scheduling offset for any scheduled PDSCH < timeDurationForQCL

    - Down select one of the following alternatives

      - Alt 1: Single QCL assumption is applied for all scheduled PDSCHs

        - FFS: Details of single QCL assumption, if supported

      - Alt 2: multiple QCL assumptions are applied

        - FFS: Details of multiple QCL assumptions, if supported

We note that one choice is to simply extend and apply the default beam selection rule defined in clause 5.1.5 of TS 138.214, which says that for a PDSCH slot with offset less than timeDurationForQCL, the UE will use spatial beam used to receive symbols on monitoring occasion(s) in the latest monitored slot that are associated with the CORESET of lowest ID among all those monitored in that slot. With such a default beam selection choice there is no ambiguity in UE behaviour in terms of choosing a default RX beam for all its PDSCH slots with offsets less than timeDurationForQCL. However, such a choice can lead to varying RX beams across slots, and at least in the single TRP case, there may be a single optimized RX beam using which we can potentially realize a better SINR at the UE (and hence a better performance).  This latter performance could have been achieved if signalled value of “timeDurationForQCL” was one symbol so that the RX beam (spatial QCL-D) conveyed via the TCI could be immediately applied by the UE for all scheduled PDSCH.  There have been proposals (cf. [4]) to consider redefining of the default beam selection rule towards realizing the above ideal performance obtained with a single optimized RX beam. However, this default beam should then be sufficiently dynamically configurable (which of-course entails more signalling and perhaps more specification changes), while respecting the original actual spatial RX beam preparation related capability of that UE. Moreover, even with the default rule outlined above, the gNB via scheduling can ensure that an optimized RX beam is used by the UE for all PDSCH slots with offsets at least as large as its “timeDurationForQCL”, while good RX beams are used by that UE even for the slots with offsets less than its “timeDurationForQCL”. Together these two factors will already mitigate any potential loss.  We therefore would like to see more compelling reasons before other choices deviating from the direct extension outlined above are considered.

Fig 1. Multi-slot PDSCH scheduling

In addition, there is a connection to the beam switch gap which is illustrated via Fig. 2. Consider the two cases outlined in the possible agreement depicted before. In case 1, gNB has the option of indicating a more finely optimized beam (or equivalently TCI state), based on latest measurement reports, and hence mandating UE to follow an indicated TCI for all its slots is a natural choice. Moreover,  in absence of an indicated TCI in the DCI, we can re-use Rel.16 rules and direct the UE to use the beam (TCI state) associated with the CORESET corresponding to the search space set whose monitoring occasion contains the PDCCH that has the triggering DCI.

In case 2, the gNB does not have the option of making the UE follow an indicated beam. Here the default choice of mandating UE to follow QCL assumption derived from the most-recently monitored lowest ID CORESET has the advantage of reducing the number of UE beam switches and possibly gap symbols. To illustrate this point, in Fig. 2 we show that making UE follow the QCL assumption derived from a pre-configured default over all its remaining PDSCH slots can potentially impose up-to twice the number of beam switches on the UE, and possibly twice the number of gap symbols.

An analogous situation arises when the common beam (single QCL assumption) the UE must use is the one derived based on default selection rules for its first PDSCH slot.

Observation 1: Changes to Rel.15 default beam assumptions in single DCI multi-slot PDSCH scheduling can result in need for provisioning of larger number of gap symbols.

Fig. 2. Impact of Beam Switching gap on Multi-slot PDSCH scheduling

# RS support for LBT Procedures

The roles of periodic-RS in both beam failure check (BFC) and candidate beam evaluation (CBE) are well recognized. There are two issues---(i) mandatory LBT with the possibility of failing LBT and (ii) restricting RS transmission to within the channel occupancy time (CoT) upon succeeding---that together create several situations where adequate periodic RS support for BFC and CBE cannot be provided by the transmitter or gNB. This has led to support for triggering aperiodic CSI-RS or TRS by the gNB to substitute for a missing periodic RS transmission [5]. One key motivation behind this support is the need to prevent UE from assuming a spurious link misalignment in situations with a relatively short-term LBT failure at the gNB. Here by short-term we mean that there is a relatively high likelihood of LBT at the gNB succeeding (with the same sensing beam) before the next scheduled periodic-RS transmission. Thus, the gNB can perform an LBT and upon success (which is likely), trigger and setup an aperiodic RS transmission. The UE can then adjust failure counter it maintains and combine the measurements on this aperiodic RS and the previous periodic ones, all of which correspond to the same transmit beam. In addition, a UE implementation can also detect the absence of a periodic RS transmission. Upon such detection, it can also tentatively assume an LBT failure at the gNB and then expect a trigger for an aperiodic RS before the next scheduled periodic RS.

An argument against the approach described above which entails triggering aperiodic CSI-RS, is to rely instead on a legacy (Rel. 16) solution that is based on counters and timers.  One key reason for not relying just on such legacy solution is that it need not be agile enough. In particular,  the legacy solution may end up either declaring too many failures (when the threshold is set to be a low value and the reset timer is set to be a large value). Conversely, it may become too conservative in flagging such failures whenever the threshold is set high and/or the reset timer is set to a small value.

Proposal 5: Utilize aperiodic CSI-RS transmission to address impact of LBT failure on periodic RS transmissions intended to support beam failure recovery.

Fig. 3: Persistent interferer (TX-B) can cause sustained LBT failure at TX-A. Switching to alternate beam is preferable for TX-A--RX-A link.

Mandatory LBT brings up another issue related to the aforementioned scenario that also merits consideration. Consider Fig. 3, which depicts the presence of an interferer (TX-B) transmitting along a beam which has a significant overlap with the sensing beam that is used by TX-A for directional LBT sensing. Suppose further that this interferer will begin transmitting for a relatively persistent or sustained duration. This scenario will violate the implicit “short-term LBT failure at the gNB” assumption made before. We thus have a situation in which using the original beam for transmission by gNB (TX-A) which might still be feasible from the UE’s (RX-A’s) point of view, becomes untenable as per LBT protocol. This is because LBT at gNB using the original sensing beam will keep classifying intended transmission by it as a potential interference to an ongoing transmission. In such a situation the gNB should instead switch to an alternate transmit beam that is available. Consequently, the gNB should perform an LBT with an alternate sensing beam (aligned with the alternate transmit beam). Since the likelihood of LBT success becomes higher using the alternate sensing beam, the gNB is more likely to be able to trigger and switch to using the alternate transmit beam. Extending this argument, the gNB can also attempt to trigger and re-evaluate a previous best beam that it had to abandon due to a persistent LBT failure in the past. In each case the gNB should be enabled to achieve a low latency beam switch for precoding periodic RS using an updated transmit beam. This will enable low latency beam switch to address sustained LBT failure.

Proposal 6: Consider support for low latency beam (QCL-TypeD) switch of periodic RS transmissions after persistent or sustained LBT failure.

# Conclusions

In this contribution, we have presented our views on beam management for shared spectrum access in beyond 52.6GHz.  Based on the discussions in the previous sections we propose the following:

Proposal 1: For both 480 kHz and 960 kHz SCS, UE is not expected to be able receive downlink data or control channel or reference signals with different QCL-TypeD properties on adjacent symbols within a slot if that violates its signaled beam switch capability or if this capability is not signaled.

Proposal 2: Precedence relations must be defined to allow a UE incapable of adjacent symbol beam switch, to determine which symbol(s), if any, to switch on for all instances entailing  adjacent symbol beam switch.

Proposal  3: The timing related parameter capability indicated by a UE for a SCS on which it cannot perform adjacent symbol beam switch is only guaranteed whenever to meet that capability it is not required to perform an adjacent symbol beam switch.

Proposal 4: For maxNumberRxTxBeamSwitchDL we propose the set {1, 2, 4, 7,[14]}.

Observation 1: Changes to Rel.15 default beam assumptions in single DCI multi-slot PDSCH scheduling can result in need for provisioning of larger number of gap symbols.

Proposal 5: Utilize aperiodic CSI-RS transmission to address impact of LBT failure on periodic RS transmissions intended to support beam failure recovery.

Proposal 6: Consider support for low latency beam (QCL-TypeD) switch of periodic RS transmissions after persistent or sustained LBT failure.

# References

RP-202925, “Revised WID: Extending current NR operation to 71 GHz”, CMCC, RAN#90-e.

R4-2107985, “LS on 60 GHz time-related issues”, RAN4#99-e.