3GPP TSG RAN WG1 #106-e			R1-2107404

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.5.2

Source: 	CMCC

Title: 	Discussion on UL-AoA enhancements

Document for:	Discussion and Decision

Introduction

In RAN#90-e meeting, a new work item for NR positioning enhancements was approved [1], in which the motivation to support positioning enhancements to attain Rel-17 target positioning requirement was justified for both commercial and IIoT use cases.

In RAN1#105-e meeting, the following agreements were made on the accuracy improvements for UL-AoA positioning solutions:

Agreement: Both GCS and LCS are supported for UL AoA/ZoA assistance information indication. Note: Existing signaling can be used for obtaining LCS to GCS translation information Agreement: Granularity of 0.1 degrees is applied for the expected AoA (φAOA), expected ZoA (θZOA ) and the corresponding uncertainty values Agreement: LMF to gNB signaling of UL AoA/ZoA assistance information (expected value and uncertainty range) is supported for UL-TDOA and Multi-RTT positioning methods Agreement: Send an LS to RAN3 (potentially also to RAN2 at least as cc) capturing RAN1 agreements on UL AOA/ZOA assistance information (expected value and uncertainty range) and request them to define signaling Agreement: NR supports gNB reporting of the first arrival path UL-AOA/ZOA measurement per SRS for positioning resource and SRS for MIMO resource - Note: The use of SRS for MIMO resource is transparent to the UE

In this contribution, we continually provide our views on the remaining issues on UL-AoA enhancements.

Discussion

In the previous meetings, some companies proposed to further enhance the UL AoA reporting, by extending the UL AoA measurements to include the other information, such as timing and RSRP. An intermedium proposal on this issue was left open in the last RAN1 meeting:

Proposal 3.9-1

- For UL-AOA positioning, select alternative for NR support of path-specific RSRP measurements based on SRS (for positioning, MIMO)

  - Alt.1: NR supports reporting of path specific RSRP measurements for the first arrival path only

  - Alt.2: NR supports reporting of path specific RSRP measurements for the first arrival path and for additional paths

  - Alt.3: NR supports reporting of RSRP measurements in a pre-configured time window

    - Power of paths outside of the window is excluded. FFS details of time window configuration

  - FFS definition of path specific RSRP measurements and whether it is supported per SRS resource or SRS resource set (for positioning, MIMO)

- For UL-AOA positioning, select alternative for support of path-specific UL-RTOA measurements based on SRS (for positioning, MIMO)

  - Alt.1: NR supports reporting of path specific UL-RTOA measurements for the first arrival path only

    - Note: UL-RTOA for the first arrival path is supported for UL-TDOA (it is discussed in the context of UL-AOA)

  - Alt.2: NR supports reporting of path specific UL-RTOA measurements for the first arrival path and for additional paths

  - FFS definition of path specific RTOA measurement and whether it is supported per SRS resource or SRS resource set (for positioning, MIMO)

During the discussion, companies shared diverse views on the necessity and benefit of this enhancement. In addition, some companies were confused with the definition of the “path-specific RSRP”, and expressed concerns on the feasibility of the gNB to separate and measure RSRP of different paths. Hence, no consensus was made regarding this issue.

From our perspective, the intention of this reporting enhancement is not clear. Firstly, the benefit of extending the AoA measurement reporting along with RSRP and/or timing information on the positioning performance seems not significant. If the reporting enhancement is to assist the gNB to detect/identify LOS/NLOS paths, we believe that the enhancement can be discussed under the NLOS/multipath mitigation AI. On the other hand, for the additional reporting of UL-RTOA, we think that it can be achieved by configuration of hybrid UL-AoA and UL-TDoA positioning, and therefore, no more reporting enhancement is necessary.

Observation 1: For UL-AOA positioning, the benefit and necessity on supporting of path-specific RSRP and UL-RTOA measurements based on SRS (for positioning, MIMO) should be further clarified.

Proposal 1: For UL-AOA positioning, support of path-specific UL-RTOA measurements reporting based on SRS (for positioning, MIMO) can be achieve by hybrid UL-AoA and UL-TDoA.

Conclusions

In this contribution, we provide our views on the UL-AoA enhancements, and the following observation and proposal is provided:

Observation 1: For UL-AOA positioning, the benefit and necessity on supporting of path-specific RSRP and UL-RTOA measurements based on SRS (for positioning, MIMO) should be further clarified.

Proposal 1: For UL-AOA positioning, support of path-specific UL-RTOA measurements reporting based on SRS (for positioning, MIMO) can be achieve by hybrid UL-AoA and UL-TDoA.

References

- RP-202900, New WID on NR positioning enhancements, CATT, Intel Corporation, Ericsson.