3GPP TSG RAN WG1 #106-e			                          R1-2107415

e-Meeting, August 16th – 27th, 2021

Source: 	CMCC

Title:	Discussion on TRS/CSI-RS occasion(s) for IDLE/INACTIVE-mode UEs

Agenda item:	8.7.1.2

Document for:	Discussion & Decision

- Introduction

In last RAN1 meeting, some agreements about TRS/CSI-RS for IDLE/INCTIVE UEs were made as the following [1],

Agreement:

Confirm the following working assumption:

- Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

  - FFS details, including paging DCI and/or PEI for L1 based signaling

  - FFS SIB-based signaling/configuration

    - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement:

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreement:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include:

- periodicityAndOffset {10, 20, 40, 80} ms

- frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB

- FFS Configuration index

  - details,

    - E.g. Per resource or resource set or group of resource sets

    - E.g. explicit or implicit indication based on QCL source

Agreement:

Further study supporting SIB based signalling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

- FFS whether and how SIB based signalling and L1 based signalling can be configured simultaneously

In this contribution, we will further discuss availability indication of TRS/CSI-RS including the indication signalling and information content.

- Discussion

2.1 Availability indication signalling

In last RAN1 meeting, there were a working assumption about L1 based signalling for the availability indication of TRS/CSI-RS at the configured occasion(s) and an agreement about further study SIB based signalling, we will further discuss these issues as the following.

2.1.1 L1 based availability indication

Paging PDCCH

The advantages of paging PDCCH are having enough reserved DCI bits to indicate the availability information and independent on whether PEI is configured or not. But one concern is that paging PDCCH will cause “always-on” signal if it is transmitted each PO to indicate the TRS availability information. One simple solution is that we should not allow gNB transmitting paging PDCCH carrying TRS availability indication-only without the short message and/or scheduling information, that is the TRS availability indication can only be carried when at least one UE needs waked up to receive paging. But this restriction will also introduce another problem that if one UE has never been paged, it cannot receive the availability information.

PDCCH-based PEI

The advantages of PDCCH-based PEI are that it will not cause “always-on” signalling if Behv-A is supported and the UE power saving gain is the biggest to reduce the AGC and T/F tracking power consumption before receiving PO. In order to minimize the payload size of PDCCH-based PEI to guarantee the detection performance, some pre-defined availability valid time can be jointly used, e.g., only indicating the availability of TRS before associated PO. Whether to continue transmitting the TRS after PO or not is up to NW’s implementation, but UE should assume the TRS is not transmitted after PO because there is no availability information anymore.

Although we support both paging PDCCH and PDCCH-based PEI as the availability indication, one issue still needs discussion is whether they are mutually exclusive when PEI is configured. As the discussion above, one disadvantage of paging PDCCH as availability indication is “always-on” signal issue if gNB wants to guarantee all UEs receiving the availability information. In order to avoid “always-on” signal, one solution is that don’t allow indicating the availability of TRS/CSI-RS only in paging PDCCH without short message and/or scheduling information. That is if there is no short message or paging message, the availability information can not be transmitted. But if PEI is configured and the availability information is carried in it, the “always-on” issue can be solved, because only waked up UE needs the TRS/CSI-RS assisting PO reception.

Therefore, we think if PDCCH-based PEI is configured by SIB, the availability indication is carried in PDCCH-based PEI, else, the availability indication is carried in paging PDCCH.

Proposal 1. Confirm the following working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS how to enable/disable L1 based availability indication configurable by SIB

Proposal 2. Don’t allow indicating the availability of TRS/CSI-RS only in paging DCI without short message and/or scheduling information.

Proposal 3. If PDCCH-based PEI is configured by SIB, the availability indication is carried in PDCCH-based PEI, else, the availability indication is carried in paging PDCCH.

2.1.2 SIB based availability information

The SIB based availability information was also discussed in last meeting for further study, which also including the combination with L1 based availability indication.

Although the SIB based availability information may impact on legacy UEs which they will also wake up to update the SI with unnecessary UE power consumption even they don’t support the Rel-17 TRS/CSI-RS feature, we think SIB based availability information can be used in the scenario which non-frequent TRS/CSI-RS availability indication and the L1 based availability indication is not configured.

Proposal 4. Supporting SIB based signalling for availability information of TRS/CSI-RS occasions for idle/inactive UEs based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

If the availability/unavailability information change is much frequent, gNB should configure the L1 based signalling to dynamically indicate which TRS/CSI-RS are available or unavailable. That is UE assumes all TRS/CSI-RS occasions configured in SIB are available until L1 based availability information changes the assumption. In this method, there is no SI update procedure which reduces the impact on legacy UEs. Therefore, if L1 based signalling is configured, there is no SI update and only L1 based signalling providing the availability/unavailability information.

Proposal 5. SIB based signaling provides availability indication for a default assumption of the availability information for all configured TRS/CSI-RS occasions, and L1 based signaling provide updates relatively to the default assumption.

2.2 Availability indication validity time

There were some discussions about validity time in last RAN1 meeting, but no consensus was achieved. As the discussion in section 2.1, we confirm both paging DCI and PDCCH-based PEI for availability indication should be supported, and the discussion of validity time is also related to the availability indication design.

Alt 1: Configured by higher layer

The first alternative is a configured value by higher layer, which is much suitable for paging PDCCH based availability indication without using additional L1 signalling indication to reduce DCI bits overhead. For example, the valid time is configured as one or more paging cycle(s).

Alt 2: A window before a PO

Alt 2 is much suitable for PDCCH-based PEI as availability indication. As we all know, the most important function of TRS/CSI-RS is assisting AGC and T/F tracking before receiving PO, it is a straightforward method to define a window before PO as validity time combined with PEI. For example, if PEI indicates some TRS/CSI-RS are available, UE can assume these RS(s) are available in a window before the PO which associated the PEI, else, UE can only use SSB for AGC and T/F tracking.

Alt 3: Included in the availability indication

Alt 3 is explicit validity time indication in the availability indication. It was agreed that UE subgrouping is also carried in PDCCH-based PEI, in order to guarantee the detection performance, the DCI size of it should not be increased too much, therefore Alt 3 is not suitable for PDCCH-based PEI as the availability indication. For paging PDCCH, there are 6 reserved bits in paging DCI, and the availability indication and UE subgrouping indication will use some reserved bits, whether there are still some reserved bits can be used to indicate the validity time is not clear.

In a summary, we think if paging PDCCH is used as availability indication, the validity time should be configured by higher layer to avoid additional L1 signalling indication. If PDCCH-based PEI is using as availability indication, the validity time should be pre-defined as a time window before the PO associated with the PEI.

Proposal 6. For paging PDCCH based availability indication, the validity time is configured by higher layer.

Proposal 7. For PEI based availability indication, the validity time is a window before the PO which associated with the PEI.

- Conclusions

In this contribution, availability information of TRS/CSI-RS are discussed, and the following proposals are made.

Proposal 1. Confirm the following working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS how to enable/disable L1 based availability indication configurable by SIB

Proposal 2. Don’t allow indicating the availability of TRS/CSI-RS only in paging DCI without short message and/or scheduling information.

Proposal 3. If PDCCH-based PEI is configured by SIB, the availability indication is carried in PDCCH-based PEI, else, the availability indication is carried in paging PDCCH.

Proposal 4. Supporting SIB based signalling for availability information of TRS/CSI-RS occasions for idle/inactive UEs based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

Proposal 5. SIB based signaling provides availability indication for a default assumption of the availability information for all configured TRS/CSI-RS occasions, and L1 based signaling provide updates relatively to the default assumption.

Proposal 6. For paging PDCCH based availability indication, the validity time is configured by higher layer.

Proposal 7. For PEI based availability indication, the validity time is a window before the PO which associated with the PEI.

- References

- Chairman's Notes RAN1#105-e