3GPP TSG RAN WG1 #106-e		R1-2107727

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.2.2

Source:	Apple Inc.

Title:	PDCCH Enhancements for above 52.6 GHz

Document for:	Discussion/Decision

# Introduction

Based on the SI for RAN1 above 52.6 GHz [1], an updated WID was agreed to in RAN #90-e with the following objectives [3] :

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

  - Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

- Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, we address enhancements to PDDCH monitoring for the new SCSs (480 kHz and 960 kHz) for NR operation between 52.6GHz and 71 GHz.

# Discussion

## PDCCH Monitoring Background

In RAN1 #104-e, the following agreements were made [6]:

Agreement: Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: A fixed pattern of N slots. - Alt 2: Use the Rel-16 capability (pdcch-Monitoring-r16, (X, Y) span) as the baseline to define the new capability - FFS: Values of X and Y and units in which they are defined - FFS: Whether number of slots within which the number of monitoring occasions is counted is needed and if needed, the value of the number of slots - Alt 3: A sliding window of N slots for defining multi-slot PDCCH monitoring capability. - FFS: Increments in which sliding occurs - Specific numbers for X, Y and N may depend on UE capability and gNB configuration - Examples: - N = [4] slots for 480 kHz SCS and N = [8] slots for 960 kHz SCS - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

The  agreement was modified in RAN1 #104b-e as follows:

Agreement: Previous agreement is modified as follows: Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability. - Each slot group consists of X slots - Slot groups are consecutive and non-overlapping - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately - FFS: Supported values/constraints of X and Y, e.g., Y<=X, Y=X - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g., the Y [symbols or slots] always start at the first slot within a slot group - FFS: Further definition of capabilities - Alt 2: Use an (X, Y) span as the baseline to define the new capability - X is the minimum time separation between the start of two consecutive spans - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots] - Y <= X - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary. - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots. - FFS: Further definition of capabilities - Alt 3: Use a sliding window of X slots as the baseline to define the new capability. - The capability indicates the BD/CCE budget within the sliding window - The sliding unit of the sliding window is [1] slot. - FFS: Further definition of capabilities - Specific numbers for X, Y may depend on UE capability and gNB configuration - Examples: - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

## Multi-slot Monitoring (MSM) and MSM capability indication for Rel-17

In Rel-15, PDCCH monitoring of search spaces is defined per slot with different usage models defining a monitoring periodicity of once per slot (classified as case-1) or multiple times per slot or per span (classified as case-2). In Rel-16, PDCCH monitoring of search spaces for URLLC is defined per span where a span is a sub-set of a slot. This allows multiple spans to be processed within a slot for the 15 kHz and 30 kHz SCSs only. The UE processing limits and procedures (including overbooking and dropping) are defined based on the PDCCH monitoring capability unit.

A Rel-17 UE may be required to increase its PDCCH processing capabilities for the new SCSs (480 kHz and 960 kHz) compared with Rel-15/Rel-16 operation with a maximum data/control SCS of 120 kHz as the symbol duration decreases with the new SCSs. In one simple example, an increase in SCS to 960 kHz (480 kHz) may require processing 8 times (4 times) as many slots as a 120 kHz system in the same amount of time. To  account for the complexity increases over the same time duration when compared with Rel-15 and Rel-16, three alternatives were defined in the agreements in RAN1 #104-e [6]. We discuss our view on the three alternatives below:

For Alternative 1, the following should be considered:

- The values of X and Y should be set to allow flexibility in the placement of the CSS and UE-SS without undue burden on UE power consumption

- For 480 kHz: X = 4 slots, for 960 kHz, X =  8 slots.

  - Additional values smaller than 4/8 slots for 480/960 kHz can be supported based on UE capability.

  - Larger values than 4/8 slots for 480/960 kHz are not supported.

  - Single-slot PDCCH monitoring for 480 kHz and 960 kHz is not supported. Supporting single slot monitoring removes the complexity reduction benefits of multi-slot monitoring. In addition, the low BD/CCE values that can reasonably be selected will result in low aggregation levels and high blocking probabilities when scheduling.

- The value Y should be defined based on slots. The capability should indicate the BD/CCE budget within Y consecutive slots in each slot group.

  - The SS sets are within the first Y slots of the slot group and Y < X i.e. no back-to-back monitoring of PDCCH between slot groups is supported.

    - The value of Y will depend on the value of X selected. Ideally Y = 1. Note that for Y > 1, there is a possibility for the monitoring occasions to be distributed over multiple slots in the X window. This will be problematic for the UE’s power consumption.

  - PDCCH monitoring of all SS sets monitored in the Y slots occurs within N consecutive symbols of Y:

    - Case 1: PDCCH monitoring limited to within the first N consecutive symbols  of Y

    - Case 2: PDCCH monitoring is on any span of up to N consecutive symbols of Y

- For Rel-17, there is no need to increase the CORESET duration i.e., the maximum CORESET duration ≤ 3. This is because (a) any increase in PDCCH capacity can be done in the frequency domain and (b) modification of the CORESET duration will result in additional specification impact and may require implementation changes.

- The positions of CSS and UE-SS in Y should be clarified.

  - In Rel-15, for FG 3-1, type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS are located in the first three symbols of the slot while  type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS. For simplicity let us define type 1 CSS with dedicated RRC configuration, type 3 CSS as Type A CSS and type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS as Type B CSS.

  - Type A CSS can tolerate some limitation in its location but should be common to all UEs in the group. This can inherit its  FG 3-1 behaviour i.e., Case -1 at the beginning of the slot.

  - For the UE-SS, given the shift to MSM, there may be a need to configure different UE-SSs at different locations to give the network some flexibility but not at the cost of UE power consumption increases due to MO distributed over multiple slots i.e., Case-2 anywhere within the slot.  This is different from its FG 3-1 behavior.

  - Type B CSS needs flexibility in its location (i.e., cannot be limited to the 1st 3 symbols in a slot group) as it may be tied to the SS/PBCH block depending on the multiplexing pattern (e.g., for multiplexing pattern 3 that is FDM with the SS/PBCH block, fixing the location of the Type B CSS will limit the location of the SS/PBCH block). The agreement in RAN  #92-e, i.e. “Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis” captured in  [10] does not eliminate the need for multiplexing pattern 3 and as such limiting the flexibility of the Type B CSS should not be agreed to yet.

  - Note that the CSS and UE-SS may be set with different capabilities and RAN1 should enable signaling of  different MSM PDCCH monitoring capabilities for different PDCCH types.

- The number of unicast DCIs for single and multi-PxSCH transmission within a slot group should be a UE  capability.

- The Max # of monitored PDCCH candidates per (multi-)slot per CC (BD) can and  the Max # of non-overlapped CCEs per (multi-)slot per CC can be set as follows:

Table 1: BD/CCE Budget for Alt. 1

| μ | Maximum number of monitored PDCCH candidates |  |  |  | Maximum number of non-overlapping CCEs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | X=1 | X=2 | X=4 | X=8 | X=1 | X=2 | X=4 | X=8 |
| 3 | 20 | - | - | - | 32 | - | - | - |
| 5 | N/A | 10 | 20 | - | N/A | 18 | 32 | - |
| 6 | N/A | 8 | 10 | 20 | N/A | 14 | 18 | 32 |

For Alternative 2, the following should be considered:

- For 480 kHz: X = 4 slots (56 symbols), for 960 kHz, X =  8 slots (112 symbols). The maximum values of X should be set to the corresponding duration of a 120 kHz SCS symbol

  - Additional values smaller than 4/8 slots for 480/960 kHz can be supported based on UE capability.

  - Larger values than 4/8 slots for 480/960 kHz are not supported.

  - Single-slot PDCCH monitoring for 480 kHz and 960 kHz are not supported.

- The value Y should be defined based on symbols. The capability indicates how much BD/CCE budget is available within Y consecutive symbols in a span.

  - The duration of Y < X  with Y ≤ 3 symbols similar to the Rel-16 definition.

- A span should not cross a slot boundary. It should be entirely contained within a slot.

- The number of unicast DCIs for single and multi-PxSCH transmission within a span should be defined as a capability.

- The Max # of monitored PDCCH candidate per slot/span per CC (BD) and  the Max # of non-overlapped CCEs per slot/span per CC can be set as follows:

Table 2: BD/CCE Budget for Alt. 2

|  | Max. # of monitored PDCCH candidates per slot/span per combination (X,Y) and per serving cell |  |  |  | Max. # of non-overlapped CCEs per slot/span for per combination (X,Y) and per serving cell |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| μ | Slot-based | (28, Y) | (56, Y) | (112, Y) | Slot based | (28, Y) | (56, Y) | (112, Y) |
| 3 | 20 | - | - | - | 32 | - | - | - |
| 5 | N/A | 10 | 20 | - | N/A | 18 | 32 | - |
| 6 | N/A | 8 | 10 | 20 | N/A | 14 | 18 | 32 |

For Alternative 3, although there is a limitation on the complexity within a sliding window, the lack of a fixed pattern within this window could result in a large increase in UE processing complexity as the UE may have to dimension its processing to occur at any time. As such, we do not see a need for using this framework as it is a big departure from the existing framework and could result in a need for implementation changes. This concurs with the  WID motivation that supports minimizing  the specification burden and maximizing the leverage of FR2 based implementations.

Procedures on overbooking and dropping may be discussed once  the MSM methodology is finalized.

Proposal 1: The MSM PDCCH monitoring capability should be based on  Alt 1 or Alt 2. Alt. 3 should not be supported.

Proposal 2: The corresponding values of X and Y are set to allow flexibility in the placement of the CSS and UE-SS without undue burden on UE power consumption e.g. Y ≤1.

Proposal 3: The values of X for Alt-1 and Alt-2 are as follows:

- For 480 kHz: X = 4 slots, for 960 kHz, X =  8 slots.

  - Additional values smaller than 4/8 slots for 480/960 kHz can be supported based on UE capability.

  - Larger values than 4/8 slots for 480/960 kHz are not supported.

- Single-slot PDCCH monitoring for 480 kHz and 960 kHz are not supported.

Proposal 4: For Alt-1, the value Y should be defined based on slots.

- the SS sets are within the first Y slots of the slot group and Y < X i.e., no back-to-back monitoring of PDCCH between slot groups is supported. With Y = 1

- PDCCH monitoring of all SS sets monitored in the Y slots occurs within N consecutive symbols of Y:

  - Case 1: PDCCH monitoring limited to within the first N consecutive symbols  of Y

  - Case 2: PDCCH monitoring is on any span of up to N consecutive symbols of Y

Proposal 5: For Alt-1, the positions of CSS and UE-SS in Y should be clarified based on both network flexibility and UE power consumption.

Proposal 6: For Alt-1, the Max # of monitored PDCCH candidate per (multi-)slot per CC (BD) can and the Max # of non-overlapped CCEs per (multi-)slot per CC can be set as  in Table 1.

Proposal 7: For Alt-2, the value Y should be defined based on symbols

- The duration of Y < X  with Y ≤ 3 symbols

Proposal 8: The Max # of monitored PDCCH candidate per slot/span per CC (BD) and  the Max # of non-overlapped CCEs per slot/span per CC can be set as in Table 2.

## Cross Carrier Scheduling and HARQ transmission

In RAN1 #104-e, the following discussion was started but was not concluded:

- Cross-carrier scheduling of a cell within 52.6-71 GHz from/[to] a cell outside 52.6-71 GHz is supported.

- FFS: potential limitations on the applicable SCS(s) of the scheduling and scheduled cells/BWPs.

For cross carrier scheduling the following issues should be studied:

- RAN1 should modify the parameter Npdsch  to account for the new SCSs: The parameter Npdsch, i.e., the # of PDCCH symbols after the end of the PDCCH scheduling the PDSCH needs to be modified for the new SCSs.

- RAN1 should study the effect of a large differential between the SCSs of the carriers involved in the cross carrier scheduling procedure. In a scenario with different numerologies between PDSCH and PUCCH, a large differential between the SCSs may result in a large gap between a transmitted PDSCH(s) and its corresponding PUCCH. In one simple example, assume that the transmission occurs such that the HARQ is on FR1 with the SCS set to 15 kHz which is equivalent to 32 480 kHz slots. A frame structure of DDDSU would require an aggregation of up to 96 slots. The maximum differential changes from 8 (120 kHz to 15 kHz) to 64 (960 kHz to 15 kHz).

- The maximum number of carriers that can be simultaneously scheduled from a single carrier should be defined as a UE capability. This may be necessary given the possible increase in the bandwidth of the different transmissions, and the increase in data rate for the new SCSs.

Proposal 9: RAN1 should modify the parameter Npdsch, i.e. the # of PDCCH symbols after the end of the PDCCH scheduling the PDSCH, to account for the new SCSs.

Proposal 10: RAN1 should study the effect of a large differential between the SCSs of the carriers involved in the cross-carrier scheduling procedure.

Proposal 11: for cross-carrier scheduling, the max number of CCs that can be scheduled from a single CC is reported as UE capability.

## DCI Design related to channel Access

### Search Space Set Group Switching

Search Space Set Group Switching was introduced in Rel-16 NR-U for power saving. It operates on CSS Type 3 and UE-SS and allows the UE to switch between two groups of search space sets to either increase or decrease the UE search space set monitoring to save power as needed e.g. within and outside a COT in unlicensed access.

With the introduction of the new SCSs, there may be a need to modify timeline parameters such as the searchSpaceSwitchDelay and searchSpaceSwitchTimer. This may be set in units of slots or multi-slots based on the UE capability and the SCS.

In Rel-16, the switching boundary and the timer decrement value are on the order of slots. In the case of MSM PDCCH monitoring, as the PDCCH may be on the order of multiple slots, both the switching boundary and the timer decrements can be modified to be on the order of multi-slots as needed. The effect of MSM on the transition boundary and the time unit of multiple slots (4 slot) is illustrated in Figure 1.

Figure 1: Example of SSSG switching with multi-slot monitoring limitations

Proposal 12: Consider the effect of the change in SCS and of MSS PDCCH monitoring on SSSG switching.

### DCI format 2-0 design

In R16 NR-U, DCI format 2-0 is enhanced to carry channel access related information: RB allocation, COT duration and search space set switching indication. While RB allocation information may not be needed depending on LBT bandwidth discussion, the COT duration and SSSG switching should be supported.

COT duration and SSSG switching information should be sent at the beginning of the COT as shown in Fig.1. However, current design of DCI format 2-0 transmission limit to one beam per slot. Therefore, it takes multiple slots to finish the beam sweeping transmission of DCI format 2-0. For example, with 120KHz SCS and 32 beams, it takes 4ms to finish beam sweeping. Considering maximum COT duration is 5ms defined by EN 302 567, more efficient transmission scheme of DCI format 2-0 is needed.

Proposal 13: Consider enhancement of DCI 2-0 transmission to signal COT duration and SS adaptation at the beginning of the COT.

# Conclusion

In this contribution, we have discussed PDCCH monitoring enhancements for NR between 52.6GHz and 71 GHz and have the following proposals:

Proposal 1: The MSM PDCCH monitoring capability should be based on a Alt 1 or Alt 2. Alt. 3 should not be supported.

Proposal 2: The corresponding values of X and Y are set to allow flexibility in the placement of the CSS and UE-SS without undue burden on UE power consumption e.g. Y ≤1.

Proposal 3: The values of X for Alt-1 and Alt-2 are as follows:

- For 480 kHz: X = 4 slots, for 960 kHz, X =  8 slots.

  - Additional values smaller than 4/8 slots for 480/960 kHz can be supported based on UE capability.

  - Larger values than 4/8 slots for 480/960 kHz are not supported.

- Single-slot PDCCH monitoring for 480 kHz and 960 kHz are not supported.

Proposal 4: For Alt-1, the value Y should be defined based on slots.

- the SS sets are within the first Y slots of the slot group and Y < X i.e., no back-to-back monitoring of PDCCH between slot groups is supported. With Y = 1

- PDCCH monitoring of all SS sets monitored in the Y slots occurs within N consecutive symbols of Y:

  - Case 1: PDCCH monitoring limited to within the first N consecutive symbols  of Y

  - Case 2: PDCCH monitoring is on any span of up to N consecutive symbols of Y

Proposal 5: For Alt-1, the positions of CSS and UE-SS in Y should be clarified based on both network flexibility and UE power consumption.

Proposal 6: For Alt-1, the Max # of monitored PDCCH candidate per (multi-)slot per CC (BD) can and the Max # of non-overlapped CCEs per (multi-)slot per CC can be set as  in Table 1.

Proposal 7: For Alt-2, the value Y should be defined based on symbols

- The duration of Y < X  with Y ≤ 3 symbols

Proposal 8: The Max # of monitored PDCCH candidate per slot/span per CC (BD) and  the Max # of non-overlapped CCEs per slot/span per CC can be set as in Table 2.

Proposal 9: RAN1 should modify the parameter Npdsch, i.e. the # of PDCCH symbols after the end of the PDCCH scheduling the PDSCH, to account for the new SCSs.

Proposal 10: RAN1 should study the effect of a large differential between the SCSs of the carriers involved in the cross-carrier scheduling procedure.

Proposal 11: for cross carrier scheduling, the max number of CCs that can be scheduled from a single CC is reported as UE capability.

Proposal 12: Consider the effect of the change in SCS and of MSS PDCCH monitoring on SSSG switching.

Proposal 13: Consider enhancement of DCI 2-0 transmission to signal COT duration and SS adaptation at the beginning of the COT.

# Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz

- 3GPP TR 38.808 V1.0.0 (2020-12, Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)

- RP-202925,  3GPP Work Item Description on Extending current NR operation to 71GHz

- Chairman’s Notes, RAN1 #102-e, August 2020

- Chairman’s Notes, RAN1 #103-e, October 2020

- Chairman’s Notes, RAN1 #104-e, January 2021

- R1-2102242, Feature lead summary#3 for [104-e-NR-52-71GHz-02] on PDCCH monitoring enhancements, FL discussion, January 2021

- 3GPP TR 38.822 V15.0.1 (2019-07), User Equipment (UE) feature list (Release 15)

- R1-2007326, Updated RAN1 UE features list for Rel-16 NR including remaining RAN1 issues

- RP-211540, Summary of email discussions on Extending NR operation to 71GHz, Moderator (Qualcomm)