3GPP TSG RAN WG1 #106-e                                                R1-2108259

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.8.3

Title: 	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3

Source: 	Moderator (ZTE Corporation)

Document for:	Discussion and Decision

- Introduction

One objective of the coverage enhancement WID is to specify mechanism(s) to support Type A PUSCH repetitions for Msg3. This contribution provides a summary of proposed Msg3 enhancements in contributions submitted under AI 8.8.3 and AI 8.8.4.

- Proposals for GTW session

Proposal_v5 for Issue#7:

Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions.

- Option 1:

- When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Support: Sharp, Qualcomm, OPPO, Panasonic, Xiaomi, Ericsson, China Telecom, vivo, ZTE, Intel, WILUS, Samsung (if the indication of repetition number is not in TDRA table), DCM (if the indication of repetition number is in TDRA table), LG

- Option 2:

- When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB.

  - FFS details, e.g. implicit or explicit indication or predefined.

  - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

Support: Nokia/NSB, Apple, CATT, Samsung (if the indication of repetition number is in TDRA table), DCM (if the indication of repetition number is not in TDRA table)

Proposal_v1 for Issue#9:

- Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition.

- FFS whether to support other values, e.g., 8.

- Note: K=1 is supported and how to support K=1 is FFS.

Proposal for Issue#10: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon can be regarded as available symbols for Msg3 PUSCH transmission.

- Option 2-A: Additional explicit indication is introduced to indicate whether flexible slots/symbols indicated via TDD-UL-DL-Configcommon are available for Msg3 repetition.

- Option 2-C: No need additional indication, i.e., leave to gNB/UE implementation.

Proposal for Issue 11:

If a symbol for Msg3 repetition in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot.

Proposal for issue 14: For frequency hopping for Msg3 repetition, down-select one of the two options below.

- Option 1: Support intra-slot frequency hopping for Msg3 with repetition.

- Intra-slot frequency hopping and inter-slot frequency hopping cannot be enabled simultaneously.

- When intra-slot frequency hopping is configured, the UE assumes the same starting RB and the same frequency offset for Msg3 PUSCH repetitions within a transmission.

- Option 2: Support only intra-slot frequency hopping for Msg3 PUSCH without repetition and only inter-slot frequency hopping for Msg3 PUSCH with repetition.

- If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping.

- Summary of Tdocs

  - Differentiation and triggering mechanisms for Msg3 repetition

In RAN1#105-e, it was agreed to use separate preamble with share ROs for requesting Msg3 repetition while other solutions are FFS.

Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

[Closed] Issue#1: Additional support of using separate RO for requesting Msg3 repetition

In addition to using separate preamble, there are two additional candidate options for a UE requesting Msg3 PUSCH repetition. In this section, companies’ detailed views are summarized as follows.

Option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

- Support: [3, Spreadtrum Communications], [8, Xiaomi], [10, Intel], [13, Panasonic], [16, Nokia/NSB], [17, Ericsson], [22, LG], [23, WILUS]

  - [5, vivo], [17, Ericsson]: If separate ROs are configured for requesting Msg3 repetition, RA-RNTI overlapping issues should be avoided, e.g., introduce a new RA-RNTI computation method.

  - [18, Sharp]: If separate RO is supported, Option 2 is preferred.

- Depending on RAN2 discussion: [1, Huawei, HiSilicon], [4, ZTE], [17, Ericsson]?, [8, Xiaomi], [25, Xiaomi], [12, Qualcomm]?

  - [4, ZTE]: Do not support additional options (i.e., using separate RO) for requesting Msg3 repetition, unless RAN2 concludes to support with taking other features into account.

  - [17, Ericsson]: A unified solution for indicating all new features requiring early indication via PRACH transmission in NR Rel-17 is pursued.

  - [8, Xiaomi], [25, Xiaomi] and [12, Qualcomm] propose to design the PRACH partitioning together with RedCap. FL’s understanding is this will be discussed in the common session in RAN2.

- [19, CMCC]: The option 2 and 3 using separate RO should be deprioritized.

- Option 3: Option 2 + using remaining RO

- Support: [6, CATT]

    - The number of invalid ROs for each PRACH configuration is summarized in Table 3 and Table 4 in [6, CATT].

- Postpone to RAN2 discussion: [1, Huawei, HiSilicon], [4, ZTE]

- [19, CMCC]: The option 2 and 3 using separate RO should be deprioritized.

There are 6 companies pointing out that this issue is related to other Rel-17 features, which will be discussed in the common session in RAN2. That is, how to design PRACH resources partitioning for multiple Rel-17 features (SDT, CovEnh, RedCap and RAN slicing), with also taking into account the legacy features that using PRACH partitioning, i.e., selected SSB, contention mode (CFRA vs CBRA), payload size (preamble group A vs group B), random access type (2-step vs 4-step RA). Therefore, FL suggests holding on the discussion for this issue for now.

[Closed] Issue#2: Details of using separate preambles of shared RO for requesting Msg3 repetition

Regarding the FFS points of using separate preambles for requesting Msg3 repetition, companies’ views are summarized below.

- [Closed] Issue#2-1: Support of a PRACH mask

Introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

- Support: [4, ZTE], [8, Xiaomi],[10, Intel], [14, Samsung], [18, Sharp], [23, WILUS]

- Not support: [6, CATT], [16, Nokia/NSB]

- Leave to RAN2: [1, Huawei, HiSilicon], [5, vivo]

For issue 2-1, FL’s understanding is it may also depend on PRACH partitioning under discussion in the common session in RAN2. Therefore, FL suggests holding on the discussion for issue 2-1 for now.

- [Closed] Issue#2-2: Indication of preambles for shared RO

- Option 1: Explicitly indicates the preamble number and preamble starting index

  - Support: [14, Samsung] and [18, Sharp]

- Option 2: Only indicates the preamble number, the preamble starting index is from the end of the preamble group for a SSB in a RO

  - Support: [14, Samsung]

[17, Ericsson]: Whether the preambles for requesting Msg3 repetition should be outside the total number of preambles configured should be discussed. A unified solution of preamble grouping should be supported for requesting Msg3 repetition as well as for indicating other new features in NR Rel-17.

For issue 2-2, FL’s understanding is this is also related to PRACH partitioning under discussion in the common session in RAN2. Therefore, FL suggests holding on the discussion for issue 2-2 for now.

- [Closed] Issue#2-3: Definition of shared RO

There are following three cases for the RO shared for Msg3 repetition.

- Case 1: The shared RO has only preambles for 4-step RACH (except for CFRA preambles).

  - E.g., when only 4-step RACH is configured.

  - Support: [4, ZTE], [7, China Telecom], [10, Intel]?, [12, Qualcomm], [16, Nokia/NSB], [22, LG], [18, Sharp]

- Case 2: The shared RO has preambles for both 4-step RACH and 2-step RACH

  - E.g., when one SSB is associated with 2 ROs for 4 step RACH, and 2-step RACH is configured with shared RO in both 2 ROs.

  - Support: [4, ZTE], [7, China Telecom], [10, Intel]?, [12, Qualcomm], [22, LG], [18, Sharp]

- Case 3: The shared RO has only preambles for 2-step RACH

  - Case 3-1: Only 2-step RACH is configured. As discussed in [105-e-NR-2step-RACH-01] in [3], it is RAN1 common understanding that 4-step RACH should be configured at least on the initial BWP in Rel-16. While, it is possible that only 2-step RACH is configured in an active BWP.

  - Case 3-2: Both 2-step RACH and 4 step RACH are configured, while a separate PRACH configuration with separate ROs is configured for 2-step RACH. For these separate ROs, there is only preambles for 2-step RACH.

  - Support: [18, Sharp]?

[4, ZTE]: As long as the ROs have configured with 4-step RACH, a UE may require Msg3 repetition for coverage enhancement when the UE is in the cell edge. For Case 3, there may be no need to support as the UE would be in good coverage if the RO is only configured with 2-step RACH procedure.

First round

For issue 2-3, it is about the RO that can be shared for separate preambles for requesting Msg3 repetition. This is more RAN1 related and could be discussed here. Based on companies input, FL suggests discussing the following proposal as a starting point, i.e., only support Case 1 and Case 2.

Proposal for issue 2-3: The separate preambles for Msg3 repetition could be configured in a shared RO only when it is configured with 4-step RACH preambles.

| Company | Comments |
| --- | --- |
| Samsung | A little confused about the wording in discussion and proposal, why should we bother defining a “shared RO” by whether it is configured with 4step RACH preambles and/or 2step RACH preambles. One understanding of the proposal might exclude the case 2. We can simply using 4step RACH RO Suggest change is: The separate preambles for Msg3 repetition could be configured in a shared RO only in a RO when it is configured with for 4-step RACH preambles. FL: Thanks for the suggestion, which aligns the intention but seems more preferable by companies. In addition, the proposal is further updated based on suggestion from Apple and Ericsson to clarify the 4 step RACH is for legacy Msg3 PUSCH without repetition. |
| Sharp | We think it’s better to hold this issue for now due to the relation with RACH partitioning discussion in RAN2. Regarding FL proposal, we are fine basically if “only” is removed from the proposal. We think it’s early to preclude Case 3. FL: Hope the comments from other companies, e.g., Nokia, Intel, LG, Ericsson, could address your concern. |
| Nokia/NSB | Support the spirit (and fine with Samsung’s modifications). The technical challenge of sharing ROs between UEs attempting access via 2SR (i.e., a “2SR UE”) and UEs attempting access via 4SR plus Msg3 repetition request (i.e., a “CovEnh UE”) is nontrivial, due to the so called “near-far” problem. Such problem causes the following three issues: - MsgA PUSCH transmission is not time aligned at gNB (clause 5.3.2 in TS 38.212). This is different from Msg3 transmission, for instance. This may cause synchronization issues at gNB, if a R17 CovEnh UE (typically far from gNB) transmits a preamble over a RO configured for 2SR and a 2SR UE transmits a preamble over the same RO (this UE would be typically close to gNB). In this case, the delay spread of the channel plus the propagation delay experienced by the preamble transmitted by the CovEnh UE, and as observed at gNB, would be much larger than the delay spread of the channel plus the propagation delay experienced by the preamble transmitted by the 2SR UE. This would cause very poor synchronization performance at gNB, w.r.t. the 2SR UE, which could cause impossibility to decode the subsequent MsgA PUSCH, due to (i) the aforementioned lack of time alignment in this case, and (ii) the very short CP that is used for MsgA PUSCH, regardless of the SCS, which would not be enough to mitigate the effect of the poor synchronization. Please note that situation would be even worse “if multiple to one” RO-to-PO mapping is configured by gNB. In summary, this issue would impact performance of 2SR. - The impact of the previous issue may likely depend on the path-loss difference (always from gNB’s perspective) between the CovEnh UE and 2SR UE. If the latter is very close to gNB, received preamble RSRP may be sufficient to filter out the preamble sent by the CovEnh UE. This could preserve decodability of MsgA. However, this would hinder the detectability of the preamble sent by the CovEnh UE, which could be seen as noise at gNB and thus ignored. In summary, this issue would impact the effectiveness of the msg3 repetition request via CovEnh 4SR procedure. - Any attempt to solve either of the two issues above would cause non-negligible implementation impact at gNB, entailed by the nontrivial technical challenges associated to the detection of the preambles transmitted according to the two procedures over the same shared RO. @Sharp: as you can see from above, we believe that the problem is technical, with non-negligible physical layer issues to account for. We think RAN1 should discuss this and provide RAN2 clear guidance in this sense. FL: Thanks for the good comment! It provides good insights about the impact between 2SR UE and 4SR CovEnh UE. |
| Intel | We are fine with the proposal with the update from Samsung. 2-step RACH is not targeted for coverage enhancement. |
| Qualcomm | Agree with Samsung |
| LG Electronics | Because it is hard that UE who has not 2-step RACH related capability knows whether configured RO(s) are used for both 4-step RACH and 2-step RACH, or for only 4-step RACH. Hence, it is quite reasonable that the UE for coverage enhancement uses the configured RO which may configured for both 4-step RACH and 2-step RACH, or for only 4-step RACH. We see that majority view is to support both Case1 and Case 2. But, FL’s proposal for issues 2-3 is intended to select only one option (i.e., Case1). Hence, we propose to revise the FL’s proposal for issue 2-3 as follow: Proposal for issue 2-3: The separate preambles for Msg3 repetition could be configured in a shared RO only when it is configured with 4-step RACH preambles. FL: FL’s intention is to select both Case 1 and Case 2 (I still think my original proposal does not preclude Case 2, as the ‘only’ intends to preclude Case 3). Anyway, it seems the suggestion from Samsung is better and could also address your concern. |
| Apple | The original wording by FL is not addressing the intention. Proposed text by Samsung is better but still leaves some questions, like “separate” preambles to what? I assume legacy UEs. If so we suggest this The separate preambles for Msg3 repetition could be configured only in a RO shared with 4-step RACH preambles without Msg3 repetition. The separate preambles for Msg3 repetition could be configured in a shared RO only in a RO when it is configured with for 4-step RACH preambles. FL: Will be reflected in the updated proposal. |
| NTT DOCOMO | Fine with the proposal and update by Samsung. |
| OPPO | Fine with the proposal with the update from Samsung. |
| Panasonic | We are fine with the update from Samsung. |
| Xiaomi | Agree with the proposal with Samsung’s update. |
| China Telecom | Fine with the FL proposal and the updated version by Samsung. |
| Ericsson | Regarding restrictions of what kind of RA type in general can coexist with the msg3 repetition RA in one RO, these are discussed in the RACH partitioning agenda. Not only 2-step RACH, but also SDT RA, RA of a network slice, RedCap RA introduced in Rel-17 should be considered together in RAN2 discussions. A unified solution is pursued for the preamble partitioning considering both flexibility and complexity and it would be good to let RAN2 to handle this though we also think some guidance from RAN1 would be good on which features may not be necessary in combination with which other features. Regarding whether an RO configured with 2-step RA preambles can be used by msg3 repetition RA, we think it should be fine to support such case as 2-step RACH is only selected by a UE when the RSRP threshold is above some threshold defined by the network and there’s also some gap configured between preamble part and PUSCH part, it can be up to gNB implementation to make sure the interference mitigated. Regarding whether an RO configured with only 2-step RA should support msg3 repetition RA, we also agree that it seems this is not necessary as even legacy 4-step RA is not configured in such RO. But as long as 4-step RA is configured in one RO, the msg3 repetition RA should be fine to be supported in our understanding. FL: This is also FL’s intention (i.e. support both case 1 and case 2). Given above, on top of updates from Samsung, we propose: Proposal for issue 2-3: The separate preambles for requesting Msg3 repetition could be configured only in an shared RO only when it is configured with 4-step RACH preambles not for requesting Msg3 repetition. FL: Will be reflected in the updated proposal. |
| CATT | We are fine with the proposal in principle. Usually, a UE would adopt 2-step RACH when the channel condition is well (e.g. SSB RSRP > threshold1). However, a UE may request Msg3 repetition for better performance when the UE is in the coverage limited scenarios such as cell edge (e.g. SSB RSRP < threshold2). Hence, it may be redundant and strange for a 2-step UE to request Msg3 repetition. On the other hand, if a RO shared with 2-step RACH and 4-step RACH is used to further distinguish between the legacy UE and the CE UE, fewer preambles would be allocated for each case which would impact the performance of random access. FL: Thanks for the support. |
| vivo | Agree with the proposal with revision from Samsung and Ericsson. |
| Huawei, HiSilicon | Only support case 1 and case 2. Fine with the updated version by Samsung. |
| ZTE | Prefer to only support case 1 and case 2. |
| FL | All companies support the intention of the proposal, while only one company prefers to support Case 3. FL proposal is updated based on the suggested revisions. Proposal_v1 for Issue#2-3: The separate preambles for requesting Msg3 repetition could be configured only in an shared RO only when it is configured with 4-step RACH preambles not for requesting Msg3 repetition. FL suggests companies to be constructive as there only two meetings left, and this proposal could provide good guidance to facilitate RAN2 discussion. So, please comment only if you have strong concerns on Proposal_v1 for Issue#2-3. Thanks! |
| Samsung2 | Thx FL for the update, but since the “4-step RACH preambles” is still there, our concern is not addressed. So let me be more specific: If we say “an RO configured with 4-step RACH preambles not for requesting Msg3 repetition”, does such RO includes the case that it also includes the 2step RACH preambles? From our understanding, it should be included, so that’s why we suggest to use “a RO configured for 4step RACH”, because such RO will be configured for sure, and it may or may not be shared with 2step RACH, so include both cases. FL: The current wording also includes Case 2. To FL’s understanding, there is no difference between ‘a RO configured for 4step RACH’ and ‘a RO configured with 4step RACH preambles’. So, as long as the RO is configured with 4-step RACH preambles, regardless of whether there is 2-step RACH preambles, it will be a valid RO here. Hope it clarifies. To apple’s comments on the “separate”, it should be clear that the preambles in one RO could be partitioned for many features (unfortunately), so the “separate” means an isolated preamble set will be configured for our feature here (msg3 repetition request), it will be separated to any other preambles used for other features. FL: The ‘separate’ here at least means different/isolated preamble compared to legacy Msg3 without repetition. While, whether it will be separate preamble to other Rel-17 features is still up to RAN2 discussion. In this sense, I think it should be ok to go with current wording, though it is a bit redundant. Suggested: Proposal_v1 for Issue#2-3: The separate preambles for requesting Msg3 repetition could be configured only in an shared RO only when it is configured withfor 4-step RACH preambles not for requesting Msg3 repetition. |
| CATT | In our view, here the 2 cases are: - If dedicated ROs are configured for 2-step RACH, then the separate preambles for requesting Msg3 repetition will only be configured in the ROs for 4-step RACH. FL: Correct - But if 4-step RACH and 2-step RACH share ROs, then, the separate preambles for requesting Msg3 repetition will still be configured in these shared ROs; however, such separate preambles for requesting Msg3 repetition will only be within the preamble group for 4-step RACH, and will not overlap with the preamble group for 2-step RACH. FL: Correct for the first part. For the second part, I am not quire sure what the preamble group means. But you are right that such separate preambles for requesting Msg3 repetition will not overlap with the preambles for 2-step RACH. I think we are the same page overall. If these is the common understanding, then we are fine with either FLs’ or Samsung’s version. |
| Xiaomi2 | We are fine with Samsung2’s version. |
| Panasonic2 | We are fine with either FL’s or Samsung2’s version. |
| FL | From FL perspective, either the proposal or Samsung2’s version is ok. Fundamentally, we are all on the same page. Based on my reply to Samsung2 above, FL suggests keeping the proposal as it is. Below, the proposal is still my last proposal with clean up the revisions. Proposal_v1 for Issue#2-3: The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition. |
| Ericsson2 | Fine with FL’s latest proposal, which already covers both 4-step RACH only case and 4-step+2-step RACH case. One thing that we may be noted is that if we have only PRACH preambles for requesting msg3 repetition configured in one RO on top of PRACH preambles for 2-step RACH, is this then an RO configured for 4-step RACH? If the answer is yes, we’d better to have red text in “4-step RACH preambles not for requesting Msg3 repetition” to make it clear as is captured in the latest proposal. |
| Apple | Support FL’s Proposal_v1 |
| Intel | We are fine with Proposal_v1 |
| LG | Support FL’s Proposal_v1 |
| FL | This issue is closed with reaching the following agreement: Agreement The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition. |

[Closed] Issue#3: Conditions of requesting Msg3 repetition

In RAN1#105-e, it was agreed a UE can request Msg3 repetition when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold.

Companies’ detailed views are summarized as follows.

[4, ZTE], [17, Ericsson]: Leave to RAN2 for decision. - [17, Ericsson]:RAN2 is to discuss the details of how the procedure using the RSRP threshold shall work for determining whether Msg3 repetition shall be requested or not. [5, vivo]: Rel-17 CE UE should request for Msg3 PUSCH repetition when SS-RSRP measured on all SSBs are lower than certain threshold which is lower than rsrp-ThresholdSSB. [9, InterDigital], [13, Panasonic], [14, Samsung], [18, Sharp], [20, NTT DOCOMO]: RSRP threshold for triggering Msg.3 repetition should be configured (e.g., Msg3Reptition-RSRP-Threshold). [12, Qualcomm]: The RSRP threshold depends on SS-RSRP and/or UE power class. FFS: whether RSRP thresholds are separate for NUL and SUL. [16, Nokia/NSB]: A UE requests Msg3 PUSCH repetitions at least when the RSRP of the downlink pathloss reference belongs to the range {rsrp-ThresholdSSB, rsrp-ThresholdSSB + msg3-DeltaRepetitionRequest}, extremes included. - A UE cannot request Msg3 PUSCH repetitions if the RSRP of the downlink pathloss reference is lower than rsrp-ThresholdSSB. [20, NTT DOCOMO]: Additionally, threshold of the number of Msg3 attempts should be supported as a condition to request Msg3 repetitions. [22, LG]: The RSRP threshold is determined by the UE side considering on UE capability (e.g., UE maximum Tx power, UE Tx beam gain, number of UE Tx antenna.)

FL notices that this issue is also under discussion in RAN2, as the determination of the RSRP threshold would impact the selection of RACH procedure defined in RAN2 (TS 38.321), with considering the conjunction with existing RSRP thresholds (e.g., SUL or 2-step RACH). Therefore, FL suggests holding on the discussion for this issue for now.

[Closed] Issue#4: UE capability reporting after initial access procedure

If a UE requests Msg3 repetition, it implicitly means the UE reports its capability. However, gNB would not know how many of UEs in the cell is capable of Msg3 repetition. Many companies observe that allowing Rel-17 UEs report its capability of Msg3 repetition after initial access could be beneficial for gNB to optimize the PRACH configuration to reduce the collision probability.

- UE capability of supporting Msg3 PUSCH repetition is reported after initial access procedure

- Support: [1, Huawei, HiSilicon], [3, Spreadtrum Communications], [4, ZTE], [19, CMCC]

- Not support: [11, Apple]

- Postpone the discussion: [12, Qualcomm],

Based on the input, FL suggests to postpone the discussion on this issue in a later phase.

  - Indication of the number of repetitions for Msg3

[Open] Issue#5: Indication of the number of repetitions for Msg3 initial transmission

For Msg3 initial transmission, the following WA was reached for repetition indication in RAN1#105-e.

Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

Companies’ views are summarized in the following table.

Table 2.2-1 Summary of companies’ views for repetition indication for Msg3

| Bit field | Support | Concern | Pros and Cons |
| --- | --- | --- | --- |
| TDRA | Sharp, CATT, Ericsson, vivo, DCM, Xiaomi, Panasonic, ZTE, CMCC, OPPO, China Telecom, InterDigital, Samsung, ETRI, LG | Apple, Nokia, NSB, Intel, Huawei, HiSilicon, Qualcomm | Pros: - Use similar mechanism defined for regular PUSCH repetition in Rel-16, which may require less spec efforts if no new default TDRA table is introduced. Cons: - Impact the flexibility of time domain resource indication; - Either more signaling overhead or may have impact on legacy UEs - If a new TDRA table is introduced including separate new indication for K2, mapping type, SLIV and repetition factor, the overall overhead increase may be relatively large. E.g., about 18% in case of 2 bits for repetition indication and 1000 bits for the total number of bits for SIB1. - If the new TDRA table reuses legacy indication for K2, mapping type and SLIV, with only additionally introducing one new field for repetition indication, then additional overhead seems minor. E.g., 2*16/1000= 3.2%, where 2 bits are used for repetition indication and 1000 bits are the total number of bits for SIB1. However, this may have impact on legacy UEs which would share the same configuration (K2, mapping type and SLIV) with Rel-17 CE UEs. - Larger spec impact if a separate new default TDRA table is specified. |
| MCS | Apple, Intel, Samsung, Huawei, HiSilicon Nokia, NSB, DCM, China Telecom, WILUS | [CATT, Ericsson, LG, OPPO], Sharp | Pros: - Less signaling overhead Cons: - Impact the flexibility of MCS indication; - New rules on split of MCS bit field |
| TPC | QC, WILUS, Nokia, NSB, Xiaomi, InterDigital, Apple | [CATT, Ericsson, DCM, LG, OPPO] Huawei, HiSilicon, Sharp, Samsung | Pros: - Less signaling overhead Cons: - Impact the flexibility of TPC indication; - New rules on split of TPC bit field |
| CSI request | WILUS | [Ericsson], Nokia, NSB Huawei, HiSilicon, China Telecom, Sharp | Pros: - Less signaling overhead Cons: - Less flexibility as only two repetition factors can be indicated. |
| FDRA | Apple, Intel, WILUS, Sharp, DCN | [CATT, Ericsson, LG], Nokia, NSB, Huawei, HiSilicon, Samsung, xiaomi | Pros: - Less signaling overhead; - Use similar mechanism defined for NR-U in Rel-16, may require less spec efforts. Cons: - Impact the flexibility of frequency domain resource indication |

Based on the input, FL provides some details for each of the information field for repetition indication as follows.

- If TDRA information field is chosen, introducing a new TDRA table including the repetition factors.

  - The new TDRA table is configure by SIB1, with selecting one of the two options below.

    - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor.

    - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor.

      - FL note: This is supported by current signaling structure of SIB1. The intention is to reduce the SIB1 signaling overhead.

  - If the repetition factor is not included in the TDRA table, the legacy default TDRA table is used, and a predefined repetition factor K is applied.

    - FFS the value of K, e.g., K=1.

    - FL note: if a new default TDRA table is supported, it would have too many spec impacts and it expects very difficult to conclude.

- If MCS information field is chosen, repurpose the MCS information field by selecting one of the two options below.

  - Option 1: X MSB bits of the MCS information field is used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

  - Option 2: A predefined MCS table with including repetition factor K is introduced.

    - FFS the value of K for each row of the MCS table.

    - FL note: it needs to define different MCS tables with or without enabling transform precoding.

- If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below.

  - Option 1: X LSB bits of the TPC information field is used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

    - FL note: the 2 LSB bits of TPC are used for indicating non-positive values in Rel-16.

  - Option 2: A predefined TPC command table with including repetition factor K is introduced.

    - FFS details.

    - FL note: [12, Qualcomm] and [16, Nokia/NSB] provide the detailed design as follows respectively.

| TPC command | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TPC value [dB] | 2 | 4 | 6 | 8 | 2 | 4 | 6 | 8 |
| Number of repetitions | 2 |  |  |  | 4 |  |  |  |

- If FDRA information field is chosen, the X bits for FDRA is truncated and used for repetition indication.

  - FFS the value of X.

  - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

First round

For indication of the number of repetition of Msg3 initial transmission, FL encourage companies to provide comments for the following aspects.

- Update your company position in Table 2.2-1 if needed.

- FL strongly encourages companies to focus on TDRA/MCS/TPC based solution. Any concern?

- It seems only one company proposes to use ‘CSI request’ information field, while several companies have concerns. Note, this doesn’t preclude to use ‘CSI request’ information field for indicating the legacy interpretation or repurposed interpretation, which will be discussed under Issue#7.

- Several companies show interests on using ‘FDRA information field’ while they also have alternative preference for other information field.

- For TDRA/MCS/TPC based solution, there are two options listed above. Which option do you prefer?

- Any other views?

| Company | Comments |
| --- | --- |
| Samsung | 1. update to position for concern, we have concerns on FDRA and TPC; 2. for MCS, we prefer option 1, link specific repetition number and specific MCS index is again putting additional constraints; 3. even though the concerns for repurpose existing field has all “impact the flexibility”, we feel the impact might be quite different, e.g., for TPC, all the values will be needed regardless of covenh or not as discussed in our tdoc, thus the impact will be large; and for MCS, some index will be much less necessary in coverage-limited case, thus the impact will be quite small. That’s why we prefer MCS. |
| Sharp | a. Update on our preference b. For msg3, only small number of RBs would be allocated. Therefore, FDRA is one possibility. c. We are fine with either for TDRA-based indication |
| Nokia/NSB | In our view, specification effort for repurposing MCS and/or TPC tables is lighter than the specification effort to repurpose TDRA. It could be claimed that both approaches require agreeing on which repetition value is mapped to each codepoint of the corresponding information field. Indeed, this would apply to TDRA MCS and/or TPC, with no conceptual difference. However, while one codepoint of the MCS and TPC information field is associated to a single parameter which affects only the single UE UL performance, i.e., the MCS index and the TPC command, each codepoint of the TDRA information field is associated to three values potentially affecting the UL performance of several UEs in the cell, i.e., mapping type, K2 and SLIV. In this context, finding ways to minimize impact on the flexibility for the scheduler at gNB and possible UL cell performance reduction, while maximizing performance of the Msg3 transmission for the single UE is nontrivial and could take a long time. Situation above would be even worse if some codepoints of the TDRA information field (or, equivalently, some rows of the TDRA table) were to be mapped to repetition value K=1 (to ensure gNB can decide not to schedule Msg3 repetitions to a given UE, even if a request in this sense has been issued by UE via Msg1). This would imply that only very limited choices in terms of mapping type, K2 and SLIV would be available at gNB in this case, instead of the legacy 16 choices. |
| Intel | We support MCS or FDRA based approach to indicate the number of repetitions for Msg3 PUSCH initial transmission. Using FDRA is similar to what was defined for NR-U and the spec impact is similar to MCS based indication. For MCS based approach, we support Option 1. We do not think we need to define a new MCS table, which would minimize the spec impact. For TDRA based approach, we share similar view as Nokia that spec impact for TDRA based approach has large spec impact or signalling overhead, e.g., either configuration from SIB for TDRA or defining a new table. |
| Qualcomm | For TDRA based approach, we share the same views as Intel & Nokia. In addition, it should be clarified that the new TDRA is optionally configured, and a default behaviour has to be specified. FL: It is clarified by the following texts for TDRA based solution. - If the repetition factor is not included in the TDRA table, the legacy default TDRA table is used, and a predefined repetition factor K is applied. - FFS the value of K, e.g., K=1. - FL note: if a new default TDRA table is supported, it would have too many spec impacts and it expects very difficult to conclude. |
| LG Electronics | We are fine with down-scoping as proposed candidates (i.e., TDRA, MCS, TPC). Also, we prefer to use TDRA table which is similar with the indication method for normal PUSCH repetition. In addition, we prefer to reuse legacy table as far as possible to minimize spec impact. |
| Apple | We support MCS, TCP, FDRA (with lower priority). For TDRA based scheme, we share similar views as Nokia/NSB. |
| NTT DOCOMO | Number of available bits representing the repetition number is important. Small number of available bits confines the selection of candidate repetition values. To achieve the flexibility in scheduling msg3 repetition, the information field for repetition indication should be large enough to partition bits for other purposes on top of the main purpose. From this aspect, TDRA, MCS, and FDRA could be potential information fields for repetition indication. Anyway, this discussion should be treated after deciding the candidate values of repetition factors in our view. On the other hand, the overhead of SIB1 is one of concerns. If complete new TDRA table (Opt1) is supported, the large overhead increase is expected. Hence, Option 2 is preferred, if TDRA is supported. |
| OPPO | We support TDRA based solution. TDRA based repetition number indication has been used for URLLC PUSCH transmission in Rel-16. We do not see much standard efforts to reuse this existing solution. For MCS, TCP, FDRA based solution, RAN1 need study how many bits should be repurposed for repetition number indication, and how the left bits are used for legacy purpose. It will require more standard efforts than TDRA based solution. |
| InterDigital | Our preferences are TPC and TDRA. |
| Panasonic | We are fine to focus on TDRA/MCS/TPC based solution for further discussion. We support TDRA based solution and Option 2 is preferred. |
| Xiaomi | TDRA and TPC fields are our preference. For TDRA based scheme, option 2 introduces less signalling overhead, which is the better choice than option 1. For FDRA based approach, due to its inflexible scheduling, the frequency domain selective gain may be affected. |
| China Telecom | a. We prefer TDRA and MCS. c. For TDRA, we prefer option 2. For MCS, we prefer option 1. |
| Ericsson | TDRA based method is clean, simple and good enough, there’s no need to change the meaning of DCI fields nor RAR fields for repetition factor signalling. We do not see SIB1 PDSCH is a bottleneck channel compared to PUSCH and do not believe adding a TDRA table in SIB1 will make SIB1 PDSCH cannot survive (also note that all the newly added TDRA tables are optional), instead RAR/DCI repurposing should be tried to be avoided unless really necessary. |
| CATT | TDRA based method (+ reserved CSI field indicating the chosen TDRA table between legacy and new TDRA table) is preferred for better flexibility, and less impact to other DCI fields for other purpose. |
| WILUS | a. We updated our views. b. For ‘CSI request’ field, we’ve proposed to use multiple bit fields to have less impact on flexibility of it’s original information field. For ‘FDRA’ field, some bits/code-points can be repurposed to indicate the repetition number since CE UE may be scheduled with small number of PRBs. c. Among three options, we prefer at least TPC-based indication. FL: I would suggest to keep TPC based solution on the table while exclude CSI based solution. I assume this could be acceptable for you. |
| vivo | b. For repurposing MCS field, the field length (4 bits) in RAR UL grant is already shorter than that in DCI format 1-0 with TC-RNTI, i.e., 5 bits. Further limiting the bit field length is not preferred. And if repurposed, it may result to different indication method for initial transmission and retransmission for Msg3 PUSCH repetition. c. TDRA based solution is simple and straightforward. And to minimize the overhead in SIB, default TDRA table with repetition can be considered. Besides, repetition number can be configured on top of legacy k2, SLIV. |
| Huawei, HiSilicon | 1.Support MCS information field 2. For MCS information field, we prefer option 1. 3. Regarding the TDRA solution listed in the FL summary, if a new TDRA table is introduced in SIB1, the increase of payload of SIB1 cannot be acceptable. On the other hand, reusing the legacy TDRA table or defining a new default TDRA table for Msg3 repetition, the scheduling flexibility will be degraded. Therefore, the TDRA solution is not good. |
| FL | - No companies showed concerns to remove CSI and FDRA based solution as long as their other alternative preference is still on the table. - No concerns about the pros&cons in Table 2.2-1, though different companies may have different understandings on the flexibility impact for different information filed. - It seems no companies show support for Option 2 of MCS based solution. So, Option 2 is deleted. Based on above, FL suggests discussing the following proposal. Proposal for Issue#5 Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, introducing a new TDRA table including the repetition factors. - The new TDRA table is configure by SIB1, with selecting one of the two options below. - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor. Support: Sharp, CATT, xx - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor. Support: Sharp, China Telecom, CATT, Panasonic, xx - If the repetition factor is not included in the TDRA table, the legacy default TDRA table is used, and a predefined repetition factor K is applied. - FFS the value of K, e.g., K=1. - Alt 2: If MCS information field is chosen, repurpose the MCS information field by selecting one of the two options below. - X MSB bits of the MCS information field is used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below. - Option 1: X LSB bits of the TPC information field is used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. Support: WILUS - Option 2: A predefined TPC command table with including repetition factor K is introduced. - FFS details. Support: xx FL suggests companies to be constructive as there only two meetings left, while there are still many FFS points to be addressed for each alternative. So, please comment only if you have strong concerns on Proposal for Issue#5. While for proponents of TDRA or TPC based solution, it is encouraged to further provide your preference with adding your companies’ name under the two options listed above. It would be helpful for our further discussion. |
| FL | The proposal is updated with correcting several typos. Proposal_v1 for Issue#5 Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, introducing a new TDRA table including the repetition factors. - The new TDRA table is configured by SIB1, with selecting one of the two options below. - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor. Support: Sharp, CATT, xx - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor. Support: Sharp, China Telecom, CATT, Panasonic, DCM, xx - If the repetition factor is not included in the TDRA table, the legacy default TDRA table is used, and a predefined repetition factor K is applied. - FFS the value of K, e.g., K=1. - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows by selecting one of the two options below. - X MSB bits of the MCS information field is are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below. - Option 1: X LSB bits of the TPC information field is are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. Support: WILUS, InterDigital (1st preference), DCM - Option 2: A predefined TPC command table with including repetition factor K is introduced. - FFS details. Support: InterDigital (2nd preference) |
| InterDigital | Our view is indicated in the FL’s entry above. |
| LG | Support Proposal_v1 for Issue#5 and prefer Option2 in Alt1. |
| FL | We have reached the following WA in GTW session for the first round of discussion. Working Assumption Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors. - The new TDRA table is configured by SIB1, with selecting one of the two options below. - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor. - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor. - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied. - K=1. - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - X MSB bits of the MCS information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below. - Option 1: X LSB bits of the TPC information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Option 2: A predefined TPC command table with including repetition factor K is introduced. - FFS details. |

Second round

Before down-selection, FL suggests discussing the FFS points of each alternative first, to make sure companies all understand each alternative better.

Q1:

For TDRA based solution, FL invites the proponents to provide your preference about Option 1/Option2 in the following table, and what’s your views if any about the pros&cons summarized below.

                - Option 1 has relatively larger SIB1 signaling overhead compared to Option 2.

                - Option 2 has less flexibility compared to Option 1, as Rel-17 CE UEs and legacy UEs share the same configuration (K2, mapping type and SLIV).

For TPC based solution, FL invites the proponents to provide your preference about Option 1/Option2 in the following table, and what’s your views if any about the pros&cons summarized below.

                - Option 2 is more like a super set of Option 1, depending how to design the predefined TPC command table.

                - Option 2 would require relatively more spec efforts, as it needs to define a totally new TPC command table with including repetition factor K.

Note: For MCS/TPC based solution, the value of X and how to use X highly depend on the candidate values for repetition factor. Thus, the FFS points will be discussed after more progress on Issue#9.

| Information field | Support | Concern | Detailed views |
| --- | --- | --- | --- |
| TDRA | Sharp, CATT, Ericsson, vivo, DCM, Xiaomi, Panasonic, ZTE, CMCC, OPPO, China Telecom, InterDigital, Samsung, ETRI, LG | Apple, Nokia, NSB, Intel, Huawei, HiSilicon, Qualcomm | Option 1: Sharp, CATT, Ericsson Option 2: Sharp, China Telecom, CATT, Panasonic, DCM, OPPO, Xiaomi, LG, Lenovo, Motorola Mobility |
| TPC | QC, WILUS, Nokia, NSB, Xiaomi, InterDigital, Apple | [CATT, Ericsson, DCM, LG, OPPO] Huawei, HiSilicon, Sharp, Samsung | Option 1: WILUS, InterDigital (1st preference), DCM, Xiaomi, WILUS Option 2: InterDigital (2nd preference), Nokia/NSB, Qualcomm |

| Company | Comments |
| --- | --- |
| China Telecom | For TDRA based solution, we prefer option 2. Since SIB1 is a broadcast information, we think option1 cost too much overhead. |
| CATT | Since the trade-off is rather obvious, i.e. payload v.s. flexibility, probably a middle ground of Option 1 and Option 2 can be considered: (1) If the new TDRA table (including separate new indication for K2, mapping type, SLIV and repetition factor) is indicated in SIB1, then use the new TDRA table. (2) Else if such new TDRA table in (1) is not indicated, and instead a set of repetition factor is indicated, then use the legacy TDRA table + new repetition factor. |
| Sharp | Preference: Either Option 1 and Option 2 Reason for Options: SIB1 overhead is not a big issue. SIB1 can take up to 2976 bits. Up to tens of bits of increased overhead due to configuring new TDRA table shouldn’t be an issue. If we agree with TDRA-based approach, the only change would be like copy-paste of Rel-16 TDRA table for repetition type A into SIB1. Commonality with Rel-16 features would be maintained. |
| OPPO | Our preference is TDRA based solution. Option 2 is slightly preferred. For TDRA based solution, option 1 may require more overhead in SIB1. The overhead issue can be further checked in RAN2 if option 1 is agreed in RAN1. For option2, the overhead of only new repetition factor is neglectable. It is acceptable for us. For the issue on the TDRA flexibility of option2, TDRA table is also shared with legacy UE for TPC/MCS based solution, which is the same situation with option2 of TDRA based solution. On the other hand, TDRA based solution has no spec efforts for the repurpose of bit fields in RAR UL grant. For TPC/MCS based solution, TDRA table is also shared with legacy UE, which is the same situation with option2 of TDRA based solution. As FL summarized, it requires more spec efforts, as it needs to define a totally new TPC command table with including repetition factor K, at least for option 2. It is not preferred for us. |
| Xiaomi | For TDRA based solution, option 2 with less signalling overhead is our preference. For TPC-based option 1, part or all of the TPC bits are directly used to indicate the number of repetitions is our preference. |
| Ericsson3 | Option 2 requires the legacy TDRA table must be configured so that new table can be configured, while legacy TDRA table is actually optional in current spec. should be also be absent when Rel-1 TDRA table is configured. Option 1 means the R17 TDRA table is independent from the legacy TDRA table and is more flexible. It does not require the old TDRA table must be configured for configuring a new table. This doesn’t mean overhead is larger either, instead, if old TDRA table is not configured, while new TDRA table is supported, the signalling overhead is reduced. By the way, we’re not convinced by the argument (without justification from evaluations) that introducing a new optional TDRA table on top of the legacy optional TDRA table will make SIB1 dead or the link budget of downlink SIB1 PDSCH will be worse than uplink PUSCH. Furthermore, it’s completely up to gNB to schedule which table should be present in SIB1 or not, same as legacy. Given above, option 1 with TDRA based method is preferred by us. |
| NTT DOCOMO | For TDRA approach, Option1 and Option2 are trade-off relationship between signaling overhead and scheduling flexibility. For MCS/TPC, Option 1 can be viewed as one mapping example of Option2, because nothing about Option2 design has been decided yet. In other words, Option1 has constraints that indication bits for MCS/TPC and repetition factors are separate, while Option2 are not limited by it. In our views, Option 1 is preferred, because mapping determination in Option 2 is more controversial and Option 1 is flexible enough. |
| WILUS | For TPC based solution, Option 1 seems to be aligned with our preference. Repetition number can be indicated in more flexible way via X LSBs within the configured repetition factors, which are discussed in Issue#9. |
| LG | We prefer option 2 which can reuse legacy table as far as possible to minimize spec impact. |
| Lenovo, Motorola Mobility | We prefer option 2 of TDRA based approach for the sake of lower signaling overhead and less standard impact. |

Q2: For proponents of Option 2 of Alt 3 (TPC based solution), please elaborate your detailed design below.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Effort for TPC-based Option 2 is much lower than effort for TDRA-based Option 1. The scale is absolutely not comparable. Option 1 for TDRA-based requires the definition of a new table with 4 parameters per row to agree on, and 16 rows. Granularity of each parameter is also extremely fine and a huge amount of combinations could be considered/proposed. FL: For TDRA-based solution, each row of the TDRA is configured by SIB1. There is no need to define each row in a predefined manner. Conversely, Option 2 for TPC-based Option 2 has just 2 parameters to agree on, and their granularity is very coarse. The rationale behind their selection would also be more straightforward and “objective”, i.e., it would just be a rather simple mapping exercise (please see examples that Nokia/NSB or Qualcomm proposed) with a very limited number of physically meaningful solutions. In other words, out of the two options for TDRA-based, which is the one who reduces scheduling options for gNB by construction, the only one which seem feasible in terms of required time to agree is the one that offers the least flexibility of the two. This shows the limitations of this approach even more. The same is not true for the TPC-based (and for the MCS-based) solution. We think acknowledging this in the description of the two options would be fairer. |
| Qualcomm | Similar to Nokia, we think TPC-based Option 2 needs much lower effort than TDRA-based approach. We think a single predefined alternative table for TPC can be used, and the usage of this alternative table is just based on the request of Msg3 repetition by UE (i.e. no separate indication is needed for alternative interpretation of the TPC bitfield). Example of this alternative TPC table is the following: |

Update

Based on companies input, the overall situation of the WA is summarized below. It would be quite changeable to down select of the alternatives right now, especially considering the dependency of Issue#9.

The discussion is still open, if any additional comments/suggestions for the summarized table below.

| Information field | Support | Concern | Detailed views |
| --- | --- | --- | --- |
| TDRA | Sharp, CATT, Ericsson, vivo, DCM, Xiaomi, Panasonic, ZTE, CMCC, OPPO, China Telecom, InterDigital, Samsung, ETRI, LG | Apple, Nokia, NSB, Intel, Huawei, HiSilicon, Qualcomm | Option 1: Sharp, CATT, Ericsson Option 2: Sharp, China Telecom, CATT, Panasonic, DCM, OPPO, Xiaomi, LG, Lenovo, Motorola Mobility Pros: - Use similar mechanism defined for regular PUSCH repetition in Rel-16, which may require less spec efforts Cons: - Impact the flexibility of time domain resource indication; - Either more signaling overhead (Option 1) or may have impact on legacy UEs (Option 2) |
| MCS | Apple, Intel, Samsung, Huawei, HiSilicon Nokia, NSB, DCM, China Telecom, WILUS | [CATT, Ericsson, LG, OPPO], Sharp | Pros: - Less signaling overhead or impact to legacy UEs Cons: - Impact the flexibility of MCS indication; - New rules on split of MCS bit field |
| TPC | QC, WILUS, Nokia, NSB, Xiaomi, InterDigital, Apple | [CATT, Ericsson, DCM, LG, OPPO] Huawei, HiSilicon, Sharp, Samsung | Option 1: WILUS, InterDigital (1st preference), DCM, Xiaomi, WILUS Option 2: InterDigital (2nd preference), Nokia/NSB, Qualcomm Pros: - Less signaling overhead or impact to legacy UEs Cons: - Impact the flexibility of TPC indication; - New rules on split of TPC bit field |

| Company | Comments |
| --- | --- |

[Closed] Issue#6: Indication of the number of repetitions for Msg3 re-transmission

For Msg3 re-transmission, the following agreements were reached for repetition indication in RAN1#105-e.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Companies’ preference for the two options about repetition indication for Msg3 re-transmission are summarized below.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission

  - [1, Huawei, HiSilicon], [ 2, OPPO], [4, ZTE], [5, vivo], [6, CATT], [8, Xiaomi], [13, Panasonic], [14, Samsung] (if initial transmission adopts TDRA based method), [15, ETRI], [17, Ericsson], [22, LG]

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

    - [10, Intel], [11, Apple], [12, Qualcomm], [14, Samsung] (if initial transmission doesn’t adopt TDRA based method), [23, WILUS]

[16, Nokia/NSB]: Discussion on the indication of the repetition number for Msg3 re-transmission should be postponed until a solution for the indication of repetition number for Msg3 initial transmission is agreed.

FL suggests to postpone the discussion on this issue until there is a clear solution for initial transmission.

[Open] Issue#7: How to interpret the information field (legacy or new interpretation)

As for how a UE is managed to know whether to use legacy interpretation or new interpretation on the information field indicating the number of repetitions, there are the following two options discussed in RAN1#105-e.

Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions in RAN1#106-e. Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Option 2: - When a UE requests Msg3 repetition, whether the UE should apply legacy or new interpretation is is determined by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - When the UE doesn’t request Msg3 repetition (including legacy UE), the UE should apply the legacy interpretation. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Companies’ support of each option is summarized below.

- Option 1:

  - Support: [4, ZTE], [5, vivo]?, [7, China Telecom], [10, Intel], [13, Panasonic], [15, ETRI], [17, Ericsson], [23, WILUS]

- Option 2:

  - Support: [6, CATT], [11, Apple], [14, Samsung], [16, Nokia/NSB], [23, WILUS]

  - [6, CATT]: CSI request bit field can be used as an explicit indication of which TDRA table is used for time domain resource allocation.

  - [16, Nokia/NSB]: UL grant interpretation differentiation in case of Msg3 re-transmission scheduling is provided via a specific set/range of TC-RNTI values used to scramble CRC of DCI 0_0 which schedules Msg3 re-transmission over PUSCH.

First round

For the first round of discussion, FL suggests first to discuss on Proposal 7 with no intention for down-selection. Once agreed, we will aim for down-selection in the next round of discussion.

Proposal for issue 7: Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions in RAN1#106-e.

Option 1:

- When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Option 2:

- When a UE requests Msg3 repetition, whether the UE should apply legacy or new interpretation is is determined by gNB.

  - FFS details, e.g. implicit or explicit indication or predefined.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the UE should apply the legacy interpretation. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

| Company | Comments |
| --- | --- |
| Samsung | This issue should wait for the outcome of issue #5; if TDRA table is supported, the clear indication by gNB is needed; but if other method is adopted in issue #5, we may consider the necessity again. FL: Regardless of the outcome of issue #5, we need to discuss how should the UE interpret gNB’s indication. Even if TPC/MCS information field is chosen, whether the UE should apply legacy or new interpretation about the TPC/MCS information field should be discussed, i.e., implicit (Option 1) or explicit (Option 2) way. To address your concern, I deleted ‘in RAN1#106-e’, meaning that we do not have to make down-selection at this meeting (while still be possible based on the overall progress). |
| Sharp | We are generally fine with the proposed direction. We propose to add a note (either Note#1 or Note#2) for clarification under the sub-bullet of the first bullet in Option 1 as follows. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - Note#1: Repetition factor K=1 is determined by the UE when one row in the TDRA table indicates K=1 or when one row in the TDRA table doesn’t indicate information on repetition factor. - Note#2: Exact signaling structure for indication of Repetition factor K=1 is up to RAN2. FL: Option 1 is for both TDRA based solution and solutions based on other information fields, while your proposed note seems only apply to TDRA based solution. FL suggests discussing this under the proposal for Issue#5. If it is possible, one note could be added for one or both option for TDRA based solution, while such details may not be that necessary at this point. |
| Nokia/NSB | We prefer the spirit Option 2, but we are not sure it is formulated in a very clear way (please see below). Arguments supporting our preference are the same we explained for Issue#5 and in R1-2106659. We have a further comment. Now, we are not sure that the logic of the proposal is 100% accurate (this impacts both Option 1 and Option2). As far as we are concerned, gNB decides what UEs needs to do both in case of Option 1 and Option 2. The difference between the two Options is related to how the indications about the “whether Msg3 repetitions have been configured” and the “how many Msg3 repetitions have been configured” are provided by gNB to UE. We would like to check if this understanding is the same as the one other companies have. In our view: - In Option 1, the TDRA table provides to the UE indication both about the “whether Msg3 repetitions have been configured” and on the “how many Msg3 repetitions have been configured” at the same time. The difference is then in terms of UE behaviour and in how many repetitions can be configured (this includes the value K=1). In fact, a UE which previously requested Msg3 repetitions via Msg1 will find boalways use the new TDRA table, regardless of whether Msg3 repetitions are configured or not by gNB (the UE cannot know beforehand). A which did not request repetitions will keep using legacy table. - FL: Yes. Your understanding on Option 1 is correct. - In Option 2, the indication about the “whether Msg3 repetitions have been configured” and on the “how many Msg3 repetitions have been configured” are separate. One indication informs the UE about the “whether”, i.e., what we refer to as “UL grant interpretation”, and one indication informs the UE about “how many Msg3 repetitions have been configured”, i.e., the corresponding information field used to this end. A UE which previously requested Msg3 repetitions via Msg1 will always first check which UL grant interpretation is to be used (i.e., the “whether Msg3 repetitions have been configured”). Depending on this indication, the UE will interpret the information field possibly used to indicate the “how many Msg3 repetitions have been configured” accordingly (either in a legacy or new way). - FL: Yes. Your understanding on Option 2 is correct. Finally, if current proposal is kept as is, please note that the verb “is” is repeated in Option 2: “When a UE requests Msg3 repetition, whether the UE should apply legacy or new interpretation is is determined by gNB” FL: Thank for spotting the typo. Corrected below. |
| Intel | We are fine with the proposal. |
| Qualcomm | We think the new interpretation of the bitfield/s for repetition indication should be only based on the repetition request from UE (if UE request repetition, new interpretation should be used, otherwise, legacy should be used). So, no separate indication is needed for new interpretation of bitfield/s. Whether that bitfield is TDRA or another bitfield is a separate issue. The same applied to both initial Msg3 and the retransmission. |
| Apple | We support Option 2, as Option 1 further limits the indication of repetition factor (given that no repetition under option 1 is as part of available states). Also, given that we are considering a UE in coverage limited and assessments from such a UE is not reliable, it should be left to gNB to eventually indicate a new interpretation of the fields or “completely” same interpretation as R15/16 (so UE demands by PRACH is not sufficient in our view to guarantee gNB sets new interpretation of RAR UL grant bit-fields). |
| NTT DOCOMO | We think which field is used for the number of repetitions should be agreed, before down-selecting these options. As some information fields do not have affordable bit widths for repetition factor indication, excluding repetition factor equal to 1 might be helpful. In that case, option 2 is preferred. Otherwise, Option 1 should be supported for simplicity. FL: As you may notice, it suggests precluding CSI bit field in the latest proposal for issue #5. To address your concern, I deleted ‘in RAN1#106-e’, meaning that we do not have to make down-selection at this meeting (while still be possible based on the overall progress). |
| OPPO | We support option 1. |
| Panasonic | We are fine with the proposal and support Option 1. |
| Xiaomi | We are fine with the proposal, and option 1 is our preference. |
| China Telecom | We are fine with this proposal and we support option 1. |
| Ericsson | Looks fine. In our understanding, option 1 is enough to avoid any additional DCI/RAR changes, since gNB will make sure when a separate PRACH resource and a separate TDRA table for Msg3 repetition will be configured, once UE selects the separate PRACH resource, the TDRA table for msg3 repetition should be used. |
| CATT | We are fine with the spirit of this proposal, and support Option 2. We share the same view as Apple. Option 1 will introduce significant scheduling restriction. If a UE requested Msg3 PUSCH repetition and a TDRA table with repetition column is always applied, the SLIV can be used for Msg3 PUSCH transmission is limited when no repetition is needed. This means that any UE is able to forcing the gNB to schedule Msg3 with new TDRA table with repetition, which is unacceptable to us. On the interpretation by Nokia, we think that, in Option 1, a UE which previously requested Msg3 repetitions via Msg1 will always use new TDRA table. Whether and how many repetitions will be applied depends on the indicated TDRA entry in the new TDRA table (indicated by TDRA field). |
| WILUS | We support the proposal. |
| Vivo | Fine with the proposal, and we support opt-1. |
| ZTE | Fine with the proposal, with preference on Option 1. |
| FL | Minor updates are made based on companies’ input. Proposal_v1 for Issue#7: Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions in RAN1#106-e. Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Support: Sharp, Qualcomm, OPPO, Panasonic, Xiaomi, Ericsson, China Telecom, vivo, ZTE, Intel, WILUS?, Samsung (if the indication of repetition number is not in TDRA table), DCM (if the indication of repetition number is in TDRA table), LG Option 2: - When a UE requests Msg3 repetition, whether the UE should apply legacy or new interpretation is is determined by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - When the UE doesn’t request Msg3 repetition (including legacy UE), the UE should apply the legacy interpretation. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Support: Nokia/NSB, Apple, CATT, Samsung (if the indication of repetition number is in TDRA table) , DCM (if the indication of repetition number is not in TDRA table) The intention is not to down-select now. Please comment only if you have strong concerns on Proposal_v1 for Issue#7. |
| Samsung 2 | To us, the option 1 should be adopted if the indication of repetition number is not in TDRA table, e.g., MCS bits explicitly indicating the repetition number; and the option 2 should be adopted if the indication of repetition number is in TDRA table; so it’s difficult for us to put name in either of the options for now. FL: Thanks for the further clarification. I captured ‘Samsung (if the indication of repetition number is not in TDRA table)’ for Option 1, and ‘Samsung (if the indication of repetition number is in TDRA table)’ for Option 2. |
| CATT | Though we are still worrying the gNB may be forced to use new TDRA table as requested by the UE in Option 1, we are fine to have such proposal for progress. However, since this issue is tightly based on the outcome in issue#5 (as also point out by Samsung), we hope to see further progress in issue#5 first before down-selection here. FL: We will not down-select before more progress on issue#5. |
| Nokia/NSB2 | We would like to ask a clarification. Is Option 2 automatically implying that K=1 is not part of the values gNB can indicate to the UE for scheduling repetitions, given that this indication is implicit if gNB indicates (implicitly or explicitly) which interpretation is to be used by UE? This seems to be the general understanding, but we want to confirm it. Thanks. FL: Yes. Your understanding is correct. |
| Ericsson2 | Option 1. |
| Intel | We support Option 1. |
| LG | Support Proposal_v1 for Issue#7 and prefer Option1 |
| FL | Based on the discussion by email among (Nokia, CATT, Intel, Ericsson, vivo, Xiaomi, Sharp), Option 2 is further updated, with clarifying the main bullet to align with the wording of Option 1 and add one sub-bullet to highlight the main difference compared to Option 1. Proposal_v5 for Issue#7: Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field. |

Second round

The following proposal is also under check for potential email approval.

Proposal_v5 for Issue#7:

Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions.

Option 1:

- When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Option 2:

- When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB.

  - FFS details, e.g. implicit or explicit indication or predefined.

  - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

| Company | Comments |
| --- | --- |
| FL | Please comment only if you have strong concerns. |

[Closed] Issue#8: Support fallbackRAR UL grant in 2-step RACH for indicating Msg3 repetition

There is an FFS point in RAN1#104-e about whether to support fallbackRAR UL grant in 2-step RACH for indicating Msg3 repetition, and if supported no specific optimization is considered.

Agreements: - For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below. - Option1: UL grant scheduling Msg3. - FFS details. - FFS fallbackRAR UL grant. - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported.

Whether to support fallbackRAR UL grant in 2-step RACH for indicating Msg3 repetition would impact the RAN2 discussion on related RACH procedure. In [4, ZTE], it is proposed to not support fallbackRAR UL grant in 2-step RACH for indicating Msg3 repetition, with the following reason.

- Given a UE requires separate PRACH preambles for requesting Msg3 repetition, in addition to the separate preambles for 4-step RACH, it would need additional preambles dedicated for 2-step RACH if Msg3 repetition is supported under fallback mode. It means a UE needs first to use such dedicated separate preambles for 2-step RACH for capability reporting, and then gNB may or may not schedule Msg3 repetition only if the UE is witched to fallback mode. In addition, it would also impact on RACH procedure selection, e.g., a new RSRP threshold for triggering Msg3 repetition for 2-step RACH may be needed. All of these would conflict the agreement about not considering any specific optimization. Thus, it is proposed to not support fallbackRAR UL grant in 2-step RACH for indicating Msg3 repetition.

First round

Companies are encouraged to provide views on the following proposal.

Proposal for Issue#8: Do NOT support fallbackRAR UL grant in 2-step RACH for indicating Msg3 repetition.

| Company | Comments |
| --- | --- |
| Samsung | Seems fine. |
| Sharp | Support |
| Nokia/NSB | Support. |
| Intel | We are fine with the proposal. 2-step RACH is not targeted for coverage enhancement. |
| Qualcomm | Support |
| LG Electronics | We are fine with FL’s proposal for issue 8. |
| NTT DOCOMO | Support the proposal. |
| OPPO | Fine with the proposal |
| Panasonic | We are fine with the proposal. |
| Xiaomi | Support |
| Ericsson | For fallback RAR scheduled PUSCH, when fallback happens, it means the coverage is already poor, it would be good to not preclude such msg3 repetition. For CBRA of 2-step RA, when fallback happens, gNB is not aware of whether UE is capable of msg3 repetition, so no msg3 repetition scheduling should be assumed to avoid any 2-step RA specific optimization. However, for CFRA of 2-step RA, when fallback happens, gNB can be aware of the UE capability based on capability report in RRC connected state. At least in this case, msg3 repetition can be supported to improve the reliability of msg3 after it (as MsgA PUSCH) already failed during 2-step RA. FL: Firstly, as commented by companies, 2-step RACH is not targeted for coverage enhancement. Even in fallback mode, it may be still in relatively good coverage compared to the case for requesting 4-step RACH with Msg3 repetition. In addition, it would also impact on RACH procedure selection, e.g., a new RSRP threshold for triggering Msg3 repetition for 2-step RACH may be needed. As long as it is open for support Msg3 repetition for 2-step RACH, it will have big impacts about RAN2 discussion. So, it’s better to give more guidance for RAN2 discussion as soon as possible. |
| CATT | Fine with the proposal. This seems related to Issue#2-3. |
| WILUS | We support the proposal. |
| Vivo | Support. |
| ZTE | Support |
| FL | Based on the input, FL feels current Proposal for Issue#8 is the right direction to go. FL encourages companies could be constructive here. Thanks. |
| Ericsson2 | We can understand that in CBRA case, gNB is not aware of which UE has msg3 repetition or not and to avoid 2-step RACH CBRA resource partitioning, it’s better to not support this msg3 repetition scheduled by fallback RAR. For CFRA case, it’s already known by gNB if the capability of msg3 repetition is reported, in such case there’s no need to introduce new RSRP threshold nor separate PRACH resource. Although we do not think such msg3 repetition scheduled by fallback RAR can be treated as a 2-step RACH enhancement since the 2-step RA already fallbacks to 4-step RA and the PUSCH is Msg 3, and we do not believe when fallback happens the UE is still in a good coverage mentioned by feature leader, we’re also fine to not support this given no other companies want it to reduce some RAN2 work. |
| Apple | Support FL’s proposal |
| FL | The issue is closed with reaching the following agreements. Agreement Do NOT support fallback RAR UL grant in 2-step RACH for indicating Msg3 repetition. |

[Open] Issue#9: Candidate values for Msg3 initial/re-transmission repetitions

In Rel-16, the candidate values for the number of repetitions of PUSCH repetition Type A/B are copied as follows. The maximum number of repetitions for enhanced PUSCH repetition type A in Rel-17 is 32 as agreed in AI 8.8.1.1.

numberOfRepetitions-r16 ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

For Msg3 repetition, the candidate values including the maximum number of repetitions should be discussed.

- [1, Huawei, HiSilicon]: The maximal repetition number up to 16 can be considered for Msg3 PUSCH repetition.

- [5, vivo]: The set of Msg3 PUSCH repetition numbers should be 1, 2, 4. The performance gain is 2.79dB and 5.70dB respectively for Msg3 transmission with 2 repetitions and 4 repetitions.

- [6, CATT]: Support {2, 4, 8} for the repetition factors of Msg3 PUSCH repetition.

- [7, China Telecom]: Support at least {1, 8} for the repetition factors of Msg3 PUSCH repetition.

- [8, Xiaomi]: The maximum number of repetitions for type A PUSCH repetition in release 17 can be adopt for Msg.3 repetition.

- [10, Intel]: ~2dB performance gain can be achieved for Msg3 PUSCH when the repetition level is doubled.

- [16, Nokia/NSB]: Support K=1 via R17 UL grant according to legacy interpretation and K = {2,4} indicated via R17 UL grant according to interpretation based on repurposed information fields.

  - FFS whether other values, e.g., {8, 12}, are supported

  - Msg3 repetitions yield non-negligible coverage benefits which increase with the number of repetitions, however diminishing returns are observed for N>12.

  - Msg3 repetitions yield coverage benefits at the cost of higher latency, possible lower efficiency, and flexibility of UL resources utilization prior to RRC connection.

- [18, Sharp]: RAR UL grant should indicate a single value from 1, 2, 4 and 8

- [20, NTT DOCOMO]: Make an agreement on the number of candidate values of Msg3 repetitions, before down-select the information field indicating the number of repetitions.

First round

As pointing out by several companies, the number of candidate values for repetition factor K for Msg3 repetition would have impacts on the down-selection of the information field for indicating K, and therefore should be discussed first. Based on the input, FL suggests discussing the following proposal as a starting point. Though we may not be able to decide the final candidate values at this point, proving a general direction would be also helpful.

Companies are encouraged to provide views on the following proposal.

Proposal for issue 9: Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition.

  - FFS whether to support other values, e.g., 1 or 8.

FL note: Support of K=1 would depend on the decision on Issue#7. So, there is no need to comment on whether or not to support K=1 here.

| Company | Comments |
| --- | --- |
| Samsung | Seems fine. |
| Sharp | We are OK with FL proposal. |
| Nokia/NSB | We fully understand the spirit of the proposal, and agree on the proposed numbers but we think value “1” would be supported either way, right? The question should be if “1” is explicitly indicated or simply the result of legacy ehaviour, i.e., no repetitions. Is this understanding is correct? If yes, can we rephrase as follows? Proposal for issue 9: Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition. - FFS whether to support other values, e.g., 1 or 8. FL note: How to support K=1 would depend on the decision on Issue#7. So, there is no need to comment on whether or not to support K=1 here |
| Intel | We are fine with the proposal. |
| Qualcomm | We do not support the proposal. We think in some scenarios, larger repetition number (e.g. 16) will be needed. Indeed, for NTN it might be useful to also include 32. FL: It is still open based on FFS, while we are trying to conclude at least on the ones with common understanding now. We think we should first discuss if the set of repetition numbers is a fixed set or one set among multiple options can be selected by gNB (configured by SIB1). FL: FL thinks this has already been included in the proposal for Issue#5. |
| LG Electronics | We are fine with FL’s proposal for issue 9. |
| NTT DOCOMO | Support the proposal. |
| OPPO | Fine with the proposal. For the support of K=1, we think it is also related to the option 1 of issue 7. |
| Panasonic | We are fine with the proposal. |
| Xiaomi | We share the same view as Qualcomm. FL: Same comments as replied to Qualcomm. |
| China Telecom | We are generally fine with the proposal. As for the value “1”, it is related to Issue #7. Also, we share the similar view as Qualcomm’ comment, if the set of repetition numbers is a fixed set, we think at most 4 kinds of repetition numbers can be configured. If the set can be selected, we are open to discuss it. |
| Ericsson | It’s up to gNB to configure the repetition factors in a TDRA table, similar to what we did in NR Rel-16, the maximum number of repetitions should be the same as a normal PUSCH repetition in Rel-16, i.e. 16. FL: It is still open based on FFS, while we are trying to conclude at least on the ones with common understanding now. |
| CATT | Fine with the proposal and Nokia’s update. |
| Vivo | Support the proposal. And K=1 should also be supported. |
| Huawei, HiSilicon | 8 should be supported. If any transmission in a counted available slot will be omitted, 4 repetitions can be underestimated and insufficient. Share similar view with Nokia, “1” should be removed from the subbullet. |
| ZTE | Prefer to support ‘8’. |
| FL | The proposal is further updated as follows based on the input. Proposal_v1 for Issue#9: - Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition. - FFS whether to support other values, e.g., 1 or 8. - Note: K=1 is supported and how to support K=1 is FFS. |
| CATT | Fine with the proposal. |
| Panasonic2 | We are fine with the Proposal_v1. |
| Ericsson2 | All candidate values of number of repetitions used by legacy normal PUSCH repetition can be supported, up to 16. |
| Intel2 | We are fine with Proposal_v1 |
| LG | We support Proposal_v1 for Issue#9. We prefer to keep K = 1 also we don’t see the motivation of using K = 8. So the motivation of K = 8 should be clarified if it is supported. |
| FL | Proposal_v1 for Issue#9 is under check for potential email approval. |

Second round

Based on the guidance from Chairman, we need make more progress on this issue before we making down-selection of the information field for repetition indication.

For TDRA based solution, it’s clear that the repetition factor is configurable via SIB1. In such case, the follow-up questions would be: Do we really need to limit the repetition factor has to be a small set of values? Do you think NR Rel-16 repetition factor set {1, 2, 3, 4, 7, 8, 12, 16} could be reused here, as it anyway could be up to network configuration?

For MCS based solution or option 1 of TPC based method, there are X bits used for repetition indication. There could be the following three indication ways:

- the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor).

- the X bits are used for selecting one repetition factor from a predefined set.

- the X bits are used for selecting one repetition factor from a SIB1 configured set.

For 1), the value of the repetition factor would be no larger than 2^X.

For 2), the overall number of candidate values is limited to 2^X or 2^X+1, depending on how to support K=1. It can be a subset of Rel-16 set {1, 2, 3, 4, 7, 8, 12, 16}. Further discussion on which sub-set is needed.

For 3), similar to TDRA based solution, do we need to further limit the the candidate value for each chosen repetition factor compared to the Rel-16 set, as long as the overall number of configured repetition factors is no larger than 2^X. In other words, could NR Rel-16 repetition factor set {1, 2, 3, 4, 7, 8, 12, 16} also be considered, while SIB1 can configure a sub-set of 2^X values.

On top of Proposal_v1 for Issue#9, FL would like to ask companies the following questions.

Q1: For the proponents of TDRA based solution, do you agree that NR Rel-16 repetition factor set {1, 2, 3, 4, 7, 8, 12, 16} could also be supported for Msg3 repetition, as it is anyway configurable by SIB1?

| Company | Comments |
| --- | --- |
| China Telecom | For TDRA based solution, since it is configurable, we think there is no need to limit the size of the set. It’s up to gNB to configure the proper value. |
| CATT | Our first preference would be {2, 4, 8}. |
| Panasonic | In our view, as far as the mechanism is same as Rel.17 PUSCH repetition Type A, repetition factor set for Msg.3 would cover the same range, such as [1, 16]. We are open to limit the size of the set to reduce the overhead. |
| vivo | Although number of repetitions are configurable, we believe the values for repetition number for Msg3 should be limited. PUSCH with 16 repetitions connected mode is configured to achieved relatively higher data rate, which is not useful for Msg3 enhancement. Based on our contribution, 4 repetitions are sufficient for Msg3 coverage enh. Hence, {1, 2, 4} are sufficient for Msg3 repetition numbers. |
| Xiaomi | We share the same view as China Telecom. The size of the set shouldn’t be limited and which values in the set will be selected depends on the gNB. |
| Ericsson3 | Yes, TDRA based method is more flexible and there’s no need to have a reduced set of values compared to R16 supported number of repetitions. |
| NTT DOCOMO | Agree. |
| LG | We don’t see any motivation for supporting all the NR Rel-16 repetition number {1,2,3,4,5,6,12,16}. First of all, as some companies analysed in their contributions, 4 repetition gives enough enhancement Msg3. Second, regarding the typical initial access situation where Msg3 transmission occurs, it is not easy for gNB to fully utilize such fine granularity of repetition number since it is hard for gNB to have much information about each UE’s geometry. So we support to use at least repetition number {2,4}. |
| Lenovo, Motorola Mobility | We are Ok to reuse Rel.16 repetition factors. |

Update for Q1

Current situation is summarized as follows.

- Yes: China Telecom, Panasonic, Xiaomi, Ericsson, NTT DOCOMO, Lenovo, Motorola Mobility

- No: CATT, vivo, LG

Given we have to make a binary decision for this issue so that we can move on for the further down-selection of Issue #5. Per FL understanding, the majority view seems more reasonable and could be considered due to the following reasoning.

- It provides gNB more scheduling flexibility, e.g., by support value like 3 or 7.

- It could provide better performance by supporting the relatively larger values. This could be potentially useful and necessary for FR2 or Rel-17 small data transmission on Msg3. Any, it’s up to gNB configuration whether to configure the relatively larger values or not.

- The signaling overhead increase (from 2 bits to 3 bits indication) is minor.

With summarized above, FL would like to check whether the following proposal is acceptable, though it may not be perfect for you.

Proposal 1 for Issue 9:

- If TDRA information field is chosen, NR Rel-16 repetition factor set {1, 2, 3, 4, 7, 8, 12, 16} is supported for Msg3 repetition.

| Company | Comments |
| --- | --- |

Q2: For the proponents of MCS based solution or option 1 of TPC based method, which way do you prefer about how to use the X bits? For your preference, please also indicate your preferred set of repetition factors.

- the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor).

- the X bits are used for selecting one repetition factor from a predefined set.

- the X bits are used for selecting one repetition factor from a SIB1 configured set.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Q2: 3) is our first preference, can live with 2). |
| China Telecom | For MCS based solution, we are ok with 2) and 3), and we prefer the value of X <=2. |
| Intel | We prefer 3). |
| Xiaomi | For TPC based solution, 3) is our preference. Based on 3), X is determined by the size of the set configured by gNB. |
| NTT DOCOMO | We prefer 2). 1) cannot cover a large value of repetition factors. Also, if the SIB1 overhead is the motivation not to support TDRA based solution, the solution should avoid overhead which 3) causes. Our preferred set of repetition factors are [2,4,8,16]. |
| Qualcomm | We prefer option 2 of TPC based method. For the set of repetition factors, we think that at least 1 should be included in that set (i.e. no extra indication for presence or absence of repetition). Also, we think that gNB should have some way to indicate a large repetition number (e.g. 16). |
| WILUS | 3) is aligned with our preference. |

Update for Q2

Current situation is summarized as follows.

| Ways for use of X | Support | Can live with |
| --- | --- | --- |
| - Direct indication by X bits |  |  |
| - Predefined | China Telecom, NTT DOCOMO, Qualcomm | Nokia/NSB |
| - Configured by SIB1 | Nokia/NSB, China Telecom, Intel, Xiaomi, WILUS |  |

To further move on to address the FFS points of MCS/TPC based solution, we have to choose one of the alternative solutions here.

We have already have common understanding that K={1,2,4} should be supported. It may anyway need 2 bits for indication if Option 1 of Issue #7 (supported by the majority) is chosen. In addition, many proponents show that relatively larger value could be required such as 8, FL’s understanding is at least of 2 bits are required to indicate the repetition factor.

Considering one main concern on TDRA based solution from the proponents of MCS/TPC based solution is about SIB1 overhead, similar philosophy could be applied here. In such sense, FL would like to check whether the following proposal could be potentially acceptable for the proponents, if it is not perfect.

Proposal 2 for Issue 9:

- If MCS information field is chosen or if Option 1 is supported in case TPC information field is chosen, X=2, which is used for selecting one repetition factor from a predefined set.

| Company | Comments |
| --- | --- |

  - Counting on the basis of available slots for Msg3 repetition

In RAN1#105-e, the following agreements were reached for counting the number of repetitions on the basis of available slots for Msg3 repetition.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

[Open] Issue #10 Use of flexible symbol for counting on the basis of available slots

- Regarding the FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon, companies’ views are summarized below.

- Option 1: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon cannot be regarded as available symbols for Msg3 PUSCH transmission,.

  - Support: [6, CATT]

- Option 2: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon, not for reception of SS/PBCH blocks, can be regarded as available symbols for Msg3 PUSCH transmission.

  - FL note: The red texts are not from the proponents, while it should be a common understanding.

  - Option 2-A: Additional explicit indication is introduced to indicate whether flexible slots indicated via TDD-UL-DL-Configcommon are available for Msg3 repetition.

    - Support: [2, OPPO], [5, vivo], [12, Qualcomm], [14, Samsung], [18, Sharp]

    - [2, OPPO], [5, vivo]: Support indication in RAR UL grant or DCI format 1-0 with RA-RNTI for initial Msg3 transmission, and DCI format 0_0 scrambled with TC-RNTI for Msg3 re-transmission

    - [12, Qualcomm], [14, Samsung]: A bit map or valid/invalid symbol pattern is signaled to the UE for indicating whether flexible symbols in a slot is considered for PUSCH repetition transmission or not.

  - Option 2-B: The first N (e.g. N=1) slots, determined as available according to type-A repetition mechanism, are also considered as available for Msg3 PUSCH repetition. The remaining repetitions are only transmitted in the UL slots.

    - Support: [5, vivo], [23, WILUS]?

  - Option 2-C: No need additional indication.

    - Support: [19, CMCC]?, [10, Intel]?, [22, LG]?, [13, Panasonic]?

    - [10, Intel]: Flexible symbols indicated by TDD-UL-DL-Configcommon for SSB transmission and CORESET0 with Type0-PDCCH CSS set cannot be used for Msg3 PUSCH repetition.

      - FL note: FL understanding is the flexible symbol configured for CORESET0 with Type0-PDCCH CSS set can be used for Msg3 PUSCH repetition in Rel-15/16. Details analysis is summarized in Appendix-B

    - [13, Panasonic]: If flexible symbol is indicated in the TDRA, Msg.3 PUSCH repetition can be transmitted on flexible symbol.

    - [22, LG]: Pre-configured DL symbols (i.e., SSB)/gap symbols cannot be used for Msg3 PUSCH repetition.

First round

Companies are encouraged to provide your views on above analysis, including,

- What’s your preference and the reasoning?

- Any wording suggestion or clarification about the options?

| Company | Comments |
| --- | --- |
| Samsung | Our initial comment is: The addition “not for reception of SS/PBCH blocks” is not clear, because the “reception” is UE implementation, we cannot ensure the symbols used for “reception”. Btw, this could be ignored if the option 2-A is adopted, the gNB could directly put these symbols to be “cannot be used”. If we have to state it, we can use the one for RO validation: “it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2,” FL: The intention was to use similar wording from the spec (copied below). Maybe, it’s better to change to “not indicated for reception of SS/PBCH blocks” . For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. In addition, considering there will be “second step” to determine the actual transmission, it seems more natural to call this “available slot” to be “valid slot”, and then apply the determination operation as discussed in next issue. This is also quite aligned with current spec logic in many place like RO validation and transmission, msgA PUSCH validation and transmission. FL: The intention here is to reuse the terminology that used in AI 8.8.1.1. So, using“available slot” which has similar meaning as valid slot, should be ok. |
| Sharp | Preference: Option 2-A. Reason: Option 1 is not applicable to the case where tdd-UL-DL-ConfigurationCommon is not signalled in the cell (e.g., dynamic TDD case). Option 2-C reduces the gNB scheduling flexibility when a pattern that is not possible only by tdd-ul-dl-ConfigCommon should be configured by using tdd-ul-dl-ConfigDedicated. Down-selection between Option 2A and Option 2-B is preferred. |
| Nokia/NSB | We would like to understand if companies expect that such optimization is of any practical relevance and, if yes, we would appreciate if a practically relevant use was described by proponents. Clearly configuration options exist to have flexible slots/symbols in the cell. However, we wonder if, given the predominance of slot structures such as DDDSU (and others considered in this WI) the relevance of flexible slots is such to justify a rather nontrivial optimization for identifying available UL resources for Msg3 repetitions. Please also note the fact the Type A repetition framework is used for such repetitions, implies that the same UL resources are used in all slots involved in the repetitions. We are not sure that flexible symbols/slots can offer this possibility in a statistically relevant number of cases. We certainly do not prefer using flexible symbols/slots only to be forced to allocate a smaller number of symbols to PUSCH, i.e., wasting useful resources in the U slot. Finally, it is worth observing that the problem is substantially different from supporting a single Msg3 transmission over the flexible symbols as per R15/R16. Therein no repetition is supported, hence the limitations (if any) given by the flexible symbols do not affect the efficiency of the use of the U slots (since only one Msg3 is transmitted in R15/R16, and hat would be transmitted over the flexible symbols, if applicable). For all these reasons, we would like to propose and support a new Option 1-B, as follows: - Option 1-B: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon cannot be regarded as available symbols for Msg3 PUSCH repetitions, if K>1 repetitions are indicated to the UE. |
| Intel | We prefer Option 2-C. This is similar to what was discussed for enhancement on PUSCH repetition type A. We think this provides more opportunities and can help reduce the latency for Msg3 PUSCH repetition compared to the case when flexible symbols cannot be used for Msg3 PUSCH repetition. Thanks for FL’s clarification. We understand that in Rel-15/16 flexible symbol configured for CORESET0 with Type0-PDCCH CSS set can be used for Msg3 PUSCH repetition. Our view is that it would be good to pose some restriction on this, similar to SSB transmission. This is different from Rel-15/16 given the fact that this is targeted for Msg3 PUSCH repetition. For Rel-15/16 Msg3 PUSCH without repetition, it is easy for gNB to avoid the collision. For Option 2-A, it is not clear to us why we need to consider additional signalling mechanism to indicate the flexible symbols. Certainly, this leads to large signalling overhead while the benefit is not clear. |
| Qualcomm | Option 2-A provides scheduling flexibility. Since signaling in Option 2-A is optional, Option 1 can be a fallback. The main issue of Option 1 is that ruling out the usage of flexible symbols may extend the Msg3 Tx latency. |
| LG Electronics | In NR, flexible symbols indicated by tdd-UL-DL-configurationCommon can be used for msg3 PUSCH transmission if UE is configured via SLIV in RAR or in DCI. In this aspect, we think the principle can be applied for msg3 PUSCH repetition. Also, if pre-configured DL symbols (i.e., SSB) /gap symbols are note overlapped the configured OFDM symbol for msg3 PUSCH repetition, UE may these symbols can be available. Hence, we think it needs that above condition is described for clear UE behaviour. |
| NTT DOCOMO | We prefer Option 1, 2-B, or 2-C. Although utilizing flexible symbols enhances the coverage performance, the efficient scenarios are limited due to TDRA of PUSCH repetition type A. Given that using flexible symbols are not so common, introducing the explicit indication to specify available symbols (Opt2-A) is not our first choice due to additional overhead. |
| OPPO | We prefer option 2A. It has efficient uplink resource usage, and low Msg3 repetition latency. For option 2B, we wonder how to determine N without the knowledge of availability of flexible symbols. For option 1, the flexible symbols actually used as uplink by gNB can not be used Msg3 repetition, which enlarges the latency of Msg3 repetition, and reduces the fficiency of uplink resource usage. |
| Panasonic | We prefer Option 2-A or Option 2-C. Both options can indicate whether flexible slots indicated via TDD-UL-DL-Configcommon is used or not for Msg3 repetition. For Option 2-C, unified handling as PUSCH repetition Type A for normal PUSCH would be possible, while TDRA with only UL and flexible + UL is necessary. Option 2-A would require only TDRA with flexible + UL and whether to use flexible is determined by flag in RAR UL grant or DCI. |
| Xiaomi | Option 2-A is our preference. Flexible symbols should be utilized for msg.3 repetition to reduce the transmission delay, while some flexible symbols indicated by TDD-UL-DL-ConfigurationCommon may be further indicated as DL symbols for other Ues by dedicated signalling, and there will be UL and DL transmission collision from different Ues at gNB side. So, additional explicit indication is needed to indicate whether flexible slots indicated via TDD-UL-DL-Configcommon are available for Msg3 repetition. |
| Ericsson | According to our agreements so far, there’s no need to mention SS/PBCH blocks, and rules for handling SSB collision with other channels are clear enough in NR Rel-16, i.e. following update is needed: “Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon, not for reception of SS/PBCH blocks, can be regarded as available symbols for Msg3 PUSCH transmission.” Furthermore, no additional signaling is needed for flexible symbol usage determination, reusing similar rules in legacy is enough. |
| CATT | Our initial preference is Option 1 for simplicity, since massive discussion will be involved on the handling of flexible symbols, as shown above. Hence Option 1 can at least be the baseline option if no consensus is reached in Option 2-A/B/C. Latency should not be a critical issue during the initial access, especially for coverage limited case. Nokia’s interpretation is also fine. If Msg3 is scheduled without repetition, flexible symbols can be used (the same as Rel-15/16). |
| WILUS | We are generally fine with FL’s proposal. However, we propose to use only for the first repetition, flexible symbols that are indicated via scheduling DCI (i.e., RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI) can be determined as available. For the remaining repetitions, it can be further discussed whether flexible symbols are considered or not for available slot determination. Therefore, Option 2-B is aligned with our preference only when N=1 case. |
| vivo | We support option 2-A (the 1st preference) and option 2-B. Considering NW may configure all symbols as flexible by tdd-UL-DL-ConfigurationCommon, for dynamic TDD, simply determine the flexible symbol as available or not available is not reasonable, which may lead to higher collision risk or limited scheduling flexibility. Option 2A provides the flexibility to schedule plenty of repetitions for coverage enhancement Ues, and collision can be avoided at the same time. |
| Huawei, HiSilicon | Prefer Option 2-C. As we have agreed in RAN1 #105 e-meeting, available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated, dynamic SFI in DCI format 2-0 and UL CI. A clarification is appreciated on what drives us to introduce additional explicit indication or method (eg. option 2-B) to determine which flexible symbol is available. And, for option 2-A, considering TDD structure DDDSU or DDSUU, the overhead of the additional explicit indication to just indicate how to use flexible symbols in one S slot is not affordable for initial access. Also, since option 2-B seems to rely on UE dedicated TDD configuration, in the first N slots, connected UEs and non-connected UEs may determine different number of available slots according to the existing type-A repetition mechanism. As a result, ambiguity on the determination of the available slot during the first N slots cannot be avoided. Therefore, both option 2-A and 2-B are not preferred. |
| FL | FL will summarize the pros&cons for this issue, and may provide a couple questions for further discussion later. |

Second round

Necessity

In Rel-16, flexible symbols configured by tdd-UL-DL-ConfigurationCommon can be used for Msg3 transmission. Details can be found in Appendix-B.

Regarding the necessity of using flexible symbols indicated by tdd-UL-DL-ConfigurationCommon for Msg3 repetition, one important question is do we intend to only address the typical TDD configuration in current deployment, e.g., DDDSU, where there are very few flexible symbols are configured. If so, there would be no much motivation to use these flexible symbols in ‘S’ slot for Msg3 type A PUSCH repetition.

However, if we’d like to also address other TDD configurations, e.g.,

- Case 1: A configuration with more flexible slots, e.g., DFFFU, is configured by tdd-UL-DL-ConfigurationCommon, or

- Case 2: tdd-UL-DL-ConfigurationCommon is not configured in the cell, i.e., all symbols are flexible according to current spec.

For Case 1, if flexible symbols cannot be used for Msg3 repetition, it would cause much delay for the transmission of Msg3 repetition. For Case 2, Msg3 repetition cannot even be transmitted.

Technically speaking, FL thinks that it’s safer that we should not only focus on the TDD configuration in current real deployment, as we are discussing Rel-17 features to be implemented in the future. Forward compatibility of a feature is very important, and should be respected here. Therefore, Option 1 is not a good approach from FL point of view. Similarly, Option 2-B is also not a good way as it can only ensure the first repetitions for Case 1 and Case 2 above.

However, if flexible symbols are to be used, it may or may not cause some ambiguity issues. An example is shown in Figure 3.3-1 below, where UE#1 and UE#2 use the same separate preamble for requesting Msg3 repetition. Both UEs would transmit Msg3 with repetition if they can both successfully decodes the corresponding Msg2. For UE1 with only provided with tdd-UL-DL-ConfigurationCommon, Msg3 repetition will be transmitted in slot #2, #3, #4, #6 if gNB schedules K=4. For UE2 in RRC connected mode, there could be two alternatives.

- Alt-1: The legacy dropping rules for Msg3 transmission is applied for Msg3 repetition. That is, if UE2 is provided with tdd-UL-DL-ConfigurationDedicated and slot #2 and 6 are changed to DL slots, the Msg3 repetition on these two slots are dropped. That is, UE2 will transmit Msg3 repetition in slot #3 and #4. As gNB is not able to be aware of which UE is transmitting Msg3 in CBRA procedure, it may cause ambiguity about whether the Msg3 repetition is actually transmitted between gNB and UE.

- Alt-2: Msg3 repetition on each available slot determined by tdd-UL-DL-ConfigurationCommon cannot be canceled by other configurations/transmissions. In such case, no ambiguity issue exits.

  - FL’s understanding is, there are two cases in Rel-16 that a Msg3 transmission can be canceled if it overlaps the following symbols (Details can be found in Appendix B):

    - Case a): Downlink symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated.

    - Case b): Symbols configured for SSB transmission.

  - In addition, it’s possible that Msg3 PUSCH may overlap PUCCH carrying HARQ-ACK/UCI as discussed in [103-e-NR-7.1CRs-06] in R1-2007802. However, there is no consensus reached about the UE behavior, i.e., it is leave to UE implementation.

Figure 3.3-1 An example of transmission of Msg3 repetition.

With summarized above, flexible symbols indicated by tdd-UL-DL-ConfigurationCommon can be regarded as available symbols for Msg3 PUSCH transmission, and there are only two ways left:

- Option 2-A: Additional explicit indication is introduced to indicate whether flexible slots/symbols indicated via TDD-UL-DL-Configcommon are available for Msg3 repetition.

  - It means, some of the flexible slots/symbols can be indicated as available for Msg3 repetition, and gNB would not indicate these slots/symbols to be downlink slots/symbols or for SSB transmissions or scheduling PUCCH. But gNB can do so for other flexible symbols.

- Option 2-C: No need additional indication. This means leave to gNB implementation to avoid any ambiguity.

  - One way is to fall back to Option 1, i.e., not configuring such TDD configurations.

  - Another way is Alt-2 above, i.e., gNB would avoid Msg3 repetition to be canceled. For instance, the flexible symbols configured by tdd-UL-DL-ConfigurationCommon would not be changed to DL symbols or configured with SSBs if gNB would like to schedule Msg3 repetition on these symbols. FL also feels it seems not easy for gNB to avoid such case.

Then, it becomes rather clear that, Option 2-A offers better flexibility while requires additional spec impacts.

Then, FL suggests discussing the following proposal as a starting point in this round of discussion.

Proposal for Issue#10: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon can be regarded as available symbols for Msg3 PUSCH transmission.

- Option 2-A: Additional explicit indication is introduced to indicate whether flexible slots/symbols indicated via TDD-UL-DL-Configcommon are available for Msg3 repetition.

- Option 2-C: No need additional indication, i.e., leave to gNB/UE implementation.

FL strongly encourages companies first check my analysis above before making any comments to the proposal. Please kindly indicate if you find any discrepancy about my analysis above, especially for Rel-16 legacy behavior.

| Company | Comments |
| --- | --- |
| Nokia/NSB | The necessity of this optimization is yet to be demonstrated. We are implicitly assuming that NW will not make use of those F slots for DL transmission which, as we know, is typically the transmission which uses the largest amount of resources. How often would then NW actually signal the availability of sufficient uplink symbols in F slots to accommodate the long PUSCH needed for msg3 transmission in case of coverage shortage (remember that we are talking about PUSCH Type A repetition framework)? It surely is always interesting intellectually to engage in this kind of optimizations. However, we do not think that the practical relevance of this scenario is such to justify such optimization. Option 2-C is thus the best choice in our view. Even more, if we consider that latency has never been an important KPI in AI 8.8 for R17, both in the SI and in the WI (for instance, when discussing PRACH resource fragmentations, PRACH retransmissions as opposed to PRACH repetitions and so). Given the limited time available and the existence of several open issues, whose relevance is well acknowledged by every company, we do not find justification to anything more than Option 2-C. |
| CATT | If the majority is willing to utilize flexible symbols, we are fine with this direction. For spec simplicity we slightly prefer Option 2-C, with leaving to gNB scheduling. Assuming the latest Issue#11 is agreed, for a unified design as much as possible, we suggest the following update for more precise: Proposal for Issue#10: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated in ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH transmission. FL: It is clear that the flexible symbols configured for SSB cannot be used for Msg3. FL suggests clarifying it under Issue#12. In addition, we also didn’t explicitly spell out this for the agreements in AI 8.8.1.1, and it should be also ok here. Agreement For PUSCH repetition Type A for Rel-17 CG-PUSCH, semi-static flexible symbol is considered as available. |
| Sharp | Our understanding of Option 2-A is that “flexible” symbols are configured as not available for Case 1 and “flexible” symbols are configured as available. Therefore, for Option 2-A, Indication in SIB1 for the additional explicit indication for indicating availability of “flexible” symbols. If Option 2-C is assumed, we are not sure why the gNB schedules msg3 repetition in “flexible” symbols as in Figure 3.3-1. Is the gNB operating with full duplex in slot#2 and #6? Downlink configuration with uplink transmission should be avoided by the gNB. In that sense, we are fine with Alt 2-C when such a collision will be avoided by the gNB. Therefore, we are fine with the current FL proposal. We are also OK with CATT’s modification. |
| OPPO | The usage of flexible symbols for Msg3 repetition can reduce the latency of Msg3 repetition. It is not flexible to hardly assume the flexible symbol is available or not. Option 2A introduces a flexible way to use the flexible symbols for Msg3 repetition. gNB can determine and indicate the utilization of flexible symbols flexibly. It is applicable to all the TDD configurations. Therefore, we prefer option 2A. For option 2C, gNB has to find the available flexible symbols for Msg3 repetition transmission scheduling, which has much scheduling restriction. |
| Intel | Using flexible symbols can provide more opportunities and can help reduce the latency for Msg3 PUSCH repetition compared to the case when flexible symbols cannot be used for Msg3 PUSCH repetition. For Option 2-A, it is still not clear to us why we need additional signalling. Given that Msg3 PUSCH repetition is based on available slots, how can gNB make prediction on the traffic or DL/UL patterns more than 10 slots earlier (in typical TDD UL/DL scenario) in order to provide indication of flexible symbol? We do not think this is reasonable assumption. In addition, we have strong concern on the signalling overhead and mechanism to indicate the flexible symbol pattern. We suggest to list all detailed design for Option 2-A, e.g., what is the detailed signalling, e.g., bitmap, or some patterns? Where is this signaled, e.g., in RAR or other places? Before we have clear picture of the complete solution, we do not think we can agree on this. We support 2-C, which is based on existing mechanism. |
| vivo | Fine with Proposal for Issue#10, and we support Option 2-A. Agree with FL’s analysis that NW can hardly avoid the collision between Msg3 repetitions and semi-static DL in the flexible symbols, if the available symbols is simply determined use the same rule defined for type-A repetition for connected UEs, especially when there are many flexible slots, or even all symbols/slots are flexible. For option 2-C, it is not easy for NW to avoid such collision. In our understanding, option 2-C means NW need to carefully arrange semi-static DL transmissions on flexible symbols for connected UEs, due to Msg3 repetitions are enabled for the cell. At the same time, the starting slots, number of slots for Msg3 scheduling are also quite restrictive to avoid collision. As a result, the flexibility of configuring higher ratio of flexible symbols/slots are lost, when Msg3 repetition feature is enabled with option 2-C. |
| Xiaomi | For the main bullet, we are fine with CATT’s version. In order to avoid gNB decoding failure, It is important that the gNB and the UEs can have a consistent determination of the slots which can be used for the actual transmission. For the two solutions proposed by FL, Option 2C will restrict the flexibility of gNB configuration, so option2A is our preference. |
| Ericsson3 | Our understanding of option 2-c means that available slot will not consider whether flexible symbols are not expected to be used in a first step, and after available slot is determined, whichever rules applied to msg3 transmission in NR Rel-16 will be applied to NR Rel-17 msg3 repetitions, which is what we preferred. |
| NTT DOCOMO | We support Option2-C. We are not sure if additional explicit indication is worth utilizing the optimization at the cost of overhead. Option 2-C seems fine, as long as network scheduling is properly managed. |
| Qualcomm | We prefer option 2-A. |
| WILUS | We are generally fine with FL’s proposal. Please clarify that both options are aligned with legacy procedure at least for the first repetition. It is worth noting that the flexible symbols by tdd-UL-DL-ConfigurationCommon can be used to indicate the Msg3 PUSCH transmission without repetition in Rel-15/16. Therefore, the first repetition of Rel-17 Msg3 PUSCH should be designed same with Rel-15/16 Msg3 PUSCH. |
| LG | We prefer option 2-C. We believe that gNB’s scheduler can avoid such situation. |
| Lenovo, Motorola Mobility | We prefer option 2-C, leave to gNB/UE implementation |
| FL | It seems companies are fine with the following proposal. FL suggests first agreeing on this proposal and then make down-selection in the next step. Proposal for Issue#10: Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon can be regarded as available symbols for Msg3 PUSCH transmission. - Option 2-A: Additional explicit indication is introduced to indicate whether flexible slots/symbols indicated via TDD-UL-DL-Configcommon are available for Msg3 repetition. - Option 2-C: No need additional indication, i.e., leave to gNB/UE implementation. |

[Open] Issue #11 Transmission of Msg3 repetition on the available slots

In addition to the determination of ‘available slot’ for Msg3 repetition, it also needs to determine the rules for ‘actual transmission’, i.e., whether to drop a Msg3 repetition or not on the ‘available slot’. For enhanced PUSCH repetition type A, one alternative (Alt 1-B) which is supported by majority companies is copied below. If similar approach is reused for Msg3 repetition, it would means the available slots for K repetitions of Msg3 is based on TDD-UL-DL-Configcommon. Then, the UE determines whether to drop a Msg3 repetition or not according to Rel-15/16 PUSCH dropping rules, but the Msg3 repetition is still counted in the K repetitions.

- Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

However, some companies propose that Rel-15/16 dropping rules cannot be fully reused. Detailed companies views are summarized below.

- Step 1: Determine available slots for K repetitions based on TDD-UL-DL-Configcommon in addition to TDRA in the RAR UL grant scheduling the Msg3 PUSCH repetition.

  - FFS potential other signaling introduced for handling of flexible symbols indicated by TDD-UL-DL-Configcommon.

  - FL note: The FFS depends on the discussion in Issue#10. If any comments about the necessity of such signaling, please comment under Issue#10 instead of here.

- Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules (FFS some exceptional cases), but the PUSCH repetition is still counted in the K repetitions.

  - [1, Huawei, HiSilicon], [4, ZTE] and [17, Ericsson] propose some exceptional cases as follows. While [18, Sharp]? Prefer to follow legacy Rel-15/16 rules, including the rules related to SFI, UL signals and CI etc.

    - [1, Huawei, HiSilicon]: The actual transmission of Msg3 PUSCH repetition in an available slot cannot be canceled by tdd-UL-DL-ConfigurationDedicated.

    - [4, ZTE]: If one or more Msg3 repetitions overlap with a PUCCH carrying HARQ-ACK/CSI, the UE transmits PUCCH and drops the overlapped one or more Msg3 repetitions.

      - Note: it’s up to UE implementation in Rel-15/16 for overlapping of PUCCH and Msg3 without repetition.

    - [17, Ericsson]: RAN1 is to further discuss whether Msg3 repetition can be cancelled by dynamic cancellation indication, whether flexible symbols for Msg3 repetition can be dynamically changed to be downlink, and the collision rules when Msg3 repetition collides with other UL channels and signals.

First round

Companies are encouraged to provide your views on above analysis.

| Company | Comments |
| --- | --- |
| Sharp | In our view, different from normal PUSCH discussed under AI8.8.1.1, Step 2 itself is not necessary. Firstly, we are not sure why the gNB schedules msg3 repetition in downlink region signalled by DCI 2_0? If the UE is not expected to be scheduled a PUSCH in a region indicated as downlink by DCI format 2_0, the step 2 above is not necessary. Please check the following quoted from Rel-15 specification TS38.213. TS38.213 For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - … - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, DCI format 2_3, or a RAR UL grant indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot FL: For your copied texts, collision between Msg3 PUSCH and dynamic SFI is not expected. This is the same as the collision handling DG PUSCH and dynamic SFI. However, CBRA Msg3 PUSCH could be transmitted during RRC connected mode, where collision with other signals, e.g., tdd-UL-DL-ConfigurationDedicated and PUCCH with HARQ-ACK, is possible. Step 2 is to reuse the legacy dropping rules as much as possible, with checking whether there are exceptional cases. Sharp: CBRA in RRC connected state would be triggered only by RACH SR (in case of PUCCH SR failure) or Pcell BFR. For RACH SR, all other uplink transmissions are released according to TS38.321. For Pcell BFR, since beam failure happened already, any transmission (e.g., HARQ-ACK feedback) in the ongoing random-access procedure other than msg3 will not be expected. Therefore, optimization only for BFR is not necessary. Regarding potential collision with downlink region configured by tdd-UL-DL-ConfigurationDedicated, which is indicated as flexible in tdd-UL-DL-ConfigurationCommon, the gNB can avoid such collision by scheduling, or the gNB can configure those flexible region as “not available” as proposed in Issue#10. As commented above, legacy rule is that the UE doesn’t expect such collisions. Therefore, if dropping is supported, new procedure should be introduced. |
| Nokia/NSB | We think legacy rules should apply as much as possible. If the outcome of Issue #10 is such that new rules are needed, then new rules could be discussed. Prior to this, it is difficult to justify further optimizations. |
| Intel | We share similar view as Sharp that Step 2 may not be needed. Msg3 PUSCH is for initial access where UE does not have dedicated RRC configuration. |
| LG Electronics | We are fine with the proposal. |
| Xiaomi | For RACH procedure in RRC connected modes, the actual msg.3 repetition should be omitted if the occupied symbols is DL symbols indicated by TDD-UL-DL-ConfigurationDedicated, SFI or CI, but the PUSCH repetition is still counted in the K repetitions. For RACH procedure in idle/inactive state, there is no CI/SFI signalling, and additional explicit indication for available slots determination is enghoug to handle the msg.3 transmission collision if it is introduced, and actual repetition omitting is not needed. |
| Ericsson | Regarding step 1, TDD-UL-DL-Configcommon and TDRA are enough, there’s no need of that FFS. Regarding step 2, a UE doing random access in CBRA doesn’t have to be initial random access, it could be in RRC connected state already. Furthermore, in CFRA, the UE is in RRC connected state, the PUSCH scheduled by RAR may also be repeated as we discussed given that in RAN1 spec. we only specify PUSCH schedule by RAR. |
| WILUS | We share the similar view with Sharp. Step 1 is enough for available slot determination. |
| FL | Please check my reply to Sharp. Companies are encouraged provide more input, including - Your views about above two steps - Except for TDD-UL-DL-Configcommon, do you think whether flexible symbols indicated by TDD-UL-DL-Configcommon for SSB transmission and/or CORESET0 with Type0-PDCCH CSS set could be used for determination of available slot? |
| Sharp2 | Thanks FL for your feedback. Please find my reply inline above in red text. On FL’s follow-up questions; Regarding (1), our view is Step 2 itself is not necessary. Regarding (2), flexible symbols indicated TDD-UL-DL-ConfigCommon for SSB transmission indicated by ssb-PositionsInBurst should be treated as “not available”, which is in line with the agreement in AI8.8.1.1. |
| CATT | Consider this issue case by case: (1) For UEs in RRC_IDLE/INACTIVE mode, only Step 1 should be enough. The gNB is responsible to guarantee the correct scheduling. (2) For UEs in RRC_CONNECTED mode, Msg3 repetition is similar to that in AI 8.8.1.1. We may consider reusing the same mechanism in AI 8.8.1.1 (where not only determination of available slot but also the dropping rules may be reused). Another thing we would like to remind that, in AI 8.8.1.1, the available slots is also determined based on SSB symbols (when overlapped with flexible symbols), as agreed in RAN1#105-e: Agreement: - If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.. Of course, in AI 8.8.3, since the use of flexible symbols is still discussing under Issue #10, we may consider this latter. (For example, if it is decided that only UL symbols can be used for Msg3 repetition, then no need to consider SSB symbols since it will not be configured as UL) |
| Xiaomi2 | 1) As mentioned above, step 2 is not necessary for RACH procedure in RRC_IDLE/RRC_INACTIVE state, but it is necessary in RRC_CONNECETED state, and at least the actual transmission should be canceled when the allocated symbols indicated by TDD-UL-DL-ConfigurationDedicated, SFI or CI is DL symbols. 2) Yes. |
| Nokia/NSB2 | In our view, the rationale to use flexible symbols for transmitting msg3 repetitions is unclear. We provided our view on this for issue#10. We should really avoid optimizations which will not really impact the success of the Msg3 repetitions consistently. This would require a lot of work and specification impact. It really does not seem justified, especially because, as we explained, the UL resource available on flexible slots is not expected to be as large as the resource available in a U slot. This would then force gNB to schedule shorter PUSCH to use the flexible slots as well, to ensure the same number of symbols is allocated for Msg3 repetitions in each slot, which is very counterproductive for coverage. In our view, only step (1) is needed. |
| Panasonic2 | We agree to Sharp’s comments. |
| Ericsson2 | - Both steps are needed, if you search “a RAR UL grant” in 38.213, it can be found e.g. in many places in section 11.1 when describing when a PUSCH scheduled by RAR and other signals should be transmitted or not. We should reuse those rules instead of skipping them when msg3 is repeated. - Available slot determination is just first step, whether flexible symbols will be actually used for transmission is still based on legacy rules in step 2. |
| Intel2 | - We share similar view as other companies that for UE in RRC_CONNECTED mode, two-step approach may still be needed, e.g., for PRACH based SR. For RRC_INACTIVE or IDLE mode UEs, second step is not needed as UE does not have dedicated RRC signalling yet to receive dynamic SFI, CI, etc.. - Yes. If we follow the agreement as for PUSCH repetition type A enhancement for Msg3 repetition, in the first step, flexible symbols indicated by TDD-UL-DL-Configcommon for SSB transmission and/or CORESET0 with Type0-PDCCH CSS set could be used for determination of available slot. |

Second round

It seems all companies Step 1 as copied below is needed, while no consensus about Step 2. Considering whether to consider Step 2 would also depend on the discussion in Issue#10, FL suggests to first focus on Step 1 here.

- Step 1: Determine available slots for K repetitions based on TDD-UL-DL-Configcommon in addition to TDRA in the RAR UL grant scheduling the Msg3 PUSCH repetition.

For Step 1, the remaining issue is whether do we need to consider other configuration/signaling for available slot determination in addition to TDD-UL-DL-Configcommon. One additional signaling could be the one in Option 2-A discussed in Issue#10. This could be further discussed after we conclude on that issue. Another one is about flexible symbols indicated by TDD-UL-DL-Configcommon for SSB transmission and/or CORESET0 with Type0-PDCCH CSS set.

Given what we have been reached in AI. 8.8.1.1., FL would like to ask the following questions:

Q1: Do you agree that available slot determination for Msg3 repetition depends on flexible symbols indicated by TDD-UL-DL-Configcommon for SSB transmission, i.e. applying similar agreements as copied below to Msg3 repetition?

Agreement: - If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Agreed. |
| CATT | Agree. |
| Sharp | Agree when “flexible” symbols are available. When “flexible” symbols are not available, no dependency exists. |
| OPPO | Agree |
| Panasonic | Agree |
| vivo | Agree. |
| Xiaomi | Agree. |
| Ericsson3 | The rules for SSB overlapping with PUSCH is already specified in following text in section 11 of 38.213, which can be in the second step when discussing omission rules for Msg3 transmission. There’s no need to align with PUSCH in 8.8.1.1. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.. |
| NTT DOCOMO | Agree. |
| Qualcomm | Agree |
| WILUS | Agree. |
| LG | Agree. |
| Lenovo, Motorola Mobility | Agree |
| FL | It seems all companies are fine with the intention except for Ericsson. Based on the views from the majority and what we have agreed under AI 8.8.1.1, FL still suggests the following proposal. Proposal for Issue 11: If a symbol for Msg3 repetition in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot. |

[Closed] Issue #12 Applicability to paired spectrum

- In [18, Sharp], it is proposed to restrict applicability of the counting based on available slots to the unpaired spectrum for msg3 repetition. That is,

- For msg3 repetition, counting based on available slots is only applicable to unpaired spectrum. For paired spectrum, counting based on physical slots is applied.

- FL’s understanding is, the available slots for K repetitions is only based on TDD-UL-DL-Configcommon or potential other signaling introduced for handling of flexible symbols indicated by TDD-UL-DL-Configcommon as discussed under Issue#10. However, these signaling are TDD specific. It means, for FDD, it seems there is no difference between ‘counting based on physical slots’ and ‘counting based on available slots’, as the only concerned signaling for available slots are TDD specific. With this understanding, there seems no need to limit for TDD. Instead, the two steps discussed in Issue#11 can apply for both TDD and FDD.

First round

Companies are encouraged to provide your views on about FL’s understanding above.

| Company | Comments |
| --- | --- |
| Sharp | Counting based on available slots are not necessary for paired spectrum. One of the reasons is that the two steps are not necessary for msg3 repetition as discussed in Issue#11. |
| Panasonic | We agree to FL’s view that the two steps discussed in Issue#11 can apply for both TDD and FDD. |
| Xiaomi | Counting based on available slots are necessary for paired spectrum. Since the redcap WI stated that the methods in CE WI will be adopted for redcap UE coverage recovery, and HD-FDD is supported in redcap WI, which needs to identify the available slots for msg.3 transmission. |
| Ericsson | Seems that this is obvious based on the agreement so far for available slot determination. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon. |
| CATT | We think the UL slots in paired spectrum are all available slots for Msg3 repetition. There should not be any difference whether the Msg3 is transmitted on the basis of available slots or physical slots in paired spectrum. The spirit has been wildly applied in the current spec, e.g. PUCCH repetition, RO validation, and now possible in AI 8.8.1.1 (counting on the basis of available slot only applies to TDD) |
| FL | FL suggests first waiting the discussion in AI 8.8.1.1, and therefore closing this issue for now. |

  - RV pattern

[Paused] Issue#13: RV pattern for Msg3 repetition

In NR Rel-15/16, a UE shall use RV0 for Msg3 initial transmission, and use the 2-bit RV bit field in DCI format 0_0 scrambled by TC-RNTI for RV indication. If Msg3 repetition is enabled, the following rules were agreed in RAN1#105-e.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission. - The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214. - The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214. - FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Regarding the FFS point, all companies raising this issue propose that similar approach that defined for enhanced PUSCH repetition type A can be reused for Msg3 repetition. It is summarized as follows.

Each available slot identified by the UE is considered as a transmission occasion for Msg3 PUSCH repetition.

- RV is cycled across transmission occasions, irrespective of whether Msg3 PUSCH transmission in the transmission occasion is further omitted or not.

- Support: [1, Huawei, HiSilicon], [4, ZTE], [6, CATT], [14, Samsung], [18, Sharp], [17, Ericsson], [22, LG]

First round

Companies are encouraged to provide views on the following proposal.

Proposal for Issue#13: Each available slot identified by the UE is considered as a transmission occasion for Msg3 PUSCH repetition.

- RV is cycled across transmission occasions, irrespective of whether Msg3 PUSCH transmission in the transmission occasion is further omitted or not.

| Company | Comments |
| --- | --- |
| Samsung | Seems fine. |
| Sharp | Support |
| Nokia/NSB | Support |
| Intel | We are fine with the proposal. |
| LG Electronics | We are fine with the proposal. |
| NTT DOCOMO | Support the proposal. |
| OPPO | Fine with the proposal |
| Panasonic | We are fine with the proposal. |
| Xiaomi | Fine with the proposal. |
| China Telecom | Support. |
| Ericsson | Fine. |
| CATT | Fine. |
| vivo | Support. |
| Huawei, HiSilicon | Support. |
| ZTE | Support |
| FL | It seems Proposal for Issue#13 is agreeable. |
| Apple2 | Do not support. The main bullet needs clarification, as it does not define which occasions are considered as available occasions for Msg3 repetition. In our view, based on what we agreed in previous meeting, available occasions for Msg3 are cell-specific determined and known to UE and will not be dropped/cancelled later. FL: No matter how we determine available slot and whether/how the UE will drop Msg3 repetition on an available slot, the following always applies: - Each available slot is a transmission occasion. There is no dropping/cancelling of a transmission occasion. - RV is cycled across transmission occasions. In other words, it means RV cycling is based on available slot for Msg3 repetition, regardless whether/how the UE will drop Msg3 repetition on an available slot. Note that, this is similar as we have agreed for enhanced PUSCH repetition type A. Agreement: - Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. |
| Apple3 | For Issue#13, we completely understand the intention that “if” a Msg3 repetition is dropped then RV is cycled anyway. But, we cannot agree with this proposal as mentioned before: the proposal is meaningful only if there are conditions/scenarios that a Msg3 repetition is cancelled (otherwise no point to discuss whether/how RV is cycled). To us, and based on agreements in 105-e on how Msg3 occasions for repetitions are determined, such scenarios will never happen, or at least it is not discussed in RAN1. We are against (dynamically) cancelling Msg3 occasion, so we cannot agree with a proposal which is only valid when/if a Msg3 is dropped. |
| FL | Let's leave it for now considering the concerns from Apple, though I think the proposal doesn't imply we will support cancellation of Msg3 repetition or not. Given this issue would be rather clear after resolving other issues. Let's leave it and save our time a bit. |

  - Frequency hopping related issues.

[Paused] Issue#14: Support of intra-slot frequency hopping for Msg3 PUSCH with repetition

In Rel-15/16, intra-slot FH is supported for Msg3 transmission without repetition. If repetition is enabled for Msg3, it has been agreed to support inter-slot FH. Then, it needs to discuss whether intra-FH could be still supported.

This issue have been discussed in the past three RAN1 meetings without any consensus. In RAN1#105-e, the following conclusion was reached to encourage companies provide addition evaluations.

Conclusion: - Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously)

[1, Huawei, HiSilicon], [17, Ericsson], [20, NTT DOCOMO] and [22, LG] provide some evaluation results, and the observations are summarized as follows.

[1, Huawei, HiSilicon]: For the inter-slot frequency hopping only with 1 symbol front-loaded DMRS and the intra-slot frequency hopping with 1 symbol front-loaded DMRS as well as 1 symbol additional DMRS, the performance of both is approximately the same. Inter-slot frequency hopping provides about 1 dB gain over intra-slot frequency hopping, when both are configured with 1 symbol front-loaded DMRS and 1 symbol additional DMRS. [17, Ericsson]: Initial link level results show 1 dB gain from inter-slot hopping over two frequencies compared to repetition with intra-slot FH. [20, NTT DOCOMO]: Intra-slot frequency hopping does not provide the gain in terms of coverage performance under the simulation assumptions made in coverage enhancements. [22, LG]: Inter-slot frequency hopping shows better performance than intra-slot frequency hopping when msg3 PUSCH repetition is applied.

Companies’ views on the following two options are summarized below.

- Option 1: Support intra-slot frequency hopping for Msg3 with repetition.

- Intra-slot frequency hopping and inter-slot frequency hopping cannot be enabled simultaneously.

- When intra-slot frequency hopping is configured, the UE assumes the same starting RB and the same frequency offset for Msg3 PUSCH repetitions within a transmission.

  - Some companies argued that intra-slot FH could provide additional flexibility for UE multiplexing. An example is shown in Figure 2.5.1.

  - [4, ZTE], [7, China Telecom],[8, Xiaomi],[10, Intel]: If both intra-slot and inter-slot FH is supported for Msg3 PUSCH repetition, one of the FH mechanisms can be configured by higher layers via SIB1. Further, FH flag in the RAR UL grant and DCI format 0_0 can be used to enable FH.

  - [11, Apple]: FH is always enabled for a UE with Msg3 transmission with repetitions.The single bit for FH flag is repurposed to indicate the FH mode.

  - [12, Qualcomm]: Use the existing FH flag for indicating intra-slot FH, and reinterpret another bit filed for inter-slot FH.

Figure 2.5.1 Multiplexing among two legacy UEs without Msg3 repetition and one Rel-17 UEs with Msg3 repetition

- Option 2: Support only intra-slot frequency hopping for Msg3 PUSCH without repetition and only inter-slot frequency hopping for Msg3 PUSCH with repetition.

  - [1, Huawei, HiSilicon], [6, CATT], [8, Xiaomi],[14, Samsung], [16, Nokia/NSB], [17, Ericsson], [23, WILUS]

  - [1, Huawei, HiSilicon]: If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant is reused to indicate inter-slot frequency hopping.

Table 2.5-1 Summary of inter-slot FH for Msg3 repetition

|  | Support | Can live with | Have strong concern | Pros&Cons |
| --- | --- | --- | --- | --- |
| Option 1 | Qualcomm, Panasonic, Intel,Xiaomi, ZTE, Apple [Vivo, Sharp, OPPO, China Telecom, Spreadtrum], | DCM | Ericsson | Pros: - Better multiplexing with legacy UEs. Cons: - 1 dB SNR loss in some cases - More spec efforts on determining the signaling design. |
| Option 2 | CATT, Samsung, Ericsson, WILUS, Nokia/NSB, Huawei, Hisilicon, | [OPPO], Apple, DCM | Intel | Pros: - Less spec effort. Cons: - Less flexibility regarding multiplexing with legacy UEs. |

First round

This issue has been discussed from the first WI meeting. The pros and cons are quite well with each other.

Proposal for issue 14: For frequency hopping for Msg3 repetition, down-select one of the two options below.

- Option 1: Support intra-slot frequency hopping for Msg3 with repetition.

- Intra-slot frequency hopping and inter-slot frequency hopping cannot be enabled simultaneously.

- When intra-slot frequency hopping is configured, the UE assumes the same starting RB and the same frequency offset for Msg3 PUSCH repetitions within a transmission.

- Option 2: Support only intra-slot frequency hopping for Msg3 PUSCH without repetition and only inter-slot frequency hopping for Msg3 PUSCH with repetition.

- If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping.

Note: The proposal is updated with adding the red part. Please do NOT comment unless you have concerns on the current wording, especially the newly added part. While you are encouraged to update your position in Table 2.5-1 if needed. FL’s intention is to discuss this issue in GTW session.

| Company | Comments |
| --- | --- |

  - Support of other enhancements studied for regular PUSCH for Msg3 repetition

[Closed] Issue#15: Support of TB processing over multi-slot PUSCH for Msg3

Based on companies’ input, the support of TB processing over multi-slot PUSCH for Msg3 initial/re-transmission is summarized as follows.

- Support TB processing over multi-slot PUSCH for Msg3 repetition

  - Support: [2, OPPO]

  - Not support: [4, ZTE], [13, Panasonic], [21, CATT], [12, Qualcomm]

  - FFS: [14, Samsung]

Given the limited interests and this is also related to the discussion in AI 8.8.2, FL suggests postponing the discussion to later RAN1 meetings.

[Closed] Issue#16: Support of joint channel estimation for Msg3 repetition

Based on companies’ input, the support of joint channel estimation for Msg3 initial/re-transmission is summarized as follows.

- Support joint channel estimation for Msg3 repetition

  - Support: [2, OPPO], [13, Panasonic], [19, CMCC], [20, NTT DOCOMO]

  - FFS: [4, ZTE], [14, Samsung], [21, CATT]

  - [13, Panasonic]: Msg.3 repetition capable UE should always support joint channel estimation

Some companies also provide evaluation results for joint channel estimation for Msg3 with the following observations.

[4, ZTE]: Cross-slot channel estimation among 4 Msg3 repetitions can provide about 1 dB gain. [19, CMCC]: The joint channel estimation could bring additional 1.75dB coverage gain when 2 slot repetitions are considered. [22, LG]: When DMRS bundling is enabled, inter-bundle frequency hopping can achieve much better performance than intra-slot frequency hopping or inter-slot frequency hopping.

[12, Qualcomm]: If JCE is supported for Msg3 PUSCH repetitions with subject to power consistency and phase continuity requirements, only back-to-back PUSCH transmission is supported and the UE indicates its capability of supporting JCE during RACH procedure. UE is not expected to monitor Msg2 PDCCH between the repetitions of Msg3. gNB should be able to expect no UL beam switching among repetitions of Msg3. Support transmission of PTRS inside Msg3 repetitions.

The essence of this issue is whether early UE capability reporting for joint channel estimation of Msg3 repetition is needed. If the answer is yes, then additional PRACH partitioning would be required, which would need quite lots of spec efforts. Considering it’s still under discussion in AI 8.8.2 about whether the maximum duration may be reported by UE, FL suggests postponing the discussion to later RAN1 meetings.

  - Early termination of Msg3 repetition

[Closed] Issue#17: Early termination of Msg3 repetition

In Rel-15/16 RACH procedure, a UE starts the ra-ContentionResolutionTimer and restart the ra-ContentionResolutionTimer at each HARQ retransmission in the first symbol after the end of the Msg3 transmission. The UE shall monitor PDCCH for Contention Resolution while the ra-ContentionResolutionTimer is running. If Msg3 repetition is enabled, then it needs to discuss whether to support early termination, i.e., whether the ra-ContentionResolutionTimer can start or re-start after each repetition. An example is shown in Figure 2.7.1.

Figure 2.4.1 Early termination for Msg3 repetition

In this meeting, [14, Samsung] proposes to support early termination while [2, OPPO] and [6, CATT] raise their concerns on the contrary. Considering low interests in RAN1 and RAN2 is discussing their interpretation about the contention resolution timer, FL suggests postponing the discussion to later RAN1 meetings.

  - Other issues

Support of PUSCH repetition for CFRA PUSCH scheduled by RAR UL grant.

[4, ZTE]: It was agreed in SI phase that enhancement to PUSCH scheduled by RAR UL grant will not consider the optimization specific for CFRA case. [4, ZTE] interprets that PUSCH repetition can be supported for CFRA while RAN1 will not do any optimization specific for CFRA. In most places of the current NR specifications, ‘a PUSCH scheduled by RAR UL grant’, which includes both Msg3 initial transmission and CFRA PUSCH, is widely used. That is, there is no differentiation of PHY layer handling for Msg3 PUSCH and CFRA PUSCH in most typical cases. Therefore, it is proposed that PUSCH repetition is supported for a PUSCH scheduled by RAR UL grant including CFRA PUSCH, while no optimization specific for CFRA PUSCH is considered.

Support of qam64-LowSE MCS

In [8, Xiaomi]: QAM64-LowSE MCS table provides lower coding rate, which is benefit for Msg.3 coverage enhancement with lower required SNR. So, QAM64-LowSE MCS table can be used for Msg.3 transmission in bad coverage. Therefore, it proposes to support the use of QAM64-LowSE MCS table for Msg.3 transmission with repetitions.

Waveform indication for Msg3

[11, Apple]: Specify a UE-specific procedure to enable/disable transformprecoder for Msg3 transmission, via:

- Alt1: explicit indication, e.g., repurpose some bits in RAR UL grant (for initial Msg3 transmission) or DCI format 0_0 with CRC scrambled by TC-RNTI (for Msg3 retransmission) to indicate whether transformprecoder enabled or not

- Alt2: implicit indication, for example, transformprecoder is enabled if UE indicates to require coverage enhancement/recovery.

Spatial domain transmission relation

[12, Qualcomm]: Consider one of the following options on spatial domain transmission relation for Msg3 PUSCH transmission:

- Option 1: The UE transmits the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the same spatial domain transmission relation.

- Option 2: The UE may transmit the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the different spatial domain transmission relations.

[14, Samsung]: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission. On the other hand, the UE can select the beam for msg3 re-transmissions.

Cell selection criterion S for CE

In [13, Panasonic], it raises that the cell selection criterion S for CE should be specified, similar to LTE eMTC.

This is RAN2 related and FL suggests proponents to raise the issue in RAN2.

Support of additional C-RNTI and HARQ-ACK resource for MSG.4 PDSCH

[24, vivo] observes that NW may decode MSG.3 PUSCH from multiple UEs, which have transmitted the same preamble on the same RO, if MSG.3 PUSCH repetition is introduced. It is beneficial to support contention resolution for multiple UEs simultaneously to reduce the access delay of CBRA procedure, if MSG.3 PUSCH repetition is supported. Therefore, it proposes additional additional (T)C-RNTI can be provided UE to support contention resolution for multiple UEs without initiating a new RACH attempt.

Appendix - A: Previous agreements

RAN1#104-e

Agreements: - For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below. - Option1: UL grant scheduling Msg3. - FFS details. - FFS fallbackRAR UL grant. - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported. - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI - FFS details. - Option3: SIB1 only - Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively Agreements: - For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below. Option1: DCI format 0_0 with CRC scrambled by TC-RNTI. FFS details. Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI. Option2: Can be determined based on the repetition number for Msg3 initial transmission Agreements: Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission. FFS details, e.g., signaling etc. Agreements: For Msg3 PUSCH repetition, the following options are considered, aiming for down-selection in RAN1#104b-e: - Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request, - A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions. - For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - FFS details if any. - Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request, - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition - For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways) - Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc. - gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition. - FFS details if any. - Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions. - Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS details if any. - Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways) - Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - FFS details if any. - Other options are not precluded.

RAN1#104b-e

Agreement: For Msg3 PUSCH repetition, support the following modified Option 2-1. - Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.). - Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual - FFS details if any. Agreements: For the determination of RV for Msg3 PUSCH repetition, - RV of the first repetition is determined in the same way as legacy. - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission. - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission. - FFS determination of the RV sequence. Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted. - FFS additionally using MAC RAR for indication. Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted. Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3. - FFS: the determination of available slots.

RAN1#105-e

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold. Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission. Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc. Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI. Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission. - The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214. - The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214. - FFS: The RV cycling for Msg3 is based on transmission occasions on available slot. Conclusion: - Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously) Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

RAN1#106-e

Agreement Do NOT support fallback RAR UL grant in 2-step RACH for indicating Msg3 repetition. Agreement The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition. Working Assumption Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors. - The new TDRA table is configured by SIB1, with selecting one of the two options below. - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor. - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor. - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied. - K=1. - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - X MSB bits of the MCS information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below. - Option 1: X LSB bits of the TPC information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Option 2: A predefined TPC command table with including repetition factor K is introduced. - FFS details.

Appendix - B: Legacy rules for use of flexible symbol for Msg3 transmission

- FL’s understanding about Rel-15/16 rules for use of flexible symbol for Msg3 transmission:

If SFI is not configured, the following symbols are available symbols for Msg3 transmission according to current specification. - 1) Uplink symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated - 2) Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided. - If a UE is only provided by tdd-UL-DL-ConfigurationCommon, and a symbol is indicated as flexible symbol by tdd-UL-DL-ConfigurationCommon, it is a common understanding that the flexible symbol can be used for Msg3 transmission. - If a UE is provided by both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, and a symbol is indicated as flexible symbol by both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, the flexible symbol is available for Msg3 transmission. If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1 - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, DCI format 2_3, or a RAR UL grant. Meanwhile, the following symbols are not available for Msg3 transmission if SFI is not configured. - 1) Downlink symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.. - 2) Symbols configured for SSB transmission For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.. - Note that, a symbol for a CORESET for Type0-PDCCH CSS set indicated by pdcch-ConfigSIB1 in MIB can be indicated as flexible symbol by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, which can be used for Msg3 transmission. In other words, as long as it is a flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, it can be used for Msg3 transmission. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated..

If dynamic SFI is configured, the Rel-15/16 legacy UE behavior for collision handling of Msg3 transmission is summarized below. - If dynamic SFI is configured, a UE does not expect collision between Msg3 transmission and SFI indication. - If dynamic SFI is configured and the DCI format 2_0 is detected by UE, the flexible symbols indicated by the DCI format 2_0 are available symbols for Msg3 transmission. - If dynamic SFI is configured and while DCI format 2_0 is not detected by UE, the flexible symbols indicated by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided are available symbols for Msg3 transmission. The related specification texts are also copied below. For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols in the slot as downlink and to detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot. For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - ... - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot - ... For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR.

Reference

R1-2106499 	Discussion on Msg3 repetition for coverage enhancement 	Huawei, HiSilicon

R1-2107260	 Type A PUSCH repetitions for Msg3 coverage	 OPPO

R1-2106713 	Discussion on type A PUSCH repetitions for Msg3	 Spreadtrum Communications

R1-2106743 	Discussion on support of Type A PUSCH repetitions for Msg3	 ZTE

R1-2106615	 Discussion on Type A PUSCH repetitions for Msg3 	vivo

R1-2106992	 Discussion on Type A PUSCH repetitions for Msg3 	CATT

R1-2107127	 Discussion on type A PUSCH repetitions for Msg3 	China Telecom

R1-2107939 	Discussion on Type A PUSCH repetition for Msg3	 Xiaomi

R1-2107654 	Type A PUSCH repetitions for Msg3 	InterDigital, Inc.

R1-2107606 	On Msg3 PUSCH repetition 	Intel Corporation

R1-2107757 	Discussion on Msg3 Coverage Enhancement 	Apple

R1-2107363	 Type A PUSCH repetition for Msg3 Qualcomm Incorporated

R1-2107119	 Discussion on Type A PUSCH repetitions for Msg.3	 Panasonic Corporation

R1-2106906	 Type A PUSCH repetitions for Msg3 	Samsung

R1-2107478	 Type A PUSCH repetitions for Msg3 	ETRI

R1-2106659	 Approaches and solutions for Type A PUSCH repetitions for Msg3 	Nokia, Nokia Shanghai Bell

R1-2107563	 Type A PUSCH Repetition for Msg3 	Ericsson

R1-2107803	 Type A PUSCH repetition for msg3 	Sharp

R1-2107421	 Discussion on type A PUSCH repetitions for Msg3	 CMCC

R1-2107876	 Type A PUSCH repetitions for Msg3 	NTT DOCOMO, INC.

R1-2106993	 Views on reusing PUSCH enhancements for Msg3	 CATT

R1-2107552	 Discussion on coverage enhancement for Msg3 PUSCH 	LG Electronics

R1-2108160	 Discussion on Type A PUSCH repetitions for Msg3 	WILUS Inc.

R1-2106616 	Enhanced contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition vivo

R1-2107940	 Other considerations for coverage enhancement	 xiaomi