3GPP TSG-RAN WG1 Meeting #106-e              	R1-2108453

e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0254 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Sub-slot Based HARQ-ACK Feedback for MAC CE Activation/deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), CATT, vivo, Ericsson, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#106-e, the following agreement and conclusion were made to address the issue of HARQ-ACK timing corresponding to a PDSCH carrying MAC commands. For all relevant MAC CE, this avoids a ‘slot’ being mis-interpreted as ‘subslot’ in determining the HARQ-ACK timing when subslotLengthForPUCCH is configured. Agreement For the purpose of determining the HARQ-ACK timing corresponding to a PDSCH carrying MAC commands [38.321], a slot with PUCCH transmission carrying HARQ-ACK information is considered as a slot with symbols as defined in [TS 38.211], irrespective of presence or absence of subslotLengthForPUCCH configuration. Conclusion Outside of TS 38.213 Clause 9, a “slot” consists of symbols as defined in TS 38.211, even if the “slot” is described to have PUCCH transmission carrying HARQ-ACK information, and subslotLengthForPUCCH is configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 38.213 section 4.3 is edited to reflect the understanding described by the Conclusion. Thus, MAC CE for secondary cell activation / deactivation is treated the same as all other MAC CEs, i.e., no need to explicitly state that a slot is composed of symbols. - For 38.213 section 9 and 9.2.2, it’s clarified that for the MAC CE of PUCCH spatial setting, a slot related to HARQ-ACK timing is composed of symbols, not subslotLengthForPUCCH symbols. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mismatch in interpetion of HARQ-ACK timing of all relevant MAC CE. Risk of different interpretation of the specification between UE and network vendors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 9, 9.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.3	Timing for secondary cell activation / deactivation

With reference to slots for PUCCH transmissionsNsymbslot, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following:

-	the actions related to CSI reporting on a serving cell that is active in slot

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot

-	the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after  in which the serving cell is active.

The value of  is m+3 Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3 and  is a number of slots per subframe for the SCS configuration  of the PUCCH transmission as defined in [4, TS 38.211].

With reference to slots for PUCCH transmissionsNsymbslot, if a UE receives a deactivation command [11, TS 38.321] for a secondary cell ending in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on an activated serving cell which the UE applies in slot .

*** Unchanged text is omitted ***

## 9	UE procedure for reporting control information

*** Unchanged text is omitted ***

In the remaining of this clause, if a UE is provided subslotLengthForPUCCH, a slot for an associated PUCCH resource of a PUCCH transmission with HARQ-ACK information includes a number of symbols indicated by subslotLengthForPUCCH, unless stated otherwise.

*** Unchanged text is omitted ***

## 9.2.2	PUCCH Formats for UCI transmission

*** Unchanged text is omitted ***

A spatial setting for a PUCCH transmission is provided by PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId; otherwise, if the UE is provided multiple values for PUCCH-SpatialRelationInfo, the UE determines a spatial setting for the PUCCH transmission as described in [11, TS 38.321]. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo, each slot consists of Nsymbslot symbols as defined in [4, TS 38.211], and  is the SCS configuration for the PUCCH

*** Unchanged text is omitted ***