3GPP TSG-RAN WG1 Meeting #106-e	R1- 2108492

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.4.1

Source:	Moderator (Ericsson)

Title:	Feature lead summary#4 on timing relationship enhancements

Document for:	Discussion

# 1	Introduction

At RAN1#106-e, two rounds of email discussion on timing relationship enhancements for NTN have been conducted. This contribution presents proposals that can be considered for discussion and potential endorsement at the GTW session on Wednesday, August 25, 2021 (if time permits).

# 2	Proposals

PDCCH ordered PRACH:

Proposal 1:

For random access procedure initiated by a PDCCH order received in downlink slot n, UE determines the next available PRACH occasion after uplink slot n+Koffset to transmit the ordered PRACH.

Note: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc, where NTA=0 is assumed for PDCCH ordered PRACH.

FFS: which value of Koffset should be applied

FFS: whether the n+Koffset timing relationship is impacted by UE behavior within or after the validity duration.

UE reporting of information about the UE specific TA pre-compensation:

Proposal 2:

At least for uplink scheduling adaptation, the exact content of UE reporting of information about the UE specific TA pre-compensation is UE specific TA.

- The unit of reported UE specific TA is slot

- FFS: absolute value and/or differential value

BFR timing relationships:

Proposal 3:

- On beam failure recovery procedure, for PRACH transmission in uplink slot n, UE monitors the corresponding PDCCH starting from downlink slot “n + K_mac + 4” within a corresponding RAR window.

- FFS: the timing between PDCCH reception and application of new PUCCH beam in the beam failure recovery procedure.