3GPP TSG RAN WG1 #106-e			R1-2108679

e-Meeting, August 16th – 27th, 2021

Agenda item:	8

Source: 	Moderators (AT&T, NTT DOCOMO, INC.)

Title: 	Preliminary RAN1 UE features list for Rel-17 NR

Document for:	Information

- Introduction

This contribution includes preliminary RAN1 UE features for Rel-17 NR provided from WI rapporterurs.

- NR_FeMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI for intra- and inter-cell beam management | For both intra- and inter-cell beam management: 1. Support of joint TCI and separate DL/UL TCI with their components (configuration mechanism, QCL rules, applicable source and target signals) 2. Support of association between TCI state and UL PC settings (PLRS and other, including handling of beam misalignment for PLRS) 3. Support of MAC-CE+DCI-based beam indication (including TCI state activation, use of DCI formats 1_1/1_2 with and wthout DL assignment) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM and mTRP) | 1. Support of beam measurement on SSB(s) with PCI(s) different from serving cell PCI 2. Support of beam reporting up to Kmax=4 SSBRI-RSRP pairs, with at least one pair associated with a PCI different serving cell PCI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and associated P-MPR in PHR reporting) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-2-1 | Multi-TRP PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Maximum number of such overlaps in 3 |  |  |  |  |  |  |  |  | Component1: {Support, Not support} Component2: {2, 3, [{2,3}]} Component3: {monitored, not monitored} Component4: details FFS | Optional |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition |  |  |  |  |  |  |  | FR2 only | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) 2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B, 3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of including other cell info in RRC TCI state config 2. Support of X>1 (max number of PCIs different from serving cell) |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-5-1 | Beam reporting enhancements | Support of the maximum number of beam groups |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional |
| 23. NR_FeMIMO | 23-5-2 | Maximum number of BFD-RS resources per set for MTRP BFR | Support of the maximum number of BFD-RS resources per set |  |  |  |  |  |  |  |  | Candidate values: {1, [2,…]} | Optional |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. Support of scheme A for PDCCH 2. Support of scheme A for PDSCH 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B for PDCCH and PDSCH 2. Support of dynamic switching between single-TRP and SFN PDSCH scheme B for DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST) | Support of simultaneous activation of two TCI states in all BWPs across set of configured component carriers |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4 | Default beam setup for HST | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of Nmax=1 - FFS others |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports per source in a resource set for Multi-TRP CSI | {24 ports, 32 ports} |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-3 | Support of max # resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI |  |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-7-5 | Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses | FR2 only |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | Support of SRS antenna switching xTyR with y>4 |  |  |  |  |  |  |  |  | {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} | Optional |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets for antenna switching |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency sounding |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for MIMO SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=1 - FFS others |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II |  |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II |  |  |  |  |  |  |  |  |  | {Support, Not support} | Optional |

- NR_ext_to_71GHz

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1 | General FR2-2 support | 1. Support 120KHz SCS transmission and reception 2. Support multi-RB PUCCH format 0/1/4 3. PRACH with 120KHz SCS and length 139/571/1151 |  |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-2 | 120KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-3 | 480KHz SSB based stand-alone support | 1. Support 480KHz SSB for initial access | 24-1, 24-2, 24-4 |  |  |  |  |  |  |  | From WID: - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) |  |
| 24. NR_ext_to_71GHz | 24-4 | 480KHz SCS support | 1. 480KHz SCS for UL transmission 2. 480KH SCS for DL reception 3. 480KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 480KHz with X=4 5. PRACH with 480KHz and length 139/[571] |  |  |  |  |  |  |  |  | From WID: In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-5 | 960KHz SCS support | 1. 960KHz SCS for UL transmission 2. 960KH SCS for DL reception 3. 960KHz for SSB monitoring 4. Multiple-slot PDCCH monitoring for 960KHz with X=8 5. PRACH with 960KHz and length 139 |  |  |  |  |  |  |  |  | From WID - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported. |  |
| 24. NR_ext_to_71GHz | 24-? | Cat 3 or Cat 4 LBT support for FR2-2 unlicensed operation | 1. Cat 3 or 4 LBT support (not agreed yet if CW is supported, so it can be either Cat 3 or Cat 4 LBT for now. Will update when we have agreement) | 24-1 |  |  |  |  |  |  |  |  |  |
| 24. NR_ext_to_71GHz | 24-? | Cat 2 LBT support for FR2-2 unlicensed operation | 1. Support Cat 2 LBT | 24-1 |  |  |  |  |  |  |  |  |  |

- NR_IIOT_URLLC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-1 | SPS HARQ-ACK deferral in case of TDD collision | 1. Idenfify HARQ-ACK bits of active SPS configurations for deferral in the initial PUCCH slot 2. Determination of the target PUCCH slot for SPS HARQ-ACK deferral 3. Multiplexing and transmission of deferred SPS HARQ-ACK information in the target PUCCH slot | 5-18 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-2 | Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 0 and 2 over multiple slots with K = 2, 4, 8 | 4-23 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8 | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with RRC configured repetition factor K = 2, 4, 8 | 4-23 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-3a | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication | Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication. | 25-3 30-X | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-4 | One-shot HARQ ACK feedback triggered by DCI format 1_2 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_2 without scheduling a PDSCH using a reserved FDRA value | 10-16 11-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-5 | PHY priority handling for one-shot HARQ ACK feedback | Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI | 10-16 11-4 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-6 | Enhanced type 3 HARQ-ACK codebook feedback | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support configuration of up to X enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_1 and DCI 1_2 (for a UE supporting DCI format 1_2, 11-1) 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) | 10-16 | Yes | N/A |  | Per UE | No | No | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1,...,X} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-7 | Triggered HARQ-ACK codebook re-transmission | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_1 and DCI format 1_2 (for a UE supporting DCI format 1_2, 11-1) 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-8 | Semi-static HARQ-ACK codebook for sub-slot PUCCH | Semi-static (Type 1) HARQ-ACK codebook for sub-slot based PUCCH configuration | 4-11 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-9 | Semi-static PUCCH cell (FFS or PUCCH carrier) switching | Semi-static PUCCH cell (FFS or PUCCH carrier) switching using configured time-domain domain pattern of applicable PUCCH cell / carrier |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-10 | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication | PUCCH cell (FFS or PUCCH carrier) switching based on dynamic indication in the DCI scheduling the PUCCH |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling [This FG is a part of basic operation for following scenarios defined in TS38.300 Scenario A2, B, C, D and E with semi-static channel access mode] |
| 25. NR_IIOT_URLLC_enh | 25-13 | UE initiating a semi-static channel occupancy with independent configurations from gNB semi-static channel access configurations | Support initiating a semi-static channel access occupancy by the UE where the corresponding period is independently configured from the period configured for a semi-static channel occupancy that can be initiated by gNB. | 10-1a | Yes | N/A |  | Per band | N/A | N/A | N/A | The signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-14 | PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-15 | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell | 12-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-16 | HARQ-ACK with different priorities multiplexing using a PUCCH | 1. Support multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH. Support separate coding for the two HARQ-ACKs. 2. [Support multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations).] 3. [Support multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH.] | 11-3 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-17 | HARQ-ACK piggybacked on a PUSCH of a different priority | 1. Support multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 2. Support multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only). Support separate beta_offset values for this priority combination. 3. Support multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. 4. Support multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. | 11-3 12-1 | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. |  | Yes | N/A |  | Per UE | No | No | N/A |  | Optional with capability signaling |

- NR_NTN_solutions

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE and RRC_INACTIVE states based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. | Mandatory |
| 26. NR_NTN_solutions | 26-2 | Uplink Frequency/Doppler pre-compensation | - In RRC_IDLE and RRC_INACTIVE states calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED states, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. |  | No | No | Release 17 UE cannot access NTN |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 | An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | Mandatory |
| 26. NR_NTN_solutions | 26-3 | Start of RAR window | - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow - delaying the start of msgB-ResponseWindow | 26-2 | No | No | Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 26-2 | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | [Optional] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is 32 for both UL and DL - Support on the maximal HARQ process number is up to UE capability |  | Yes | No | Increasing the number of HARQ processes avoids HARQ stalling, |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |
| 26. NR_NTN_solutions | 26-6 | HARQ CODEBOOK Enhancement | - Enhancement on Type-2 HARQ codebook in NTN - Enhancement on Type-1 HARQ codebook in NTN |  | Yes | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Mandatory |
| 26. NR_NTN_solutions | 26-7 | NTN Performance enhancement | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 8, 16 or 32 |  | Yes | No | Particularly beneficial in case HARQ feedback is disabled |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - UE polarization capability (RHCP, LHCP, Linear) - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. |  | No | No |  |  | No | No | support mixture of FDD/TDD (for HAPS) and/or FR1/FR2 |  | Optional |

- NR_pos_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-x1 | Mitigation of UE Rx timing delays | The maximum number of UE-RxTEG per UE, which is supported and reported by UE for DL TDOA FFS: the values (>1). FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFF: whether to have separate values for DL TDOA and/or Multi-RTT positioning |  | No |  | Mitigation of UE Rx timing delays is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x2 | Mitigation of UE Tx timing delays | The maximum number of UE-TxTEG per UE, which is supported and reported by UE for UL TDOA and/or Multi-RTT positioning FFS; the values (>1). FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FF: whether a UE supports different values for UL TDOA and/or Multi-RTT positioning |  | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. FFS: whether gNB needs to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x3 | Mitigation of UE RxTx timing delays | The maximum number of UE-RxTxTEG per UE, which is supported and reported by UE for Multi-RTT positioning FFS; the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated |  | No |  | Mitigation of UE RxTx timing delays is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-x4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS; The values (>1). | 27-x1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. |  |
| 27. NR_pos_enh | 27-z1 | Support of PRS RSRP of the first path | UE’s capability to support providing the PRS RSRP of the first path |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-z2 | Support of DL PRS RSRP reporting for more than 8 measurements. | UE capability to support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Limit the maximum number of DL PRS RSRP associated with the same Rx beam index |  | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u1 | Support of M-sample measurements. | The capability to support providing a measurement based on measuring M samples (instances) of a DL PRS resurce set M=[1, 4]. FFS: other values. If the UE does not provide the capability, the UE is assume to support M=4 only. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u5 | PRS measurement outside MG | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG. Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 2B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2C refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-u6 | PRS Processing Capability outside MG | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-u5 | No |  |  | Per band | n/a |  |  | FFS |  |
| 27. NR_pos_enh | 27-v1 | LOS/NLOS Indicator | UE’s capability to support reporting LoS/NLoS indicator to LMF for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning. FFS: whether to have separate capability component for RSTD and UE Rx-Tx time difference measurements. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-w1 | Support of on-demand PRS | UE’s capability to support UE-initiated on-demand PRS. |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |

- NR_redcap

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. |  | Yes |  | Impact on UE complexity | Per UE | No | [No] |  | RedCap UEs do not support carrier aggregation or dual connectivity. | Optional with capability signaling |
| 28. NR_redcap | 28-2 | Number of UE Rx branches and DL MIMO layers for RedCap UE | 1. For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. 2. For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. | 28-1 | Yes |  | Impact on UE complexity | [Per band] | No | [No] |  | For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. Detailed signalling is up to RAN2. | Optional with capability signaling |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) for RedCap UE | 28-1 | Yes |  | Impact on UE complexity | [Per band] | FDD only | FR1 only |  |  | Optional with capability signaling |
| 28. NR_redcap | 28-4 | DL 256QAM support for RedCap UE | 1. Support of 256QAM for PDSCH for RedCap UE 2. Support of 256QAM MCS table (Table 5.1.3.1-2 in TS 38.214) for PDSCH for RedCap UE 3. Support of CQI table 2 (Table 5.2.2.1-3 in TS 38.214) for RedCap UE | 28-1 | Yes |  | Impacts on UE complexity and DL link performance at high SNR | [Per band] | No | [No] |  | For RedCap UEs, the 256QAM MCS table for PDSCH and CQI table 2 are only supported if the UE supports 256QAM for PDSCH. | Optional with capability signaling |
| 28. NR_redcap | 28-5 | UL 256QAM support for RedCap UE | 1. Support of 256QAM for PUSCH for RedCap UE 2. Support of 256QAM MCS table (Table 5.1.3.1-2 in TS 38.214) for PUSCH for RedCap UE | 28-1 | Yes |  | Impact on UE complexity and UL link performance at high SNR | [Per band] | No | [No] |  | For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH. | Optional with capability signaling |

- NR_UE_pow_sav_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | - Support of PDCCH monitoring adaptation behaviour 1/1A - Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] |  | Y |  | PDCCH monitoring adaptation within an active BWP is not supported | Per UE | N | N | N | FFS: Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A/2B | Optional |

- NR_cov_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-5 | Dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |

- NR_IAB_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception |  | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | Restricted DU beam indication | Support [Simultaneous Operation DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | Recommended/[Non-preferred] MT beam indication | Support [Simultaneous Operation MT Beam Indication] transmission |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | Timing case indication | Support [Timing Case Indication] reception |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing offset | Support [Case7 Timing Offset] reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | support [Desired DL TX Power Adjustment] reporting |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting, is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | DL TX power adjustment | support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-8 | Desired UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |

- NR_SL_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by NR Uu or preconfiguration. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [32-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [32-1], [32-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |
| 32. NR_SL_enh | 32-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [32-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported. |

- NR_MBS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  | Yes |  |  | Per UE | No | No |  |  | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. - Support PTM retransmission for multicast. - Support PTP retransmission for multicast. |  | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Intra-slot TDM-ed multiple unicast PDSCH and/or multiple group-common PDSCH | - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

- NR_DSS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 |  |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions - CIF configured on sSCell for CCS from sSCell to PCell/PSCell - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling | 6-5 | Yes |  |  | Per BC | N | N |  | sSCell can only be configured in FR1 |  |

- LTE_NR_DC_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | Temporary RS based SCell activation | SCell activation with the assistance of temporary RS - Temporary RS is aperiodic and triggered by MAC CE. | 6-5 | Yes | N/A |  | Per UE[/Per BC] | No | No |  |  | Optional with capability signalling |

- NR_DL1024QAM_FR1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. |  |  |  |  | Per Band |  | No |  | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability |  |

- [NR_RF_FR1_enh]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. [NR_RF_FR1_enh] | 37-x |  |  |  |  |  |  |  |  |  |  |  |  |

Note: Placeholder as there seems no RAN1 UE features for Rel-17 Tx switching in rapporteur’s understanding so far, but companies can provide their views on potential RAN1 UE features in RAN1#106bis-e.

- [NR_SmallData_INACTIVE]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. [NR_SmallData_INACTIVE] | 38-x |  |  |  |  |  |  |  |  |  |  |  |  |

Note: Placeholder as there seems no no RAN1 UE features for SDT in rapporteur’s understanding so far, but companies can provide their views on potential RAN1 UE features in RAN1#106bis-e.