3GPP TSG-RAN WG1 Meeting #106-e	R1-210xxxx

e-Meeting, August 16th – August 27th, 2021

Agenda item:		7.2.5

Source:		Moderator (Apple Inc.)

Title:	Summary of email discussion [106-e-NR-L1enh-URLLC-03] on sub-slot-based HARQ-ACK timing in Rel-16

Document for:		Discussion and Decision

# 1	Introduction

This contribution provides the summary for the following email discussion in RAN1#106-e:

- [106-e-NR-L1enh-URLLC-03] Issue#7: HARQ-ACK timing for sub-slot based HARQ-ACK feedback by August 20 – Sigen (Apple)

Section 2 provides the background information. Section 3 captures the detailed email discussions. Section 4 summarizes the outcome of the email discussion.

# 2	Background

For HARQ-ACK, the PUCCH for HARQ-ACK is transmitted in UL slot n+k, where k is indicated in UL DCI, and n is determined based on PDSCH. When UL SCS is larger than DL SCS, two different interpretations existed in the history of RAN1 discussions. This was discussed in RAN1#104b-e [1] and RAN1#105-e [2], and it was concluded that the two different interpretations can exist in Rel-15, but for Rel-16, a working assumption was made to adopt interpretation 2 below. The main reasons for adopting interpretation 2 are that it is aligned with earlier RAN1 agreement and it is also aligned with the Type-1 HARQ-ACK codebook construction in TS 38.213.

Conclusion: (RAN1#104b-e)

For HARQ-ACK timing in Rel-15, in case UL SCS is larger than DL SCS, there are two different interpretations:

-       Interpretation 1: k = 0 corresponds to the last UL slot that overlaps with the PDSCH

-       Interpretation 2: k = 0 corresponds to the last UL slot that overlaps with the DL slot for the PDSCH

Further discuss this issue for Rel-16 in future meetings.

Working Assumption (RAN1#105-e)

For HARQ-ACK timing in Rel-16 with slot-based HARQ-ACK feedback, in case UL SCS is larger than DL SCS, k = 0 corresponds to the last UL slot that overlaps with the DL slot for the PDSCH.

- Further discuss the HARQ-ACK timing for sub-slot-based HARQ-ACK feedback

- FFS specification impact

What remains open is the HARQ-ACK timing for sub-slot-based HARQ-ACK feedback. As discussed in [2], similar to slot-based HARQ-ACK feedback, two options are available:

For HARQ-ACK timing in Rel-16 with sub-slot-based HARQ-ACK feedback,

- Option 1: k = 0 corresponds to the last UL sub-slot that overlaps with the PDSCH.

- Option 2: k = 0 corresponds to the last UL sub-slot that overlaps with the DL slot for the PDSCH.

Option 1 is aligned with the following RAN1#97 agreement, while Option 2 is aligned with the working assumption that was made for slot-based HARQ-ACK timing in RAN1#105-e.

Agreements: (RAN1#97)

For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH.

- Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association.

- FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth.

- Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot.

This issue was discussed in [3]-[6] submitted to RAN1#106-e. A draft CR is also provided in [3].

Option 2 is preferred in [3][6], while Option 1 is preferred in [4][5]. The arguments for each option are summarized as follows:

- Option 1

  - Reduced latency in some cases (e.g. when PDSCH is at the beginning of the DL slot and the earliest symbols for HARQ-ACK feedback are UL symbols) compared to Option 2, as shown in Figure 1.

- Option 2

  - Consistent behavior between slot-based and sub-slot-based HARQ-ACK feedback timing

  - Principle of Type 1 HARQ-ACK codebook construction can be reused for sub-slot-based HARQ-ACK feedback.

  - Less gNB/UE complexity and less specification impact

Figure 1 k = 0 for Option 1 and Option 2

# 3	Email Discussions

## 3.1	First Round of Email Discussion

For HARQ-ACK timing in Rel-16 with sub-slot-based HARQ-ACK feedback,

- Option 1: k = 0 corresponds to the last UL sub-slot that overlaps with the PDSCH.

- Option 2: k = 0 corresponds to the last UL sub-slot that overlaps with the DL slot for the PDSCH.

Companies please indicate which option you support.

| Option 1 |  |
| --- | --- |
| Option 2 |  |

Companies please provide detailed reasons why you support Option 1 or 2.

| Company | Comments |
| --- | --- |

The TP for both slot-based and sub-slot-based HARQ-ACK feedback will be discussed together after this issue is concluded.

## 3.2	Second Round of Email Discussion

# 4	Outcome of the Email Discussion

# References

- R1-2104105, Summary of email discussion [104b-e-NR-7.1CRs-04] on the correction for HARQ-ACK timing, Moderator (Apple Inc.), RAN1#104b-e, April 2021.

- R1-2106301, Summary of email discussion [105-e-NR-7.1CRs-13] on the correction for HARQ-ACK timing in Rel-16, Moderator (Apple Inc.), RAN1#105-e, May 2021.

- R1-2107063, Draft CR on Timing for slot-based and sub-slot-based HARQ-ACK Feedback, Ericsson, RAN1#106-e, Aug. 2021.

- R1-2107266, Remaining issues on HARQ-ACK timing for sub-slot based HARQ-ACK feedback, OPPO, RAN1#106-e, Aug. 2021.

- R1-2107681, Correction for HARQ-ACK timing in Rel-16, Huawei, HiSilicon, RAN1#106-e, Aug. 2021.

- R1-2107713, Discussions on HARQ-ACK timing for sub-slot-based HARQ-ACK feedback in Rel-16, Apple, RAN1#106-e, Aug. 2021.