3GPP TSG-RAN WG1 Meeting #106-e	R1-210xxxx

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.2.8

Source:	Moderator (Huawei)

Title:	Summary of [106-e-NR-Pos-02] DL PRS antenna ports and editorial corrections for SRS

Document for:	Discussion and decision

Introduction

This document provides the summary for [106-e-NR-Pos-02] on the PRS antenna ports and some editorial changes.

[106-e-NR-Pos-02] Email discussion/approval on DL PRS antenna ports and editorial corrections for SRS (Aspect #2) until August 20 – Su (Huawei)

The related submission of contribution includes

- R1-2106504	Draft CR on PRS antenna ports	Huawei, HiSilicon

The email discussion is divided into two rounds, with the intermediate summary at 23:59 UTC, Aug. 18.

General information

In [1], it is noticed that in TS 38.211, the antenna ports description lacks that for DL PRS, particularly for the cases when (slot-level) repetition is configured, whereas the DM-RS have the dedicated restriction for the same port “in the same slot”. Therefore, 1) the description for PRS antenna ports is added and 2) some editorial corrections to the SRS are provided as shown below:

### 4.4.1 Antenna ports ========================= Unchanged parts ========================= For DM-RS associated with a PBCH, the channel over which a PBCH symbol on one antenna port is conveyed can be inferred from the channel over which a DM-RS symbol on the same antenna port is conveyed only if the two symbols are within a SS/PBCH block transmitted within the same slot, and with the same block index according to clause 7.4.3.1. For PRS, the channel over which a PRS symbol on one antenna port is conveyed can be inferred from the channel over which a PRS symbol on the same antenna port is conveyed only if the two symbols are within a DL PRS resource within the same slot. Two antenna ports are said to be quasi co-located if the large-scale properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters. ========================= Unchanged parts ========================= ##### 6.4.1.4.2 Sequence generation ========================= Unchanged parts ========================= The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Resource IE or the SRS-PosResource IE. The SRS sequence identity is given by the higher layer parameter sequenceId in the SRS-Resource IE, in which case nIDSRS∈0, 1, …, 1023, or the SRS-PosResource IE, in which case nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. ========================= Unchanged parts ========================= ##### 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. ========================= Unchanged parts =========================

Discussion

PRS antenna ports across slots

The reason for the change given by [1] is that in principle, the UE should not assume that the PRS of the same DL PRS resource repeated in different slots are from the same antenna port, which means that coherent combining for the same PRS resource across the repetition slots should not be expected.

- Proposal: Decide whether to adopt the following change.

For PRS, the channel over which a PRS symbol on one antenna port is conveyed can be inferred from the channel over which a PRS symbol on the same antenna port is conveyed only if the two symbols are within a DL PRS resource within the same slot.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Has concern | If the restriction of “within the same slot” included, then the UE will not be allowed to combine the repetitions of one same DL PRS resource to improve the coverage performance of PRS and thus impair the NR positioning performance. The same channel shall be expected on repetitions of the same PRS resource within each transmission periodicity. |
| Huawei, HiSilicon | Support. | To OPPO, We think coherent combining may not always be possible across repetitions. Unless explicitly indicated, we think UE should make such assumption. Note that for repetition, we may also repetition in non-adjacent slots. |

Editorial changes on SRS

The change removes the “-r16” suffix and the duplicated periodicity and offset for periodic and semi-persistent positioning SRS.

- Proposal: Decide whether to adopt the following change.

##### 6.4.1.4.2 Sequence generation ========================= Unchanged parts ========================= The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Resource IE or the SRS-PosResource IE. The SRS sequence identity is given by the higher layer parameter sequenceId in the SRS-Resource IE, in which case nIDSRS∈0, 1, …, 1023, or the SRS-PosResource IE, in which case nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. ========================= Unchanged parts ========================= ##### 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE, or in the SRS-PosResource IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 11.1 of [5, TS 38.213].

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | NO | The current spec has no issue. |
| Huawei, HiSilicon | Yes | We do not need a standalone CR for the change, but can be the accompanying changes with other essential corrections, which I believe should be a normal procedure. From our side, we are fine to also leave the change to an editor alignment CR. |

Conclusion

TBD