3GPP TSG RAN WG1 #106-e	R1-2106438

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.12.1

Source:	Huawei, HiSilicon, CBN

Title:	Resource configuration and group scheduling for RRC_CONNECTED UEs

Document for:	Discussion and Decision

Introduction

This contribution continues the discussion of scheduling multicast for RRC_CONNECTED UEs including multicast resource configuration, scheduling scheme, and detailed issues etc., per the agreements achieved in the last meeting [1].

Discussion on multicast scheduling

  - Frequency resource configuration

Down selection regarding the common frequency resource between options 2A/2B has been discussed for several meetings and the last meeting agreed Option 2B as a working assumption:

Working assumption: Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs. - FFS: CFR associated with initial BWP - FFS: CFR larger than initial BWP

Given there is no fundamental issue identified from our perspective, we propose to confirm this working assumption. Regarding the two FFSs CFR associated with initial BWP or CFR is larger than initial BWP, we view that the CFR for multicast should be contained within the initial BWP.

Firstly, the configured resource for any transmission is supposed to be within one BWP because BWP is usually taken to consider the impact to implementation for power saving. Secondly, SIB1 configured initial BWP is usually configured to UE which supports a single BWP. Therefore, this BWP is usually large enough and the necessity of CFR being larger than this initial BWP is questionable. Considering the above two reasons, granted a CFR larger than SIB1 configured initial BWP is needed, it is probably called one BWP as well. In such a case, when UE receives multicast in this specific BWP and unicast in initial BWP simultaneously, whether there is BWP switching time is still controversial according to previous discussion. Per our understanding of RAN4 specification, we think the BWP switching time is needed in such a case.

Overall, in addition to confirming the working assumption, we think this should be extended to initial BWP case as well.

Proposal 1: For the common frequency resource configuration for multicast

- Confirm the Working Assumption of Option 2B:

  - The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

- Support CFR associated with initial BWP and is not larger than the SIB1 configured initial BWP.

Regarding the configured CORESETS and CFR configured for multicast, there is a WA made in RAN1#105e.

Working assumption: The maximum number of CORESETs per BWP is not increased for support of MBS, and the number of CORESETs configured within the CFR is left to gNB implementation.

Regarding the CORESET used for unicast and multicast, CORESETs are configured by gNB and whether a CORESET is used for multicast or unicast scheduling is also up to gNB. Depending on whether the CFR fully contains the CORESETS in frequency domain configured for unicast scheduling within the unicast dedicated BWP, additional CORESETS may or may not be needed. For example, if does not contain, additional CORESET(s) for multicast scheduling is needed; otherwise, gNB can configure the same CORESET for both unicast and multicast scheduling in the CFR if needed. It should be noted that even if additional CORESET(s) is configured, the total number of CORESETs configured to UE is not expected to exceed UE capability at least in this Release for commercialization.

Regarding the number of configured CFR per dedicated unicast BWP, the need to support more than one CFR in Rel-17 is not strong and is not preferred since simplified specification support for early commercial deployment is more important.

Proposal 2: For CFR for multicast scheduling confined within a dedicated unicast BWP,

- One CFR per a dedicated BWP is sufficient in Rel-17.

- It is up to gNB to configure the same or different CORESETs for unicast and multicast scheduling within the CFR.

- The total number of CORESETs is not expected to be increased in Rel-17 comparing to the number UE supported in Rel-16.

  - The same HPN for PTM and PTP

Regarding HARQ process number management between unicast and multicast, it was agreed up to gNB to allocate the HPN between unicast and multicast. For the same TB association, the same HPN and NDI are used.

Agreement:

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

Conclusion:

The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception.

- How to allocate HARQ processes between unicast and multicast is up to gNB.

However, considering the multicast may target a large group of UEs, coordinating a HPN that is not used for any UE for unicast with the group is difficult and is not necessary, especially because supporting PTP retransmission for multicast is enabled. Next PTM initial transmission can use the same HPN if the earlier multicast TB is failed in decoding for some UEs for which the TB can be retransmitted by PTP.

With the assumption that the same HPN can be used for PTM and PTP, this section discusses two issues that were discussed in the last meeting as well.

    - PTP retransmission for multicast and PTP transmission for unicast

For HARQ process management for unicast and multicast, there was an agreement made in the previous meeting:

Agreement: For HARQ process management, further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

When the same HPID is used for unicast and multicast, what if UE missed PTM1 DCI as shown in Fig. 1?

Fig. 1: PTM1 DCI is missed when the same HPN is used for PTM and PTP

Take UE-b shown in Fig. 1 as an example, there are two sub cases depending on whether the TB transmitted by PTP1 is ACK-ed or NACK-ed:

- Subcase1: If TB of PTP1 at time “t1” is ACK-ed, the GC-PDCCH scheduling PTM1 at time ”t2” is missed, and PTP2 at time “t3” is a retransmission for PTM1, UE-b will not soft-combine the PTP1 transmission and PTP2 retransmission because PTP1 transmission is ACK-ed. UE-b will assume gNB missed the ACK of TB transmitted by PTP1, and then UE-b will directly report an ACK for PTP2 instead of decoding the PTP2. In this case, the TB of PTM1 will be lost.

- Subcase2: If TB of PTP1 at time “t1” is NACK-ed, the GC-PDCCH scheduling PTM1 at time ”t2” is missed, and PTP2 at time “t3” is a retransmission for PTM1, UE-b will wrongly soft-combine the PTP1 transmission and PTP2 retransmission due to the same NDI and the same HPN.

For solving this problem, the straightforward approach is that the scheduling DCI indicates the TB of PTP2 is for multicast retransmission or unicast (re)transmission.

Regarding RNTI configured for multicast, there was an agreement made from RAN2 [2]. Multiple G-RNTI/G-CS-RNTI can be configured for different MBS services.

One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS One-to-one mapping between G-CS-RNTI and MBS session is supported in NR MBS. Other mappings FFS. A UE can support multiple G-RNTIs/G-CS-RNTIs, It is FFS whether this depends on UE capability. Inform RAN1 of this agreement.

Based on what we described above, a PTP transmission should be indicated for unicast (re)transmission or for multicast retransmission. If multiple G-RNTIs are configured for a UE, a PTP transmission indicated for a multicast retransmission should be further indicated for which G-RNTI.

Proposal 3: Support DCI scheduling PTP transmission indicates whether the transmission is for unicast (re)transmission or for multicast retransmission.

- For UE configured with multiple G-RNTIs, the DCI should further differentiate the PTP transmission is for which G-RNTI retransmission.

    - PTP retransmission for multicast and PTM1 initial transmission for multicast

It was agreed to use the same HPN in the DCI scheduling retransmission, regardless the DCI is scrambled by C-RNTI (PTP) or G-RNTI (PTM). It was also discussed that whether the same HPN can be reused for initial transmission for of a new TB for MBS if the earlier TB for MBS using the same HPN was failed decoding but the discussion does not lead to any conclusion/FFS. We continue this discussion in this section.

It should be noted that PTM scheduling targets a group of UE to receive the same TB, there could be many cases that some of UEs failed decoding, missed the scheduling, or the HARQ-ACK feedback taking long time for some reasons, it does not make sense to restrict network to use different HPN for such cases because the HPN for initial transmission of a new TB also needs to be different from the HPN for unicast for each of UEs in the group.

Enabling network scheduling anther PDSCH for a given HARQ process before the end of the expected transmission of HARQ-ACK for that HARQ process would help alleviate network scheduling difficulties in some extent as illustrated in Fig. 2. UE#1 in the group failed decoding multicast TB#1 with HPN 000 at time “t1”, the network can still use HPN 000 for TB#2 initial transmission by PTM sheme1 and use HPN 000 for UE1’s TB1 retransmission by PTP at time “t2”.

Fig. 2: HPN coordination for multicast between PTM scheme 1 initial transmission and PTP retransmission

It was argued in the last meeting that a UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process in both Rel-15 and Rel-16. However, it does not work well for the cases we are discussing for PTM, for which network should not be restricted to use a different HPN for the new TB initial transmission. In other words, when UE is scheduled to receive a multicast initial transmission and a PTP retransmission with the same HPN, especially at the same time, UE’s behavior should be defined. Considering the probability of receiving successfully, PTP retransmission should be received, because UE can do soft-combine with the soft information buffered in the HARQ process. The ignored multicast initial transition at time “t2” can be transmitted later.

Proposal 4: For multicast services, when UE is scheduled to receive a PTM1 initial transmission and a PTP retransmission with the same HPN at the same time, UE should receive the PTP retransmission.

  - Scheduling DCI for multicast

For PDCCH scrambling, nID and nRNTI for initializing scrambling sequence generator equal to NIDcell and zero, respectively, for CSS, according to the current specification [3] as follows:

#### 7.3.2.3 Scrambling The UE shall assume the block of bits b0, …, b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, is scrambled prior to modulation, resulting in a block of scrambled bits b0, …, b(Mbit-1) according to bi=bi+c(i) mod 2 where the scrambling sequence is given by clause 5.2.1. The scrambling sequence generator shall be initialized with where - for a UE-specific search space as defined in clause 10 of [5, TS 38.213], equals the higher-layer parameter pdcch-DMRS-ScramblingID if configured, - nID=NIDcell otherwise and where - is given by the C-RNTI for a PDCCH in a UE-specific search space if the higher-layer parameter pdcch-DMRS-ScramblingID is configured, and - otherwise.

As agreed in RAN1#105-e meeting, Type-x CSS is used for group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, which is supposed to include the second DCI format which takes the “non-fallback” DCI as the baseline. For such a case, the nID and nRNTI for initializing scrambling sequence generator for scheduling multicast in Type-x CSS should be configurable via higher layer.

Proposal 5: For initializing scrambling sequence generator for group common PDCCH for scheduling multicast in Type-x CSS,

- nID should be configurable, and equals to a higher layer parameter pdcch-DMRS-ScramblingID if configured.

- nRNTI is given by the G-RNTI.

  - Simultaneous retransmissions

In the last meeting, for dynamic or SPS scheduling for multicast, one issue discussed without conclusion is whether PTM and PTP retransmission can be configured to UE simultaneously.

From UE perspective, if UE is configured for both unicast and multicast reception, UE will always monitor C-RNTI and G-RNTI scrambled DCI. Until the DCI is decoded, UE knows the scheduling is for initial transmission or retransmission by the HARQ process number and NDI. From this sense, UE is always ready to receive PTM and PTP for retransmission with no need of explicit/default configuration of which scheme being used for retransmission. In other words, as long as UE supports both unicast and multicast reception, PTM and PTP retransmission schemes are simultaneously supported. Meanwhile, it is up to gNB to retransmit the failed TB via PTM or PTP.

Proposal 6: It is up to gNB to retransmit the failed TB via PTM scheme 1 or PTP.

  - UE does not need to be configured with PTM scheme 1 or PTP or both for retransmission.

  - SPS for multicast

For SPS group-common PDSCH for multicast for RRC_CONNECTED UEs, the agreement is shown as follows:

Agreement: For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support at least one of the following alternatives. - Alt 1: retransmit the activation command via group-common PDCCH. - Alt 2: retransmit the activation command via UE-specific PDCCH. - Alt 3: retransmit the activation command via MAC-CE. - FFS other details. - Note: Down-selection can take into account the HARQ-ACK feedback scheme for SPS activation

For multicast SPS, at least ACK/NACK based HARQ-ACK feedback is supported. NACK-only is also agreed for multicast SPS PDSCH without PDCCH scheduling and FFS NACK-only for SPS activation/deactivation. The discussion for this issue can refer to our contribution in AI 8.12.2 [4], where the proposal is that NACK-only is not supported for activation/deactivation since otherwise network cannot differentiate the activation/deactivation is missed or ACK-ed by UEs.

For SPS reception, the PDSCH transmission occasions depend on the periodicity of the SPS configuration and the occasion where the PDCCH for activation is transmitted. The HPID calculation also needs the slot index of the slot where the first scheduled PDSCH is transmitted. Regarding the listed three alternatives, as shown in Fig. 3, both Alt 1 and Alt 2 have the similar problem that the opportunities to retransmit the activation command would be restricted to recover the SPS PDSCH reception, especially when UE is configured with DRX. The retransmission of the activation command should be transmitted as soon as possible before UE enters the inactive period of DRX after on-duration timer expires. k0 indicated by DCI indicating the first PDSCH transmission occasion cannot match all possible periodicities of the SPS configuration to achieve the purpose of retransmitting the activation command as early as possible.

Fig. 3: Example of Alt 1 and Alt 2: retransmit the activation command via PDCCH.

As shown in Fig. 4, for Alt 3, MAC-CE message can include some necessary information, e.g., the slot index of the slot where the first activation DCI was transmitted for recovering the SPS PDSCH reception. Moreover, the MAC-CE can be transmitted instantly after the first transmission is missed and before UE enters the inactive period of DRX because it is transmitted via PTP. Therefore, Alt 3 is preferred.

Fig. 4: Example of Alt 3: retransmit the activation command via MAC-CE.

Proposal 7: Support Alt 3: Retransmit the activation command via MAC-CE for reliability of multicast SPS.

Conclusions

This paper focuses on discussion on multicast scheduling for RRC_CONNECTED UEs, which leads to the proposals as follows:

Proposal 1: For the common frequency resource configuration for multicast

- Confirm the Working Assumption of Option 2B:

  - The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

- Support CFR associated with initial BWP and is not larger than the SIB1 configured initial BWP.

Proposal 2: For CFR for multicast scheduling confined within a dedicated unicast BWP,

- One CFR per a dedicated BWP is sufficient in Rel-17.

- It is up to gNB to configure the same or different CORESETs for unicast and multicast scheduling within the CFR.

- The total number of CORESETs is not expected to be increased in Rel-17 comparing to the number UE supported in Rel-16.

Proposal 3: Support DCI scheduling PTP transmission indicates whether the transmission is for unicast (re)transmission or for multicast retransmission.

- For UE configured with multiple G-RNTIs, the DCI should further differentiate the PTP transmission is for which G-RNTI retransmission.

Proposal 4: For multicast services, when UE is scheduled to receive a PTM1 initial transmission and a PTP retransmission with the same HPN at the same time, UE should receive the PTP retransmission.

Proposal 5: For initializing scrambling sequence generator for group common PDCCH for scheduling multicast in Type-x CSS,

- nID should be configurable, and equals to a higher layer parameter pdcch-DMRS-ScramblingID if configured.

- nRNTI is given by the G-RNTI.

Proposal 6: It is up to gNB to retransmit the failed TB via PTM scheme 1 or PTP.

  - UE does not need to be configured with PTM scheme 1 or PTP or both for retransmission.

Proposal 7: Support Alt 3: Retransmit the activation command via MAC-CE for reliability of multicast SPS.

- References

- RAN1 Chairman’s Notes, RAN1#105-e, e-Meeting, May 19 – May 27, 2021.

- RAN2 Chairman’ Notes, RAN2#114-e, e-Meeting, May 19 – May 27, 2021.

- 3GPP TS 38.211, “Physical channels and modulation”.

- R1-2106439, Mechanisms to improve reliability for RRC_CONNECTED UEs, RAN1#106-e, Huawei, HiSilicon, CBN, August 16 –August 27, 2021.