3GPP TSG RAN WG1 #106-e	R1-2106439

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon, CBN

Title:	Mechanisms to improve reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

Introduction

Last meeting progressed further in aspect of separate codebooks for unicast and multicast, Type-1 codebook generation for TDM-ed and FDM-ed cases in detail, NACK-only based feedback, HARQ-ACK feedback for SPS, and HARQ-ACK enabling/disabling, etc., with some of the agreements as follows:

Agreement:

- The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

This contribution pursues the discussion regarding the FFS of such agreements.

Separate codebooks/PUCCH-config

UE can be configured to generate separate HARQ-ACK codebooks corresponding to different priorities for unicast including both Type-1 and Type-2 codebooks as specified in Rel-16. It was agreed in the last meeting that UE supporting multicast can also be configured to generate separate codebooks for unicast and multicast, respectively, for Type-2 HARQ codebook. Such an agreement can naturally be extended to support Type-1 codebook.

Proposal 1: UE can be configured to generate separate Type-1 HARQ-ACK codebooks for unicast and multicast with different priorities, respectively.

In general, multicast scheduling targets a group of UEs for reception but unicast scheduling may only consider the single given UE. Therefore, from network perspective, it would be favorable to authorize network the freedom to manage the unicast scheduling and multicast scheduling separately. Given that HARQ-ACK feedback could affect the subsequent scheduling, network may accordingly request UE to generate the HARQ-ACK codebooks separately regardless of the same or different priorities, e.g., by configuring separate PUCCH-Config for multicast.

Another benefit of generating separate codebooks for the same priority is that it can simplify the issue for the case of slot-based PUCCH for multicast and subslot-based PUCCH for unicast. Specifically, when UE is configured with a separate PUCCH-Config for multicast from that for unicast and slot-based PUCCH and subslot-based PUCCH are configured wherein, respectively, as shown in Fig. 1, if the HARQ-ACK bits for unicast and multicast are of the same priority, if only support multiplexing the HARQ-ACK bits for generating a single codebook for the same priority, how the HARQ-ACK bits are multiplexed needs to be defined for such case in  Fig. 1 which needs to take UE processing timeline into account and as such many specification efforts are anticipated. Instead, generating two separate codebooks despite the same priority can simplify this issue and the mechanism as specified in URLLC for different priorities can be reused. In details, both codebooks can be transmitted as long as the resources are not overlapped and one of them is transmitted otherwise.

Fig. 1: Subslot-based PUCCH for unicast and slot-based PUCCH for multicast

Proposal 2: UE can be configured to generate separate HARQ-ACK codebooks (including both Type-1 and Type-2 codebooks cases) for unicast and multicast with the same priority.

Proposal 3: When UE is configured with a separate PUCCH-Config for multicast from that for unicast and slot-based PUCCH and subslot-based PUCCH are configured wherein, respectively, two separate codebooks are generated independently and one of them is transmitted when the PUCCH resources are overlapped including the cases of same or different priorities.

Regarding whether separate HARQ-ACK codebooks apply to separate PUCCH transmissions only, if the codebooks are of different priorities, separate codebooks should apply to separate PUCCH transmissions only at this stage since the feature of multiplexing codebooks with different priorities into one PUCCH transmission is still under discussion in Rel-17 URLLC topic.

Proposal 4: The separate HARQ-ACK codebooks for unicast and multicast should apply to separate PUCCH transmissions only.

FDM/TDM scheduling for Type-1 HARQ codebook

Now RAN1 has progressed on how Type-1 HARQ-ACK codebook is constructed for unicast and multicast multiplexing in FDM-ed and TDM-ed cases, respectively, when UE generates a single HARQ-ACK codebook for unicast and multicast. For TDM-ed multiplexing, Type-1 codebook is based on the union of the PDSCH TDRA sets of the unicast and the multicast. For FDM-ed multiplexing, Type-1 codebook is constructed by concatenating the HARQ-ACK sub-codebook for unicast followed by the sub-codebook for multicast.

Type-1 HARQ-ACK codebook size is determined by higher layer configurations (e.g., TDRA set, K1 set) semi-statically instead of physical layer scheduling dynamically. However, whether unicast and multicast are multiplexed in TDM-ed or FDM-ed manner is in general up to network scheduling dynamically. Therefore, in order to keep Type-1 codebook size independent of scheduling, UE needs to be aware of the multiplexing manner. For example, UE generates the Type-1 codebook via “FDM-ed” manner only when UE is configured to do so. Otherwise, UE will not expect unicast and multicast to be scheduled in FDM.

Proposal 5: UE generates the Type-1 codebook via “FDM-ed” manner only when UE is configured to do so. Otherwise, UE will not expect unicast and multicast to be scheduled in FDM.

Priority indication for “fallback”

Regarding priorities for the HARQ-ACK codebook for dynamic scheduling, it was agreed at RAN1#104e meeting that the priority index (0 or 1) can be included in the DCI formats scheduling the group-common PDSCH.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

What DCI formats are used for scheduling multicast is still under discussion. Given that multicast reception needs UE to obtain the necessary configurations in RRC_CONNECTED state in advance, the fields in DCI formats can depend on the configuration. In case the “fallback DCI” is needed for which some fields of DCI formats should not rely on variable higher layer configuration, how the priority index is indicated is a question. Taking the “default” value should be a straightforward resolution and the default value can be configured when G-RNTI and the corresponding configurations are configured.

Proposal 6: The priority index, if not included in the DCI formats, takes the default value that can be configured when G-RNTI is configured.

NACK-only based HARQ-ACK feedback

PUCCH resources

According to the discussion of PUCCH resources for NACK-only in the last meeting [1], different companies have different views on whether the resources should be specific for NACK-only or can be shared with ACK/NACK based PUCCH resources. Also, there was one sort of view that the resource configuration is eventually up to network anyhow, so there is no need to discuss it due to no specification impact. At least from the perspective of introducing NACK-only for low resource overhead, the PUCCH resources can be shared by UEs transmitting NACK-only as concluded in the last meeting.

Conclusion:

PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

From UE perspective, one minor specification impact way is keeping the maximum number of resource sets and the maximum number of resources per resource set unchanged. If the resources for NACK-only and ACK/NACK are shared and, for example, associated with the same PUCCH resource ID, when the ACK/NACK bits and NACK-only feedback are both available to be transmitted on the same PUCCH resource, either NACK-only is transformed into ACK/NACK based or NACK-only is dropped always or other ways not precluded. From this sense, the PUCCH resources for NACK-only are not necessarily dedicated. Alternatively, network CAN configure a dedicated PUCCH resource for all UEs to feedback NACK when failed in decoding PDSCH. Overall, it can be up to network for a proper configuration.

However, an issue we need to consider is whether the NACK-only feedback can be of different priorities. From the sense that NACK-only can benefit reducing UE power consumption and resource overhead, it can apply to both low and high priority services. Therefore, it makes sense to configure different PUCCH resources for low and high priority of NACK-only feedback.

Proposal 7: A separate PUCCH-Config or PUCCH-ConfigurationList for multicast can be but is not necessarily configured for NACK-only based HARQ-ACK feedback.

Proposal 8: For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

Overlapping PUCCH/PUSCH

There was a short discussion in the last meeting that how NACK-only is transmitted when more than one NACK-only based feedback are available for transmission in the same PUCCH slot [1] without achieving agreement but a list of alternatives are identified for further discussion as follows:

When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, down-select from the following alternatives:

- Alt1: Support multiplexing the HARQ-ACK bits.

- Alt2: Support UE transmitting more than one PUCCHs in the same slot-based PUCCH slot.

- Alt3: NACK-only bundling

- Alt4: support sub-slot based PUCCH for NACK-only based feedback.

- other alternatives are not precluded.

Furthermore, when the PUCCH resources for NACK-only are overlapped with other PUCCH or PUSCH transmissions, how NACK-only is transmitted also needs to be considered. Since NACK-only can be of either low or high priority based on the priority index indication, the straightforward resolution is that the NACK-only feedback will be dropped if it is of low priority. If the NACK-only is of high priority, there could be multiple alternatives worth considering as listed above. Likewise, such issue also exists for the case of NACK-only for multicast SPS without PDCCH scheduling as discussed in 5.4.

Overall, comparing all of these alternatives, Alt1 is favorable because it seems the simplest and can solve all these issues together with minimum specification impact.

Proposal 9: When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, or the NACK-only feedback are available with other UCI feedback on the same PUCCH resource, or the PUCCH resources are overlapped with PUSCH transmission,  the NACK-only feedback is transformed into the ACK/NACK based feedback.

PUCCH format

Both PUCCH format 0 and format 1 are supported for NACK-only based HARQ-ACK feedback. These two formats both support 1 or 2 HARQ-ACK bits and are transmitted on one PRB. PUCCH format 0 occupy one or two symbols but PUCCH format 1 can be transmitted over four and more symbols for better coverage. The common questions for both formats are whether two bits or only one bit is used to NACK-only feedback and what the cyclic shift is used to represent NACK.

The minor specification impact way is only supporting one HARQ-ACK bit for NACK-only given that NACK-only can be transformed into the ACK/NACK based feedback when more than one NACK-only based feedback are available for transmission in the same PUCCH slot, or the NACK-only feedback are available with other UCI feedback on the same PUCCH resource, or the PUCCH resources are overlapped with PUSCH transmission according to the discussions in the above sections. Accordingly, the sequence cyclic shift for NACK-only can be hard-coded into.

Proposal 10: Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

Proposal 11: The sequence cyclic shift for NACK-only is .

NACK-only for multicast SPS

For multicast SPS PDSCH without PDCCH scheduling, it has been agreed to support NACK-only feedback.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Support of ACK/NACK based feedback for multicast SPS has been agreed earlier. Now the question is whether NACK-only is supported for SPS activation/deactivation. Given that multicast SPS is targeted to a group of UEs and the transmission of the activation/deactivation command should be robust and as such the PDSCH could be transmitted as configured or stopped. From this sense, most of UEs are very likely to decode the activation/deactivation command successfully. If NACK-only is supported for the activation/deactivation command, network could not differentiate whether the command is missing or all UEs decode it successfully. Hence, NACK-only based HARQ-ACK feedback is not supported for multicast SPS activation/deactivation.

Proposal 12: For multicast SPS activation/deactivation, NACK-only based HARQ-ACK feedback is not supported.

It is worth noting that NACK-only for SPS without PDCCH scheduling is more naturally applied to the case that the HARQ feedback of one multicast SPS configuration is transmitted solely. When UE needs to transmit the HARQ-ACK feedback in the same PUCCH slot for multiple multicast SPS configurations, multicast SPS and unicast SPS, or multicast SPS and dynamic scheduling of unicast/multicast, one issue as discussed in section 5.2 needs to be solved, i.e., how the transmission will be when there are more than one PDSCH needs to report NACK-only. The simple and straightforward resolution is that the NACK-only feedback is transformed into the ACK/NACK based feedback in such cases.

Proposal 13: When UE is configured with NACK-only for multicast SPS without PDCCH scheduling and when the HARQ-ACK bits for more than one multicast SPS configuration, for multicast SPS and unicast SPS, or for multicast SPS and dynamic scheduling of unicast/multicast, needs to be transmitted in the same PUCCH slot, the NACK-only based feedback for multicast SPS is transformed into the ACK/NACK based feedback.

Enabling/disabling HARQ feedback

Regarding enabling/disabling HARQ feedback, after discussed in several meetings, the following working assumption was agreed in RAN1#105e:

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

During the discussion, there was one comment that how the group-common DCI indicating enabling/disabling HARQ-ACK feedback is implemented per UE [2]. Since the RRC configuration is per UE signalling, UEs can be independently configured to or not to react to the DCI indication since the DCI format or size needs to be unique due to group-common DCI.

From UE perspective, when RRC configuring the group-common indicating the function, UE reacts to the DCI indication to enable or disable the feedback. Otherwise, RRC can configure UE always to enable or disable the feedback and not react to the potential indication from the DCI because network may configure other UEs to listen to the DCI indication.

Proposal 14: Confirm the working assumption regarding the enabling/disabling HARQ-ACK feedback.

Conclusions

This contribution, based on the agreements achieved and the discussions without conclusion happened in the last meeting, continues to discuss the separate HARQ-ACK codebooks, FDM/TDM scheduling for Type-1 HARQ codebook, priority indication for the “fallback DCI”, some issues regarding NACK-only based including NACK-only for multicast SPS, and the follow-up issues regarding enabling/disabling HARQ-ACK feedback. The discussion in this paper leads to the following proposals:

Proposal 1: UE can be configured to generate separate Type-1 HARQ-ACK codebooks for unicast and multicast with different priorities, respectively.

Proposal 2: UE can be configured to generate separate HARQ-ACK codebooks (including both Type-1 and Type-2 codebooks cases) for unicast and multicast with the same priority.

Proposal 3: When UE is configured with a separate PUCCH-Config for multicast from that for unicast and slot-based PUCCH and subslot-based PUCCH are configured wherein, respectively, two separate codebooks are generated independently and one of them is transmitted when the PUCCH resources are overlapped including the cases of same or different priorities.

Proposal 4: The separate HARQ-ACK codebooks for unicast and multicast should apply to separate PUCCH transmissions only.

Proposal 5: UE generates the Type-1 codebook via “FDM-ed” manner only when UE is configured to do so. Otherwise, UE will not expect unicast and multicast to be scheduled in FDM.

Proposal 6: The priority index, if not included in the DCI formats, takes the default value that can be configured when G-RNTI is configured.

Proposal 7: A separate PUCCH-Config or PUCCH-ConfigurationList for multicast can be but is not necessarily configured for NACK-only based HARQ-ACK feedback.

Proposal 8: For a separate PUCCH-ConfigurationList for multicast that is configured for NACK-only based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority and high priority feedback, respectively.

Proposal 9: When more than one NACK-only based feedback are available for transmission in the same PUCCH slot, or the NACK-only feedback are available with other UCI feedback on the same PUCCH resource, or the PUCCH resources are overlapped with PUSCH transmission,  the NACK-only feedback is transformed into the ACK/NACK based feedback.

Proposal 10: Support only one HARQ-ACK bit for NACK-only feedback for PUCCH format 0 or format 1.

Proposal 11: The sequence cyclic shift for NACK-only is .

Proposal 12: For multicast SPS activation/deactivation, NACK-only based HARQ-ACK feedback is not supported.

Proposal 13: When UE is configured with NACK-only for multicast SPS without PDCCH scheduling and when the HARQ-ACK bits for more than one multicast SPS configuration, for multicast SPS and unicast SPS, or for multicast SPS and dynamic scheduling of unicast/multicast, needs to be transmitted in the same PUCCH slot, the NACK-only based feedback for multicast SPS is transformed into the ACK/NACK based feedback.

Proposal 14: Confirm the working assumption regarding the enabling/disabling HARQ-ACK feedback.

- References

R1-2106113, FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs, Moderator (Huawei), RAN1#105-e, May 10-27, 2021.

R1-2106330, “FL summary#6 on improving reliability for MBS for RRC_CONNECTED UEs,” Moderator (Huawei), RAN1#105-e, May 10-27, 2021.