3GPP TSG RAN WG1 Meeting #106-e	R1-2106492

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.3.3

Source:	Huawei, HiSilicon

Title:	Intra-UE multiplexing enhancements

Document for:	Discussion and Decision

Introduction

In the RAN plenary meeting #88e, the scope of Industrial IoT and URLLC was revised in [1]. The description for intra-UE multiplexing and prioritization was captured as follows:

- Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]: - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

In previous meetings, some agreements have been achieved, mainly on potential multiplexing methods for HP SR and LP HARQ-ACK, and some mechanisms to multiplex HP HARQ-ACK and LP HARQ-ACK on PUCCH/PUSCH [2][3][4][5][6]. In this paper, we will discuss the remaining issues for these two aspects and provide our views on the details of the overall procedure for UCI/data multiplexing. Additionally, we will discuss the multiplexing rule for multiple PUCCHs/PUSCHs, the prioritization rule for CG PUSCH and DG PUSCH of different priorities and the simultaneous PUCCH/PUSCH transmission in this paper.

Mechanism to enable/disable intra-UE MUX

In the RAN1#103-e meeting, the following agreements have been achieved for multiplexing HARQ-ACK of different priorities on PUCCH and on PUSCH [3].

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI. Agreements: For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing. - FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0 - FFS: Interaction between the enable/disable mechanism and other multiplexing conditions - FFS for other types of UCI.

In principle, a unified solution is preferred to enable/disable the multiplexing of HP UCI and LP UCI, regardless if they are carried on PUCCH or on PUSCH. This would minimize the standard effort and simplify the implementation and network management complexity. In the following we discuss further details and present a proposal for common handling:

For the case of multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH, it is not preferred to introduce DCI indication. The reason is that such a mechanism could not be applicable for some situations, e.g. for HARQ-ACK feedback for PDSCH(s) that are scheduled by fallback DCI or for SPS HARQ-ACKs. In addition, an extra DCI field would be needed for indicating the multiplexing, which brings unnecessary overhead. Hence, it is better to use RRC signaling to semi-statically enable/disable the MUX.

For the case of multiplexing HARQ-ACK of different priorities on PUSCH, similar to the case of multiplexing HARQ-ACK of different priorities on PUCCH, newly introduced DCI indication is not preferred since it is not applicable in some cases similar to the PUCCH scenario.

Thus, RRC configuration is preferred as a unified method that the gNB can use to enable/disable the multiplexing. In addition, extra conditions on latency and reliability can be defined to judge whether the multiplexing should be allowed or not allowed for certain cases as discussed in section 3.1.4.

Proposal 1: Adopt RRC configuration to enable/disable the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH, and the multiplexing of HARQ-ACK on PUSCH with different priorities.

- Additional conditions shall be specified to further guarantee the latency/reliability of the HP HARQ-ACK for the overlapping case.

UCI multiplexing on PUCCH

In the RAN1 #102-e meeting [2], three use cases are identified as high priority, i.e. HP HARQ-ACK vs LP HARQ-ACK, HP SR vs LP HARQ-ACK, and HP HARQ-ACK/SR vs LP HARQ-ACK. This section provides our views on the details of multiplexing for each case.

3.1 Case 1: HP HARQ-ACK and LP HARQ-ACK

In the RAN1 #105-e meeting [6], the following agreements have been achieved for multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH.

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority.  Rel-15 design (for PF0 and PF1) is baseline.  Note: QC has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. QC accept the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

3.1.1 HP HARQ-ACK vs LP HARQ-ACK with 2 bits total payload

In the RAN1 #105-e meeting, it has been agreed that for HP HARQ-ACK and LP HARQ-ACK with 2 bits total payload, treat the 2 bits as HP HARQ-ACK, and apply the Rel-15 mechanism as baseline. One potential remaining issue for this scenario is the LP DCI missing which may impact the performance of HP HARQ-ACK in case the multiplexing is enabled. It will result in degraded BLER for the HP HARQ-ACK with respect to the mismatched understanding on the HARQ-ACK codebook size between gNB and UE. One way to resolve the issue is to use a dedicated PUCCH resource set for the multiplexed HP/LP transmission, as explained in Section 3.1.3.

3.1.2 HP HARQ-ACK vs LP HARQ-ACK with more than 2 bits total payload

In the RAN1 #104bis-e meeting, it has been agreed to support separate coding in case the total number of LP and HP HARQ-ACK bits is more than 2, with the target of reusing the legacy encoding chain. And the possible coding algorithm was discussed in the RAN1 #105-e meeting. In the following we will discuss the FFS issues for the separate coding method. It should be noted that the same rule can be also applied for HP HARQ-ACK and HP SR vs LP HARQ-ACK with more than 2 bits total payload.

First issue is that the legacy PUCCH format 2 does not support the transmission of CSI part 2 for Rel-15/Rel-16., i.e., there is only one encoding chain for PUCCH format 2. However, regarding the PUCCH format 2 is typically adopted for serving latency sensitive traffics, it is useful to relax the restriction by supporting PUCCH format 2 to carry separately coded HP HARQ-ACK and LP HARQ-ACK in Rel-17. From the implementation perspective, the two encoding chains for PUCCH format 3/4 can be applied for format 2 without much challenge. In this condition, the RRC configuration of PUCCH format 2 can also include separate coding rates for HP HARQ-ACK and LP HARQ-ACK, as described in section 3.1.3.

Proposal 2: For PUCCH format 2, support 2 encoding chains for the case of HP HARQ-ACK and LP HARQ-ACK multiplexing.

- Support separate code rates for HP HARQ-ACK and LP HARQ-ACK for PUCCH format 2.

The next and significant issue is the general principle on the reuse of the legacy encoding chains and valuable enhancements for multiplexing HP HARQ-ACK and LP HARQ-ACK. We discuss below from the perspectives of encoding, rate matching and RE mapping.

Encoder

In general, 2 separate encoding chains exist for the multiplexing of the UCIs with the same priority on PUCCH. The concatenated bit sequence of HARQ-ACK/SR/CSI part 1 is encoded through encoding chain #1, and the information bit sequence of CSI part 2, if any, is encoded independently through encoding chain #2. So for the multiplexed HP HARQ-ACK and LP HARQ-ACK, it is straightforward to reuse the encoding chain #1 for HP HARQ-ACK, and to reuse the encoding chain #2 for LP HARQ-ACK.

One more remaining issue is the encoding algorithm for HP HARQ-ACK or LP HARQ-ACK with 1-2 bit(s) payload. 2 candidate encoding options were agreed for further down selection in the last meeting. Option 1 is 1 bit repetition code and 2 bits simplex code which reuses the coding of UCI on PUSCH, while Option 2 is padding to 3 bits and using RM coding which reuses the coding method for < 3 bits CSI part 2 (e.g., for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports) on PUCCH. From the implementation perspective, both of the two above encoding approaches have been implemented for UCI encoders irrespective of the channel (PUSCH/PUCCH) or the content (CSI part 2/HARQ-ACK), and can be reused in PUCCH with the minor change of relaxing the restriction of the applicable channel or the content. We slightly prefer option 1, because from the perspective of reliability performance, repetition or simplex code would bring more encoding gain and better performance than zero padding.

Rate matching

For rate matching, since HP HARQ-ACK uses encoding chain #1 which is originally intended for HARQ-ACK/SR/CSI part 1, it is straightforward that the rule of rate matching for HP HARQ-ACK follows the equation applied to CSI part 1 in 38.212, restricted by a configured maximum PUCCH coding rate as shown below, where L=0 in case of the HP payload no larger than 11.

EHP AN=min⁡(Etot,(OHP AN+L)/RHP ANmax/Qm∙Qm)

For LP HARQ-ACK, the number of coded bits after rate matching is equal to the total number of coded bits on all REs minus the number of coded bits of HP HARQ-ACK:

ELP AN=Etot-min⁡(Etot,(OHP AN+L)/RHP ANmax/Qm∙Qm)

Proposal 3: For the encoders of LP and HP HARQ-ACK bits with more than 2 bits total payload,

- HP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 HARQ-ACK+CSI part 1.

- LP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 CSI part 2.

- For the encoding method of HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support Option 1, i.e., 1 bit repetition code and 2 bits simplex code.

RE mapping

Different from the encoding/rate matching procedure where the changes are challenging and complex, the RE mapping procedure can be flexibly changed with the evolving of the 3GPP releases. Therefore, from our understanding it is feasible to consider flexible and enhanced RE mapping rule in Rel-17 to achieve gains.

Observation 1: It is feasible to consider enhanced RE mapping rule in Rel-17.

As the total bandwidth would be enlarged by the multiplexing of HP HARQ-ACK and LP HARQ-ACK as compared to HP only or LP only, it is beneficial to adopt interleaved frequency mapping between HP HARQ-ACK and LP HARQ-ACK to achieve the diversity gain in frequency domain. Similar as the existing mechanism of UCI mapping rule on PUSCH, the HP HARQ-ACK(s) can be mapped as distributed manner in frequency domain, followed by the LP HARQ-ACK(s) which are mapped on the remaining REs. An example of distributed RE mapping on PUCCH with 2 RBs and one symbol is shown in figure 1 below. Note that the distributed mapping of HP HARQ-ACK in frequency is especially valuable for PUCCH format 2 where the time length is too short to improve the reliability from the time domain.

Figure 1. Distributed RE mapping for HP UCI and LP UCI

Proposal 4: For multiplexing of HP HARQ-ACK and LP HARQ-ACK, the distributed mapping between HARQ-ACK and LP HARQ-ACK could be considered for PUCCH format 2.

Power control

For the case of HP HARQ-ACK and LP HARQ-ACK multiplexed on PUCCH resource from the second PUCCH-config, it is a straightforward way to use the power control parameters for the HP HARQ-ACK. However, this solution may worsen the transmission reliability of LP HARQ-ACK compared to the case where there is only LP HARQ-ACK transmission.

According to the formula in section 7.2.1 of TS38.213, the transmit power includes a power adjustment component ∆TF,b,f,ci. For PUCCH format 2/3/4, if the number of UCI bits is smaller than or equal to 11, the ∆TF,b,f,ci is calculated as in the following formula,

And if the number of UCI bits is larger than 11, the ∆TF,b,f,ci is calculated as in the following formula,

where .

For the two formulas mentioned above, we can observe that ∆TF,b,f,ci is determined by the equivalent UCI code rate, i.e., BPRE, where higher code rate would lead to larger transmission power compensation and vice versa. However, for the case of multiplexed HP HARQ-ACK and LP HARQ-ACK, how to determine the BPRE should be further studied as there would be two separate code rates.

Proposal 5: The BPRE calculation can be further studied for the transmission power of multiplexed HP HARQ-ACK and LP HARQ-ACK.

3.1.3 PUCCH resource determination

In the RAN1 #104-e meeting [4], the following agreements are achieved.

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details

Extra code rate for LP HARQ-ACK

To achieve differentiated reliability, an extra coding rate for LP HARQ-ACK should be adopted for PUCCH format 2/3/4. In the last meeting, the mechanism of configuring an extra code rate was discussed. Considering that two PUCCH-configs have already been configured including both the max code rate of HP HARQ-ACK and that of LP HARQ-ACK on PUCCH, the extra coding rate for LP HARQ-ACK under the multiplexing situation could directly reuse the max code rate in the first PUCCH-config. With the configured coding rate from both the first PUCCH-config and the second PUCCH-config, the UE can apply differentiated coding rate for HP HARQ-ACK and LP HARQ-ACK. Then the UE can select the PUCCH resource set as well as the number of total RBs according to the separate payload sizes and the separate coding rates for HP HARQ-ACK and LP HARQ-ACK.

Proposal 6: The code rate configured in the first PUCCH-config should be reused for LP HARQ-ACK transmission under the case of HP HARQ-ACK and LP HARQ-ACK multiplexing.

PUCCH PRB number determination

For the determination of the PUCCH PRB number for format 2/3, the existing mechanism would refer to original information bits, modulation order and code rate. For example, if CSI part 2 would be mapped on PUCCH, the number of RB should meet the requirement in TS 38.213

and

So similarly, to reuse the existing mechanism as far as possible, if HP HARQ-ACK use encoding chain #1 and LP HARQ-ACK use encoding chain #2, the minimum PRB should be calculated with separate coding rate.

OHP ACKQm∙rHP+OLP ACKQm∙rLP=MRBPUCCH∙Nsc,ctrlRB∙Nsymb,UCIPUCCH

One special case mentioned by companies that, PUCCH may not have sufficient resource to carry multiplexing HP and LP HARQ-ACK, that is OHP ACKQm∙rHP+OLP ACKQm∙rLP>MRBPUCCH∙Nsc,ctrlRB∙Nsymb,UCIPUCCH. One alternative is to drop part of LP HARQ-ACK bits or compress LP HARQ-ACK bits, as mentioned by some companies. Comparing to existing mechanism on CSI dropping based on the priority of each CSI report in Rel-16, LP HARQ-ACK bits or codebook are a complete combination. It could include TB-level HARQ-ACK bits, CBG-level HARQ-ACK bits and reserved bits, which is hard to judge which bit should be dropped. From the perspective of implementation, the method of de-constructing a HARQ-ACK codebook, selecting HARQ-ACK information bits and then re-constructing a new HARQ-ACK codebook would be extremely complicated. We should avoid too much specification effort on over optimization.  To this end, we think the UE should not compress or drop partial information bits of the LP HARQ-ACK in case the configured LP code rate cannot be satisfied. One option is to drop all LP HARQ-ACK; another option is to transmit LP HARQ-ACK with higher code rate than its configured code rate unless it exceeds the maximum code rate (e.g., 948/1024).

Proposal 7: For HP HARQ-ACK and LP HARQ-ACK multiplexing on PUCCH format 2/3, the minimum PRB numbers for HP and LP are separately determined based on their coding rates, respectively.

- In case the remaining rate matching resources cannot guarantee the LP configured code rate, drop all LP HARQ-ACKs or transmit LP HARQ-ACK with higher code rate than configured.

Ambiguity of the LP HARQ-ACK due to LP DCI missing

Another open issue is how to determine the PUCCH resource for the multiplexed of HP HARQ-ACK and LP HARQ-ACK. One candidate is to transmit the multiplexed HP HARQ-ACK and LP HARQ-ACK on to the HP PUCCH resource which is the same resource with HP HARQ-ACK only. However, the ambiguity of the existence of LP HARQ-ACK due to missing LP DCI will harm the reliability of the HP decoding.

For example, for the case that 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK on PUCCH format 0, if the UE misses the LP DCI it transmits only the LP HARQ-ACK with the sequence CS #0 (for ‘0’) or CS #6 (for ‘1’). However the gNB, assuming the UE still transmit 2 bits HP and LP, will make a decision among CS #0 (for ‘00’), #3 (for ‘01’), #6 (for ‘11’), #9 (for ‘10’) and would be easier to mistake CS #0 (HP state ‘0’) as CS #9 (HP state ‘1’), or CS #6 (HP state ‘1’) as CS #3 (HP state ‘0’), compared to 1 bit case. As figure 2 shows below, such mismatch will lead to a larger error probability for HP. Similarly, if the UE transmits only 1 bit BPSK (‘0’ or ‘1’) but the gNB still makes decision based on 2 bits QPSK (‘00’, ‘01’, ‘10’, ‘11’), then it would be easier to mistake ‘00’ as ‘10’ or ‘11’ as ‘01’ compared to the BPSK case, leading to a larger error probability for HP.

Figure 2. UE transmits sequence with CS #0 or CS #6, while gNB receives the sequence with CS #0/#3/#6/#9 pattern, leading to judgement error of HP when the received sequence falls in the red line part

For the case that more than 2 bits HP HARQ-ACK and LP HARQ-ACK on PUCCH format 2/3/4, missing LP DCI may lead to several cases if we assume the rate matching is performed between LP HARQ-ACK and HP HARQ-ACK.

Case 1: The UE misses all the LP DCIs, i.e., ambiguity between HP only and HP+LP. Generally, different rate matching outcome, i.e., a different number of coded bits E, would result into different polar coding parameters. For this case, the rate matching formulas for HP HARQ-ACK under the HP only case and the HP + LP case are different, as shown in the following figures. Since LP DCI is missed, the UE wrongly assumes that only HP HARQ-ACKs should be transmitted, which would occupy all REs with respect to the HP only rate matching formula. But the gNB may still assume that the UE transmits HP and LP, wherein HP HARQ-ACK occupies only parts of the REs according to the configured maximum coding rate. Thus the decoding for HP HARQ-ACK will be incorrect due to the wrong rate matching parameters used by the gNB.

Figure 3. UE misses all LP DCIs and transmits HP only

Case 2: The UE misses part of the LP DCIs and there is ambiguity on the LP HARQ-ACK payload size. There are two sub-cases as follows:

- Case 2-1: The total RB number calculated for format 2/3 is the same between gNB (with actual LP payload) and UE (with smaller LP payload) for rate matching, i.e. the wrong LP HARQ-ACK payload does not impact the calculated PRB number. Under this case, we think it does not impact the accuracy of decoding the HP HARQ-ACK since the HP HARQ-ACK REs are mapped in higher priority and independent with LP HARQ-ACK REs.

Figure 4. UE misses part of LP HARQ-ACK but no impact on RB number

- Case 2-2: The total RB numbers calculated for format 2/3 are different between gNB (with actual LP payload) and UE (with smaller LP payload, resulting in smaller RB number) for rate matching. Under this case, the gNB can perform the PUCCH DMRS blind detection to identify the bandwidth of the PUCCH transmitted by the UE, then perform the decoding according to the detected bandwidth, so it does not impact the accuracy of decoding the HP HARQ-ACK.

Figure 5. UE misses part of LP HARQ-ACK which impacts the RB number

Based on the above analysis, we think Case 1: ambiguity on the existence of LP HARQ-ACK should be the target case for considering the solutions, while Case 2 can be resolved by gNB implementation, i.e., PUCCH DMRS blind detection.

Observation 2: Ambiguity on the existence of LP HARQ-ACK should be the target case that needs to be resolved by specification.

Based on the ambiguity case analysis, separate PUCCH resources for HP only and hybrid HP+LP would be a simple way to resolve the problem, and applicable also for the multiplexing between semi-static HP UCI and LP UCI where there is no HP DCI. The gNB can configure different PUCCH resources (RB/CS/OCC) for HP only and for hybrid HP+LP, respectively, and simply perform the blind detection of PUCCH DMRS on the hypotheses for an easy identification if the LP DCI has been missed. Following this approach, in addition to the up to 4 PUCCH resource sets currently configured for HP HARQ-ACK only, up to 4 extra PUCCH resource sets can be additionally configured for multiplexing  hybrid HP+LP in the second PUCCH-Config.

Figure 6 below is an illustration of the PUCCH resource configuration in the second PUCCH-Config. The dedicated PUCCH set for hybrid HP+LP can be on different RBs or CSs than the HP only PUCCH resource, partially overlap or totally orthogonal, which can be up to the gNB configuration. This method can achieve flexible PUCCH configuration for hybrid HP+LP and avoid the ambiguity issue that would occur in the case of a missed LP DCI.

Figure 6 Different PUCCH resource sets for HP HARQ-ACK and multiplexed HARQ-ACKs

Proposal 8: Support a group of dedicated PUCCH sets in the second PUCCH-Configuration to carry the multiplexed HP HARQ-ACK and LP HARQ-ACK.

3.1.4 MUX conditions

As analyzed above, by configuring dedicated PUCCH resources for hybrid HP+LP at the second PUCCH-Config, the transmission reliability of HP HARQ-ACK can be guaranteed. Then the remaining key problem is how to guarantee the latency. The latency of the HP HARQ-ACK depends mainly on the ending symbol of the PUCCH resource carrying the multiplexed UCI, or more specifically, the ending symbols of REs carrying HP HARQ-ACK in the PUCCH resource. Therefore, if the multiplexing is only allowed when the ending symbol of the PUCCH resource carrying multiplexed UCI or the ending symbol of REs carrying HP HARQ-ACK is no later than X symbols after the ending symbol of the PUCCH resource carrying HP HARQ-ACK, then the impact on the latency for HP HARQ-ACK can be avoided. For simplicity, it is preferred to define X = 0 and to select the ending symbol of the PUCCH resource for the HP HARQ-ACK as the reference symbol to make it immune to mapping rules.

Proposal 9: For HP HARQ-ACK overlapping with LP HARQ-ACK, the multiplexing is allowed only when the PUCCH carrying the multiplexed UCI ends no later than the PUCCH carrying HP HARQ-ACK.

3.2 Case 2: HP SR and LP HARQ-ACK

In Rel-15/Rel-16, the multiplexing of SR and HARQ-ACK has been discussed in two sub-cases. Similarly, the study of Case 2 can still follow these two sub-cases.

- Case 2-1: HARQ-ACK is of 1~2 bits and carried on PUCCH format 0 or 1

- Case 2-2: HARQ-ACK is of more than 2 bits and carried on PUCCH format 2, 3 or 4

For Case 2-1, the following agreements were achieved in the RAN1 #104-e meeting [4].

Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.3: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - FFS on conditions of multiplexing. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.5: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. - Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

In the above agreements, the option to transmit SR and HARQ-ACK on SR resources is listed for all cases. It is true that transmitting HP SR on SR resources can guarantee the reliability by reusing the power control of HP SR and also guarantee the latency since the same resource is used. However, for SR with PF0, this would lead to a great resource waste since the PUCCH resource for SR is pre-configured by the gNB and the gNB needs to reserve more RBs/CSs which are not useful in most of the time. Such waste is not negligible for the IIoT scenario where massive robots/AGVs would be connected. Similarly, for SR with PF1, this would lead to multiplexing as many as three bits on PF1 resource.

In principle, we prefer to reuse the Rel-15 mechanism as much as possible unless the Rel-15 way will result in the dropping of HP SR.

- For HP SR of PF0/PF1 and LP HARQ-ACK of PF0, SR can be multiplexed on the PUCCH resource of the LP HARQ-ACK through CS changing. The latency increase is at most 1 OFDM symbol and hence can be negligible since the PUCCH resource of PF0 spans at most two symbols. The reliability can be guaranteed by employing power control for HP UCI. So we think Opt.2c should be supported for cases that HP SR of PF0/PF1 and LP HARQ-ACK of PF0.

- For HP SR of PF1 and LP HARQ-ACK of PF1, the resource selection method in Rel-15 can guarantee the latency and reliability of the multiplexed UCI since the multiplexed UCI is transmitted on the PUCCH resource of SR.

- For HP SR of PF0 and LP HARQ-ACK of PF1, the Rel-15 rule would lead to SR dropping and hence is unacceptable in Rel-17. The simplest solution would be to drop the LP HARQ-ACK when the HP SR is positive, i.e., the prioritization rule of HP UCI vs LP UCI in Rel-16 should be applied.

Proposal 10: For multiplexing HP SR and LP HARQ-ACK with PF0/PF1,

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF0, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF1, drop LP HARQ-ACK if HP SR is positive (i.e. option 4/5);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF0, Opt.2c should be supported.  That is SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF1, reuse the resource selection method in Rel-15.

For Case 2-2, the multiplexing rule in Rel-15 is to add log2(K+1) bits to represent which SR configuration is positive into the HARQ-ACK bit sequence where K denotes the number of SR configurations whose PUCCH resources overlap with the PUCCH resource carrying HARQ-ACK. In Rel-17, a straightforward way is to employ separate coding for the HP SR and the LP HARQ-ACK on one PUCCH resource to provide different reliability protection and improved resource utilization. For resource determination, we can reuse the dedicated PUCCH resource for hybrid HP HARQ-ACK and LP HARQ-ACK mentioned in 3.1.3 as a unified solution. With respect to the MUX condition, we can reuse the condition for multiplexing the HP HARQ-ACK and the LP HARQ-ACK explained above. That is, the multiplexing is only allowed when the Rel-15 timeline requirement is satisfied and the latency of HP SR is not enlarged.

Proposal 11: For multiplexing HP SR and LP HARQ-ACK with PUCCH format 2/3/4,

- Adopt separate coding to HP SR and LP HARQ-ACK on one PUCCH resource

- The PUCCH resource is selected from dedicated PUCCH resource sets in the second PUCCH-Config for multiplexing HP HARQ-ACK and LP HARQ-ACK

- The multiplexing is only allowed if the ending symbol of the PUCCH resource carrying multiplexed SR and HARQ-ACK is no later than the ending symbol of the PUCCH resource carrying SR.

3.3 Case 3: HP/LP HARQ-ACK multiplexed with a HP SR and a LP SR

On top of the above case 2, there is a further case where a PUCCH format 0 or 1 of HP HARQ-ACK/LP HARQ-ACK overlaps with both a HP SR and a LP SR. Normally, from the UE side, whenever there is requirement for HP or LP UL-SCH transmission, the UE would trigger the positive SR transmission on the HP or LP SR PUCCH resource. Meanwhile, in NR Rel-15, only one SR can be multiplexed with HARQ-ACK on PUCCH format 0 or 1, even though multiple SRs overlap with the HARQ-ACK on the PUCCH format 0 or 1. In this case, in order to guarantee the reliability and latency requirement of the high priority (i.e. URLLC) uplink service, gNB would probably choose to schedule a HP PUSCH using a conservative MCS based on the positive SR transmitted from the UE, as the gNB does not know the SR priority type is HP or LP at the UE side. This would harm the system efficiency if the positive SR is triggered by the LP UL-SCH. Thus, for the above case, we support the UE dropping the LP SR, and multiplexing the HP SR with the HP or LP HARQ-ACK, and the specific method for the multiplexing can follow the conclusion from the case 2 discussed in the above section.

Proposal 12: When a PUCCH format 0 or 1 carrying HP/LP HARQ-ACK overlaps with both a HP SR and a LP SR, drop the LP SR, and multiplex the HP SR and the HP/LP HARQ-ACK.

In case of a PUCCH format 2 or 3 or 4 overlapping with both a HP SR and a LP SR, two options can be considered. One is reusing the unified solution as the above case for HARQ-ACK PUCCH format 0 or 1, where the LP SR is dropped, and the HP SR is jointly encoded with the HP HARQ-ACK. The other option is using the agreed separate coding scheme for the HARQ-ACK and SR with the respective priorities.

UCI multiplexing on PUSCH

In the RAN1 #102-e meeting [2], three use cases are identified as high priority, i.e. LP HARQ-ACK vs HP PUSCH (conveying data only), HP HARQ-ACK vs LP PUSCH (conveying data only), and LP HARQ-ACK and HP HARQ-ACK vs HP/LP PUSCH (conveying data and A-CSI).

4.1 Case 4: LP HARQ-ACK on HP PUSCH

In the RAN1 #104-e meeting [4], it was agreed to support 0< beta-offset <1 to enable better protection of data reliability.

From RAN1#104-e:

Agreements: For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16)

Since the bit-field for the beta-offset already exists in the DCI, it is useful to also support beta-offset = 0 to implicitly disable multiplexing of UCI on HP PUSCH. However, enabling/disabling multiplexing based on RRC signaling should be supported also considering uniform solution for all scenarios, since using beta-offset=0 for disabling is not applied in some cases, e.g. configured grant PUSCH.

Proposal 13: For multiplexing LP HARQ-ACK on HP PUSCH scheduled dynamically by UL grant, support beta-offset = 0 to disable the multiplexing.

4.2 Case 5: HP HARQ-ACK on LP PUSCH

For multiplexing HP HARQ-ACK on LP PUSCH, the reliability can be guaranteed by configuring/indicating a large beta-offset, but the latency may be greatly enlarged, especially when frequency hopping is configured for the LP PUSCH. A simple solution would be to introduce extra conditions and to allow multiplexing only for the case when the latency would not be extended. For that, the method in proposal 6 could be re-used, i.e., allowing the multiplexing only if the ending symbol of the LP PUSCH (or ending of the symbols used for carrying HP HARQ-ACK in LP PUSCH) is no later than the ending symbol of the PUCCH carrying HP HARQ-ACK.

Proposal 14: For HP HARQ-ACK overlapping with LP PUSCH, the multiplexing is only allowed when the ending symbol of the LP PUSCH is no later than the ending symbol of the PUCCH carrying HP HARQ-ACK.

4.3 Case 6: LP HARQ-ACK/CSI & HP HARQ-ACK/CSI on one PUSCH

The following agreements [5] have been achieved in the RAN1 #104bis-e meeting.

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

Since separate beta-offsets can be configured for different multiplexing combinations, e.g. HP UCI vs HP PUSCH, HP UCI vs LP PUSCH, LP UCI vs HP PUSCH, LP UCI vs LP PUSCH, it is straightforward to still use two different beta-offsets for HP HARQ-ACK/CSI and LP HARQ-ACK/CSI when being multiplexed on one PUSCH.

Proposal 15: For multiplexing HP HARQ-ACK/CSI and LP HARQ-ACK/CSI on one PUSCH, support different beta-offsets for these two UCIs.

For the configuration/indication of different beta-offsets, for fallback DCI or when the DCI format 0_1/0_2 does not include the beta-offset bit-field, the beta-offset values for HP HARQ-ACK and LP HARQ-ACK can be configured by RRC. For DCI format 0_1/0_2 with existing beta-offset bit-field, the bit-field is used to dynamically indicate one of beta-offset values configured by RRC. Specifically, one codepoint value n of the beta-offset field is linked to a triplet {βoffsetHARQ-ACK(n), βoffsetCSI-1(n), βoffsetCSI-2(n)}. For Rel-17, if both HP HARQ-ACK and LP HARQ-ACK would be multiplexed on one PUSCH, we can split the column for HARQ-ACK into two columns for HP HARQ-ACK and LP HARQ-ACK separately. That is, one codepoint value is linked to a quadruple {βoffsetHARQ-ACK,βoffsetCSI-1,βoffsetCSI-2, βoffsetHARQ-ACK,1} as shown in the following table.

Table 1 Rel-17 beta-offset indication methods

| Beta-offset bit-field | HARQ-ACK of the same priority | CSI part 1 (of same priority) | CSI part 2 (of same priority) | HARQ-ACK of the different priority |
| --- | --- | --- | --- | --- |
| 0 (1-bit beta-offset indicator) 00 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(0) | βoffsetCSI-1(0) | βoffsetCSI-2(0) | βoffsetHARQ-ACK,1(0) |
| 1 (1-bit beta-offset indicator) 01 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(1) | βoffsetCSI-1(1) | βoffsetCSI-2(1) | βoffsetHARQ-ACK,1(1) |
| 10 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(2) | βoffsetCSI-1(2) | βoffsetCSI-2(2) | βoffsetHARQ-ACK,1(2) |
| 11 (2-bit beta-offset indicator) | βoffsetHARQ-ACK(3) | βoffsetCSI-1(3) | βoffsetCSI-2(3) | βoffsetHARQ-ACK,1(3) |

Proposal 16: For DCI format 0_1/0_2 with existing beta-offset bit-field, one codepoint of the field is linked to a quadruple {βoffsetHARQ-ACK,βoffsetCSI-1,βoffsetCSI-2, βoffsetHARQ-ACK,1} to jointly indicate the beta-offset values for HP HARQ-ACK, LP HARQ-ACK, CSI part 1 and CSI part 2.

Another issue that needs to be discussed is how to reuse the PUSCH encoding chains for HP UCI and LP UCI multiplexed on PUSCH in Rel-17. In the legacy system, there are in total 3 encoding chains available for UCI on PUSCH: encoding chain #1 is used to encode HARQ-ACK and SR, encoding chain #2 for CSI part 1, and encoding chain #3 for CSI part 2. For Rel-17, the rules for reusing the legacy encoding chains and the dropping rules in case of collision with the LP HARQ-ACK need to be discussed, to avoid exceeding the number of the UCI encoding chains.

HP HARQ-ACK, and LP HARQ-ACK would be transmitted on HP/LP PUSCH

In Rel-17, if the HP HARQ-ACK and LP HARQ-ACK collide on one PUSCH without CSI, there is no need to drop any UCI as the maximum encoder number is not exceeded. The LP HARQ-ACK can reuse the encoding chain #2 or #3 in Rel-15/Rel-16, and it can be FFS whether it should be encoding chain #2 or #3. We recommend to adopt the same rule as for other cases to achieve a unified solution for simplicity.

In the last meeting, whether to study HARQ-ACK ambiguity issue for type 1 HARQ-ACK codebook remains FFS. For type 1 HARQ-ACK codebook transmitted on PUSCH, DAI_UL is used to enable/disable the codebook transmission. However, one fallback situation is that if DAI_UL is 0 and HARQ-ACK information bit is 1bit, this 1 HARQ-ACK bit is still transmitted on PUSCH for efficiency. This fallback situation may be unacceptable for the case of multiplexing LP HARQ-ACK on HP PUSCH, because UE cannot guarantee the right LP HARQ-ACK codebook size if the feedback relies on whether DCI is received or not. The LP HARQ-ACK codebook size ambiguity could impact the reliability of HP PUSCH if rate match is adopted for LP HARQ-ACK mapping. One simple solution is always not to transmit HARQ-ACK if DAI_UL is 0.

Proposal 17: For multiplexing LP HARQ-ACK with type 1 HARQ-ACK codebook on HP PUSCH, LP HARQ-ACK is not transmitted if DAI_UL is 0.

HP HARQ-ACK, LP HARQ-ACK and CSI would be transmitted on LP PUSCH

CSI should be dropped to avoid an increased number of encoders. But as opposed to CSI on PUCCH, the HARQ-ACK, CSI part 1 and CSI part 2 are all separately encoded on PUSCH in Rel-15/Rel-16. Thus, we recommend to only drop CSI part 2, while CSI part 1 can be kept. Two candidate principles can be considered for the reuse of the legacy encoders, where the encoding chain in the following includes the behaviors of encoding, rate matching and RE mapping:

- Candidate 1: HP HARQ-ACK reuses the encoding chain #1 and LP HARQ-ACK reuses the encoding chain #3, i.e., LP HARQ-ACK is swapped in the CSI part 2 encoder. It has minor impact to the specification, where only the CSI part 2 related step in multiplexing is replaced with the content of LP HARQ-ACK, while the CSI part 1 is left unchanged.

- Candidate 2: HP HARQ-ACK reuses the encoding chain #1, LP HARQ-ACK reuses encoding chain #2 and CSI part 1 reuses encoding chain #3. This principle can achieve a better performance for LP HARQ-ACK compared to Candidate 1. For example, there is no risk for the LP HARQ-ACK to be punctured by HP HARQ-ACK. But it results in more spec efforts where the CSI part 1 related step also needs to be revisited.

In addition, to minimize the specification impact for LP HARQ-ACK with 1-2 bit(s) payload, it should be rate matched with UL-SCH and/or CSI part 1 instead of being punctured.

HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on HP PUSCH

It includes two sub-cases:

1) The CSI is semi-static CSI, i.e., the CSI could be categorized as LP CSI. As the priority of CSI is lower than that of the HARQ-ACK with the same priority, it is intuitive to drop the CSI to give the priority to LP HARQ-ACK. Similar to the multiplexing of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH, the CSI part 2 should be dropped, and the reorganization of the encoding chains can refer to Candidate 1 or Candidate 2.

2) The CSI is A-CSI triggered on the HP PUSCH, i.e. the CSI is categorized as HP CSI. There may be two options for this case according to the discussion in the last meeting. The first option is that the HP CSI triggered by HP DCI should be prioritized over the LP HARQ-ACK, thus the LP HARQ-ACK should be dropped in this sub-case. The second option is that HP CSI part 2 should be replaced by LP HARQ-ACK, since CSI part 1 could be sufficient for the measurement of the first codeword, and all the HARQ-ACK information should be ensured. We slightly prefer option 1, because meeting the requirements for URLLC is critical and should always been guaranteed if overlapping with eMBB. If it is difficult to make a decision of the priority/importance between HP CSI part 2 and LP HARQ-ACK, we can also consider a flexible solution to introduce a configured priority order list for these candidate UCIs, and the UE would select the first 3 UCIs according to this configured priority order list and then assign them to the corresponding encoding chains on PUSCH.

Proposal 18: If HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, the LP HARQ-ACK can be multiplexed by reusing the encoding chain for legacy CSI part 1 or CSI part 2.

Proposal 19: If HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on LP PUSCH, or, if HP HARQ-ACK, LP HARQ-ACK, and semi-static CSI would be transmitted on HP PUSCH, the CSI part 2 should be dropped, and following two candidates can be further studied:

- Candidate 1: HP HARQ-ACK reuses the encoding chain for legacy HARQ-ACK, and LP HARQ-ACK reuses the encoding chain for legacy CSI part 2.

- Candidate 2: HP HARQ-ACK reuses the encoding chain for legacy HARQ-ACK, LP HARQ-ACK reuses the encoding chain for legacy CSI part 1, and CSI part 1 reuses the encoding chain for legacy CSI part 2.

Proposal 20: If HP HARQ-ACK, LP HARQ-ACK, and A-CSI would be transmitted on HP PUSCH, the LP HARQ-ACK should be dropped.

Proposal 21: For collision of HP HARQ-ACK and LP HARQ-ACK with PUSCH, if the LP HARQ-ACK is to be multiplexed on PUSCH, it should be rate matched with the UL-SCH and/or CSI regardless of the LP HARQ-ACK payload.

Multiplexing order and rules for multiple PUCCHs/PUSCHs

In the RAN1 #102-e meeting [2], an agreement has been achieved to study the multiplexing rules for multiple UCIs with different priorities.

Agreements: For multiplexing UCIs of different priorities in a PUCCH in R17, - Support of multiplexing between different resources not confined within a sub-slot if conditions are met - FFS: Details - Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met - FFS details

Separate UE capabilities for prioritization and multiplexing

The framework of handling the inter-priority multiplexing of overlapped PUCCHs/PUSCHs has been discussed in the last meeting. Till the end of Rel-17, we will have 3 general mechanisms in NR to handle multiple overlapping PUCCHs/PUSCHs: Rel-15 intra-priority multiplexing, Rel-16 inter-priority prioritization and Rel-17 inter-priority multiplexing. The processing could become extremely challenged from the perspective of implementation if all the 3 mechanism has to be supported for the Rel-17 UE. To reduce the complexity and difficulty on implementation, a set of various UE capabilities on supporting multiplexing/prioritization should be introduced. UE would report to support one of the following capabilities:

- Rel-16 inter-priority prioritization and Rel-15 intra-priority multiplexing

- Rel-17 inter-priority multiplexing and Rel-15 intra-priority multiplexing

- Rel-17 inter-priority multiplexing, Rel-16 inter-priority prioritization and Rel-15 intra-priority multiplexing

Actually, the capability 1) mentioned above represents the legacy Rel-16 UE. And for Rel-17 UE, either capability 2) or 3) could be reported. With UE reporting capability 2), gNB should always guarantee the proper scheduling to satisfy one specific multiplex timeline, which would simplify the gNB and UE implementation complexity. Of course, for high capability UE, both Rel-17 inter-priority multiplexing and Rel-16 inter-priority prioritization could be simultaneously supported.

Proposal 22: Support flexible UE capability configuration on handling inter-priority multiplex/prioritization:

- 1)	Rel-16 inter-priority prioritization and Rel-15 intra-priority multiplexing.

- 2)	Rel-17 inter-priority multiplexing and Rel-15 intra-priority multiplexing.

- 3)	Rel-17 inter-priority multiplexing, Rel-16 inter-priority prioritization and Rel-15 intra-priority multiplexing.

Multiplexing order for multiple PUCCHs/PUSCHs

For handling the scenarios with more than two overlapping PUCCHs of different priorities, i.e. the Rel-17 inter-priority multiplexing order and rule, two candidate options have been proposed, as the figure shown below. The first one is a Rel-15-like method to handle the multiplexing of all overlapped PUCCHs first, including HP PUCCHs and LP PUCCHs. The second option is a Rel-16-like method, that is to handle the multiplexing of overlapped PUCCHs with intra-priority first, and then handle the multiplexing between inter-priorities. We prefer the second option, considering that the method of final step/checking is consistent and compatible with the procedure of simultaneous PUCCH/PUSCH transmission in Section 7 of this contribution.

Figure 7. Candidate options for Rel-17 multiplexing order

Proposal 23: For Rel-17 inter-priority multiplexing order, support the second option, i.e. to handle the multiplexing of overlapped PUCCHs with intra-priority first, and then handle the multiplexing between inter-priority.

Prioritization between CG PUSCHs and DG PUSCHs

In the RAN1 #102-e meeting, the following agreement was achieved [2].

Agreements: Support PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH in R17. - FFS details - Clarify R16 baseline if needed.

In the RAN1 # 103-e meeting, the following agreements are achieved [3].

Agreements: Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17. - FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details. - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered. - FFS the timeline requirements. - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled. - FFS UE capability for this feature. Note: The main bullet has been agreed in the WID by RAN Plenary

For collision between HP CG vs LP DG, the LP DG should be cancelled at least from the first overlapping symbol in case that the CG PUSCH begins later than the DG PUSCH (if CG would begin earlier, then the UE can directly stop the DG PUSCH). With respect to the cancellation timeline, we think it is up to UE implementation to guarantee that there is enough time in the PHY layer to cancel a LP DG and to prepare the transmission HP CG.

Proposal 24: For collision between HP CG PUSCH and LP DG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the overlapping DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH.

For collision between LP CG PUSCH and HP DG PUSCH, the Rel-16 timeline for HP DG PUSCH and LP PUCCH for SPS HARQ-ACK can be reused, i.e. the UE is expected to cancel the LP CG PUSCH at least from the first symbol of the HP DG PUSCH, and the cancellation should start no later than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH. Meanwhile, an extra processing time of d2 would be introduced for the preparation of the DG PUSCH due to the cancellation process.

Proposal 25: For collision between HP DG PUSCH and LP CG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the DG PUSCH and cancel the CG PUSCH by the first overlapping symbol at the latest.

- The UE expects to transmit the DG PUSCH no later than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH.

Simultaneous PUCCH /PUSCH transmission on different cells

For simultaneous PUCCH/PUSCH transmission on different cells, the following agreements were achieved in the previous meeting.

Agreements (RAN1#102-e meeting[2]): Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA. Agreements (RAN1#104-e meeting[4]): Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

The focus of the feature for simultaneous PUCCH/PUSCH transmission on different cells is limited to the inter-band CA case and also the PUCCH and PUSCH must have different priorities. Note, that we do not see the motivation to support simultaneous PUCCH/PUSCH transmission on different cells for the same priority. Based on this, we discuss how this feature will work with the Rel-16 intra-UE prioritization rule.

In Rel-16, the processing of intra-UE prioritization is as follows:

- Step 1: Perform the multiplexing of LP PUCCH(s)/PUSCH assuming no overlapping HP UL channels and determine the final LP PUCCH/PUSCH;

- Step 2: Judge whether the final LP PUCCH/PUSCH is overlapping with any HP UL channel before and/or after multiplexing of HP UL channels, and if an overlapping happens, the LP PUCCH/PUSCH is dropped

With the support of simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells, the processing can be slightly changed: the UE performs Step 1 as in Rel-16 and determines the final LP PUCCH/PUSCH; and in Step 2, the UE would still check whether the final LP PUCCH/PUSCH overlaps with any HP UL channel, but only when the overlapping happens and the HP UL channel is in a CC within the same band as the final LP PUCCH/PUSCH, the LP PUCCH/PUSCH will be dropped. That is, if a HP UL channel overlaps with the final LP PUCCH/PUSCH but lies in a CC in a different band, the LP PUCCH/PUSCH can remain and is transmitted simultaneously with the HP UL channel.

Observation 3: If simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells is configured, the prioritization of LP PUCCH/PUSCH can be performed as follows.

- Step 1: Perform the multiplexing of LP PUCCH(s)/PUSCH assuming no overlapping HP UL channels and determine the final LP PUCCH/PUSCH;

- Step 2: Judge whether the final LP PUCCH/PUSCH is overlapping with any HP UL channels before and/or after multiplexing of HP UL channels, and if an overlapping happens on the same serving cell or cells within the same band, the LP PUCCH/PUSCH is dropped.

Conclusions

In this paper, we discussed the possible multiplexing rules for PUCCHs/PUSCHs with different priorities. The following observations and proposals are given:

Proposal 1: Adopt RRC configuration to enable/disable the multiplexing of HP HARQ-ACK and LP HARQ-ACK on PUCCH, and the multiplexing of HARQ-ACK on PUSCH with different priorities.

- Additional conditions shall be specified to further guarantee the latency/reliability of the HP HARQ-ACK for the overlapping case.

Proposal 2: For PUCCH format 2, support 2 encoding chains for the case of HP HARQ-ACK and LP HARQ-ACK multiplexing.

- Support separate code rates for HP HARQ-ACK and LP HARQ-ACK for PUCCH format 2.

Proposal 3: For the encoders of LP and HP HARQ-ACK bits with more than 2 bits total payload,

- HP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 HARQ-ACK+CSI part 1.

- LP HARQ-ACK can reuse the encoder and rate matching equation of Rel-15 CSI part 2.

- For the encoding method of HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support Option 1, i.e., 1 bit repetition code and 2 bits simplex code.

Observation 1: It is feasible to consider enhanced RE mapping rule in Rel-17.

Proposal 4: For multiplexing of HP HARQ-ACK and LP HARQ-ACK, the distributed mapping between HARQ-ACK and LP HARQ-ACK could be considered for PUCCH format 2.

Proposal 5: The BPRE calculation can be further studied for the transmission power of multiplexed HP HARQ-ACK and LP HARQ-ACK.

Proposal 6: The code rate configured in the first PUCCH-config should be reused for LP HARQ-ACK transmission under the case of HP HARQ-ACK and LP HARQ-ACK multiplexing.

Proposal 7: For HP HARQ-ACK and LP HARQ-ACK multiplexing on PUCCH format 2/3, the minimum PRB numbers for HP and LP are separately determined based on their coding rates, respectively.

- In case the remaining rate matching resources cannot guarantee the LP configured code rate, drop all LP HARQ-ACKs or transmit LP HARQ-ACK with higher code rate than configured.

Observation 2: Ambiguity on the existence of LP HARQ-ACK should be the target case that needs to be resolved by specification.

Proposal 8: Support a group of dedicated PUCCH sets in the second PUCCH-Configuration to carry the multiplexed HP HARQ-ACK and LP HARQ-ACK.

Proposal 9: For HP HARQ-ACK overlapping with LP HARQ-ACK, the multiplexing is allowed only when the PUCCH carrying the multiplexed UCI ends no later than the PUCCH carrying HP HARQ-ACK.

Proposal 10: For multiplexing HP SR and LP HARQ-ACK with PF0/PF1,

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF0, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF0 vs LP HARQ-ACK with PF1, drop LP HARQ-ACK if HP SR is positive (i.e. option 4/5);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF0, Opt.2c should be supported.  That is SR is multiplexed on HARQ-ACK resource in the same way as Rel-15 if SR is positive and transmit only HARQ-ACK on HARQ-ACK resource if SR is negative (i.e. option 2c);

- For the case of HP SR with PF1 vs LP HARQ-ACK with PF1, reuse the resource selection method in Rel-15.

Proposal 11: For multiplexing HP SR and LP HARQ-ACK with PUCCH format 2/3/4,

- Adopt separate coding to HP SR and LP HARQ-ACK on one PUCCH resource

- The PUCCH resource is selected from dedicated PUCCH resource sets in the second PUCCH-Config for multiplexing HP HARQ-ACK and LP HARQ-ACK

- The multiplexing is only allowed if the ending symbol of the PUCCH resource carrying multiplexed SR and HARQ-ACK is no later than the ending symbol of the PUCCH resource carrying SR.

Proposal 12: When a PUCCH format 0 or 1 carrying HP/LP HARQ-ACK overlaps with both a HP SR and a LP SR, drop the LP SR, and multiplex the HP SR and the HP/LP HARQ-ACK.

Proposal 13: For multiplexing LP HARQ-ACK on HP PUSCH scheduled dynamically by UL grant, support beta-offset = 0 to disable the multiplexing.

Proposal 14: For HP HARQ-ACK overlapping with LP PUSCH, the multiplexing is only allowed when the ending symbol of the LP PUSCH is no later than the ending symbol of the PUCCH carrying HP HARQ-ACK.

Proposal 15: For multiplexing HP HARQ-ACK/CSI and LP HARQ-ACK/CSI on one PUSCH, support different beta-offsets for these two UCIs.

Proposal 16: For DCI format 0_1/0_2 with existing beta-offset bit-field, one codepoint of the field is linked to a quadruple {βoffsetHARQ-ACK,βoffsetCSI-1,βoffsetCSI-2, βoffsetHARQ-ACK,1} to jointly indicate the beta-offset values for HP HARQ-ACK, LP HARQ-ACK, CSI part 1 and CSI part 2.

Proposal 17: For multiplexing LP HARQ-ACK with type 1 HARQ-ACK codebook on HP PUSCH, LP HARQ-ACK is not transmitted if DAI_UL is 0.

Proposal 18: If HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, the LP HARQ-ACK can be multiplexed by reusing the encoding chain for legacy CSI part 1 or CSI part 2.

Proposal 19: If HP HARQ-ACK, LP HARQ-ACK, and CSI would be transmitted on LP PUSCH, or, if HP HARQ-ACK, LP HARQ-ACK, and semi-static CSI would be transmitted on HP PUSCH, the CSI part 2 should be dropped, and following two candidates can be further studied:

- Candidate 1: HP HARQ-ACK reuses the encoding chain for legacy HARQ-ACK, and LP HARQ-ACK reuses the encoding chain for legacy CSI part 2.

- Candidate 2: HP HARQ-ACK reuses the encoding chain for legacy HARQ-ACK, LP HARQ-ACK reuses the encoding chain for legacy CSI part 1, and CSI part 1 reuses the encoding chain for legacy CSI part 2.

Proposal 20: If HP HARQ-ACK, LP HARQ-ACK, and A-CSI would be transmitted on HP PUSCH, the LP HARQ-ACK should be dropped.

Proposal 21: For collision of HP HARQ-ACK and LP HARQ-ACK with PUSCH, if the LP HARQ-ACK is to be multiplexed on PUSCH, it should be rate matched with the UL-SCH and/or CSI regardless of the LP HARQ-ACK payload.

Proposal 22: Support flexible UE capability configuration on handling inter-priority multiplex/prioritization:

- 1)	Rel-16 inter-priority prioritization and Rel-15 intra-priority multiplexing.

- 2)	Rel-17 inter-priority multiplexing and Rel-15 intra-priority multiplexing.

- 3)	Rel-17 inter-priority multiplexing, Rel-16 inter-priority prioritization and Rel-15 intra-priority multiplexing.

Proposal 23: For Rel-17 inter-priority multiplexing order, support the second option, i.e. to handle the multiplexing of overlapped PUCCHs with intra-priority first, and then handle the multiplexing between inter-priority.

Proposal 24: For collision between HP CG PUSCH and LP DG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the overlapping DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH.

Proposal 25: For collision between HP DG PUSCH and LP CG PUSCH, PHY layer can make the prioritization so that the UE is expected to transmit the DG PUSCH and cancel the CG PUSCH by the first overlapping symbol at the latest.

- The UE expects to transmit the DG PUSCH no later than Tproc,2+d1 after the last symbol of the PDCCH scheduling the DG PUSCH.

Observation 3: If simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells is configured, the prioritization of LP PUCCH/PUSCH can be performed as follows.

- Step 1: Perform the multiplexing of LP PUCCH(s)/PUSCH assuming no overlapping HP UL channels and determine the final LP PUCCH/PUSCH;

- Step 2: Judge whether the final LP PUCCH/PUSCH is overlapping with any HP UL channels before and/or after multiplexing of HP UL channels, and if an overlapping happens on the same serving cell or cells within the same band, the LP PUCCH/PUSCH is dropped.

- References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #83, June 2020.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 102-e, August 2020.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 103-e, October 2020.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 104-e, January 2021.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 104bis-e, April 2021.

- RAN1 Chairman’s notes, 3GPP TSG RAN WG1 Meeting 105-e, May 2021.