3GPP TSG RAN WG1 #106-e			R1-2106759

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.9.2

Source: 	Qualcomm Incorporated

Title: 	Support of 14 HARQ processes and scheduling delay

Document for:	Discussion and Decision

# Background

A new work item on additional enhancements for NB-IoT and LTE-MTC was approved in [1]. One of the objectives is to increase the eMTC effective throughput by means of the following technique:

- Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL, for HD-FDD Cat M1 UEs. [LTE-MTC] [RAN1]

We would like to point out that this objective was already discussed during Rel-16 TEI [2,3], but was finally postponed to Rel-17.

In RAN1#105-e the following agreements were reached:

Agreement

In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be supported with multiple solutions: Alt-1 for full flexibility and Alt-2e for support of legacy delay:

- Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

  - Without using more than 6 bits

  - FFS: How to minimize the overhead by using joint encoding

- Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

  - The HARQ-ACK delay values and the length of the HARQ-ACK delay set will be based on

    - Alt-2e: “3 bits (same as legacy)”

    - FFS: Whether HARQ delay set is to use range1 or range2

- RRC signaling will be used to configure between Alt-1 and Alt-2e

- FFS: Signaling details

- FFS: Joint encoding

R1-2106029	Feature Lead Summary [105-e-LTE-Rel17_NB_IoT_eMTC-02] checkpoint#2	Moderator (Ericsson)

Working Assumption

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field:

- The field uses no more than 7 bits if Alt-1 is configured.

- The field is 5 bits if Alt-2e is configured.

- FFS: Details of the joint encoding.

- FFS: Legacy DCI fields that might be re-purposed for the jointly encoded solution of Alt-1 and Alt-2e respectively.

Note: Alt-1 expresses the HARQ-ACK delay as: (y) BL/CE DL subframe + 1 subframe + (z) BL/CE UL subframes, where y = {0, 1, 2, … 11} and z = {1, 2, 3}.

Conclusion:

In Rel-17, for the 14 HARQ processes feature:

When the HARQ-ACK delay is configured to use Alt-1 “PUCCH using Repetition = 1 is postponed”, whereas when the HARQ-ACK delay is configured to use Alt-2e “PUCCH using Repetition = 1 is not postponed (legacy behavior)”.

Agreement

In Rel-17, the 14 HARQ processes feature is applicable for HD-FDD Cat M1 UEs in CE Mode A only.

For discussion in future meetings:

Whether 14 HARQ processes feature can be enabled for PDSCH repetition case

- Signaling of HARQ-ACK and PDSCH scheduling delay

In the last meeting, a working assumption on the PDSCH scheduling delay was reached. This working assumption was a compromise between the different alternatives and was the result of long online and offline discussions. We propose to confirm the working assumption before proceeding to discussing the details.

Proposal 1: Confirm the working assumption on joint encoding of PDSCH scheduling delay and HARQ-ACK delay.

In the following we present our views on the details of the joint encoding:

## Details for Alt-2e

The objective of Alt-2e is to offer low overhead in the case of small number of non-BL/CE subframes. In the case of no invalid subframes, as analyzed in [4], we can reuse range1 in TS 36.213. Since we have additional states, we will propose to add a few more values to the HARQ-CK delay.

We propose to jointly encode the PDSCH scheduling delay as

IDCI=IHARQ-Delay+11IPDSCH-Delay

Where IPDSCH-Delay∈{0,1,2} represents the three options for the PDSCH scheduling delay, for example with the following mapping:

- 0: 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

- 1 : 1 subframe + 3 BL/CE UL subframes+ 1 subframe + 2 BL/CE DL subframes.

- 2:  2 BL/CE DL subframes.

IHARQ-Delay∈{0,…,10}, with the combination of IHARQ-Delay=10 and IPDSCH-Delay=2 being disallowed. The mapping from delay index to delays can be done following the table below:

Table 1 Mapping of HARQ-ACK delay index to HARQ-ACK delay value

| IHARQ-Delay | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | 4 | 5 | 6 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 |

The entries highlighted in red are the new ones added on top of ‘range1’ in current TS 36.213.

Proposal 2: When Alt-2e is configured:

- The DCI includes the following index: IDCI=IHARQ-Delay+11IPDSCH-Delay

- IPDSCH-Delay∈{0,1,2} indicates the PDSCH scheduling delay among the following:

  - IPDSCH-Delay= 0: 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

  - IPDSCH-Delay= 1 : 1 subframe + 3 BL/CE UL subframes+ 1 subframe + 2 BL/CE DL subframes.

  - IPDSCH-Delay= 2:  2 BL/CE DL subframes.

- IHARQ-Delay∈{0,…,10}, with the mapping from indices to HARQ-ACK delay values as in Table 1.

- The combination of IHARQ-Delay=10 and IPDSCH-Delay=2 is disallowed

## Details for Alt-1

The encoding for Alt-1 is more straightforward. We need to encode 3 values for z, 12 values for y, and 3 values for the PDSCH scheduling delay.

Similar to Alt-2e, we propose to encode the index sent in DCI as

IDCI=y+12z-1+36IPDSCH-Delay

With y∈{0,…,11}, z={1,2,3} and IPDSCH-Delay∈{0,1,2}, where the mapping between IPDSCH-Delay and the actual delay is as in proposal 2.

Note that this leads to a total of 108 combinations, so we could still add 20 more entries within the 7 bits.

Proposal 3: When Alt-1 is configured:

- The DCI includes the following index: IDCI=y+12z-1+36IPDSCH-Delay

- IPDSCH-Delay∈{0,1,2} indicates the PDSCH scheduling delay among the following:

  - IPDSCH-Delay= 0: 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

  - IPDSCH-Delay= 1 : 1 subframe + 3 BL/CE UL subframes+ 1 subframe + 2 BL/CE DL subframes.

  - IPDSCH-Delay= 2:  2 BL/CE DL subframes.

- y∈{0,…,11}, z={1,2,3}

- Support of PDSCH repetition

The WID objective under discussion in this agenda item was motivated by increasing the data rate for the case of PDSCH without repetitions. In some cases, however, it may be necessary to enable PDSCH repetition (e.g. in the case of rapid deterioration of the radio conditions). Thus, the UE could be configured simultaneously with PDSCH repetition and 14 HARQ processes, although the operation of 14 HARQ processes simultaneously with PDSCH repetition may not be optimized.

Proposal 4: Do not introduce optimizations for the support of PDSCH repetition.

- Other changes

In this section we describe other second-order changes needed to enable higher throughput with 14 HARQ processes:

- When 14 HARQ processes are enabled, the “HARQ process number” field in DCI format 6-1A is 4 bits.

- The PDSCH scheduling delay applies only to PDSCH scheduled from USS. The new fields in DCI do not apply to CSS.

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 in Sect. 7.3.1 of TS 36.213 when the UE is configured with 14 HARQ processes.

- A UE configured with 14 HARQ processes is also configured with HARQ-ACK bundling.

Proposal 5: Introduce the following specification changes for supporting 14 HARQ processes:

- When 14 HARQ processes are enabled, the “HARQ process number” field in DCI format 6-1A is 4 bits.

- The PDSCH scheduling delay applies only to PDSCH scheduled from USS. The new fields in DCI do not apply to CSS

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes.

- A UE configured with 14 HARQ processes is also configured with HARQ-ACK bundling.

- Conclusions

In this contribution we presented our views on the necessary changes to enable increased throughput with 14 HARQ processes and PDSCH scheduling delay. We made the following proposals:

Proposal 1: Confirm the working assumption on joint encoding of PDSCH scheduling delay and HARQ-ACK delay.

Proposal 2: When Alt-2e is configured:

- The DCI includes the following index: IDCI=IHARQ-Delay+11IPDSCH-Delay

- IPDSCH-Delay∈{0,1,2} indicates the PDSCH scheduling delay among the following:

  - IPDSCH-Delay= 0: 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

  - IPDSCH-Delay= 1 : 1 subframe + 3 BL/CE UL subframes+ 1 subframe + 2 BL/CE DL subframes.

  - IPDSCH-Delay= 2:  2 BL/CE DL subframes.

- IHARQ-Delay∈{0,…,10}, with the mapping from indices to HARQ-ACK delay values as in Table 1.

- The combination of IHARQ-Delay=10 and IPDSCH-Delay=2 is disallowed

Proposal 3: When Alt-1 is configured:

- The DCI includes the following index: IDCI=y+12z-1+36IPDSCH-Delay

- IPDSCH-Delay∈{0,1,2} indicates the PDSCH scheduling delay among the following:

  - IPDSCH-Delay= 0: 1 BL/CE DL subframe + 1 subframe + 3 BL/CE UL subframes + 1 subframe + 1 BL/CE DL subframe.

  - IPDSCH-Delay= 1 : 1 subframe + 3 BL/CE UL subframes+ 1 subframe + 2 BL/CE DL subframes.

  - IPDSCH-Delay= 2:  2 BL/CE DL subframes.

- y∈{0,…,11}, z={1,2,3}

Proposal 4: Do not introduce optimizations for the support of PDSCH repetition.

Proposal 5: Introduce the following specification changes for supporting 14 HARQ processes:

- When 14 HARQ processes are enabled, the “HARQ process number” field in DCI format 6-1A is 4 bits.

- The PDSCH scheduling delay applies only to PDSCH scheduled from USS. The new fields in DCI do not apply to CSS

- The maximum number of received PDSCH receptions pending HARQ-ACK is set to W = 12 (in Sect. 7.3.1 of TS 36.213) when the UE is configured with 14 HARQ processes.

- A UE configured with 14 HARQ processes is also configured with HARQ-ACK bundling.

# References

[1] RP-201306, WID revision: Additional enhancements for NB-IoT and LTE-MTC

[2] RP-192390, LS on support of high peak data rate for Cat-M1 UEs in HD-FDD

[3] R1-1912694, Increased peak data rate for HD-FDD MTC UEs

[4] R1-2104821, Support of 14 HARQ processes and scheduling delay