3GPP TSG RAN WG1 #106-e			R1-2107311

e-Meeting, August 16th – 27th, 2021

Agenda item:	7.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on SRS carrier switching

Document for:	Discussion and Decision

# Background

In RAN1#104b-e, there was a discussion on prioritization rules for SRS carrier switching under [104b-e-NR-7.1CRs-02]. During this discussion, there seemed to be a misalignment between companies on how the prioritization is performed for SRS carrier switching. More specifically, the following two interpretations were mentioned:

- Alt 1: The prioritization rules apply only between the source CC and the target CC.

- Alt 2: The source CC is always dropped. The prioritization rules are between the target CC and other CCs, depending on UE capability.

In RAN1#105-e the discussion continued, and the following agreements were reached:

Agreement

The prioritization rules of SRS carrier switching apply to at least the source CC.

- FFS : Whether the specification needs to be updated or not

Agreement

Down select one from the following two options in RAN1#106-e meeting to determine which UL CCs other than the source CC should be used for SRS carrier switching priority rules:

- Option 1: The UL CCs  in the same band as the source CC

- Option 2: The UL CCs can be any carriers which result in uplink transmissions beyond the UE ’s indicated uplink carrier aggregation capability.

  - Companies should indicate how “UE’s indicated uplink CA capability” is derived based on current ASN.1 signaling.

In this contribution we present our views on the above issues.

- Current specification

In this section, we highlight some relevant parts of the specification that we will reference in the remaining of the contribution.

The current specification (TS 38.214) defines the relationship between “source CC” and “target CC” and “switching times” as follows:

[T1] – 38.214

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

The specification also defines a set of priority rules. For instance, the following text specified the UE behavior in the case of collision between SRS and high priority UCI:

[T2] – 38.214

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

The RAN4 specifications (38.133) specify the following interruption related to SRS CS (example for FR1):

[T3] – 38.214

When SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell in SCG if UE is not capable of Per-FR gap, or on active serving cell(s) in SCG in FR1 if UE is capable of Per-FR gap, during the switching to the carrier of a serving cell in FR1 not configured for PUCCH/PUSCH transmission,

-     with up to X1 slot as specified in Table 8.2.1.2.12-1.

[…]

Table 8.2.1.2.12-1: Interruption length X1 (slot)

|  | NR Slot length | SRS carrier | Interruption length X1 (slots) |  |
| --- | --- | --- | --- | --- |
|  | (ms) of victim cell | switching time (us)Note 1 | Sub carrier spacing for agressor cell (kHz) |  |
|  |  |  | 15 | 30 |
| 0 | 1 | ≤ 200 | 2 | 2 |
|  |  | 300, 500 | 2 | 2 |
|  |  | 900 | 3 | 3 |
| 1 | 0.5 | ≤ 200 | 3 | 2 |
|  |  | 300, 500 | 3 | 3 |
|  |  | 900 | 4 | 4 |
| 2 | 0.25 | ≤ 200 | 4 | 3 |
|  |  | 300, 500 | 5 | 4 |
|  |  | 900 | 7 | 6 |
| 3 | 0.125 | ≤ 200 | 7 | 5 |
|  |  | 300, 500 | 9 | 7 |
|  |  | 900 | 12 | 10 |
| Note1: NR SRS carrier switching time is UE capability indicated by higher layer parameter SRS-SwitchingTimeNR. |  |  |  |  |

We note that, in the above text T3, the “aggressor cell” is the target cell (i.e., the cell in which SRS transmission with carrier switching happens), and “victim cell” is any other cell in the band combination.

Our reading of the three texts above is as follows:

- T1: Specifies the relationship between source and target CCs, i.e., the source CC is the CC that is interrupted (from RAN1 specification point of view) *if* SRS is transmitted in the target CC.

- T2: Specifies the collision handling between source and target CC, i.e., if there is a conflict between source and target CC, it defines which channel/signal is prioritized and which one shall be dropped.

- T3: Specifies that, in the event of doing SRS carrier switching, an interruption of X1 slots is allowed in any other CC (in the same FR only if the UE supports Per-FR gap).

Note the difference between T2 and T3: T2 forces the UE to drop a channel/signal, while T3 only talks about interruption (i.e., the UE behavior is left undefined during that period).

Thus, our view is that Alt.1 is correct, i.e., the UE performs prioritization between source and target CC, but the UE may have an interruption in any carrier (other than target CC).

Observation 1: Under current rules for SRS carrier switching:

- The UE performs prioritization between source and target CCs. RAN1 specifications define which channel the UE SHALL drop

- For CCs other than source and target, RAN4 specifications define an interruption period during which the UE MAY skip transmission.

- The case of intra-band CA

In RAN1#104-b, a CR (R1-2103759) for adding the intra-band CCs into the prioritization rules was discussed in [104b-e-NR-7.1CRs-02].

Let us assume the same UE capability as in the previous example, but now B1 allows 2CCs. The configuration is as follows:

| CC | Band | Uplink config |
| --- | --- | --- |
| CC1 (source CC) | B1 | Yes (PUSCH/PUCCH) |
| CC1A | B1 | Yes (PUSCH/PUCCH) |
| CC2 (target CC) | B2 | SRS CS from CC1 |
| CC3 | B3 | Yes (PUSCH/PUCCH) |

According to current specifications, the UE behavior would be as follows:

- UE performs prioritization between CC1 and CC2 (T1/T2).

- CC1A and CC3 may be interrupted (T3)

Figure 1 Current UE behavior for intra-band CA with SRS CS

From RAN1 point of view, it should be clear that for the case of intra-band contiguous CA, CC1A is always interrupted if CC1 is switched, since they both are assumed to share the same Tx chain. Thus, we think that the proposed change in x3759 would be beneficial, since it would allow to perform prioritization between {CC1, CC1A} and CC2. With the change, the UE behavior would be as follows:

Figure 2 UE behavior if CCs in the same band are jointly prioritized (as per CR in x3579).

Proposal 1: For Rel-16, define joint prioritization rules for carriers that are in the same band as the source CC, taking as baseline the CR in R1-2103759.

- Handling of capabilities under “Option 2”

In RAN1#105-e, several companies expressed that the capability of simultaneous transmission in bands other than the target CC could be derived based on the signaled uplink CA capability. In the following, we will show that such capability does not exist explicitly in current ASN.1 signaling, and that deriving it implicitly based on other band combination presents several issues.

First, consider the following case as an example:

- B1: DL + UL

- B2: DL only, SRS CS from B1

- B3: DL + UL

Let’s denote this band combination as BC1={B1[D+U], B2[D+switch from B1], B3[D+U]}.

For this particular band combination (entry in bandCombionationList), the UE would signal 3 bands, indicating that B1/2/3 have downlink (i.e., including the IE ca-BandwidthClassDL-NR), B1/3 have uplink (i.e., including the IE ca-BandwidthClassUL-NR) and B2 does not (i.e., not including the IE ca-BandwidthClassUR-NR). An example with the signaling for this band can be found in the appendix.

Given this capability, there is no way to indicate that, while operating in BC1, when the UE switches from B1 to B2 it cannot simultaneously transmit in B3.

Observation 2: There is no explicit capability in current ASN.1 indicating that, while operating in a band combination with SRS carrier switching, the UE cannot perform simultaneous transmission in the target band and a third band (other than the source and target).

One possibility would be to indicate the above capability in an implicit way by signaling a separate band combination. For example, the UE may signal that it supports BC2 = {B1[D], B2[D+U], B3[D+U]}, and from this capability the gNB may infer that the UE supports simultaneous transmission in B2 and B3 while switching. This approach of signaling a dummy band combination, however, has the following issues:

- Issue 1: By signaling the above, the UE now shall support BC2 with ULCA – there is no way to indicate that BC2 is a “dummy” band combination that is just included to signal support of simultaneous transmission during SRS carrier switching in BC1. Thus, the UE must be tested against this band combination in case a future gNB wants to enable it. It is possible that no operator in the deploys this band combination, so testing is not possible.

- Issue 2: Even if we ignore issue 1, the signaling is much more complicated than what has been shown above. In the above example, a band combination is depicted as a combination of bands, but it is more than that: It is a combination of bands with a corresponding combination of features. A typical example is when the UE is baseband limited, and indicates support of different number of layers for different band combinations (or any other features, e.g. support of timelines, CSI processes, etc.). When we say “BC2 is the switched version of BC1”, not only the bands have to be compatible (as above), but also all the capabilities have to be compatible. This makes the definition of “switched band combination” very complicated.

- Issue 3: When the UE reports capabilities, it may not include all the band combinations (this is typically set by handset manufacturers, and there may be further filtering depends on the request by gNB). In this case, even if the UE supports BC2 by hardware, it may not include BC2 in the capability report.

In view of the above, we conclude that the “implicit method” has several issues and should be avoided.

Observation 3: Using an implicit method (based on a separate band combination) to determine support of simultaneous transmission during SRS CS has several issues and should be avoided.

For the issue of handling UEs that may not be capable of simultaneous transmission while switching (e.g. B2 and B3 in BC1), the following solutions are possible:

- Solution 1: No specification change, the UE will drop B3 (as allowed by current RAN4 specifications). This has the drawback that B3 is not taken into account for prioritization.

- Solution 2: Introduce a new UE capability that explicitly indicates whether the UE is capable of simultaneous transmission while switching.

In view of the points above, we make the following proposal.

Proposal 3: RAN1 concludes that current ASN.1 does not include a capability to indicate “beyond UE’s indicated uplink CA capability”

- Based on current specification, UEs not supporting simultaneous transmission in the target band and a third band (other than source and target bands) are allowed to drop transmissions in the third band (per RAN4 requirements)

- FFS: RAN1 to consider discuss the need of introducing a new UE capability for indicating simultaneous transmission while switching.

- Summary

In this contribution we presented our views on some issues related to SRS carrier switching discussed in RAN1#104b and RAN1#105-e. We made the following observations and proposals:

Observation 1: Under current rules for SRS carrier switching:

- The UE performs prioritization between source and target CCs. RAN1 specifications define which channel the UE SHALL drop

- For CCs other than source and target, RAN4 specifications define an interruption period during which the UE MAY skip transmission.

Proposal 1: For Rel-16, define joint prioritization rules for carriers that are in the same band as the source CC, taking as baseline the CR in R1-2103759.

Observation 2: There is no explicit capability in current ASN.1 indicating that, while operating in a band combination with SRS carrier switching, the UE cannot perform simultaneous transmission in the target band and a third band (other than the source and target).

Observation 3: Using an implicit method (based on a separate band combination) to determine support of simultaneous transmission during SRS CS has several issues and shall be avoided.

Proposal 3: RAN1 concludes that current ASN.1 does not include a capability to indicate “beyond UE’s indicated uplink CA capability”

- Based on current specification, UEs not supporting simultaneous transmission in the target band and a third band (other than source and target bands) are allowed to drop transmissions in the third band (per RAN4 requirements)

- FFS: RAN1 to consider discuss the need of introducing a new UE capability for indicating simultaneous transmission while switching.

# Appendix: ASN.1 example for BC1

The following ASN.1 examples (comments in red) include the BandParameters (3 entries, one per band) the UE will include to signal the band combination BC1 described in Section 4.

First entry:

BandParameters ::= CHOICE {

eutra SEQUENCE {

bandEUTRA                 FreqBandIndicatorEUTRA,

* ca-BandwidthClassDL-EUTRA CA-BandwidthClassEUTRA OPTIONAL,

* ca-BandwidthClassUL-EUTRA CA-BandwidthClassEUTRA OPTIONAL

},

nr SEQUENCE {

bandNR                 FreqBandIndicatorNR, è B1

* ca-BandwidthClassDL-NR CA-BandwidthClassNR OPTIONAL, è Present (indicates the UE supports DL in this band)

* ca-BandwidthClassUL-NR CA-BandwidthClassNR OPTIONAL è Present (indicates the UE supports UL in this band)

}

}

Second entry:

BandParameters ::= CHOICE {

eutra SEQUENCE {

bandEUTRA                 FreqBandIndicatorEUTRA,

* ca-BandwidthClassDL-EUTRA CA-BandwidthClassEUTRA OPTIONAL,

* ca-BandwidthClassUL-EUTRA CA-BandwidthClassEUTRA OPTIONAL

},

nr SEQUENCE {

bandNR                 FreqBandIndicatorNR, è B2

* ca-BandwidthClassDL-NR CA-BandwidthClassNR OPTIONAL, è Present (indicates the UE supports DL in this band)

* ca-BandwidthClassUL-NR CA-BandwidthClassNR OPTIONAL è Absent (no UL in this band)

}

}

Third entry:

BandParameters ::= CHOICE {

eutra SEQUENCE {

bandEUTRA                 FreqBandIndicatorEUTRA,

* ca-BandwidthClassDL-EUTRA CA-BandwidthClassEUTRA OPTIONAL,

* ca-BandwidthClassUL-EUTRA CA-BandwidthClassEUTRA OPTIONAL

},

nr SEQUENCE {

bandNR                 FreqBandIndicatorNR, è B3

* ca-BandwidthClassDL-NR CA-BandwidthClassNR OPTIONAL, è Present (indicates the UE supports DL in this band)

* ca-BandwidthClassUL-NR CA-BandwidthClassNR OPTIONAL è Present (indicates the UE supports UL in this band)

}

}