3GPP TSG RAN WG1 #106-e																																																				R1-2107402

e-Meeting, August 16th – 27th, 2021

Source:	CMCC

Title:	Other aspects for NTN

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In RAN1#105-e meeting, agreements were achieved on other aspects for NTN [1]

Agreement: Same beam layout in BWP#0 and BWP#x (Option 1) and hierarchical beam for BWP#0 (Option 2) should be supported by the specifications for NR-NTN. - FFS: Whether any specification changes are needed specifically to support this functionality Agreement: For explicit indication of polarization information for DL by the network, support indication in SIB - FFS: Signaling details for indication in SIB Agreement: - Polarization information for UL may be indicated in SIB by the network - UE assumes a same polarization for UL and DL, when the UL polarization information is absent. - FFS: Signaling details for indication in SIB

In this contribution, we will share our view on beam management, polarization indication and support of ATG for NTN.

NTN Beam management

NR-NTN beam management issues

In RAN1#105-e meeting, there was a discussion on UE BWP/beam switching enhancement triggered by gNB or UE, but no consensus was achieved. Moderator suggested for further discussion on the following alternatives [2].

FL proposal DP1-1 For beam management in NR-NTN, support at least one of the following enhancements - Alt-1: UE BWP/beam switching is triggered by gNB relying on prediction on gNB side. - FFS: whether BWP/beam switching can be done for a group of UE. - Alt-2: UE BWP/beam switching is performed by UE automonously relying on assistance information. - FSS: details on assistance information

Regarding Alt-1, its benefit was challenged because different UEs may have different BWP/beam switching time.

(a)																																					(b)

Figure 1: Illustration of un-synchronous BWP/beam switching time for a group of UEs.

As shown in Figure 1 (a) for earth moving beam scenario, assume at time t, UE1 is just at the middle line of two satellite beams, and tine t is right time to be switched UE1 from satellite beam A to satellite beam B.

Nevertheless, in order to reduce the signalling overhead for beams switching, a group of UEs in the yellow rectangle was trigged by the gNB to synchronously switch from satellite beam A to satellite beam B at time t. It is obvious that time t is not the right time to perform beam switching for all UEs in the group. For example, for UE2, time t is too late for beam switching; while for UE 3, it is too early.

Thus, the group BWP/beam switching mechanism trigged by gNB may save signalling overhead in the cost of a period of ineffective time interval when some UE is not served by the correct beam.

Assume one side length of the yellow rectangle along the beam switching direction as shown in Figure 1 (a) is L, it corresponds to a time interval δt, during which the beam footprint on the ground moves from A to B, as shown in Figure 1 (b). The key question is: For an acceptable time interval δt (e.g., ±0.5 slot), how long is L, which is the key factor of the size of UE group for synchronous switching.

Let us have a simple calculation for LEO 600 km with 90° elevation angle. The relative speed v of satellite with respect to earth is about 7.56 km/s. As shown in Figure 1 (b), for SCS 30 kHz, δt = 0.5 ms, thus,

CD=7.56 km/s *0.5 ms

L=AB=CD*RERE+H=CD*6371 km6371+600 km

where RE is the earth radius, H is the satellite altitude. It can be calculated that L≈3.5 m.

It can be seen that when the acceptable time interval when some UE is not served by the correct beam is considered, the coverage range of UE group to be synchronously trigged by gNB for beam switching may be quite limited. Thus, the motivation of Alt-1 (UE BWP/beam switching is triggered by gNB relying on prediction on gNB side) needs more clarification.

Observation 1: When the acceptable time interval when some UE is not served by the correct beam is considered, the coverage range of UE group to be synchronously trigged by gNB for beam switching may be quite limited.

Proposal 1: For beam management in NR-NTN, the motivation of Alt-1 (UE BWP/beam switching is triggered by gNB relying on prediction on gNB side) needs more clarification.

NR-NTN beam measurement and reporting

In RAN1#105-e meeting, there was a discussion on NR-NTN beam measurement and reporting, but no consensus was achieved. Moderator suggested for further discussion on the following alternatives [2].

FL proposal DP3-1: For the deployment scenario with multiple beam per cell and frequency reuse >1, NR-NTN should support the following cases for beam measurement performed by a UE: Alt-1: support beam measurement on multiple RS associated with different beams within a same active BWP. Alt-2: support beam measurement on multiple RS associated with different beams within across BWPs.

Note that it was agreed that same beam layout in BWP#0 and BWP#x (Option 1) and hierarchical beam for BWP#0 (Option 2) should be supported in the last RAN1 meeting [1].

Agreement: Same beam layout in BWP#0 and BWP#x (Option 1) and hierarchical beam for BWP#0 (Option 2) should be supported by the specifications for NR-NTN. - FFS: Whether any specification changes are needed specifically to support this functionality

For the above both layout options, UE can perform SSB based beam measurement in BWP#0, thus Alt-1 seems workable at least for the basic functionality of beam switching. Hence, Alt-1 is preferred for less spec impact.

Nevertheless, Alt-2 seems beneficial for interference management and throughput enhancement. Thus, Alt-2 can be further studied.

Proposal 2: For the deployment scenario with multiple beam per cell and frequency reuse >1, at least support beam measurement on multiple RS associated with different beams within a same active BWP (Alt-1).

Proposal 3: For the deployment scenario with multiple beam per cell and frequency reuse >1, beam measurement on multiple RS associated with different beams within across BWPs (Alt-2) can be further studied.

Status report of implicit support of ATG

As captured in the NTN WID [3], NTN work item aims to specify the enhancements identified for NR NTN especially LEO and GEO with implicit compatibility to support HAPS and ATG.

The work item aims to specify the enhancements identified for NR NTN (non-terrestrial networks) especially LEO and GEO with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles: - FDD is assumed for core specification work for NR-NTN. - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG - Earth fixed Tracking area is assumed with Earth fixed and moving cells - UEs with GNSS capabilities are assumed.

In our understand, “implicit compatibility to support HAPS and ATG scenarios” in the WID means the enhancements for NTN can also be applicable for HAPS and ATG, although we do not need to discuss the enhancements specifically for HAPS and ATG. Furthermore, at least in principle,

- If there are several potential solutions for NTN, and some of them are more essential / important / applicable for ATG / HAPS, then these solutions should be prioritized.

In order to support HAPS and ATG scenario, the following enhancements should be supported which are essential for ATG and HAPS, and they are also beneficial for satellite application.

- Issue 1: Extend the value range of K1.

- Issue 2: Extend the maximal supported HARQ process number.

- Issue 3: Support satellite ephemeris based on satellite position and velocity state vectors.

Observation 2: In order to support HAPS and ATG scenario, the following enhancements should be supported which are essential for ATG and HAPS, and they are also beneficial for satellite application.

- Issue 1: Extend the value range of K1.

- Issue 2: Extend the maximal supported HARQ process number.

- Issue 3: Support satellite ephemeris based on satellite position and velocity state vectors.

Issue 1 (extend the value range of K1) has been agreed in RAN1#104-e meeting [4].

Agreement: For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

Observation 3: Extend the value range of K1 (Issue 1) has been agreed in RAN1#104-e meeting.

Issue 2 (Extend the maximal supported HARQ process number) has been agreed in RAN1#102-e meeting [5].

Agreement: The extension of maximal HARQ process number can be considered with following assumptions: - The maximal supported HARQ process number is up to 32. - FFS: Support on the maximal HARQ process number is up to UE capability - Minimizing the impacts on specification and scheduling

Observation 4: Extend the maximal supported HARQ process number (Issue 2) has been agreed in RAN1#102-e meeting.

Issue 3 (Support satellite ephemeris based on satellite position and velocity state vectors) has been agreed in RAN1#105-e meeting [1].

Agreement: - Specifications should support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements.

Observation 5: Support satellite ephemeris based on satellite position and velocity state vectors (Issue 3) has been agreed in RAN1#105-e meeting.

Based on the above observation, it can be concluded that at least from RAN1 perspective, all the functionalities which are essential for ATG have been supported, thus, Rel-17 NTN specification can support ATG scenarios.

Observation 6: At least from RAN1 perspective, Rel-17 NTN specification can support ATG scenarios.

Conclusions

In this contribution, we share our share our view on beam management, polarization indication and support of ATG for NTN.. The observations and proposals are summarized as follows:

Observation 1: When the acceptable time interval when some UE is not served by the correct beam is considered, the coverage range of UE group to be synchronously trigged by gNB for beam switching may be quite limited.

Observation 2: In order to support HAPS and ATG scenario, the following enhancements should be supported which are essential for ATG and HAPS, and they are also beneficial for satellite application.

- Issue 1: Extend the value range of K1.

- Issue 2: Extend the maximal supported HARQ process number.

- Issue 3: Support satellite ephemeris based on satellite position and velocity state vectors.

Observation 3: Extend the value range of K1 (Issue 1) has been agreed in RAN1#104-e meeting.

Observation 4: Extend the maximal supported HARQ process number (Issue 2) has been agreed in RAN1#102-e meeting.

Observation 5: Support satellite ephemeris based on satellite position and velocity state vectors (Issue 3) has been agreed in RAN1#105-e meeting.

Observation 6: At least from RAN1 perspective, Rel-17 NTN specification can support ATG scenarios.

Proposal 1: For beam management in NR-NTN, the motivation of Alt-1 (UE BWP/beam switching is triggered by gNB relying on prediction on gNB side) needs more clarification.

Proposal 2: For the deployment scenario with multiple beam per cell and frequency reuse >1, at least support beam measurement on multiple RS associated with different beams within a same active BWP (Alt-1).

Proposal 3: For the deployment scenario with multiple beam per cell and frequency reuse >1, beam measurement on multiple RS associated with different beams within across BWPs (Alt-2) can be further studied.

References

- Chairman's Notes RAN1#105-e, May 10th – 27th, 2021.

- R1-2106336, “Final Summary of 8.4.4 Other Aspects of NR-NTN”, e-Meeting, May 10th – 27th, 2021.

- RP-193234, “Solutions for NR to support non-terrestrial networks (NTN)”, RAN#86, Sitges, Spain, December 9-13, 2019.

- Chairman's Notes RAN1#104-e, January 25th – February 5th, 2021.

- Chairman's Notes RAN1#102-e, 17th - 28th August 2020.