3GPP TSG RAN WG1 #106-e		    R1-2107515

e-Meeting, August 16th – 27th, 2021

Source: 	MediaTek Inc.

Title:	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs

Agenda item:	8.12.2

Document for:	Discussion & Decision

- Introduction

In last previous e-meeting [1], we discussed some high priority issues about NR MBS reliable transmission mechanism and reached some agreements. However, there are still some open and controversial issues that need to be further discussed, e.g., separate HARQ ACK codebook construction for multicast and unicast, how to enable/disable HARQ feedback indicator, etc. In this contribution, we will further discuss the detailed design about the remaining issue to improve reliability for RRC_CONNECTED UEs.

- Discussion

  - HARQ-ACK feedback for multicast

Regarding the separate HARQ-ACK codebook construction for the multicast and unicast services, the following agreements were reached in last meeting:

Agreement: The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook FFS: The case for the same priority. FFS: The case of Type-1 HARQ codebook FFS: Whether this applies to separate PUCCH transmissions only

As agreed above, for the case of different priority, the signalling for URLLC can be reused to configure separate codebooks for Type-2 HARQ codebook. Besides, for the case of same priority , it was agreed that Type-1 HARQ-ACK codebook construction is based on the union of the PDSCH TDRA sets of the unicast service and the multicast service in RAN1#104 e-meeting. Therefore, we suggest Type-1 HARQ codebook construction is only applicable for the case of the same priority for multicast, regardless of whether the same or separate PUCCH resource allocation or not.

Proposal 1: For multicast HARQ-ACK codebook construction, Type-1 HARQ codebook is only applicable for the case of same priority between unicast and multicast.

It has been agreed that a separate PUCCH-ConfigurationList for multicast is optionally configured in previous RAN1 e-meeting. Assuming that separate PUCCH-ConfigurationList_ucast for unicast has been configured which is different from PUCCH-ConfigurationList_mcast configured for multicast:

- If Type-1 codebook is supported for unicast and multicast, UE is expected to generate a joint HARQ codebook with the same priority between unicast and multicast, and the PUCCH resource for the Type-1 HARQ codebook is determined according to across PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

- If Type-2 codebook is supported for unicast and multicast, the separated HARQ-ACK codebook will be constructed independently and be transmitted in respective resource.

Proposal 2: When separate PUCCH-ConfigurationList_mcast is configured for multicast:

- If Type-1 codebook is supported, UE is expected to generate a joint HARQ codebook with the same priority between unicast and multicast, and the PUCCH resource for the Type-1 HARQ codebook is determined according to across PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

- If Type-2 codebook is supported, the separated HARQ-ACK codebook will be constructed independently and be transmitted in respective resource.

  - UCI multiplexing/prioritizing

Regarding the last DCI issue when multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, it was discussed and still controversial without more progress in last meeting. The updated proposal was listed as following.

#### Updated Proposal: For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, continue discussing the following alternatives: - Alt.1: the last DCI for unicast; - Alt.2: the last DCI across unicast and multicast; - This meeting aims to down select one alternative.

From our understanding, Alt.2 is more flexible and has less spec impact than Alt.1. Besides, Alt. 2 is more efficient than Alt.1, especially for the case with less unicast DCI for multiplexing. Besides, it has been agreed that orthogonal PUCCH resources among UEs within the same group is configured for ACK/NACK based HARQ feedback for multicast. In other words, Alt.1 can be viewed as a subset of Alt.2. Therefore, we suggest Alt.2 is used for determining the last DCI.

Proposal 3: When multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the “last DCI” refers to the last DCI across unicast and multicast.

  - Configure/select of HARQ feedback mode

The ACK/NACK and NACK-only based HARQ-ACK feedback mode for multicast were agreed in RAN1#104-e, RAN1#104bis-e meeting respectively.

Agreement in RAN1#104-e meeting: For RRC_CONNECTED UEs receiving multicast, support the following: - ACK/NACK based HARQ-ACK feedback for multicast, - It is up to network to configure orthogonal PUCCH resources among UEs within the same group. - FFS: NACK-only based HARQ-ACK feedback for multicast, - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group. - FFS details. Agreement in RAN1#104bis-e meeting: Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

As agreed above, both ACK/NACK and NACK-only based HARQ feedback mode are supported for Rel-17 NR multicast service. It will incur one critical question as discussed in last RAN1 e-meeting, if both are supported, how to configure or select of ACK/NACK-based and NACK-only based HARQ-ACK feedback? Some companies may think NACK-only based HARQ feedback mode can be used for multiple UEs in one MBS group, which can reduce the PUCCH overhead, and ACK/NACK based HARQ feedback can be used for small UEs in one MBS group. However, there is no criteria to measure how many UEs can be regarded as a “multiple UEs” MBS group and how little UEs can be regarded as a “little UEs” MBS group. Maybe we can set a criteria to measure the UE number, e.g., when UE number more than available PUCCH resource for multicast service, it can regard this MBS group as “multiple UEs” type and use NACK-only based HARQ feedback mode. However, we think it is too restrictive to network scheduling, especially for some UEs newly join or leave the MBS group, gNB need to reschedule the HARQ feedback mechanism. We suggest network can flexibly schedule the HARQ feedback mode based on real transmission condition. The concrete HARQ feedback mode can be indicated dynamically by DCI field, e.g, add a new field “HARQ feedback option” within group common DCI.

Proposal 4: Network can flexibly choose the HARQ-ACK mode and the HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

  - Enable/disable HARQ-ACK feedback

Regarding enable/disable HARQ-ACK feedback mode for multicast, a working assumption was achieved in last meeting as following:

Working assumption: For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH: - RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback. - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling. - FFS details on RRC signaling and group-common DCI indicating. - FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases. - FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction. - FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission. - FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback. - FFS whether/how to apply it to SPS group-common PDSCH.

The working assumption only focus on the ACK/NACK based feedback and the NACK-only based feedback is for further study. As discussed in “Configure/select of HARQ feedback mode” part, HARQ feedback mode can be indicated dynamically by DCI field, which is independent with the enable/disable HARQ-ACK feedback. Therefore, we suggest that the ACK/NACK-based enabling/disabling HARQ-ACK feedback mechanism can be extended to apply to NACK-only based feedback mode.

Proposal 5: The ACK/NACK-based enabling/disabling HARQ-ACK feedback mechanism can be extended to apply to NACK-only based feedback enabling/disabling.

Proposal 6: Confirming the working assumption:

- RRC signalling configures the enabling/disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback is supported for multicast services.

Regarding whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback, the reason is that some UEs within multicast group have low reliability without HARQ-ACK feedback and other UEs within the same multicast group have higher reliability with HARQ-ACK feedback. However, from our understanding, the HARQ feedback mechanism is the same for all UEs within in the same multicast group. Otherwise, the scheduling processing is more complicated and is not preferred. Besides, the service priority of the same group cast services is the same for the UEs within the same group. So, it is better to define the same HARQ-ACK feedback behaviour for all the UEs in the same group.

Proposal 7: The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

As agreed that if RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. In this scenario, whether enabling or disabling the feedback as the default mode is FFS. From our understanding, HARQ ACK is an enhancement for NR MBS, and no HARQ feedback in LTE SC-PTM. Thus, if the RRC signaling doesn’t configure the function, disabling the HARQ feedback can be regarded as a default mode. If the MBS services needs the high reliability quality, the RRC signaling can enable the HARQ ACK feedback mechanism.

Proposal 8: Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

  - Multicast SPS related HARQ-ACK feedback

Regarding the multicast SPS related HARQ-ACK feedback, the following agreements were achieved in last meeting:

Agreement: For support of ACK/NACK based HARQ-ACK feedback for SPS multicast, - the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast. - UE determines a priority index from the HARQ-ACK codebook index - UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations. Agreement: Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling. - FFS for SPS activation/deactivation.

As discussed in our companion contribution in AI 8.12.1 [3], whether and how to address the missed activation/deactivation is critical issue especially when common NACK only feedback mode is used for SPS transmission. E.g., the gNB cannot be able to distinguish between UE receiving the first PDSCH successfully and failing to decode the activation PDCCH. In order to solve the issue, a ACK/NACK based HARQ ACK feedback mode can be used for SPS activation/deactivation and the normal SPS data (e.g., PDSCH without PDCCH scheduling) can support ACK/NACK or common NACK only feedback mode as indicated by corresponding group common DCI indicator field.

Proposal 9: MBS SPS activation/deactivation’s feedback mechanism only support ACK/NACK based HARQ feedback mode.

  - Retransmission combination with HARQ operation

Regarding the retransmission scheme, some agreements were achieved as following in pervious RAN1 e-meeting:

Agreements: For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support retransmission(s) can use PTM transmission scheme 1. Agreements: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

Due to limited meeting time unit, this issue is not discussed in last meeting and defer the discussion in this meeting. According to the agreements, it can be seen that PTM scheme 1 can be used for initial MBS transmission and PTM schem1 or PTP can be configured for MBS retransmission. However, which retransmission scheme will be supported for a specific HARQ-ACK feedback option is still pending in previous meeting.

For ACK/NACK based HARQ feedback mode, each UE within one MBS group is allocated orthogonal PUCCH resource and gNB can differentiate which UE feed back ACK or NACK. Thus, the gNB can flexible choose PTM scheme 1 or PTP for retransmission based on the feedback status. E.g., if many UE report NACK to gNB, the PTM scheme 1 is a recommended way for retransmission. If only little UE feed back NACK to gNB, it can use PTP for retransmission using a narrower beam toward the UE and offer higher robustness.

For NACK-only based HARQ feedback mode, all UEs within one MBS group will use the same PUCCH resource for HARQ feedback, and gNB cannot differentiate which UE feedback NACK. Thus, PTP retransmission targeting for specific UE is not supported and only PTM scheme 1 can be scheduled for retransmission.

Proposal 10: PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

Proposal 11: PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

- Conclusion

In this contribution, it further discusses the NR MBS HARQ reliable issues for RRC_CONNECTED UEs with following proposals:

Proposal 1: For multicast HARQ-ACK codebook construction, Type-1 HARQ codebook is only applicable for the case of same priority between unicast and multicast.

Proposal 2: When separate PUCCH-ConfigurationList_mcast is configured for multicast:

- If Type-1 codebook is supported, UE is expected to generate a joint HARQ codebook with the same priority between unicast and multicast, and the PUCCH resource for the Type-1 HARQ codebook is determined according to across PUCCH-config_ucast of PUCCH-ConfigurationList_ucast and PUCCH-config_mcast of PUCCH-ConfigurationList_mcast.

- If Type-2 codebook is supported, the separated HARQ-ACK codebook will be constructed independently and be transmitted in respective resource.

Proposal 3: When multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the “last DCI” refers to the last DCI across unicast and multicast.

Proposal 4: Network can flexibly choose the HARQ-ACK mode and the HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

Proposal 5: The ACK/NACK-based enabling/disabling HARQ-ACK feedback mechanism can be extended to apply to NACK-only based feedback enabling/disabling.

Proposal 6: Confirming the working assumption:

- RRC signalling configures the enabling/disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback is supported for multicast services.

Proposal 7: The same HARQ-ACK feedback behaviour for all the UEs in the same MBS group are required.

Proposal 8: Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

Proposal 9: MBS SPS activation/deactivation’s feedback mechanism only support ACK/NACK based HARQ feedback mode.

Proposal 10: PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

Proposal 11: PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

- Reference

- RAN1 Chairman’s Notes, RAN1 #105-e, May 2021.

- RAN1 Chairman’s Notes, RAN1 #104-e, Jan 2021.

- R1-2107514, “Discussion on NR MBS group scheduling for RRC_CONNECTED UEs”, MediaTek, RAN1#106-e, August 16th – 27th, 2021.