3GPP TSG RAN WG1 #106	R1-2107526

e-Meeting, 16 – 27 August, 2021

Agenda item:		8.13.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On cross-carrier scheduling from SCell to Pcell

Document for:		Discussion and Decision

# Introduction

In this document further details for the sSCell scheduling the PCell are discussed.

# Discussion

## Monitoring of 0_1/1_1/0_2/1_2 in PCell USS set

RAN1#105 made a core agreement on having two UE types, one that can simultaneously monitor the USS scheduling the PCell in both the sSCell and the PCell, and another one that doesn’t.

- For Type A UE: At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - search space sets on P(S)Cell

      - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE)

      - USS sets for DCI formats 0_0,1_0

      - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

    - search space sets on sSCell

      - USS set(s) for scheduling P(S)Cell

  - FFS: BD/CCE handling

- For Type B UE: Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell

    - search space sets on P(S)Cell

      - USS sets for DCI formats 0_0,1_0

      - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI

    - search space sets on sSCell

      - USS set(s) for scheduling P(S)Cell

  - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2

    - Alt 2-1 is adopted

  - There is no restriction on Type-0/0A/1/2-CSS sets configurations

  - FFS: BD/CCE handling

The remaining questions are related to what does the ”configured so that the UE monitors/does not monitor them in overlapping [slot/symbol”] really entail for the Type A UE.

E.g. would a slot/symbolwise TDM pattern in the search space configurations satisfy the condition? Or would it be possible to configure the UE with overlapping USS sets, but it would in the overlapping case monitor USS in one of the two cells only. This would seem to allow for USS configuration on the sSCell with no overlapping USS monitoring on the P(S)Cell, and still facilitate an efficient fall-back mechanism if the sSCell is e.g. deactivated.

Proposal 1: If a Type A UE is configured with USS sets in P(S)Cell and sSCell with search spaces in overlapping [slots/symbols], the UE monitors the sSCell search space only, if the sSCell is considered able to transmit data

Proposal 2: The conditions for sSCell is considered not to be able to transmit data are at least:

- sSCell is deactivated

- sSCell is dormant

- sSCell is experiencing a radio link failure

The question on whether the overlapping is evaluated on a symbol-by-symbol basis or slot-by-slot basis is another open issue to be resolved. The slotwise evaluation of the overlapping can lead to quite some blockage when e.g. a 30 kHz TDD with 2.5 ms TDD pattern and 15 kHz FDD are considered as depicted in the figure below. If the TDD carrier DL is to be used for USS monitoring, then the FDD carrier is never monitored. Conversely, monitoring in one FDD slot blocks 2 TDD slots from monitoring.

Further, the symbolwise determination would allow TDMing search spaces of the two carriers and still be able to schedule both carriers with overlapping slots.

Proposal 3: The overlapping search space condition is evaluated on a symbol basis, i.e: “overlapping [slots/symbols]”

For Type B UE a fall-back mechanism could also be employed to reduce the UE processing load if the sSCell is considered not to be able to transmit, e.g. a different SS configuration could be applied for the P(S)Cell.

Proposal 4: Type A and Type B UE may be configured with a different set of serach spaces to monitor on the P(S)Cell incase a fall-back mechanism is triggered due to e.g. sSCell being considered not to be able to transmit data.

## BD/CCE limits

RAN1#105 discussed the BD/CCE limit handling and the moderator compiled two alternative branches that RAN1 was then tasked to consider further in RAN1#106:

For at least Type B UE, downselect from one of the BD/CCE limit handling options below - [based on Option A/C] When UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1) - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - UE is not required to monitor more than - Alt1 - α*MPDCCHmax,slot,μ PDCCH BD candidates per P(S)Cell slot - Alt2 - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per slot of sSCell - UE is not required to monitor more than - Alt1 - β*MPDCCHmax,slot,μ PDCCH BD candidates per P(S)Cell slot - Alt2: - β*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - At least case of α+β≤1 is supported. - FFS case of α, β,α+β>1 - FFS multi-TRP case - FFS following - Selection between Alt1 vs. Alt2 above - Whether separate α and β are configured by RRC or if β=1-α and only α is configured - How the PDCCH BD candidates are distributed between multiple sSCell slots overlapping a P(S)Cell slot when μ<μ1 and whether the BD limits for sSCell are specified per sSCell slot or per P(S)Cell slot - [based on Option B] When UE is configured for CCS from sSCell to P(S)Cell and when when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1) - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per slot of P(S)Cell - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per slot of sSCell - Considering both PDCCH BD candidates for P(S)Cell self-scheduling on P(S)Cell and PDCCH BD candidates for sSCell to P(S)SCell cross-carrier scheduling on sSCell - UE is not required to monitor more than - Alt 1 - MPDCCHmax,slot,μ PDCCH BD candidates per P(S)Cell slot - Alt 2 - minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - FFS: selection between Alt-1 and Alt-2 - FFS: whether/how the definition of MPDCCHtotal,slot,μ orMPDCCHtotal,slot,μ1 is modified compared to Rel16 when UE is configured with CCS from sSCell to P(S)Cell

As discussed in [2], and expressed during the RAN1#105, Nokia preference is the Option B based solution as it offers better system flexibility than the Option A/C based solution. The search space configuration for the P(S)Cell and sSCell shall ensure that the existing BD/CCE limits are maintained and full benefit of cross-carrier scheduling from sScell-to-PCell is available to UE and network with existing limits.

Considering the scenario where both PDCCH BD candidates for P(S)Cell self-scheduling on P(S)Cell and PDCCH BD candidates for sSCell to P(S)SCell cross-carrier scheduling on sSCell, we propose to use a fixed factor, p for deriving the M_total. Where p= P for P(s)Cell and p = (1-P) for sScell. The value of P can be agreed during the discussions.

Proposal 5: Adopt the option B based solution for BD/CCE limits. For the M_total, a fixed factor, p = P for P(S)Cell and p = (1-P) for sScell is proposed. The value of P can be agreed during the discussions.

## Location of the PUCCH, PUSCH and SRS

If the uplink is not operating in CA mode, the current specifications would require that the PUxCH and SRS (setting aside the SRS carrier switching) are mapped on the PCell. If the gNB would wish to place these on the SCell carrier, this could be easily achieved by switching that carrier to be the PCell. As typically one would expect the UL coverage to be the bottleneck, having the DL PCell move at the same time would not cause any such issues. Whenever there is a coverage related need to place the PCell on the DSS carrier, but schedule it from the (higher-band) SCell, the uplink would need to be placed on the PCell as well. Considering the SRS carrier switching, the sSCell scheduling the PCell should have no relation to how the SRS carrier switching operates, and no special consideration for that is necessary.

Proposal 6: The requirement to have the PCell configured with the PUSCH/PUCCH/SRS for non-CA case are not modified.

For UL CA case, both of the uplink carriers can be used for PUSCH and SRS, but in case of a single PUCCH group the PUCCH would need to be placed on the PCell. The same coverage argument as made for the non-CA case applies, and there is no obvious reason  to change this for the case of SCell scheduling the PCell.

Proposal 7: The requirement to have the PCell configured with the PUCCH for CA case with single PUCCH group is not modified.

# Conclusion

In this contribution the following observations and proposals are made:

Proposal 1: If a Type A UE is configured with USS sets in P(S)Cell and sSCell with search spaces in overlapping [slots/symbols], the UE monitors the sSCell search space only, if the sSCell is considered able to transmit data

Proposal 2: The conditions for sSCell is considered not to be able to transmit data are at least:

- sSCell is deactivated

- sSCell is dormant

- sSCell is experiencing a radio link failure

Proposal 3: The overlapping search space condition is evaluated on a symbol basis, i.e: “overlapping [slots/symbols]”

Proposal 4: Type A and Type B UE may be configured with a different set of serach spaces to monitor on the P(S)Cell incase a fall-back mechanism is triggered due to e.g. sSCell being considered not to be able to transmit data.

Proposal 5: Adopt the option B based solution for BD/CCE limits. For the M_total, a fixed factor, p = P for P(S)Cell and p = (1-P) for sScell is proposed. The value of P can be agreed during the discussions.

Proposal 6: The requirement to have the PCell configured with the PUSCH/PUCCH/SRS for non-CA case are not modified.

Proposal 7: The requirement to have the PCell configured with the PUCCH for CA case with single PUCCH group is not modified.

# References

- R1-2106356, Summary#5 of Email discussion [105-e-NR-DSS-01], Moderator (Ericsson)

- R1-2104185, On cross-carrier scheduling from SCell to Pcell, Nokia, Nokia Shanghai Bell