3GPP TSG RAN WG1 #106-e		                                    		R1-2107536

e-Meeting, Aug 16 – Aug 27, 2021

Agenda Item:	8.14.3

Source:	InterDigital Inc.

Title:	Performance Evaluation Results for XR

Document for:	Discussion and Decision

# Introduction

Previous RAN1 meetings have agreed on several parameters for the traffic model in terms of data rates and PDB values per application, flagging some parameters as baseline and others as optional [1-4]. In this contribution, system-level evaluations for capacity and power consumption are presented for all baseline scenarios. Additionally, capacity performance for a multiple-stream model is evaluated in the UL

# UL/DL Performance Evaluation for XR/CG

System level simulations are done to evaluate capacity performance for a single DL data stream model and single and multiple UL data stream models. For the UL case, the underlying assumption is that the UEs are always ON (no power saving scheme is applied for either model). For the DL case, different CDRX cycles are applied to evaluate the baseline scenario without power savings against selected power saving schemes.

2.1 Traffic Model for RAN evaluation

Parameters used in the performance evaluation for the statistical model for XR traffic are listed in Table 1 for UL and Table 2 for DL. In the given set of evaluations, a UE is declared as satisfied only when each stream meets the requirement of 99% successful packet delivery within a given air interface PDB.

| UL |  | CG/VR (single stream: pose/control) | AR (single stream: Aggregated video) |
| --- | --- | --- | --- |
|  | Packet Arrival distribution | Periodic: 4ms (no jitter) | Periodic (periodicity: 1/60fps) (no jitter) |
|  | Data rate | 0.2 Mbps | 10 Mbps |
|  | Packet Size distribution | 100 bytes | Truncated Gaussian, same parameters as DL (Table 2) |
|  | Air Interface PDB | 10ms | 30ms |
|  | Capacity KPI [X, PDB] | [99%,10ms] (baseline) | [99%,30ms] (baseline) |
|  | Jitter | - Periodic: 4ms (no jitter) | - Jitter: same model as for DL |

Table 1: UL Traffic models for CG/VR/AR evaluations

|  |  | CG | VR | AR |
| --- | --- | --- | --- | --- |
| DL | Data Rate | 8Mbps, 30Mbps @60fps | 30Mbps, 45Mbps @60fps | 30Mbps, 45Mbps @60fps |
|  | FPS | 60 fps (baseline) 120 fps (optional) Other values, e.g., 30, 90 fps can be also optionally evaluated. |  |  |
|  | Packet Arrival Distribution (single video stream) | Periodic (with periodicity = 1/fps) - Each packet k corresponds to set of IP packets belonging to video frame k - Jitter (with random distribution) is added to arrival slot of each packet k |  |  |
|  | Packet Size Distribution | Truncated Gaussian distribution - Mean: Derived from average data rate and fps as: (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes] - STD: [10.5% of Mean] - Max packet size: [150% of Mean] - Min packet size: [50% of Mean] |  |  |
|  | Air Interface PDB | 15ms | 10ms | 10ms |
|  | Jitter (single video stream) | Arrival time of packet k is k/X x 1000 [ms] + J [ms], where X is the given fps value and J is a random variable (drawn from Truncated Gaussian Distribution) - Mean: [0], STD: [2 ms], Range: [[-4, 4]ms] |  |  |

Table 2: DL Traffic models for CG/AR/VR evaluations

2.2 Simulation assumptions

The simulation assumptions employed to evaluate system level performance for FR1 are in Table 3.

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,2,2,1,1;8,2), (dH, dV) = (0.5λ, 0.8λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | 32-port CSI-RS Type I codebook |  |  |
| Scheduling Algorithm | DL: MU-MIMO with PF scheduling UL: MU-MIMO with PF scheduling |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 3: Assumptions for System-level simulation

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Table 4: Baseline scenarios from RAN 1 #105-e meeting notes [1]:

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table 5: Power model for DL

|  | DRX cycle (ms) | Inactivity timer value (ms) | On duration timer value (ms) |
| --- | --- | --- | --- |
| CDRX1 | 16 | 4 | 12 |
| CDRX2 | 4 | 2 | 2 |

Table 6: Selected CDRX parameters for power consumption evaluations

# Simulation results

  - Uplink results

Uplink system capacity evaluations for the single stream and multiple-stream models are presented in this section.

UL Capacity results for single stream transmission

System capacity provides the maximum number of supported UEs out of which at least Y% of UEs are satisfied where Y=90 (baseline) is employed in the current simulation.

#### Indoor Hotspot

UL system capacity for Indoor hotspot (parameters defined in Table 3) is evaluated for FR1 (4 GHz) using MU-MIMO with PF scheduling. A UE is declared satisfied if more than 99% of packets are successfully transmitted within PDB values of 30ms and 10ms for AR and CG/VR applications, respectively.

Figure 1: FR1 UL AR results for Indoor Hotspot scenario

Figure 2: FR1 UL CG/VR results for Indoor Hotspot scenario

Figures 1 and 2 show UL results for system capacity for AR and CG/VR respectively in an InH deployment where a single stream is simulated. It is observed that a large number of UEs/cell (>20) can be supported when only pose/control information is sent in the UL for the CG/VR case. In the AR case, where aggregated video is sent in the UL, the capacity experiences a sharp drop as the number of UEs exceed ~10 UEs/cell. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to < 12 UEs/cell. Resource utilization (%) also follows a steeper uptake as the number of UEs/cell increases when sending aggregated video in the UL vs only pose/control information.

#### Dense Urban

Figures 3 and 4 show the deployment in a dense urban scenario (with parameters as defined in Table 3) and all other evaluation assumptions kept the same (i.e., PDB values of 30ms and 10ms for CG/VR and AR applications respectively).

Figure 3: FR1 UL AR results for Dense Urban scenario

Figure 4: FR1 UL CG/VR results for Dense Urban scenario

Observation 1: The uplink traffic of AR (aggregated video) with 10Mbps data rates is the bottleneck of the system capacity in FR1, compared to the uplink traffic of CG/VR (pose and control information).

Observation 2: Increasing the Inter-site distance (or decreasing deployment density) as we transition from InH to DU scenario has a significant impact on the capacity as shown by a large drop in #UEs/cell that can be supported.

### UL Capacity results for multiple-stream transmission

As discussed by some companies during RAN1#105-e meeting, several approaches can be used to model multiple streams in the uplink. One approach involves duplicating a physical UE into multiple virtual UEs (twin 1, twin 2, etc.) whereby each virtual UE is associated with a single flow. Another evaluation approach, that we adopted in our simulations, is based on multiplexing whereby multiple data flows are be mapped onto the same PDSCH/PUSCH. In this approach, some form of book-keeping is required to track the transmission and delay budget for each packet in a PDSCH/PUSCH.

#### Indoor Hotspot

UL system capacity for Indoor hotspot is evaluated for two steams in FR1 (4 GHz) using MU-MIMO with PF scheduling, and more specifically a pose/control stream and an aggregated video stream at data rates of 10 Mbps. A UE is declared satisfied if more than 99% of packets are successfully transmitted for each stream with their respective PDB values, i.e., stream 1: [99%, 10ms] AND stream 2: [99%, 30ms].

Figure 5: FR1 UL AR results for 2 streams in Indoor hotspot scenario

Figure 5 shows the UL results for system capacity for multiple-stream AR in an InH deployment. It is observed that a smaller number of UEs/cell can be supported compared to either single-stream scenario in the UL. The trend mirrors the aggregated video performance displaying a drastic fall-off as the number of UEs/cell increases. In this case, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to < 8 UEs/cell (compared to < 12 UEs/cell for single stream aggregated video transmission).

#### Dense Urban

Figure 6: FR1 UL AR results for 2 streams in Dense Urban scenario

Figure 6 shows UL results for system capacity for multiple-stream AR in a Dense Urban deployment. It is observed that a smaller number of UEs/cell can be supported compared to either single-stream scenario in the UL. The trend mirrors the aggregated video performance displaying a drastic fall-off as the number of UEs/cell increases. In this case, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied drops to 0 UE/cell (compared to ~ 2 UEs/cell for single stream aggregated video transmission).

Observation 3: UL capacity of multi-stream model is less than that of single-stream model.

Observation 4: Capacity trends in multi-stream model mirrors that of single-stream aggregated video in the UL (with a lower number of supported UEs/cell), leading to the following conclusions:

- Capacity bottleneck in multi-stream model is largely due to transmission of aggregated video in the UL.

- The impact of sending pose information in multi-stream model is not entirely negligible.

  - Downlink results

Downlink system capacity evaluations for the single stream model with and without power saving schemes are presented in this section.

DL Capacity results for single stream transmission

The CDRX parameters used for power consumption evaluations (i.e., periodicity, inactivity timer and on-duration timer) are provided in Table 6. The no DRX scenario (in blue) represents the case where the UE is always ON:

#### Indoor Hotspot

DL system capacity for Indoor hotspot (parameters defined in Table 3) is evaluated for FR1 (4 GHz) using MU-MIMO with PF scheduling. A UE is declared satisfied if more than 99% of packets are successfully transmitted within chosen PDB values CG and AR/VR applications.

CG @ 8 Mbps

Figure 7: FR1 DL CG results for Indoor Hotspot scenario at data rate of 8 Mbps

As expected, application of a power saving scheme reduces overall system capacity. At low data rates of 8 Mbps, the impact of the power saving schemes on the number of satisfied UEs/cell starts to become apparent after 8 UEs/cell. As a result, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied remains 12 UEs/cell.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 12 / 12 |
| Case 2 (CDRX1) | 7.572 % | 3.787 % | 7.630 % | 10.938 % | 11.33 / 12 |
| Case 3 (CDRX2) | 19.871 % | 15.983 % | 19.744 % | 23.663 % | 10.92/ 12 |

Table 6: Power savings gain for baseline vs. CDRX configs (Indoor – CG – 8 Mbps)

CG @ 30 Mbps

Figure 8: FR1 DL CG results for Indoor Hotspot scenario at data rate of 30 Mbps

As the data rate increases to 30 Mbps, the impact of the power saving schemes on the number of satisfied UEs/cell has an earlier onset (> 2 UEs/cell). As a result, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is ~ 3 UEs/cell.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 5.52 / 6 |
| Case 2 (CDRX1) | 5.277 % | 0.024 % | 5.397 % | 10.44 % | 4.0 / 6 |
| Case 3 (CDRX2) | 16.41 % | 12.184 % | 16.022 % | 22.842 % | 2.53/ 6 |

Table 7: Power savings gain for baseline vs. CDRX configs (Indoor – CG – 30 Mbps)

AR/VR @ 30 Mbps

Figure 9: FR1 DL AR/VR results for Indoor Hotspot scenario at data rate of 30 Mbps

As the PDB is reduced from 15 ms to 10 ms, overall system capacity decreases. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is 2 UEs/cell without any power saving scheme and drops to 0 UE/cell with the implementation of either power saving scheme.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 2.94 / 4 |
| Case 2 (CDRX1) | 6.42 % | 3.08 % | 6.237 % | 10.765 % | 1.384 / 4 |
| Case 3 (CDRX2) | 17.39 % | 13.90 % | 16.56 % | 23.09 % | 1.73 / 4 |

Table 8: Power savings gain for baseline vs. CDRX configs (Indoor – AR/VR – 30 Mbps)

AR/VR @ 45 Mbps

Figure 10: FR1 DL AR/VR results for Indoor Hotspot scenario at data rate of 45 Mbps

As the PDB is reduced from 15 ms to 10 ms and data rate is increased from 30 Mbps to 45 Mbps, overall system capacity further decreases. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is 2 UEs/cell without any power saving scheme and drops to 0 UE/cell with the implementation of either power saving scheme.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 1.88 / 4 |
| Case 2 (CDRX1) | 5.84 % | 2.93 % | 5.39 % | 10.17 % | 1.04 / 4 |
| Case 3 (CDRX2) | 16.3 % | 13.47 % | 15.63 % | 21.85 % | 1.17 / 4 |

Table 9: Power savings gain for baseline vs. CDRX configs (Indoor – AR/VR – 45 Mbps)

#### Dense Urban

CG @ 8 Mbps

Figure 11: FR1 DL CG results for Dense Urban scenario at data rate of 8 Mbps

As expected, application of a power saving scheme reduces overall system capacity. At low data rates of 8 Mbps, the impact of the power saving schemes on the number of satisfied UEs/cell starts to become apparent after 6 UEs/cell. As a result, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied remains 8 UEs/cell.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 4 / 4 |
| Case 2 (CDRX1) | 9.44 % | 6.67 % | 9.461 % | 11.451 % | 4 / 4 |
| Case 3 (CDRX2) | 22.425 % | 20.264 % | 22.409% | 23.221 % | 4 / 4 |

Table 10: Power savings gain for baseline vs. CDRX configs (Dense Urban – CG – 8 Mbps)

CG @ 30 Mbps

Figure 12: FR1 DL CG results for Dense Urban scenario at data rate of 30 Mbps

As the data rate increases to 30 Mbps, the impact of the power saving schemes on the number of satisfied UEs/cell has an earlier onset (> 2 UEs/cell). As a result, the maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is ~ 2 UEs/cell.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 3.9/ 4 |
| Case 2 (CDRX1) | 6.638 % | 0.781 % | 7.013 % | 10.644 % | 3.45 / 4 |
| Case 3 (CDRX2) | 17.649 % | 12.921 % | 17.380 % | 22.705 % | 3.45/ 4 |

Table 11: Power savings gain for baseline vs. CDRX configs (Dense Urban – CG – 30 Mbps)

AR/VR @ 30 Mbps

Figure 13: FR1 DL AR/VR results for Dense Urban scenario at data rate of 30 Mbps

As the PDB is reduced from 15 ms to 10 ms, overall system capacity decreases. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is 2 UEs/cell without any power saving scheme and with CDRX 2, and drastically drops to 0 UE/cell with the implementation of CDRX 1.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 3.26 / 4 |
| Case 2 (CDRX1) | 7.088 % | 2.953 % | 7.045 % | 10.655 % | 1.83 / 4 |
| Case 3 (CDRX2) | 18.052 % | 14.196% | 17.564 % | 22.807 % | 2.16 / 4 |

Table 12: Power savings gain for baseline vs. CDRX configs (Dense Urban – AR/VR – 30 Mbps)

AR/VR @ 45 Mbps

Figure 14: FR1 DL AR/VR results for Dense Urban scenario at data rate of 45 Mbps

As the PDB is reduced from 15 ms to 10 ms and data rate is increased from 30 Mbps to 45 Mbps, overall system capacity decreases. The maximum number of UEs that can be supported such that at least 90% of UEs are satisfied is 0 UEs/cell in all implemented scenarios.

| Power Saving Scheme | Power Saving Gain (PSG) compared to Case 1 |  |  |  | #satisfied UEs per cell2 / #UEs per cell3 |
| --- | --- | --- | --- | --- | --- |
|  | Baseline | Optional |  |  |  |
|  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF1 | PS gain of 50%-tile UE in PSG CDF1 | PS gain of 95%-tile UE in PSG CDF1 |  |
| Case 1 (no DRX) | - | - | - | - | 2.36/ 4 |
| Case 2 (CDRX1) | 6.388 % | 2.690 % | 6.156 % | 10.051 % | 1.31/ 4 |
| Case 3 (CDRX2) | 16.932 % | 13.602 % | 16.396 % | 21.287 % | 1.44/ 4 |

Table 13: Power savings gain for baseline vs. CDRX configs (Dense Urban – AR/VR – 45 Mbps)

Observation 5: Overall capacity performance in DL in all deployment scenarios depends on the chosen PDB and data rates. As requirements tighten, system capacity decreases.

Observation 6: There is tradeoff relation between PSG and ratio of satisfied UEs per cell. As a result, PSG and ratio of satisfied UEs should be considered together when evaluating different power saving schemes

Observation 7: Using any power saving scheme results in reducing the overall system capacity in all deployment scenarios

Observation 8: CDRX scheme 2 (4,2,2) has a lower Resource Utilization (%) and a better PSG performance for all simulated scenarios. This is expected as its on-duration is 50% of the total DRX cycle time (as opposed to 75% for CDRX scheme 1 (16,4,12))

# Conclusion

In this contribution, system level simulation results are provided for baseline scenarios for single stream uplink and downlink, as well as capacity results for a multiple-stream model in the UL. Some power saving schemes are also evaluated for the single-stream downlink scenario. The following observations are made:

Observation 1: The uplink traffic of AR (aggregated video) with 10Mbps data rates is the bottleneck of the system capacity in FR1, compared to the uplink traffic of CG/VR (pose and control information).

Observation 2: Increasing the Inter-site distance (or decreasing deployment density) as we transition from InH to DU scenario has a significant impact on the capacity as shown by a large drop in #UEs/cell that can be supported.

Observation 3: Ratio of satisfied UEs/cell drops as we transition from a single-stream model to a multi-stream model.

Observation 4: Capacity trends in multi-stream model mirrors that of single-stream aggregated video in the UL (with a lower number of supported UEs/cell), leading to the following remarks:

- Capacity bottleneck in multi-stream model is largely due to transmission of aggregated video in the UL.

- The impact of sending pose information in multi-stream model is not entirely negligible.

Observation 5: Overall capacity performance in all deployment scenarios depends on the chosen PDB and data rates. As requirements tighten, system capacity decreases.

Observation 6: There is a tradeoff relation between PSG and ratio of satisfied UEs per cell. As a result, PSG and ratio of satisfied UEs should be considered together when evaluating different power saving schemes.

Observation 7: Using any power saving scheme results in reducing the overall system capacity in all deployment scenarios

Observation 8: CDRX scheme 2 (4,2,2) has a lower Resource Utilization (%) and a better PSG performance for all simulated scenarios. This is expected as its on-duration is 50% of the total DRX cycle time (as opposed to 75% for CDRX scheme 1 (16,4,12))

From the evaluation results, it is clear that the CDRX configuration that achieves the best trade-off between power saving gain and DL capacity loss for CG and AR/VR applications depends on the configuration parameters used (i.e. cycle periodicity, inactivity timer, on-duration timer). Going forward, more candidate options for CDRX configurations can be evaluated both for the single-stream and multiple-stream models to achieve greater alignment between XR traffic and the power saving scheme cycle.

# References

RAN1 Chairman notes, RAN1#105-e, May 2021

RAN1 Chairman notes, RAN1#104-bis-e, Apr 2021

RAN1 Chairman notes, RAN1#104-e, Jan 2021

RAN1 Chairman notes, RAN1#103-e, Nov 2020