3GPP TSG RAN WG1 #106-e			  R1-2107693

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.10.2

Source:	AT&T

Title:	Enhancements for IAB interference management

Document for:	Discussion/Approval

# Introduction

As part of the Rel-17 Work Item on Enhancements for Integrated Access and Backhaul for NR [1], 3GPP has agreed to the following RAN1 led objectives:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

This contribution discusses the potential physical layer enhancements to support simultaneous operation of child and parent links including CLI measurements, timing alignment, and power control.

# Physical Layer Enhancements for IAB

An example of a network with integrated access and backhaul links is shown in Figure 1 below.

Figure 1: Integrated access and backhaul links

In Rel-16, for in-band operation IAB nodes are assumed to operate with a half-duplex constraint and operate in a TDM manner when multiplexing the resources of the IAB-DU and IAB-MT, which means they can only do the following at any given time:

- Receive on the access link (UE to IAB node) and/or backhaul link (IAB node to IAB node)

- Transmit on the access link and/or backhaul link

However, in Rel-17 two new scenarios are considered in RAN1 to support more efficient access and backhaul link multiplexing: 1) spatial division multiplexing (SDM) and 2) multi-panel transmission/reception (MPTR) which are illustrated in Figure 2.

SDM

MPTR

SDM

MPTR

Figure 2: SDM and MPTR Rel-17 IAB Scenarios

The remaining sections of this document focus on solutions for IAB to enable efficient management of interference in the context of Rel-16 and new Rel-17 scenarios.

## IAB Interference Considerations

The frame structure in IAB adapts to the DL and UL traffic, such that dynamic resource allocation on the backhaul and access links is achieved. This dynamic TDD structure introduces DU-DU and MT-MT cross link interference (CLI), which hinders the performance of IAB and requires measurement and mitigation. Depending on the frame structure design for IAB, CLI can be for example mitigated by semi-statically reserving resource blocks for backhaul transmissions, or through graph-coloring based techniques.

As a result, in RAN1#105-e the following agreements were reached:

Agreement

Rel-16 CLI coordination signalling (Intended TDD DL-UL Configuration) is extended to support IAB specific UFD patterns.

- FFS: Support the exchange of IAB-DU H/S/NA resource configuration information among neighbouring IAB-nodes/IAB-donors for CLI management purposes.

In the context of multi-panel Tx/Rx panel operations for IAB, the impact of cross-link interference on access and backhaul links is even more pronounced. In fact, CLI mitigation techniques need to manage inter IAB node interference scenarios where IAB nodes are both interfering, and victim nodes as depicted in Figure 3.

Figure 3: CLI interference case 3 with MPTRx where both IAB nodes are victims and aggressors

MT

DU

MT

DU

MT

DU

MT

DU

Backhaul UL

Backhaul UL

Access UL

Access UL

In another CLI case, MPTRx further affects the access UE, in addition to the IAB MT as shown in Figure 4.

Figure 4: CLI interference case 1 with MPTRx where both IAB and access UE are victims

MT

DU

MT

DU

MT

DU

MT

DU

Backhaul DL

Backhaul UL

Access DL

Access UL

CLI interference mitigation techniques should subsequently take into account the new CLI scenarios arising with MPTRx, ensuring that access UEs are not impacted by the different multiplexing alternatives. Thus we propose that CLI mitigation in Rel. 17 considers whether enhancements are needed to the CLI UE-UE measurement framework in Rel. 16 to support different duplexing scenarios.

Proposal 1: Specify enhancements to the UE-UE Rel. 16 CLI measurement framework to support L1 measurement reports from a child node to a parent node as well as measurement configurations which support transmit and receive beam sweeping for both TDM and non-TDM multiplexing scenarios (i.e. DL and UL RS in the same time/frequency resources).

in addition to MT-based CLI measurements, introducing DU-DU CLI measurements should also be supported. Although in many cases a co-located IAB-DU and IAB-MT may share hardware components including antenna arrays, since the access and backhaul links involve different nodes and potentially different time/frequency resources, it is not possible to always infer the interference characteristics at the DU from those measured by the MT, especially since the parent node is in control of the MT configuration and may not be aware of the available or intended beams/panels to be utilized at the DU. However, since the properties of the measurements would be fundamentally the same between the DU/MT is proposed to utilize the same framework (i.e. CLI/RRM measurements and reports) for the purpose of DU-DU CLI measurements.

Proposal 2: To support DU-to-DU measurement and reports, MT-based CLI measurements and reports are enhanced to support explicit differentiation of time/frequency/spatial resources used by a co-located MT or DU.

## Timing Alignment and Over-the-Air Synchronization

One of the key requirements of NR when operating in TDD band is the synchronization and symbol level alignment of the DL across all the DUs. For IAB, the IAB-DUs do not have any wired backhaul, therefore they must derive their timing synchronization over the air. It is possible for IAB nodes to achieve this over the air-multi hop synchronization using GNSS or by using the MAC CE based T_delta signaling introduced in Rel-16.

Figure 5: IAB Case 1 timing over multiple hops

Shown in the Figure 5 above is a typical Case 1 timing relationship between the backhaul subframes which is between the IAB-MT function of an IAB node and its parent, relative to the access subframe between which is between the DU of an IAB node and its child or a UE. As mentioned before the access subframe DL needs to be symbol level and slot level aligned across all DUs. However the backhaul subframe as seen by the IAB-MT function of the IAB node has the DL and UL shifted due to the propagation delays. However the IAB node IAB node function is aware of this shift since the timing advance is provided to the IAB-MT explicitly, so the timing advance can be used by the IAB node to align its DL access subframes. While Case 1 timing is sufficient for access links and TDM-only IAB nodes, it is not optimized for SDM or MPTR scenarios introduced in Rel-17. In order to support simultaneous operation of the access and backhaul links, additional guard symbols would need to be introduced to avoid cases where the IAB-DU or IAB-MT timing overlaps a symbol or slot boundary. If instead Case 6 or Case 7 timing are used to align the reception and transmission timing of the IAB node respectively, the overhead of the guard symbols could be reduced or avoided altogether. As a result, in RAN1#105-e the following agreements were reached:

Agreement

An IAB-node is indicated when Case 6 timing is performed at the IAB-node.

- FFS details of the indication (e.g. semi-static and/or dynamic, implicit and/or explicit, linkage to multiplexing capability, etc.).

FFS whether an IAB-node is also indicated when Case 7 timing is performed at the IAB-node.

In order to ensure backwards compatibility and avoid impact on access links and TDM-only IAB nodes, Case 1 timing should remain the default mechanism for timing alignment. As result, new timing alignment mechanisms in Rel-17 (e.g. Case 6/7) should only be used in resources which are orthogonal from those used by access or TDM-only backhaul links.

Proposal 3: An IAB-node is explicitly indicated when Case 6 and Case 7 timing is performed at the IAB-node in specific time/frequency resources of the semi-static resource configuration where non TDM multiplexing capability is supported.

## Power Control Considerations

As discussed in previous sections, when SDM/MPTR scenarios are supported, compared to TDM with a half-duplex constraint, multiplexing access and backhaul DL/UL transmissions and receptions makes power control an important consideration. In NR transmit and receive power levels of DL channels and UL channels can be very different. For example when the PHY is in receive mode then it must receive the backhaul DL transmission and the access UL transmission at the same time. These two could be MPTR or SDM. However, in this case the DL backhaul transmission is performed by the IAB-DU of the parent node whereas the UL access transmission is performed by a IAB-MT of a child node or a UE. Therefore the EIRP of DL transmission is typically much higher than the EIRP of the UL transmission. Moreover in NR the UL transmissions are power controlled whereas DL transmissions are not. This implies that the backhaul DL and access UL will arrive at the receiver at very different levels as shown in Figure 6. A similar problem happens during the transmit stage when the backhaul PUSCH and access PDSCH are SDM/MPTR. A similar problem in the different power level between the IAB-MT PHY and the IAB-DU PHY exists.

Figure 6: Power spectral density difference at the receiver for backhaul and access link

The following agreements were made in RAN1#105-e:

Agreement

Decide in RAN1#106-e whether to support an IAB-node indicating assistance information to help with its MT’s UL TX power control. The assistance information can be:

- FFS: Desired TX power

- FFS: Offset to a baseline PHR

- FFS: Desired dynamic range

FFS: whether this information is provided to the parent-node, the CU, or both.

FFS: whether the MT’s UL TX power control formula needs to be changed

The information to assist DL power allocation of the parent-node is indicated by the IAB-MT to the parent node DU in terms of desired power adjustment.

- FFS applicability of assistance information, e.g. per multiplexing scenario, per resource, etc.

If the hardware is shared between the IAB-MT and IAB-DU (e.g. same panel or same RF chain) then such a receiver PSD difference can cause significant problem. The higher PSD of the DL backhaul will likely set the AGC which implies that the UL access signal will fall well below the level set by AGC, which would impact the SINR and therefore the overall throughput of the access transmission.

In order to support SDM/MPTR using the same RF (e.g. intra panel) it is very critical to consider enhancements to the power control mechanism for IAB. One example of an enhancement is the configuration of DL power control at the parent node based information at the child about dynamic range for a given subset of resources at the child IAB-DU.

Proposal 4: DL power allocation assistance information indicated from the child to the parent node should be applicable for indicated subsets of child IAB-DU time/frequency and spatial resources (e.g. beam/panel granularity).

# Conclusion

This contribution analyzed potential physical layer enhancements for IAB to manage interference under different multiplexing scenarios. The following proposals and observations were made:

Observation 1: Multiple factors including antenna array design, beam/panel selection, and IAB node geometry can influence the extent of cross-link and self-interference experienced when non-TDM operation is supported.

Proposal 1: Specify enhancements to the UE-UE Rel. 16 CLI measurement framework to support L1 measurement reports from a child node to a parent node as well as measurement configurations which support transmit and receive beam sweeping for both TDM and non-TDM multiplexing scenarios (i.e. DL and UL RS in the same time/frequency resources).

Proposal 2: To support DU-to-DU measurement and reports, MT-based CLI measurements and reports are enhanced to support explicit differentiation of time/frequency/spatial resources used by a co-located MT or DU.

Proposal 3: An IAB-node is explicitly indicated when Case 6 and Case 7 timing is performed at the IAB-node in specific time/frequency resources of the semi-static resource configuration where non TDM multiplexing capability is supported.

Proposal 4: DL power allocation assistance information indicated from the child to the parent node should be applicable for indicated subsets of child IAB-DU time/frequency and spatial resources (e.g. beam/panel granularity).

# Reference

[1] RP-201293 New WID on Enhancements to Integrated Access and Backhaul, Qualcomm