3GPP TSG-RAN WG1 Meeting #106-e	R1-2107979

# e-Meeting,  August 16th - 27th,  2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on PUCCH Power control when the number of SL HARQ-ACK bits larger than 11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.2.1 of 38.213, it is specified that for a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for the number of UCI bits larger than 11, the PUCCH transmission power adjustment component is determined as , where - - The current specification specifies how to determine transmission power for a PUCCH for sidelink HARQ-ACK reporting when the number of HARQ-ACK bits OACK≤11. If a PUCCH format2/3/4 is used for SL HARQ-ACK reporting and the number of HARQ-ACK bits OACK is larger than 11, it is not clear how to determine the for PUCCH power determination. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The formula for transmission power determination for PUCCH with a number of UCI bits larger than 11 in clause 7.2.1 is reused for determining the power of a PUCCH with more than 11 sidelink HARQ-ACK bits, except that =OACK, and , are set to zero. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear how to determine the transmission power of a PUCCH for sidelink HARQ-ACK reporting when OACK>11 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1.1, 16.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: For gNB not implementing this CR, it shall avoid scheduling or configuring PUCCHs carrying more than 11 sidelink HARQ-ACK bits for sidelink HARQ-ACK reporting. For UEs not implementing this CR, it is not expected to be scheduled or configured with a PUCCH with more than 11 sidelink HARQ-ACK bits for sidelink HARQ-ACK reporting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

====omitted====

If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=m=0M-1Nmreceived where Nmreceived is a number of HARQ-ACK information bits determined for corresponding PSSCH transmissions with corresponding PSFCH reception occasions in PSFCH reception occasion m.

If OACK>11, and if the PUCCH transmission uses PUCCH format 2 or PUCCH format 3 or PUCCH format 4, the UE determines a transmission power for the PUCCH, as described in Clause 7.2.1, except that

-	 =OACK.

-	=0

-	=0

====omitted====

16.5.2.1	Type-2 HARQ-ACK codebook in physical uplink control channel

====omitted====

If OACK≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=VSAI,mlastSL-USAImod4+m=0M-1Nmreceived+NCG

where

-	VSAI,mlastSL is a value of a counter SAI field in a last DCI format 3_0 scheduling PSSCH transmissions associated with PSFCH reception occasions that the UE detects within the M PDCCH monitoring occasions

-	VSAI,mlastSL=0 if the UE does not detect any DCI format 3_0 scheduling PSSCH transmissions associated with PSFCH reception occasions in any of the M PDCCH monitoring occasions

-	USAI is a total number of DCI format 3_0, scheduling PSSCH transmissions associated with PSFCH reception occasions, that the UE detects within the M PDCCH monitoring occasions. USAI=0 if the UE does not detect any DCI format 3_0 scheduling PSSCH transmissions with associated PSFCH reception occasions in any of the M PDCCH monitoring occasions

-	Nmreceived is a number of DCI format 3_0 scheduling PSSCH transmissions with associated PSFCH reception occasions that the UE detects in PDCCH monitoring occasion m

-	NCG is a number of SL configured grants for which the UE transmits corresponding HARQ-ACK information in a same PUCCH as for HARQ-ACK information corresponding to PSFCH reception occasions within the M PDCCH monitoring occasions

If OACK>11, and if the PUCCH transmission uses PUCCH format 2 or PUCCH format 3 or PUCCH format 4, the UE determines a transmission power for the PUCCH, as described in Clause 7.2.1, except that

-	 =OACK.

-	=0

-	=0

====omitted====