3GPP TSG RAN WG1 Meeting #106-e	R1-2108064

e-Meeting, August 16th – 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Views on RAN3 reply LS for L1/L2 mobility

Document for:	Discussion and Decision

# Introduction

RAN1 sent an LS to RAN3 asking questions on L1/L2 mobility [1] and received feedback with questions in [3]. In this paper, we provide our views on RAN3 questions. Note that the scope of L1/L2 mobility was discussed in RAN [5] and the corresponding WID was updated in [6] (copied below), with which the agreements in WGs may need to be revisited.

- Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management for intra-cell and inter-cell scenarios to support higher UE speed and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - For inter-cell beam management, a UE can transmit to or receive from only a single cell (i.e. serving cell does not change when beam selection is done). This includes L1-only measurement/reporting (i.e. no L3 impact) and beam indication associated with cell(s) with any Physical Cell ID(s) - The beam indication is based on Rel-17 unified TCI framework - The same beam measurement/reporting mechanism will be reused for inter-cell mTRP - This work shall only consider intra-DU and intra-frequency cases

# Answer to Questions from RAN3

In [3], RAN3 asked the following questions:

Question 1: What does “non-serving cell” mean? Question 2: What is the procedure of L1/L2-centric mobility? e.g., - Configuration of a non-serving cell - Deconfiguration of non-serving cell - Change of serving cell Question 3: Whether both intra-frequency and inter-frequency scenarios are targeted for this new scheme?

Based on the discussion in RAN1, RAN, and updated WID, we propose the following answers to RAN3.

Answer 1: The term of “non-serving cell” was used to refer to a neighbor cell that is not configured as serving cell to a UE, but can be a candidate for L1/L2-centric inter-cell mobility (changed to inter-cell beam management in RAN#92 [6]) and inter-cell multi-TRP operation. As it created some confusion based on the feedback received [2], such term will not be used in future correspondence.

Answer 2: According to the updated WID, for inter-cell beam management [6], a UE can transmit to or receive from only a single cell (i.e. serving cell does not change). In this sense, for data/control reception/transmission, the UE sees the serving cell only, with which there is no need to discuss configuration/deconfiguration of non-serving/neighbor cell or change of serving cell in R17. The UE may still need to perform L1 measurement/report based on RS from cells with PCI different from the serving cell, and the procedure for configuration/triggering/release of such measurement/report are still being discussed in RAN1 and RAN2.

Answer 3: According to the updated WID, only intra-frequency scenario is considered in R17.

# Summary and conclusion

In this contribution, we propose to send a follow-up LS to RAN3, with the following answers to their questions.

Proposal 1: Send a follow-up LS to RAN3, with the following answers to their questions:

Question 1: What does “non-serving cell” mean? Answer 1: The term of “non-serving cell” was used to refer to a neighbor cell that is not configured as serving cell to a UE, but can be a candidate for L1/L2-centric inter-cell mobility (changed to inter-cell beam management in RAN#92 [6]) and inter-cell multi-TRP operation. As it created some confusion based on the feedback received [2], such term will not be used in future correspondence. Question 2: What is the procedure of L1/L2-centric mobility? e.g., - Configuration of a non-serving cell - Deconfiguration of non-serving cell - Change of serving cell Answer 2: According to the updated WID, for inter-cell beam management [6], a UE can transmit to or receive from only a single cell (i.e. serving cell does not change). In this sense, for data/control reception/transmission, the UE sees the serving cell only, with which there is no need to discuss configuration/deconfiguration of non-serving/neighbor cell or change of serving cell in R17. The UE may still need to perform L1 measurement/report based on RS from cells with PCI different from the serving cell, and the procedure for configuration/triggering/release of such measurement/report are still being discussed in RAN1 and RAN2. Question 3: Whether both intra-frequency and inter-frequency scenarios are targeted for this new scheme? Answer 3: According to the updated WID, only intra-frequency scenario is considered in R17.

# References

[1] R1-2102248, LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[2] R2-2106787, LS Reply on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[3] R3-212879, Reply LS to RAN1 LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[4] R4-2108356, Reply to RAN1 LS on L1/L2-Centric Inter-Cell Mobility

[5] RP-211551, Moderator's summary for email discussion [92-e-08-feMIMO-Scope]

[6] RP-211586, Revised WID: Further enhancements on MIMO for NR