3GPP TSG RAN WG1 #106-e	R1-2108093

e-Meeting, 16 – 27 August, 2021

Agenda item:		8.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion of other aspects for NR over NTN

Document for:		Discussion and Decision

# Introduction

RAN plenary #86 [1] approved a work item on NR solutions to support non-terrestrial networks. It has been agreed that enhancements on PRACH sequence and/or format may be specified if beneficial and needed. In this document we provide our view on PRACH format enhancements. On top of this, we discuss some of the other aspects that need further consideration in relation to NR over NTN, even that some progress on these aspects have already been seen in previous e-meetings.

# PRACH format and preamble sequence enhancements

Enhancements on PRACH preamble formats and sequence generation principle may be required in case the residual UL timing and frequency errors after optional compensation procedure exceed tolerated deviations from defined NR preamble formats. The NTN SR [2] considers four options for enhanced PRACH format and sequence design:

●	Option-1: A single Zadoff-Chu sequence based on larger SCS, repetition number. Additional usage of CP and Ncs can be further determined in normative work.

●	Option-2: A solution based on multiple Zadoff-Chu sequences with different roots.

●	Option-3: Gold/m-sequence as preamble sequence with additional process, e.g., modulation and transform precoding.

●	Option-4: A single Zadoff-Chu sequence with combination of scrambling sequence

The NTN SI provides also guidance on how to down select the different options:

●	minimize required normative work.

●	minimize implementation complexity.

●	minimize testing in device and network.

From our perspective option-1 is the preferred solution over the others, because it is the only one that fulfils all listed requirements.

A natural way to make initial access preambles more robust against residual frequency offsets is to allow higher SCS e.g. up to 240 kHz for FR2 in scenario with very large beam diameters and small elevation angles. Unambiguous preamble signature detection requires that the frequency offsets are within +/- 0.5 SCS. This range can be increased to +/-1.5 SCS by employing restricted set type A. The range can be further increased to +/-2.5 SCS by employing restricted set type B. Currently, higher SCS and restricted set types are not supported for preambles of length 139, 571 and 1151. In addition, specific subcarrier scaling factors are linked to specific preamble lengths. Therefore, we suggest to introduce SCS scaling factors {0,1,2,3,4} for preambles of length {139,571,1151} and optionally deactivate specific root sequences via restricted set type A. Table 1 provides details on the configurations.

Proposal 1: Enable additional SCS scaling factors for all formats defined in TS 38.211 [3] table 6.3.3.1-2.

Proposal 2: Support restricted set type A for formats defined in TS 38.211 table 6.3.3.1-2.

Table 1. Configurations for enhanced ZC preambles.

| Format | LRA∈ | μ∈ |  |  |  | Support for restricted sets |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 2⋅2048κ⋅2-μ | 288κ⋅2-μ | Type A |
| A2 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 4⋅2048κ⋅2-μ | 576κ⋅2-μ | Type A |
| A3 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 6⋅2048κ⋅2-μ | 864κ⋅2-μ | Type A |
| B1 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 2⋅2048κ⋅2-μ | 216κ⋅2-μ | Type A |
| B2 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 4⋅2048κ⋅2-μ | 360κ⋅2-μ | Type A |
| B3 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 6⋅2048κ⋅2-μ | 504κ⋅2-μ | Type A |
| B4 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 12⋅2048κ⋅2-μ | 936κ⋅2-μ | Type A |
| C0 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 2048κ⋅2-μ | 1240κ⋅2-μ | Type A |
| C2 | {139,571,1151} | {0,1,2,3,4} | 15∙2μ kHz | 4⋅2048κ⋅2-μ | 2048κ⋅2-μ | Type A |

From Fig. 1-3 it can be observed that the residual differential (uncompensated) delay and hence the cyclic shift (NCS) should be kept as small as possible such that a large number of preambles can be offered in the network. Employing restricted set type B is not considered as an option because this would, specifically for preambles of length 139, lead to a small number of preambles.

Figure 1. Number of available preambles for unrestricted and restricted set type A as function of NCS for LRA=139.

Figure 2. Number of available preambles for unrestricted and restricted set type A as function of NCS for LRA=571.

Figure 3. Number of available preambles for unrestricted and restricted set type A as function of NCS for LRA=1151.

# Reliance on GNSS Systems

The usage of GNSS-based solution, assumes the UE has access to a GNSS system. However, the GNSS solution is not part of the 3GPP specification standard, and therefore it is not subject to standardization. As such, the 3GPP can’t standardize how the UE implements its GNSS solution.

Observation 1: As GNSS is external to 3GPP, the standard cannot dictate how the UE implements its GNSS solution nor the system chosen (GPS, GLONASS, Galileo, Others).

Observation 2: The precision and availability provided by different systems may vary significantly.

The full-reliance on GNSS for synchronization and Random Access procedures leaves the 3GPP system implementation dependent on third part systems. This create additional room for threats to the functioning of the system:

- Spoofing: Modification of the position of the UE. Systems like GPS are know to be vulnerable to spoofing (https://nakedsecurity.sophos.com/2013/07/31/80-million-yacht-hijacked-by-students-spoofing-gps-signals/ ). For Random Access, the major threat is to prevent the UE to ever be able to get the RA process complete due to lack of synchronization.

- Jamming: Most of GNSS systems are known to be highly sensitive to jamming of their signals.

- Availability: The provider of the GNSS system may decide to turn off, modify, remove free access to their system, without notice or consideration to the NTN service provider. This will impact UEs capability to access the network and their satisfaction.

- Hacking: The 3GPP cannot ensure the third part system is not subject to malicious attacks, as its security implementations are outside the 3GPP world.

Other applications utilize GNSS for implementing or refining their solutions, but NTN seems more vulnerable to faults in GNSS systems, as the entire connectivity (the Random Access procedure) is fully relying on GNSS.

Observation 3: Full reliance on third part GNSS applications leave the 3GPP systems exposed to vulnerabilities that cannot be subject to enhancements or modifications by 3GPP standards or service provider will.

Proposal 3: NTN systems must contain a fall-back conservative solution that allows UE to access the network in case of faulty or malfunctioning GNSS systems.

# Polarization signalling and BWP aspects

At RAN1#105-e, the following agreemens were made:

Agreement:

For explicit indication of polarization information for DL by the network, support indication in SIB

- FFS: Signaling details for indication in SIB

Agreement:

- Polarization information for UL may be indicated in SIB by the network

- UE assumes a same polarization for UL and DL, when the UL polarization information is absent.

- FFS: Signaling details for indication in SIB

Agreement:

Same beam layout in BWP#0 and BWP#x (Option 1) and hierarchical beam for BWP#0 (Option 2) should be supported by the specifications for NR-NTN.

- FFS: Whether any specification changes are needed specifically to support this functionality

When providing explicit indication of the polarization information for either DL or UL it is our understanding that this information is important for the UE to be able to utilize such information prior to reading scheduling information and/or provide any uplink transmissions (such as random access preamble transmission). Hence, we would propose:

Proposal 4: The explicit indication of polarization information for DL is carried in an NTN-specific SIB that is broadcast frequently (SIB1 kind of signaling).

Proposal 5: The explicit indication of polarization information for UL is carried in an NTN-specific SIB that is broadcast frequently (SIB1 kind of signaling) and which is assumed to be read by the UE prior to performing UL access attempts.

During the e-mail discussion as part of RAN1#105-e, there was a length debate over the beam management operation for NR over NTN. As part of the initial discussion there were a number of potential issues that were raised for discussion. All of these issues (1-7) were related to whether or not enhancements would be needed for the existing NR framework in terms of enhancing the association between NR beams and NR BWPs. According to our understanding, the existing framework for BWP management and beam management should be sufficient to address the needed for supporting NR over NTN in Release 17.

Proposal 6: No NTN related enhancements for NR BWP or NR Beam management should be introduced for Rel-17.

# Conclusion

In this contribution we have presented our observations and proposals. These are as follows:

Observation 1: As GNSS is external to 3GPP, the standard cannot dictate how the UE implements its GNSS solution nor the system chosen (GPS, GLONASS, Galileo, Others).

Observation 2: The precision and availability provided by different systems may vary significantly.

Observation 3: Full reliance on third part GNSS applications leave the 3GPP systems exposed to vulnerabilities that cannot be subject to enhancements or modifications by 3GPP standards or service provider will.

Proposal 1: Enable additional SCS scaling factors for all formats defined in TS 38.211 [3] table 6.3.3.1-2.

Proposal 2: Support restricted set type A for formats defined in TS 38.211 table 6.3.3.1-2.

Proposal 3: NTN systems must contain a fall-back conservative solution that allows UE to access the network in case of faulty or malfunctioning GNSS systems.

Proposal 4: The explicit indication of polarization information for DL is carried in an NTN-specific SIB that is broadcast frequently (SIB1 kind of signaling).

Proposal 5: The explicit indication of polarization information for UL is carried in an NTN-specific SIB that is broadcast frequently (SIB1 kind of signaling) and which is assumed to be read by the UE prior to performing UL access attempts.

Proposal 6: No NTN related enhancements for NR BWP or NR Beam management should be introduced for Rel-17.

# References

- RP-193234, Thales ”Solutions for NR to support non-terrestrial networks (NTN)”

- 3GPP TR 38.821 V16.0.0 “Solutions for NR to support non-terrestrial networks (NTN)”

- 3GPP TS 38.211 V16.3.0 ”Physical channels and modulation”