3GPP TSG RAN WG1 106-e						R1-2108115

e-meeting, 16th Aug – 27th Aug 2021

Agenda item:	8.11.1.2

Source: 	CEWiT

Title: 	Feasibility and benefits for NR Sidelink mode 2 enhancements

Document for:	Discussion and Decision

- Introduction

The RAN WG approved study item on NR Sidelink Enhancements [1]. The study item includes the following objective:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89].

In this contribution, we express our views on sidelink enhancements targeting mode-2 enhancements in terms of power saving, latency, and reliability. Following agreements were made based on the discussion in meeting RAN1 104b-e[3]

- Agreement: - Support the following schemes of inter-UE coordination in Mode 2: - Inter-UE Coordination Scheme 1: - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information - FFS condition(s) in which Scheme 1 is used - Inter-UE Coordination Scheme 2: - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict - FFS condition(s) in which Scheme 2 is used - Agreement: - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination: - Details include applicable scenario(s)/inter-UE coordination scheme(s) - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc. - Including the possibility of being subject to certain conditions and/or capability - Agreement: - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission - For scheme 1: - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information - For scheme 2: - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information( sending indication) Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

No agreement was reached in the past RAN1#105-e meeting. The last plenary RAN#92-e concluded to let RAN1 continue the work for now without imposing any specific down-selection or guidance.

2 Discussion

In [2] we discussed drawbacks about Rel-16 sidelink resource allocation mode 2 such as hidden node and half duplex constraint for transmitting user equipment (UE). It is a well known fact that these problems lead to insufficient sensing results which may causes resource collision. Half duplex constraint is understood as a transmitting UE will not perform sensing during transmission of packet. Therefore, half-duplex constraint leads to some un-decoded or un-sensed SCIs. Lack of this knowledge may lead to collision while the UE attempts to transmit its own data over the resources which same resources are being occupied or overlapped by another UEs at same time.

Fig. 1a-Illustration figure of hidden-node		Fig. 1b-Co-UE transmitting assistance info.

Hidden node is also a major problem that occurs when two transmitting vehicles are not in sensing range of each other, but both of them are in the sensing range of another vehicle as shown in Fig. 1a. Here vehicle-1 and vehicle-3 are not in the sensing range of each other while vehicle-2 can sense from both, hence, it can lead to interference when vehicle-1 and vehicle-2 attempts to transmit on overlapping resources. In order to counter the above-mentioned problem, the concept of Inter-UE coordination can be introduced as shown in Fig. 1b, where Co-operative UE (Co-UE) provides assistance information to another UE. There is an agreement to provide this assistance information to UEs. Further, different aspect of assistance information is discussed. It includes the information content in the assistance information, selection of V2X UEs for sharing this information and upon receiving the assistance information what recipient UE should do on this assistance information.  Remaining aspect on these signalling is discussed in subsequent sections.

2.1 Scenario supported for assistance information sharing

In scheme 1, the coordination information sent from UE-A to UE-B is the set of preferred and/or non-preferred resources for UE-B’s transmission. Set of preferred resources may be useful in both cases of when UE-B perform sensing and not sensing. When it performs sensing it may take assistance from resource set indicated by UE-A along with own sensing results to do resource selection for transmission. When it is not sensing, the only option left is to rely on assistance information. In case of non-preferred resources, UE-B needs to have sensing mechanism in order to ignore those assistance information indicated. However, both are suitable to counter Rel-16 resource allocation issues such as hidden node and half duplex by providing additional useful information. In scheme 2, the coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI.

Observation 1: Preferred resource is suitable for UE-B with/without sensing capability

Observation 2: Non-preferred resource is suitable for UE-B having sensing capability

Proposal 1: Both preferred and non-preferred resource set should be supported for Rel-17 sidelink resource allocation mode 2 enhancement.

Scheme 1 is mainly to avoid the collision whereas scheme 2 is beneficial to detect the occurred collision or future potential/expected collison. In scheme 2, the candidate resources need not be included as it can be just an indication of conflict.

Observation 3: In scheme 2, candidate resources need not be included. It may include indication of conflict

2.2 Trigger for inter UE coordination

Trigger for inter UE coordination can be based on the request from UE-B or pre-configured. As shown in Fig. 2, when there is need for assistance, in the first case, a request/activation message-based triggering is applied when UE-B request for co-ordination information from UE-A. Based on request/ activation message UE-A responds with assistance information for UE-B.

UEB

UEA

assistance

request

Pre-configured

assistance

Fig. 2-Illustration figure of triggering

In the second case, UE-A can be configured to send assistance in periodic interval without any request for assistance from UE-B. Here UE-A may transmit co-ordination information with some fix periodicity as configured. Pre-configured periodic trigger is preferred in case of low latency requirements where UE-B cannot afford to transmit explicit request.

Proposal 2: Both request base triggering and pre-configured triggering should be supported for Rel-17 sidelink resource allocation mode 2 enhancement

Proposal 3: In case of pre-configured periodic  trigger, UE-A should be preconfigured with a periodicity to transmit co-ordination information

2.3	Method to transmit assistance information

In order to convey assistance information, a UE first prepare it after performing sensing and then convey it further through proper signaling. As shown in Fig. 3 UE-A first collects information based on its own sensing results obtained by decoding sidelink control information (SCI) received from other UEs in proximity. After sensing them UE-A derives the content based on the transmission type and transmits the assistance information for UE-B to assist in transmission/reception. These sensing results can be utilized by UE-B for resource allocation or to identify the resource conflict with another UE. Resource conflict occurs due to hidden node problem. Rel-16 supports unicast, groupcast and broadcast communication but there are certain problems we pointed out in section 2 those leads to incomplete sensing results. So, in order to improve existing Rel-16 method of resource selection in terms of enhancing reliability and achieving power saving, assistance information needs to be conveyed in all the three types of transmission, i.e. broadcast, groupcast and unicast.

Proposal 4: Assistance information signalling should be supported for unicast, groupcast and broadcast transmission in the Rel-17 sidelink.

SCI

SCI

UE1

UE2

UEA

UEB

Sensing Results

Fig. 3 -Procedure of inter-UE coordination

In order to convey assistance information, a proper signalling mechanism needs to be adopted. Assistance information consists of control and data part.  Design of a new 2nd stage SCI can be considered which can be indicated through 1st stage SCI field when there is a need to transmit the assistance information. In Rel-16 two 2nd stage SCI formats SCI-2A and SCI 2B is already used for decoding PSSCH based on the HARQ operation. SCI-2A is used when HARQ-ACK information includes ACK or NACK, HARQ-ACK information includes only NACK, or there is no feedback of HARQ-ACK information for all transmission types. SCI-2B is used in case of groupcast mainly with HARQ operation when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information. When a UE monitor new SCI format indicated by 2nd stage SCI format field, it understands assistance information is being conveyed from the two 2 bits field present in 1st stage SCI field.

In case of unicast, RRC or MAC message should be utilized for the conveying assistance information which will be followed by data in PSSCH

Proposal 5: New SCI format design should be considered for transmitting assistance information in the Rel-17 sidelink at least for broadcast and group-cast transmission.

Proposal 6: In case of unicast, assistance information is sent as RRC or MAC message

2.4 Content of the Assistance

The content of assistance information varies based on the transmission type. If assistance information is utilized among two UE (i.e., unicast), Co-UE which is UE-A can simply contain several resource sets, which can be used by the UE receiving assistance information to make decision while communicating to UE-A. Resource sets in sidelink is defined in terms of sub-channel and slots. Assistance information can consist of group of sub-channels in frequency and slots in time domain. UE transmitting assistance information (UE-A) can indicate the preferred resources in which it  wants to receive from another UE. Similarly, it can indicate the resources which should not be utilized for transmission by the receiving UE. For example, in unicast one of the UE that may send a preferred resource set in which it wants to receive data from the UE receiving assistance information. Other way is that it can indicate resources which are not suitable for Co-UE for receiving the information from another UE.

Proposal 7: Unicast assistance information should include both preferred and non-preferred resource set.

In presence of potential/expected and/or detected resource conflict on the resource indicated by UE-B’s SCI, the coordination information sent by UE-A can be a one-bit indication. Conflict is said to be potential/expected when there is chance of occurrence of collision. For example, presence of a high priority UE that may utilize the same resources as UE-B, which might cause conflict in future. In case of detected resource conflict. Since UE-A which will be sure of the resources that are causing conflict,it  will transmit them as co-ordination information to UE-B

Proposal 8: Both the cases of potential/expected and detected resource conflict should be supported in Rel-17 sidelink resource allocation enhancement

2.5 UE-B's response after decoding assistance information

UE-B after decoding assistance information will find out weather it is for the resource allocation or to indicate the collision with another UE. Either it checks it’s own results along with assistance information or it may use the assistance information directly to allocate resources. However, it is for the UE-B to decide on using the assistance information. Either it checks it’s own results along with assistance information or it may use the assistance information directly to allocate resources.  In any case, the received assistance information should help UE-B to take better decision about sidelink resource selection without restricting flexibility in resource selection. UE-B should also be able to ignore the assistance information in certain situation like high priority traffic, critical services etc.

Observation 4: UE may use the assistance information based on traffic type, priority of the traffic and own sensing results if available in Rel-17 sidelink resource allocation enhancement.

Proposal 9: UE-B should use the assistance information along with own sensing results, if available, to decide the final resources.

- Conclusion

Feasibility and benefits of NR sidelink mode 2 enhancement is presented. Inter-UE coordination can be key in order to enhance reliability and by supporting blind retransmission. Following are the observation and proposals discussed in this contribution about sidelink mode 2 resource allocation enhancement.

Observation 1: Preferred resource is suitable for UE-B with/without sensing capability

Observation 2: Non-preferred resource is suitable for UE-B having sensing capability

Observation 3: In scheme 2, candidate resources need not be included. It may include indication of conflict

Observation 4: UE may use the assistance information based on traffic type, priority of the traffic and own sensing results if available in Rel-17 sidelink resource allocation enhancement.

Proposal 1: Both preferred and non-preferred resource set should be supported for Rel-17 sidelink resource allocation mode 2 enhancement.

Proposal 2: Both request base triggering and pre-configured triggering should be supported for Rel-17 sidelink resource allocation mode 2 enhancement

Proposal 3: In case of pre-configured periodic  trigger, UE-A should be preconfigured with a periodicity to transmit co-ordination information

Proposal 4: Assistance information signalling should be supported for unicast, groupcast and broadcast transmission in the Rel-17 sidelink

Proposal 5: New SCI format design should be considered for transmitting assistance information in the Rel-17 sidelink at least for broadcast and group-cast transmission.

Proposal 6: In case of unicast, assistance information is sent as RRC or MAC message

Proposal 7: Unicast assistance information should include both preferred and non-preferred resource set.

Proposal 8: Both the cases of potential/expected and detected resource conflict should be supported in Rel-17 sidelink resource allocation enhancement

Proposal 9: UE-B should use the assistance information along with own sensing results, if available, to decide the final resources.

- References

- RP-202846 WID revision: NR sidelink enhancement

- R1-2101647 Feasibility and benefits for NR Sidelink mode 2 enhancements

- RAN1#104b-e Chairman’s notes, April 2021.

- RAN1#105-e Chairman’s notes, June 2021.