3GPP TSG-RAN WG1 Meeting #106-e	R1- 2108168

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.5.5

Source:	Ericsson

Title:	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

Document for:	Discussion, Decision

# Introduction

The updated WID for NR positioning enhancements [1] includes the following objective

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

This contribution is towards the above objective. In this contribution, we propose a few ways to enable LOS/NLOS detection in the network.

# Motivation for LOS/NLOS detection

Other features being specified in Release 17 such as DL-AoD and UL-AoA based positioning depends heavily on LOS/NLOS detection. Many of these features being specified may remain futile or not useful in the absence of information on LOS/NLOS nature of the link.  Unlike any timing estimation which may have bounded variability of say tens of meters around its mean estimate in NLOS channel, angle estimates may have uniformly distributed variability from 0 to 360 degrees in NLOS channel.

Hence while working on angle-based positioning in this Release its very important that LOS/NLOS detection mechanism of a link must be specified too.

A NLOS channel may result in bounded timing delay variation. However, it can result in an angle spread covering all directions. Hence, angle-based positioning methods are more prone to errors than timing-based methods in NLOS channel. And Release 17 focuses a lot on angle-based methods.

NLOS/LOS detection mechanism are beneficial to NR positioning enhancements since:

  - It will strengthen other features being specified in Release 17. Particularly the angle-based methods

  - It will help meeting stringent positioning accuracy requirement in Release 17.

Support NLOS/LOS detection mechanisms in release 17

Even in all timing-based positioning methods, additional random bias due to non-line-of-sight condition deteriorates performance of position estimation. For example, in the InF models defined in 38.901, the mean NLOS excess delay is 48.3ns corresponding to 14.5m. NLOS links will thus rarely contribute constructively to meter or submeter accuracy positioning. With stringent requirement on positioning accuracy for IIOT scenario in SID [1] of 20 cm, LOS detection will be very crucial in meeting the requirements.

The mean NLOS excess delay in the InF models is 48.3ns corresponding to 14.5m. NLoS links will thus rarely contribute constructively to meter or submeter accuracy positioning.

To show the potential for performance improvements based on LOS detection, we have evaluated DL TDOA positioning based on genie LOS detection. In addition, we have evaluated a simple realistic LOS detection mechanism which detects a link as LOS if the first peak in the channel impulse response is also the strongest peak. In either case, position estimation is done by including only the detected LOS links. In Figure 1 and  Table 1, we show the positioning performance in the InF-DH scenario for genie LOS detection, strongest peak LOS detection and without LOS detection.

The results for strongest peak LOS detection show promising gains compared to no LOS detection, a factor 2 improvement at the 90 percentile, a factor 4 improvement at the 80 percentile and a factor 37 improvement at the 50 percentile. This comes with a method with very low complexity. To detect the strongest peak is obviously never harder than detecting the first peak and is most likely already done by UEs for other purposes. We therefore propose that RAN1 should specify reporting of the strongest peak in rel. 17.

Strongest peak LOS detection gives fantastic gains compared to no LOS detection for the InF-DH scenario, a factor 2 improvement at the 90 percentile, a factor 4 improvement at the 80 percentile and a factor 37 improvement at the 50 percentile.

The results for genie LOS detection show enormous gains in positioning accuracy compared to no LOS detection, a 47 fold improvement at the 90 percentile and a 47 fold improvement at the 50 percentile. This shows the great potential of LOS detection techniques. It also shows that there is a big potential to improve beyond the use of strongest peak based LOS detection. No LOS detection method is fool proof and to get closer to the genie LOS detection potential a combination of multiple LOS indicators will be needed. We therefore propose that RAN1 should study LOS detection techniques and reporting of LOS indicators for potential specification in Rel. 17.

Genie LOS detection give enormous gains in positioning accuracy in the InF-DH scenario compared to no LOS detection, a 69 fold improvement at the 90 percentile and a 47 fold improvement at the 50 percentile. This shows the great potential of LOS detection techniques.

Figure 1 Comparison of DL TDOA performance for the InF-DH scenario with genie LOS detection, with LOS detection based on whether the first peak is also the strongest peak, and without LOS detection. The figure to the right is zoomed in at small positioning errors.

Table 1 Comparison of DL TDOA performance for the InF-DH scenario with genie LOS detection, with LOS detection based on whether the first peak is also the strongest peak, and without LOS detection at different percentiles.

|  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| Genie LOS detection | 0.07m | 0.11m | 0.16m | 0.26m |
| Strongest peak LOS detection | 0.09m | 0.18m | 1.9m | 8.4m |
| No LOS detection | 3.3m | 5.4m | 8.2m | 18m |

# Rich multipath reporting

- Agreement:

- Study multipath reporting enhancements for DL, UL, and DL+UL positioning to enable LoS/NLoS/multipath identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

- Agreement:

- For multipath reporting enhancements, study reporting from TRP to LMF, angle, timing, phase (of additional paths) and power for the additional N paths (value of N is part of the study).

- Note: Companies are not obligated to provide inputs for all parameters in their study

Agreement:

For multipath reporting enhancements, study reporting from UE to LMF, relative timing of additional paths (additional to the first path) and the power (at least relative power) at least per DL PRS resource per additional path for at least DL-AoD reporting (the number of paths is part of the study).

The following was agreed during RAN1#105e:

While release 16 provides a good suite of reference signals for positioning, the measurements specified in release 16 needs to be enriched through the incorporation ofadditional useful information for positioning.

Rich multipath reporting have been discussed in release 16, and there is support for up to two additional paths to be reported with the timing measurements (RSTD, UE RxTX Time Diff, gNB RxTx Time Diff and RTOA). However, in release 17, to make rich reporting useful and to meet stringent accuracy requirements, we need to increase the number of additional paths reported and unambiguously define what additional paths and what metrics the UE shall report.

Rich multipath reporting can include many parameters of the received signal which provides additional information helpful for positioning. Such parameters can be magnitudes of peaks SNRs of the peaks, Doppler frequencies, angle of arrival etc. of every reported multipath.

In order for rich reporting of additional paths to be at all useful, it’s necessary to define unambiguously what additional paths the UE should report.

- For rich multipath reporting, it shall be unambiguously defined what additional paths a UE shall report.

Different paths can be useful for different purposes, e.g.

- First path. Gives the TOA of the LOS path in case of LoS and the TOA closest to LOS in case of NLOS.

- Strongest path. Gives information that can be used for LOS detection (If the first path isn’t also the strongest path it can be assumed to be NLOS). It can be measured with best precision, and is useful for fingerprinting techniques.

- N strongest paths with shorter delay than the strongest path. Gives useful information in case the first path is misdetected with noise, interference or an out of range path. This is useful also for fingerprinting techniques.

- N strongest paths. This is useful for fingerprinting techniques.

- Alternatively, a first path based on lower noise and interference threshold.

In the remainder of this section we present different ways that rich multipath reporting can be used for LOS/NLOS detection and give proposals accordingly.

## Signature based LOS/NLOS classification

In signature-based LOS/NLOS classification, we propose detecting specific signatures(patterns) in the measurements to detect LOS/NLOS. Below we motivate a LOS/NLOS detection method inspired from a real-world measurement dataset in LOS/NLOS scenarios [2]. Figure 2 shows power delay profile in indoor factory like scenario in LOS/NLOS conditions. The zoomed version of the power delay profile is shown on the right in Fig.2 in relevant regions of the measurement.

Figure 2 Power delay profile in LOS/NLOS scenarios;  zoomed version of the PDP is shown on the right

It can be seen from above figures that in LOS scenario the highest peak of the measurement is very prominent. It is the first very clear peak arising from the measurement noise floor. Whereas, in the NLOS case, the highest peak of the measurement data is very gradual. The nature of LOS peak can be attributed to the fact that in the LOS conditions, all scattered signals arrive after the LOS component of the delay profile. Magnitudes of arriving scattered signals are always lower than the LOS reception due to higher path losses and other loss attributing effects. In NLOS cases, the highest peak can arrive possibly after many other scattering peaks. Even when the NLOS peak is prominent, typically it may be received after diffuse scattering components. Receiving diffuse scattering components along and before the prominent NLOS peak makes the rise of the peak very gradual, as evident from. Based on above discussion, following observations can be made.

LOS peak is always the first peak in power delay profile of the received signal for a LOS link.

A NLOS peak can also be a first prominent peak in a delay profile. But it appears along with smaller surrounding peaks and other components of delay profile generated from diffusely scattered signal before and after the NLOS prominent peak.

Rise of LOS highest peak is very sharp. Rise of NLOS highest peak is typically very gradual.

Figure 4 shows a possible implementation for detecting LOS/NLOS based on analyzing power delay profile of the received signal.

Figure 3 LOS/NLOS detection algorithm based on analyzing power delay profile of the received signal.

Based on the above discussion, we make the following proposal:

- Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

  - Delay and magnitude of the first peak.

  - Delay and magnitude of the highest peak.

  - Components of PDP/CIR around first/highest peak.

## Avoiding mis-detection of NLOS due to bad FFT window placement

We have seen from previous discussions the importance of first path in detecting NLOS links. The first path can be missed due to using an FFT window which are typically used for communication purposes also being used for computing CIR for positioning purposes. It is important to ensure that right CIR is used in doing NLOS detection using CIR peaks.

Fig.7 shows a NLOS impulse response where the start of impulse response, the first path falls within the FFT window. However, Fig.8 shows the case when the first path is missed as it may fall out of the FFT window. This can happen when a FFT window is placed to capture maximum energy of an impulse response, which is typically the case while placing FFT windows for communication purposes. Higher numerologies can be more prone to such issue as the length of cyclic prefix shrinks while the geometries of scenarios and the corresponding time of arrival delay remains same.

Figure 4   A typical CIR where the first path is included within the cyclic prefix. A NLOS link will be correctly detected as first peak falls within the FFT window.

Figure 5  A CIR where first path is  missed due to wrong placement of FFT window. In this case a NLOS channel will be detected as LOS channel. This can happen when the FFT windows are placed in order to capture maximum energy within the window.

Positioning may require different FFT window placement (symbol timing estimation) than typically used for communication depending on the scenario for channel impulse response based LOS/NLOS detection.

RSTD windows typically use distance estimates based on timing advanced. The timing advanced values are computed using FFT windows used for communication purposes. Hence RSTD window is not enough to capture CIR enriched with information required for LOS/NLOS detection.

It is important to ensure that CIR used in NLOS detection is not missing the first peak.

In higher numerologies, cyclic prefix shrinks but delays in channel remains same. Hence, it is more probable to miss the first path in higher numerologies.

When the reciprocity principle holds, a CIR computed by the UE in DL and a CIR computed by a TRP in UL should ideally match. This property can be used by LMF to validate the CIRs and detect errors caused by an incorrectly placed FFT window.

Channel reciprocity between the network and UE can be used for avoiding mis-detection of the first CIR/PDP peak.

A consequence of Observation 13,  corroboration of the CIRs at the two ends (Network in UL, UE in DL) based on the reciprocity principle can improve the reliability of a NLOS detector and hence the position estimation. One option is that LMF compares the complete CIRs obtained by the UE and TRP respectively, but then of course these CIRs must be signalled to LMF. Alternatively, the comparison can be done based on rich multipath reporting from the UE and TRP.

Rich multipath reporting in both UL and DL can be used by LMF for validation and to avoid mis-detection of LOS/NLOS links.

If LMF identifies that the UE or the TRP systematically fails to detect the first CIR peak due to incorrect FFT-window then LMF can feedback to the UE/TRP that another FFT window configuration is needed.

The LMF can provide the UE/TRP a configuration for FFT window placement or alternatively provide an indication that a reconfiguration is needed.

## Number of additional paths: Performance evaluations with ML based NLOS detection

Agreement:

- Study whether to support up to N>2 additional paths in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT,

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

  - FFS: LMF requesting additional M non-distinct paths corresponding to the first path.

- Note 1: This agreement applies to N additional paths (i.e., not including the “first” path).

- Note 2: Rel-16 supports N=2 already.

The following was agreed during RAN1#105e:

Machine Learning (ML) classifiers can be trained for LOS/NLOS detection. We have studied the performance of different ML classifiers that take as inputs the value and delay time of varying number of peaks in the power delay profile (PDP). The classifiers were trained and evaluated on real 5G measurement data from a macro deployment with carrier frequency 2.66 GHz and PDP obtained from reference signals with 20 MHz bandwidth. The best performance was obtained with a K-nearest neighbor method and those results are presented here.

Test cases

10 test cases were evaluated, with increasing number of reported peaks:

For case 1 to 7:

- The LOS/NLOS classifier inputs are the (i) peak value and (ii) delay time of the highest peaks in the PDP.

- The number of peaks used are: 5, 10, 15, 20, 30, 40 or 50 (Test case 1 to 7)

For case 8 to 10:

- The LOS/NLOS classifier inputs are the first X number of samples of the measured PDP.

- Using X= 100 (case 8), X=150 (case 9), and all samples in the PDP (case 10)

The K-nearest neighbor method is always used as it gives the best performance.

Results

Table 2 summarize the results of the evaluation. Since Test case 10 makes use of the full PDP, we interpret the performance results obtained there as an upper bound for what can be achieved by this method on a subset of the PDP. We can make a number of observations:

The more PDP peaks (corresponding to paths) that are used by the classifier, the better becomes the classifier performance.

Motivated by these results we think that the number N of additional paths should be as large as possible.

The number of additional paths N should be large to enable accurate LOS/NLOS detection.

For each additional path, report at least:
- the path power,
- the path delay.

Table 2: ML results for test cases 1-10. The true LOS/NLOS status of the channel is on the vertical axis, and the ML-estimate is on the horizontal axis. Consequently, the diagonal elements shows the fraction of correct NLOS resp. LOS classifications and the off-diagonal elements show false negatives and false positives.

# Algorithmic LOS/NLOS classification

Classification of measurements as LOS or NLOS and mitigation of the effects of NLOS can sometimes be done algorithmically with methods for robust estimation. The input to this kind of algorithms is typically a set of observations, like for instance TOA or TDOA measurements between different TRPs and a UE. NLOS measurements are considered as contaminated observations or outliers. While e.g. estimation with a least-squares method is highly sensitive to outliers, robust estimation methods are not. The methods only work when there is an excess of observations from which outliers can be pruned, hence algorithmic classification is particularly interesting for positioning in e.g. industrial scenarios where there is a dense deployment of TRPs. It should be stressed that the applicability of algorithmic LOS/NLOS classification to macro deployments (that are sparse) is very limited.

In this section we show results for an adaptation of the RANdom SAmple Consensus (RANSAC) algorithm proposed in [3] to the TOA/TDOA positioning method. The performance of robust estimation methods degrades as the fraction of contaminated observations increases. RANSAC can cope with a relatively large fraction of contaminated data. Algorithmic LOS/NLOS classification can benefit from rich reporting and be used in combination with e.g. signature-based LOS/NLOS detection to obtain even better performance. Here we show one such example where RANSAC is combined with a LOS/NLOS detector that compares the strength of different peaks in the power delay profile: If the first peak is the strongest, then it is likely LOS.

We evaluate the positioning performance on the InF-DH scenario specified by 3GPP in TR 38.901 with model parameters as given in TR 38.857. There are 18 TRPs in a 120x60 room which are mounted on a high level and 800 UEs are uniformly distributed over the deployment area. Each TRP broadcast DL-PRS that the UEs perform TOA measurements on by peak detection in an estimated channel impulse response. The TRPs are assumed synchronized with each other but not with the UE. The bandwidth and carrier frequency of the reference signal is 100 MHz respectively 3.5 GHz. With 800 UEs and 18 TRPs there in total 14400 TOA measurements. Figure 4 shows the TOA error distribution for LOS resp. NLOS measurements. The UE/TRP clock synchronization errors are not included in these figures, meaning that what is shown are the measurement errors and NLOS excess delays.

For LOS measurements the error distribution is well approximated by a Student´s t distribution with location, scale and shape parameters μ= 0, σ= 0.05 and ν= 1, respectively. The accuracy of these measurements in the order of decimeters, likely thanks to the high bandwidth of the DL-PRS signal. The NLOS excess delay is modeled with a log-normal distribution in the InF-DH scenario. This distribution is also plotted in Figure 4 (b) and its agreement with the simulated data is very good. Notice that the NLOS TOA errors are generally much larger than the LOS TOA errors, this is an important property for the performance of an algorithmic LOS/NLOS classifier.

Figure 4

Figure 5 shows the UE positioning error CDFs that are obtained with two different RANSAC algorithms. The figure also includes the results obtained with a least squares estimator, serving as a baseline performance reference. Moreover, the "LOS Genie'' results are obtained with a least-squares estimation based on the actual LOS measurements. Hence, these results indicate an upper limit for the accuracy that can theoretically be achieved by an estimator with a non-ideal LOS classifier.

The performance of the least-squares estimator is bad because of the large fraction of NLOS measurements in this scenario. Already the basic RANSAC algorithm gives very good improvements as compared to the LS estimator. The RANSAC+SFP algorithm combines RANSAC with the output of a signature-based LOS/NLOS classifier. The combination gives an even better positioning performance, close to the performance of “LOS Genie”.

Robust estimation methods can drastically improve the positioning performance in dense deployments with many NLOS measurements. However, since they require an excess of measurements, they cannot be used in sparse deployments.

Robust estimation methods can be combined with signature-based LOS/NLOS detectors to achieve even better positioning performance.

Figure 5

In Figure 6 we group the total 800 UEs according to how many LOS measurements there are for each of them and show how many of the UEs that different algorithms locate to a position within 1 m from the true position.

We observe a gradually reduced performance for both the robust estimation methods as the number of LOS measurements reduce. However, the combined RANSAC+SFP method performs better when there are very few LOS measurements.

The performance of robust estimation methods reduces gradually as the fraction of LOS measurements decrease.

The use of signature-based LOS/NLOS detectors becomes more important when there are only a few LOS measurements.

Figure 6

The next table summarizes the LOS/NLOS classification performance of the methods. SFP is the detector that compares the strengths of the pdp-peaks.

The SFP classifier has a much larger fraction of false LOS classifications than the other methods. In this dense deployment scenario, algorithmic LOS/NLOS classification with RANSAC works well. However, the best performance is obtained with the combined algorithmic and signature-based detector RANSAC+SFP.

Note: In our implementation, RANSAC+SFP will always “inherit” the NLOS classifications of the SFP detector and possibly also add more. However, this need not be the case depending on how the detectors are combined.

The LOS/NLOS classification performance of robust estimation methods is good in dense deployments.

The LOS/NLOS classification performance can be further improved by combining signature-based classifiers with robust estimation techniques.

Table 2 LOS/NLOS detector statistics. In total 14400 TOA measurements are simulated according to the Inf-DH scenario, 7524 of them are in LOS conditions and the other 6876 are in NLOS conditions

| Detector | LOS |  | NLOS |  |
| --- | --- | --- | --- | --- |
|  | Correct | False | Correct | False |
| SFP | 7136 | 2250 | 4626 | 388 |
| RANSAC | 7197 | 249 | 6627 | 327 |
| RANSAC+SFP | 7016 | 32 | 6844 | 508 |
| LOS Genie | 7524 | 0 | 6876 | 0 |

When performing TOA measurements on an estimated PDP, it is not always obvious which peak to report for, for instance if the first peak is not the strongest. In this situation the first peak could be a noise peak or it can correspond to the LOS path which is attenuated from e.g. penetration of a wall. However, since algorithmic LOS/NLOS classification can resolve such confusions, it is best to always report both the TOA for the first peak and additionally the N strongest peaks.

To support outlier detection types of methods (e.g. RANSAC) The UE could report TOA for the first (earliest) detected peak and additionally the N strongest detected peaks.

# Alternatives to rich multipath reporting

## Reporting of LOS/NLOS indicators

Agreement:

- Study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options (or combinations of the following options) for LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: LoS/NLoS indicators for UE-based positioning

The following was agreed during RAN1#105e:

For UE assisted positioning, LMF can perform LOS/NLOS classification and positioning based on multiple information sources, e.g. UE measurements, TRP measurements, additional sensor data etc.  Additional geometric constraints obtained from e.g. a map and the known locations of other network nodes can also be used. With different and complementary information sources the classification performance can be considerably improved compared to what can be achieved with only the measurements of a UE/TRP. We could see one example of this in Section 4, where the combined RANSAC+SFP detector outperforms the SFP detector.

A network based LOS/NLOS detector that combines measurements from multiple sources and geometrical constraints can outperform a detector that operates on the measurement of one UE or TRP alone.

In such hybrid positioning algorithm it will difficult to assess how a LOS/NLOS indicator reported from a UE or TRP should be trusted for instance if the value of the indicator contradicts some other information. It is much better if LMF can access the measurements or statistics that was the basis for the indicator value directly, because then LMF can perform optimal LOS/NLOS classification (and position estimation) by e.g. Bayesian techniques.

On the other hand, we recognize that there can be measurements performed by a UE/TRP which cannot be reported to LMF. For instance, a UE might be able to extract LOS/NLOS information from a camera. In this case a reported indicator value can be useful to LMF.

Reporting of a LOS/NLOS indicator can be useful when it is based on measurements that cannot be reported to LMF.

It should be stressed as in Observation 23 that an indicator cannot replace rich multipath reporting, only complement it.

Below we comment on Option 1 resp. 2 for the indicator format.

Option 1: A UE/TRP cannot be completely certain about the LOS/NLOS conditions. Hence, even a binary indicator must be interpreted as "probably LOS" resp. "probably NLOS".

Option 2: The value of the indicator reflects the estimated probability for LOS/NLOS. This option is a better option for use in a hybrid network based classification algorithm.

To help LMF asses such indicator we propose that the indicator of Option 2 is coupled with an information element that states what kind of measurement that was used to obtain it.

A LOS/NLOS indicator reported by a UE/TRP should be coupled with an information element that states what kind of measurement or analysis that was used to obtain it.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A NLOS channel may result in bounded timing delay variation. However, it can result in unbounded angle spread. Hence, angle-based positioning methods are more prone to errors than timing-based methods in NLOS channel. And Release 17 focuses a lot on angle-based methods.

Observation 2	NLOS/LOS detection mechanism are beneficial to NR positioning enhancements since:

a.	It will strengthen other features being specified in Release 17. Particularly the angle-based methods

b.	It will help meeting stringent positioning accuracy requirement in Release 17.

Observation 3	The mean NLOS excess delay in the InF models is 48.3ns corresponding to 14.5m. NLoS links will thus rarely contribute constructively to meter or submeter accuracy positioning.

Observation 4	Strongest peak LOS detection gives fantastic gains compared to no LOS detection for the InF-DH scenario, a factor 2 improvement at the 90 percentile, a factor 4 improvement at the 80 percentile and a factor 37 improvement at the 50 percentile.

Observation 5	Genie LOS detection give enormous gains in positioning accuracy in the InF-DH scenario compared to no LOS detection, a 69 fold improvement at the 90 percentile and a 47 fold improvement at the 50 percentile. This shows the great potential of LOS detection techniques.

Observation 6	LOS peak is always the first peak in power delay profile of the received signal for a LOS link.

Observation 7	A NLOS peak can also be a first prominent peak in a delay profile. But it appears along with smaller surrounding peaks and other components of delay profile generated from diffusely scattered signal before and after the NLOS prominent peak.

Observation 8	Rise of LOS highest peak is very sharp. Rise of NLOS highest peak is typically very gradual.

Observation 9	It is important to ensure that CIR used in NLOS detection is not missing the first peak.

Observation 10	In higher numerologies, cyclic prefix shrinks but delays in channel remains same. Hence, it is more probable to miss the first path in higher numerologies.

Observation 11	Channel reciprocity between the network and UE can be used for avoiding mis-detection of the first CIR/PDP peak.

Observation 12	Rich multipath reporting in both UL and DL can be used by LMF for validation and to avoid mis-detection of LOS/NLOS links.

Observation 13	The more PDP peaks (corresponding to paths) that are used by the classifier, the better becomes the classifier performance.

Observation 14	RAN1 to study the use of rich reporting including power, delay and DL AOA per path as a basis for LOS detection and consider these measurements for specification in Rel. 17.

Observation 15	Robust estimation methods can drastically improve the positioning performance in dense deployments with many NLOS measurements. However, since they require an excess of measurements, they cannot be used in sparse deployments.

Observation 16	Robust estimation methods can be combined with signature-based LOS/NLOS detectors to achieve even better positioning performance.

Observation 17	The performance of robust estimation methods reduces gradually as the fraction of LOS measurements decrease.

Observation 18	The use of signature-based LOS/NLOS detectors becomes more important when there are only a few LOS measurements.

Observation 19	The LOS/NLOS classification performance of robust estimation methods is good in dense deployments.

Observation 20	The LOS/NLOS classification performance can be further improved by combining signature-based classifiers with robust estimation techniques.

Observation 21	To support outlier detection types of methods (e.g. RANSAC) The UE could report TOA for the first (earliest) detected peak and additionally the N strongest detected peaks.

Observation 22	A network based LOS/NLOS detector that combines measurements from multiple sources and geometrical constraints can outperform a detector that operates on the measurement of one UE or TRP alone.

Observation 23	Reporting of a LOS/NLOS indicator can be useful when it is based on measurements that cannot be reported to LMF.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support NLOS/LOS detection mechanisms in release 17

Proposal 2	For rich multipath reporting, it shall be unambiguously defined what additional paths a UE shall report.

Proposal 3	Following measurements should be specified in Rel-17 to support signature-based methods. These measurements can be part of rich reporting.

a.	Delay and magnitude of the first peak.

b.	Delay and magnitude of the highest peak.

c.	Components of PDP/CIR around first/highest peak.

Proposal 4	The LMF can provide the UE/TRP a configuration for FFT window placement or alternatively provide an indication that a reconfiguration is needed.

Proposal 5	The number of additional paths N should be large to enable accurate LOS/NLOS detection.

Proposal 6	For each additional path, report at least: - the path power, - the path delay.

Proposal 7	A LOS/NLOS indicator reported by a UE/TRP should be coupled with an information element that states what kind of measurement or analysis that was used to obtain it.

# Bibliography

| [1] | 3GPP TSG RAN Meeting #90e, “New WID on NR Positioning Enhancements,” December 7-11, 2020. |
| --- | --- |
| [2] | 3GPP, “TR 38.901. Study on channel model for frequencies from 0.5 to 100 GHz,” 2019. |
| [3] | R. C. a. F. M. A. Bolles, “A RANSAC-based approach to model fitting and its application to finding cylinders in range data,” in IJCAI Citeseer, 1981. |