# 3GPP TSG RAN WG1 Meeting #106-e	R1-2108582

# e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0561 | rev | - | Current version: | V16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NPUSCH postponement for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (ZTE), Sanechips, Ericsson, Lenovo, Motorola Mobility, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2021-08-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#95 meeting the following was agreed. Agreement: For the new NPRACH resources introduced in Rel-15, - For non-EDT NPRACH resources, the UE postpones NPUSCH in those resources only if the UE indicates support for the corresponding feature(s). - For EDT NPRACH resources, the UE postpones NPUSCH in those resources only during an EDT procedure However in Clause 10.1.3.6 of TS36.211, the following triggering cases for NPUSCH postponement are not included: Case #; NPRACH preamble; Carrier; UE capability; RRC Configuration. 1; Legacy(i.e. Format 0/1); Non-anchor; multiCarrier-NPRACH and mixedOperationMode; SystemInformationBlockType22 > ul-ConfigListMixed-r15. 2; Format 2; Non-anchor; multiCarrier-NPRACH and nprach-Format2; SystemInformationBlockType23-NB-r15 > ul-ConfigList-v1530 > nprach-ParametersListFmt2-r15. 3; Format 2; Non-anchor; multiCarrier-NPRACH, mixedOperationMode and nprach-Format2; SystemInformationBlockType23-NB-r15 > ul-ConfigListMixed-v1530 > nprach-ParametersListFmt2-r15. 4; Format 2; Anchor; nprach-Format2; SIB2 > RadioResourceConfigCommonSIB-NB-r13 > nprach-ParametersListFmt2-r15. 5; TDD; Anchor; [NOTE1]; SIB2 > RadioResourceConfigCommonSIB-NB-r13 > nprach-ParametersListTDD-r15. 6; TDD; Non-anchor; multiCarrier-NPRACH [NOTE1]; SystemInformationBlockType22 > ul-ConfigCommonListTDD-NB-r15> nprach-ParametersListTDD-r15. NOTE1: No explicit capability for TDD NPRACH. |
| Summary of change: |  | Adding the missing triggering cases 1~6 for NPUSCH postponement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NPUSCH postponement rule is not clear for above 6 triggering cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 10.1.3.6	Mapping to physical resources

# <Unchanged parts are omitted>

If a mapping to  slots or a repetition of the mapping contains a resource element which overlaps with

-	any configured NPRACH resource according to nprach-ParametersList in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigList in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource according to nprach-ParametersList given by ul-ConfigListMixed in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH and mixedOperationMode as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 in SystemInformationBlockType2-NB and if the UE indicates nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigList in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListFmt2 given by ul-ConfigListMixed in SystemInformationBlockType23-NB and if the UE indicates multiCarrier-NPRACH, mixedOperationMode and nprach-Format2 as supported, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType2-NB, or

-	any configured NPRACH resource according to nprach-ParametersListTDD in SystemInformationBlockType22-NB and if the UE indicates multiCarrier-NPRACH as supported, or

-	any configured NPRACH resource configured for Early Data Transmission and if the NPUSCH transmission is during an Early Data Transmission procedure [12, Clause 7.3b],

then,

-	for  the NPUSCH transmission in overlapped slots is postponed until the next  slots not overlapping with any configured NPRACH resource.

-	for  the NPUSCH transmission in overlapped  slots is postponed until the next  slots starting with the first slot satisfying nsmod 2=0 nsmod 2=0and not overlapping with any configured NPRACH resource.

# <Unchanged parts are omitted>