3GPP TSG-RAN WG1 Meeting #106bis-e		R1-2108946

e-Meeting, October 11th – 19th, 2021

Title:	[DRAFT] Reply LS on RAN2 Agreements Related to Resource Selection

Response to:	R2-2108997

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	vivo [RAN1]

To:	RAN2

Cc:

Contact person:	Zichao Ji

zichao.ji@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 thanks RAN2 for the LS on the following RAN2 agreements of TX UE resource selection in DRX.

- For unicast, the TX UE maintains a set of timers per pair of source Layer-2 ID and destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. For groupcast/broadcast, the TX UE maintains a set of timers per destination Layer-2 ID corresponding to the SL DRX timers in the RX UE. The TX UE uses the timers as part of the criterion for determining the allowable transmission time for each RX UE. - For transmissions to RX UE(s) using SL DRX operation, LCP restrictions ensure that a TX UE transmits data in the active time of the RX UE(s).

Agreements: - When data is available for transmission to one or more RX UE in DRX, TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) determined by the timers maintained at the TX UE. Details are FFS. FFS whether RAN1 or RAN2 implement this restriction. - For unicast, the TX UE selects the resources for the initial transmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE. How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact. - For unicast, the TX UE can select the resources for the retransmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE. How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact. - For broadcast, the TX UE can select the resources for the initial transmission associated with any active time supported by broadcast (i.e. on duration) at the RX UE. - For broadcast, the TX UE can select the resources for the retransmission associated with any active time supported by broadcast (i.e. on duration) at the RX UE.

Regarding the question of whether/how RAN1 intends to reflect the restriction in the following RAN2 agreement “When data is available for transmission to one or more RX UE in DRX, TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) determined by the timers maintained at the TX UE.  Details are FFS. FFS whether RAN1 or RAN2 implement this restriction”, it is RAN1’s understanding that MAC would provide the active time of the RX UE to PHY when it triggers resource selection procedure, so that PHY can select and report suitable candidate resources (i.e., within the active time of the RX UE) to MAC.

2	Actions

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #107-e			November 11th – 19th, 2021   			e-meeting

TSG RAN WG1 Meeting #107bis-e			January 17th – 25th, 2022 			e-meeting