3GPP TSG RAN WG1 #106bis-e		    				R1-2109018

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE features for UE power saving enhancements

Agenda Item:	7.17.7

Document for:	Discussion and Decision

- Introduction

This contribution addresses our proposals for Rel-17 power saving UE features. The discussion is based on the latest version of UE features summary in R1-2108679 [1].

- Discussion on Rel-17 UE Power saving features

29-1 Paging enhancement

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |

- Subgroup indication

RAN2 discussed UE paging subgrouping as part of the Rel-17 work on UE power saving enhancement. For example, two subgrouping approaches have been agreed, namely CN-assigned subgrouping and UEID-based subgrouping. However, how to carry subgrouping information, is related and discussed in RAN1.

Considering this correlated way of discussion between RAN1 and RAN2, we expected the subgrouping indication is either separated from 29-1 or as a RAN2 feature to be discussed in RAN2.

- Descriptions of the components

According to the RAN plenary decision [2], PDCCH based PEI is agreed. New DCI format and only Behv-A supported is agreed. Thus it should be captured in the component descriptions.

Proposal 1:

- Support UE subgroup indication is either separated from 29-1 or as a RAN2 feature to be discussed in RAN2.

- Update the descriptions of 29-1 as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1-1 | Paging enhancement | 1. Support paging early indication 1. Support of configured window for detection of DCI format XXX with CRC scrambled with YYY for paging early indication 2. Support of Behv-A if UE does not detect PEI for all monitored PEI occasion(s) for the PO 2. Support UE subgroup indication |  |
| 29. NR_UE_pow_sav_enh | 29-1-2 | Paging enhancement | Support UE subgroup indication |  |

29-2 TRS resources for idle/inactive UEs

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |

Support of receiving L1 indication for TRS availability also requires UE to read corresponding configurations from system information. Hence it seems UE supporting component (2) would be very easy to support component (1) if no new mechanism for system information change for idle/inactive TRS from SIB is introduced.

On the other hand, only supporting component (1) is also useful, e.g., the TRS configurations does not change quite very frequently.  Component (2) is useful only if the TRS configurations change quite very frequently. Hence separating these two components in different UE feature is more appropriated.

Proposal 2:

- Update the descriptions of 29-2 as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2-1 | TRS resources for idle/inactive UEs | 1. Support reading TRS configuration from SIB for idle/inactive TRS |  |
| 29. NR_UE_pow_sav_enh | 29-2-2 | TRS resources for idle/inactive UEs | 1. Support receving L1 indication for TRS availability for idle/inactive TRS | 29-2-1 |

29-3 PDCCH monitoring adaptation within an active BWP

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3 | PDCCH monitoring adaptation within an active BWP | - Support of PDCCH monitoring adaptation behaviour 1/1A - Support of PDCCH monitoring adaptation behaviour 2/2A/[2B] |

It is agreed in RAN1#106 that,

Agreement - UE behavior after receiving PDCCH indication of monitoring adaptation can be one of the followings, - Working Assumption: Beh 1: PDCCH skipping is not activated - Beh 1A: PDCCH skipping means stopping PDCCH monitoring for a duration X - FFS the possible values for X - FFS: Whether and how to support more than one skipping duration(s) - FFS: whether to continue monitoring PDCCH scrambled by C-RNTI for Type 0/1/1A/2 CSS or not - Beh 2: stop monitoring SS sets associated with SSSG#1 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#0 (legacy behaviour) - Beh 2A: stop monitoring SS sets associated with SSSG#0 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#1 (legacy behaviour) - Working Assumption: Beh 2B(if confirmed): stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2 (if confirmed) - Note: The number of supported SSSG is left to UE feature discussion. - FFS: UE capability of supported UE behaviors - Indication of Beh 1A when SSSG(s) are not configured is supported. - Working assumption: Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported - FFS: Indication of Beh 1A when three SSSG(s) (if supported) are configured - Y bits is configured for scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) indicating PDCCH schedules data and also PDCCH monitoring adaptation - FFS how the UE behavior(s) defined above mapping to Y bits § Note: at most Y = 2 - Working Assumption at most 3 SSSGs is supported to be configured. - FFS: whether or how SSSG can be configured to be monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - FFS: whether or how non-default SSSG to another non-default SSSG - FFS details of timer(s) for switching between SSSG(s) - UE fallbacks to default SSSG (i.e., SSSG#0) after timer expiration. - R16 timer for SSSG switching and the corresponding behavior is as baseline - FFS whether the timer(s) is configured per SSSG, per BWP or other approaches. - FFS whether the skipping duration(s) is configured per SSSG, per BWP, or other approaches. - FFS PDCCH monitoring adaptation indicated by non-scheduling DCI - PDCCH based monitoring adaptation is applied to USS and type-3 CSS.

According to this further discussion on supported UE behaviours and number of supported SSSG need to be addressed. In [3], the corresponding UE capability and PDCCH adaptation mode is discussed.

- number of supported SSSG

The motivation to support more than 3 SSSG has been discussed and added as an working assumption. Considering specification effort for supporting more than 2 SSSGs and different understanding of urgency, it would be natural to have a sperate capability indication for UE supporting more than 2 SSSGs. Hence support of 2/2 and [2B] should be separated.

- supported UE behaviors

Considering the different mechanisms/usage to support PDCCH monitoring adaptation, it is natural to sperate 1/1A and 2/2A/[2B].

And it is in RAN1#106-E that indication of Beh 1A for current SSSG when two SSSG(s) are configured is WA. Considering this, another UE capability can be considered to indicate the network.

Proposal 3:

- Update the descriptions of 29-3 as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3-1 | PDCCH monitoring adaptation within an active BWP | Support of PDCCH monitoring adaptation behaviour 1/1A |  |
| 29. NR_UE_pow_sav_enh | 29-3-2 | PDCCH monitoring adaptation within an active BWP | Support of PDCCH monitoring adaptation behaviour 2/2A |  |
| 29. NR_UE_pow_sav_enh | 29-3-3 | PDCCH monitoring adaptation within an active BWP | Support of PDCCH monitoring adaptation behaviour [2B] | 29-3-2 |
| 29. NR_UE_pow_sav_enh | [29-3-4] | [PDCCH monitoring adaptation within an active BWP] | [Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A] | [29-3-1, 29-3-2] |

- Conclusions

This contribution addresses our proposals for Rel-17 power saving UE features.,

Proposals:

- Support UE subgroup indication is either separated from 29-1 or as a RAN2 feature to be discussed in RAN2.

- Update the descriptions of feature 29 as follows,

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1-1 | Paging enhancement | 1. Support paging early indication 1. Support of configured window for detection of DCI format XXX with CRC scrambled with YYY for paging early indication 2. Support of Behv-A if UE does not detect PEI for all monitored PEI occasion(s) for the PO 2. Support UE subgroup indication |  |
| 29. NR_UE_pow_sav_enh | 29-1-2 | Paging enhancement | Support UE subgroup indication |  |
| 29. NR_UE_pow_sav_enh | 29-2-1 | TRS resources for idle/inactive UEs | Support reading TRS configuration and TRS availability from SIB for idle/inactive TRS |  |
| 29. NR_UE_pow_sav_enh | 29-2-2 | TRS resources for idle/inactive UEs | Support receiving L1 indication for TRS availability for idle/inactive TRS | 29-2-1 |
| 29. NR_UE_pow_sav_enh | 29-3-1 | PDCCH monitoring adaptation within an active BWP | Support of PDCCH monitoring adaptation behaviour 1/1A |  |
| 29. NR_UE_pow_sav_enh | 29-3-2 | PDCCH monitoring adaptation within an active BWP | Support of PDCCH monitoring adaptation behaviour 2/2A |  |
| 29. NR_UE_pow_sav_enh | 29-3-3 | PDCCH monitoring adaptation within an active BWP | Support of PDCCH monitoring adaptation behaviour [2B] | 29-3-2 |
| 29. NR_UE_pow_sav_enh | [29-3-4] | [PDCCH monitoring adaptation within an active BWP] | [Support of PDCCH monitoring adaptation behaviour 1/1A/2/2A] | [29-3-1, 29-3-2] |

# Reference

- R1-2108679, 	Preliminary RAN1 UE features list for Rel-17 NR, 	AT&T, NTT DOCOMO, INC.

- RP-212611,		Moderator’s summary for discussion [93e-18-PowSav-WI]: Final Round, 	Moderator (CMCC)

- R1-2108988	,	Discussion on DCI-based power saving adaptation in connected mode, 		vivo