3GPP TSG RAN WG1 #106bis		R1-2109022

e-Meeting, October 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE features for MBS

Agenda Item:	8.17.12

Document for:	Discussion and Decision

- Introduction

Preliminary Rel-17 NR UE features are summarized in [1]. In this contribution, our view on MBS part is provided based on [1].

- Discussion

For broadcast, it can be received by UEs in RRC_IDLE/INACTIVE and RRC_CONNECTED states. For simultaneous reception of unicast PDSCH and group-common PDSCH, the following agreements have been achieved in previous meetings.

Agreements: (RAN1#102) For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. - FFS: TDM or SDM in a slot. Agreements: (RAN1#103) Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. Agreements: (RAN1#103) Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. - Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot - Case 2: support TDM among multiple group-common PDSCHs in a slot - Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 5: support FDM among multiple group-common PDSCHs in a slot - FFS: maximum number of PDSCHs in a slot simultaneous received per UE Agreement: (RAN1#104bis) At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L

We think the group-common PDSCH in the above agreements also include the group-common PDSCH for broadcast for RRC_CONNECTED UEs. Therefore, besides the support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots, support of intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s) (i.e, case 1 and case 3), or among multiple group-common PDSCHs (i.e., case 2) in a slot, and FDM between unicast PDSCH and group-common PDSCH should also be based on UE capability.

In addition, for broadcast, multiple G-RNTIs can be used for different services, whether additional UE capability is needed for the reception of group-common PDSCHs with different G-RNTIs should be discussed.

Proposal 1: For broadcast, supporting intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s), or among multiple group-common PDSCHs in a slot, and supporting FDM between unicast PDSCH and group-common PDSCH should be included in the UE feature table.

Proposal 2: Capability on number of G-RNTIs for broadcast should be discussed.

For multicast, we have the following considerations:

- One basic principle is that the capability for group-common PDSCH for multicast reception and HARQ-ACK feedback should be decoupled. That is, for UE supports reception of group-common PDSCH for multicast, it does not mean the UE has to support HARQ-ACK feedback for group-common PDSCH for multicast. In addition, similar as that in NR Rel-15, support of type 1 and support of type 2 codebook should be different UE capabilities.

- Capability on number of G-RNTI for multicast should also be considered, considering it will introduce additional complexity on PDSCH descrambling, DAI counting, HARQ-ACK codebook construction, etc.

- For SPS group-common PDSCH for multicast, the support of SPS group-common PDSCH for multicast and the support of HARQ-ACK feedback SPS group-common PDSCH for multicast should be decoupled. In addition, the support of slot-level repetition for SPS group-common PDSCH should also be decoupled from the support of SPS group-common PDSCH for multicast.

- For the retransmission for multicast, whether to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast is being discussed. If some mechanisms are supported to make the differentiation between PTP (re)transmission for unicast and PTP retransmission for multicast. Support of PTP retransmission for multicast should be based on UE capability.

Proposal 3: Supporting HARQ-ACK feedback for group-common PDSCH for multicast should be a different capability from supporting group-common PDSCH reception for multicast.

Proposal 4: Capability on number of G-RNTIs for multicast should be included in the UE features for NR MBS.

Proposal 5: Supporting slot-level repetition for SPS group-common PDSCH should be a different capability form supporting SPS group-common PDSCH configuration for multicast.

Proposal 6: UE features for MBS are suggested as following.

| Features | Index | Feature group | Components | Prerequisite feature groups | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support MCCH change notification indication via DCI. |  | Optional without capability signalling |
| 33. NR_MBS | 33-1-2 | Broadcast | Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. | 33-1-1 | Optional without capability signalling |
| 33. NR_MBS | 33-1-3 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1-1 | Optional with capability signalling |
| 33. NR_MBS | 33-1-4 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for broadcast | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1-1 | Optional with capability signalling |
| 33. NR_MBS | 33-1-5 | Multiple G-RNTIs for broadcast | Capability on number of G-RNTIs for broadcast - FFS details. | 33-1-1 | Optional without capability signalling |
| 33. NR_MBS | 33-2-1 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support PTM retransmission for multicast. - FFS: Support PTP retransmission for multicast. |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-2 | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for broadcast - FFS details. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | ACK/NACK based HARQ-ACK feedback for multicast | Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-2 | TDM-ed Type-1 HARQ-ACK codebook for multicast | Support TDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2-1, 33-4-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-3 | FDM-ed Type-1 HARQ-ACK codebook for multicast | Support FDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-3-2, 33-4-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-4 | Type 2 HARQ-ACK codebook for multicast | Support Type-2 HARQ-ACK codebook for multicast. | 33-2-1, 33-4-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-5 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 33-2-1, 33-4-1 | Optional with capability signalling |

- Conclusion

In this contribution, we focus on NR MBS UE feature discussion, and have the following proposals:

Proposal 1: For broadcast, supporting intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s), or among multiple group-common PDSCHs in a slot, and supporting FDM between unicast PDSCH and group-common PDSCH should be included in the UE feature table.

Proposal 2: Capability on number of G-RNTIs for broadcast should be discussed.

Proposal 3: Supporting HARQ-ACK feedback for group-common PDSCH for multicast should be a different capability from supporting group-common PDSCH reception for multicast.

Proposal 4: Capability on number of G-RNTIs for multicast should be included in the UE features for NR MBS.

Proposal 5: Supporting slot-level repetition for SPS group-common PDSCH should be a different capability form supporting SPS group-common PDSCH configuration for multicast.

Proposal 6: UE features for MBS are suggested as following.

| Features | Index | Feature group | Components | Prerequisite feature groups | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support MCCH change notification indication via DCI. |  | Optional without capability signalling |
| 33. NR_MBS | 33-1-2 | Broadcast | Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. | 33-1-1 | Optional without capability signalling |
| 33. NR_MBS | 33-1-3 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1-1 | Optional with capability signalling |
| 33. NR_MBS | 33-1-4 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH for broadcast | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1-1 | Optional with capability signalling |
| 33. NR_MBS | 33-1-5 | Multiple G-RNTIs for broadcast | Capability on number of G-RNTIs for broadcast - FFS details. | 33-1-1 | Optional without capability signalling |
| 33. NR_MBS | 33-2-1 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support PTM retransmission for multicast. - FFS: Support PTP retransmission for multicast. |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-2 | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for broadcast - FFS details. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | ACK/NACK based HARQ-ACK feedback for multicast | Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-2 | TDM-ed Type-1 HARQ-ACK codebook for multicast | Support TDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2-1, 33-4-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-3 | FDM-ed Type-1 HARQ-ACK codebook for multicast | Support FDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-3-2, 33-4-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-4 | Type 2 HARQ-ACK codebook for multicast | Support Type-2 HARQ-ACK codebook for multicast. | 33-2-1, 33-4-1 | Optional with capability signalling |
| 33. NR_MBS | 33-4-5 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. | 33-2-1 | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 33-2-1, 33-4-1 | Optional with capability signalling |

- References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.)