3GPP TSG RAN WG1 #106bis-e                                                                        R1-2109200

e-Meeting, October 11th – 19th, 2021

Source:              CATT

Title:                   Evaluation results of XR performance

Agenda Item:     8.14.1

Document for:   Discussion

Introduction

In RAN1#104b-e meeting [1], most evaluation methodology and simulation assumption have been agreed. In RAN1#105-e meeting [2], the traffic parameters for XR base evaluation have been agreed. The traffic model and simulation assumptions for the simulation results can be found in Appendix. This paper provides the performance results of XR based on the agreed XR traffic model and evaluation methodology. In addition, two enhanced schemes for XR capacity enhancement and UE power saving are proposed with evaluation results.

Performance results

KPI and latency model

The XR performance evaluation includes the requirements of latency, reliability and UE power consumption. The Key Performance Indexes (KPIs) are the index of the system performance for XR services. In RAN1#104b e-meeting, it was agreed that:

Agreement:

In case of single stream per UE in DL, a UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within a given air interface PDB.

- The baseline X value is 99.

- Other values of X can be optionally evaluated, e.g., X < = 95, X=99.9.

- Additional combinations of (X, PDB) values can be optionally evaluated, e.g.,

  - (99, 7), (95, 13) for VR/AR

  - (99, 12), (95, 18) for CG

- FFS: Different values for I-frame and P-frame if evaluation of them is agreed.

In this contribution, system capacity is defined as the max number of UE per cell when 90% UEs satisfy requirements. A UE is declared a satisfied UE if more than 99% of the packets are successfully transmitted within given PDB. DL latency considers the processing time, DL TTI arrival time, time for ACK/NACK feedback and retransmission. Detailed DL latency model is described in Figure 1. In Figure 1, the black line with arrow means DL first transmission process and the red line with arrow means a retransmission process. The UE is assumed to have PDSCH processing capability 1.

Figure 1: DL latency model

UL initial transmission latency considers the processing time, SR and DCI transmission time for dynamic schedule, and DL/UL TTI arrival time, which is shown in Figure 2. For UL retransmission, gNB sends DCI to schedule UE directly without SR transmission.

Figure 2: UL latency model

Simulation results

In the section, we provide evaluation results of XR and CG service based on the simulation assumptions described in Appendix. The evaluation results include the system capacity and UE power consumption. Impact of various parameters on capacity and power is evaluated as well. In the simulation, the packets exceeding the delay is still scheduled and delivered to the UE and gNB for DL and UL.

Baseline performance results

We simulate the performance of XR and CG service in Indoor Hotspot and Dense Urban scenarios, including capacity and UE power consumption.

We provide the system capacity performance results based on the following parameters in Table 1, which are shown in Figure 3, 4 and 5.

Table 1: Baseline simulation parameters

|  |  | Data rate [Mbps] | Packet arrival rate [fps] | PDB [ms] |
| --- | --- | --- | --- | --- |
| DL | AR/VR | 30 | 60 | 10 |
|  | CG | 30 | 60 | 15 |
| UL | VR/CG: Pose/control | 0.2 | 250 | 10 |
|  | AR: Option 1 (single stream model) | 10 | 60 | 30 |

Figures 3, 4 and 5 show the percentage of UEs that 99% of the packets successfully transmitted within PDB for DL and UL XR traffic. The percentage of satisfied XR UEs decreases when the system load increases.

Figure 3: The percentage of satisfied UE with DL AR/VR/CG service under different system loads

Figure 4: The percentage of satisfied UE with UL pose/control service under different system loads

Figure 5: The percentage of satisfied UE with UL AR service under different system loads

From the results shown in Figures 3, 4 and 5, it could be observed that the system capacity without any power saving scheme is 12 and 15 for DL AR/VR service and CG service of 30Mbps data rate for indoor hotspot scenario, respectively, The system capacity without any additional power saving scheme is at least 12 and 6 for UL pose/control service of 0.2Mbps data rate and UL AR service of 10 Mbps data rate, respectively. In Dense Urban scenario, the system capacity without any power saving scheme is 8 and 10 for DL AR/VR service and CG service of 30Mbps data rate, respectively.

Observation 1: In Indoor Hotspot scenario, the system capacity without any power saving scheme is 12 and 15 for DL AR/VR service and CG service of 30Mbps data rate, respectively.

Observation 2: In Indoor Hotspot scenario, the system capacity without any power saving scheme is at least 12 and 6 for UL pose/control service of 0.2Mbps data rate and UL AR service of 10 Mbps data rate, respectively.

Observation 3: In Dense Urban scenario, the system capacity without any power saving scheme is 8 and 10 for DL AR/VR service and CG service of 30Mbps data rate, respectively.

Parameters impact on capacity

Impact of date rate on capacity

Figure 6: UE satisfaction rate with different data rate

Figure 6 shows the UE satisfaction rate with different data rate of 30Mbps, 45Mbps and 60Mbps. The XR capacity decreases as the data rate increases.

Observation 4: The XR capacity decreases as the data rate increases.

Impact of PDB/PER on capacity

Figure 7: UE satisfaction rate with different PER/PDB requirements

Figure 7 shows that UE satisfaction rate with different PER/PDB requirements. XR capacity increases as PER and PDB requirement relaxes.

Observation 5: XR capacity increases as PER and PDB requirement relaxes.

Parameters impact on UE power consumption

In the section, UE power consumption result for DL AR/VR service of 30Mbps data rate in Indoor Hotspot is shown. Figure 8 shows the CDF of power consumption of different UE power states when system load is high, i.e., 12 UEs/cell.

Figure 8: CDF of power consumption of different UE power states without any power saving scheme

Impact of system load on UE power consumption

Figure 9 describes the proportion of power consumption of different UE power states, including PDCCH_only, PDCCH+PDSCH and sleep, under different system load without any power saving scheme. The ratio of power consumption in Figure 9 is calculated based on the average value of all UEs. With the increase of system load, the proportion of PDCCH monitoring power consumption decreases, while that of PDSCH reception power consumption increases.

Figure 9: Proportion of power consumption of different UE power states under different system load without any power saving scheme

Observation 6: With the increase of system load, the proportion of PDCCH monitoring power consumption decreases, while that of PDSCH reception power consumption increases.

Impact of traffic periodicity on UE power consumption

Figure 10 shows that CDF of UE power consumption with different frame rate of 60fps, 90fps and 120fps. Higher frame rate, e.g., shorter traffic periodicity, causes more UE power consumption under same data rate. This is because the traffic inter-arrival time is short, resulting that the UE needs to wake up more frequently and cannot fall into deep sleep state or even light sleep state.

Figure 10: CDF of UE power consumption with different FPS

Observation 7: Higher frame rate, e.g., shorter traffic periodicity, causes more UE power consumption under same data rate.

Impact of date rate on UE power consumption

Figure 11: CDF of UE power consumption with different data rate

Figure 11 shows that UE power consumption with different data rate. Higher data rate requires higher UE power consumption. Obviously, higher data rate means that UE takes longer time to transmit packets, which increases the UE total power consumption.

Observation 8: Higher data rate requires higher UE power consumption.

Limitation of C-DRX for XR performance

In the section, we evaluate the performance of C-DRX for XR. Based on the performance results, the limitation of C-DRX for XR performance is analysed in the section.

Table 2 shows the detailed configuration parameter of the evaluated C-DRX.

Table 2: C-DRX configuration parameters

| DRX configuration | DRX Cycle | OnDurationTimer | InactivityTimer |
| --- | --- | --- | --- |
| C-DRX 1 | 16ms | 12ms | 4ms |
| C-DRX 2 | 6ms | 4ms | 2ms |
| C-DRX 3 | 16ms | 8ms | 4ms |

Table 3 collects the satisfied UE of different DRX configuration with same system load. The DRX configuration reduces the percentage of satisfied UEs for XR service. The satisfied UE ratio of XR with the DRX configuration is reduced by 4.86%~41.66% compared to the baseline without any power saving scheme. C-DRX 1 and C-DRX 2 have 4.86% and 6.94% capacity degradation which less than C-DRX 3 due to the larger duty cycle. When C-DRX with small duty cycle is configured for UE power saving, the system capacity is drastically reduced due to the mismatch of the CDRX configuration with the XR traffic generation. In particular, the jitter delay is too small to reflect the variation of network transport and out-of-order delivery.

Table 3: Satisfied UE of different DRX configuration with same system load.

| Configuration | #satisfied UEs per cell | #UEs per cell (N) | Percentage of satisfied UE |
| --- | --- | --- | --- |
| Baseline | 11.5 | 12 | 95.83% |
| C-DRX 1 (16,12,4) | 10.9 | 12 | 90.97% |
| C-DRX 2 (6,4,2) | 10.7 | 12 | 88.89% |
| C-DRX 3 (16,8,4) | 6.48 | 12 | 54.17% |

Observation 9: For XR service in Indoor Hotspot scenario, system capacity with the DRX configuration could be reduced by 4.86%~41.66% compared to system capacity without any power saving scheme.

According to the definition of XR capacity KPI, XR capacity is affected by reliability and latency. Table 4 shows the percentage of UEs not meeting the reliability and latency requirements with different case. The percentage of UEs not meeting the reliability requirement is shown at the row of “Reliability<99%” and the percentage of UEs meeting the reliability requirement but not meeting latency requirement is shown at the row of “Reliability>99%&Latency>PDB”.

Table 4: The percentage of UEs that not meeting the reliability and latency requirement with different case

| Configuration | Duty cycle | Metric | Percentage of satisfied UE |
| --- | --- | --- | --- |
| Baseline | -- | Reliability<99% | 2.08% |
|  |  | Reliability>99%&Latency>PDB | 2.08% |
| C-DRX 1 (16,12,4) | 0.75 | Reliability<99% | 4.17% |
|  |  | Reliability>99%&Latency>PDB | 4.86% |
| C-DRX 2 (6,4,2) | 0.67 | Reliability<99% | 4.86% |
|  |  | Reliability>99%&Latency>PDB | 6.25% |
| C-DRX 3 (16,8,4) | 0.5 | Reliability<99% | 5.56% |
|  |  | Reliability>99%&Latency>PDB | 40.28% |

From Table 4, it is observed that the percentage of UE not meeting the latency requirement increases with larger duty cycle. The DRX cycle does match exactly with the traffic generation at source codec with small delay jitter. The long DRX off time would reduce the available time for UE to be scheduled by gNB. This has the results of drastic decrease in system capacity for delay-sensitive XR services.

Observation 10: DRX with larger duty cycle achieves less capacity loss when DRX cycle matches with XR traffic generation.

Figure 12 shows the CDF of UE power consumption of different UE power states under different system load with C-DRX(16,12,4) and C-DRX(6,4,2) configuration when system load equals capacity, i.e., 12 UEs per cell.

Figure 12: CDF of power consumption of different UE power states with C-DRX1 and C-DRX2 configuration

Table 5 collects the power saving performance results of C-DRX for 30Mbps XR service when there are 12 UEs per cell.

Table 5: Evaluation of UE power saving schemes for Indoor Hotspot

| Power Saving Scheme | Considered UE set | Power Saving Gain (PSG) compared to Baseline |  |  |  | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean PS gain | PS gain of 5%-tile UE in PSG CDF | PS gain of 50%-tile UE in PSG CDF | PS gain of 95%-tile UE in PSG CDF |  |
| Baseline | - | - | - | - | - | 11.5 / 12 |
| C-DRX 1 (16,12,4) | All UEs | 2.39% | 1.49% | 2.86% | 3.91% | 10.9 / 12 |
|  | Satisfied UEs | 2.4% | 1.5% | 2.87% | 3.91% |  |
| C-DRX 2 (6,4,2) | All UEs | 6.14% | 5.22% | 6.61% | 7.72% | 10.7 / 12 |
|  | Satisfied UEs | 6.18% | 5.22% | 6.66% | 7.76% |  |
| C-DRX 3 (16,8,4) | All UEs | 12.57% | 9.95% | 12.18% | 14.49% | 6.5/12 |
|  | Satisfied UEs | 12.75% | 9.32% | 13.18% | 14.51% |  |

The UE power saving gain with C-DRX(16,12,4) and C-DRX(6,4,2) is only about 2.39%~2.4% and 6.14%~6.18% with lightly capacity loss, where the power saving gain is from some reduced PDCCH monitoring. In addition, the power consumption statistics of only satisfied UEs can provide little additional power saving gain, compared to that of all UEs, because unsatisfied UEs have poor channel condition and need more transmission opportunity.

To avoid the capacity loss, DRX must be configured with short off-time to give UE more time available. However, the UE with a conservative DRX configuration has little chance to sleep, and during long On-time it needs to monitor PDCCH even if it have no data reception. For NR supporting XR services, existing C-DRX is difficult to have balance between the capacity enhancement and UE power saving performance.

Observation 11: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 2.39% and 2.4% mean power saving gain are achieved by C-DRX(16,12,4) without capacity loss for all UEs and only satisfied UEs, respectively.

Observation 12: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 6.14% and 6.18% mean power saving gain are achieved by C-DRX(6,4,2) with lightly capacity loss for all UEs and only satisfied UEs, respectively.

Observation 13: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 12.57% and 12.75% mean power saving gain are achieved by C-DRX(16,8,4) with severe capacity loss for all UEs and only satisfied UEs, respectively.

Observation 14: For XR services, existing C-DRX is difficult to have balance between the capacity and UE power saving performance.

XR enhancement for capacity and power

XR-dedicated PDCCH monitoring window

Due to the quasi-periodicity nature of XR services, the existing DRX cycle configuration cannot be fully aligned with XR traffic arrival, which results in unacceptable delay during the waiting time of scheduling. Existing Rel-15/16 DRX configuration, e.g., 64ms cycle, would give little or no time left of the Packet Delay Budget for gNB to schedule the XR packet. The DRX imposes additional delay of periodic arrival of XR traffic and decrease the number of XR packets meeting the packet delay budget. This will have the results of the XR capacity reduction. Configuring DRX cycle that matches the traffic generation at source Codec and a large duty cycle can reduce capacity loss to some extent. Since DRX is the power saving technique mostly for robust traffic arrival with delay tolerant traffic, DRX configuration might not provide power saving for periodic traffic if the DRX cycle does not match with the periodicity of the periodic traffic, such as XR traffic. The DL semi-persistent scheduling (SPS) and UL configured grant (CG) schemes are introduced for periodic service with stringent latency requirements. DL SPS and UL CG had preconfigured periodic resource for the DL and UL periodic traffic, so they are ideal candidate for periodic XR services. In NR, UE needs to decode the PDSCH at the configured SPS resource in DRX ON and OFF if DRX is configured. The DL SPS would not have the additional delay caused by DRX, which XR packets are on hold and scheduled for transmission only at DRX ON.  However, the periodic reserved resource of DL SPS and UL CG might not fit well for XR service with large variation of packet size and inter-arrival time caused by delay jitter during the network transport. DL SPS needs to be enhanced with additional resource allocation mechanism for XR service at both DRX ON and DRX OFF when the XR packets are bigger than the reserved resource. Similarly, UL CG needs to be enhanced with additional resource allocation for the adaptation of variable XR packet size.

A XR-dedicated data transmission scheme with dynamic scheduling is proposed, which is called XR-dedicated PDCCH monitoring window (XR-PMW). As shown in Figure 13, UE is configured with fixed PDCCH monitoring cycle and monitoring window disassociated with DRX. The fixed PDCCH monitoring cycle could be customized for the periodic traffic generation of XR service. The window size of PDCCH monitoring at each cycle could be dynamically adapted to the delay variation of packet arrival caused by network jitter. UE monitors only XR scheduled PDCCH during a preconfigured PDCCH monitoring window.

Figure 13: XR-dedicated PDCCH monitoring window scheme

The performance of proposed XR-dedicated PDCCH monitoring window scheme was evaluated. The evaluation parameters are shown in Table 6, and the evaluation results are shown in Table 7.

Table 6: Parameters for XR-PMW performance evaluation

| Parameters | value |  |
| --- | --- | --- |
| Period | 8ms | 16ms |
| The length of window | 6ms | 12ms |

Table 7: Evaluation results of XR-PMW

| Configuration | Considered UE set | Mean Power Saving Gain (PSG) compared to Always-on | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- |
| C-DRX(16,8,4) | All UEs | 12.57% | 6.5 / 12 |
|  | Satisfied UEs | 12.75% |  |
| XR-PMW (8,6) | All UEs | 3.87% | 10.8 / 12 |
|  | Satisfied UEs | 4.13% |  |
| XR-PMW (16,12) | All UEs | 3.85% | 10.4 / 12 |
|  | Satisfied UEs | 4.14% |  |

According to Table 7, the XR-PMW achieves 66.15% capacity gain compared to C-DRX(16,8,4). And since it only works for XR service, the data transmission of other service will not be affected.

Observation 15: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 66.15% capacity gain can be achieved by XR-dedicated PDCCH monitoring window scheme compared with C-DRX.

Observation 16: The XR-dedicated PDCCH monitoring window scheme does not affect data transmission of other service.

The power saving schemes, e.g., PDCCH skipping and go-to-sleep, are also applied with XR-dedicated PDCCH monitoring window for saving energy. The PDCCH monitoring cycle is fully matched with XR packet generation, i.e., length of the cycle is 16.67ms. The PDCCH monitoring window is set to be long enough to cover the variance range of jitter. In each cycle, UE performs PDCCH skipping with certain duration until the packet arrival at the gNB. UE is indicated to sleep state once it completes the reception of the PDSCH of XR packet and reports all ACKs for PDSCH. Table 8 shows the evaluation results of XR-PMW with PDCCH skipping and go-to-sleep.

Table 8: Evaluation results of XR-PMW with PDCCH skipping and go-to-sleep

| Configuration | Considered UE set | Mean Power Saving Gain (PSG) compared to Always-on | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- |
| Always-on | - | - | 11.5 / 12 |
| XR-PMW with go-to-sleep | All UEs | 24.01% | 10.8 / 12 |
|  | Satisfied UEs | 24.39% |  |
| XR-PMW with PDCCH skipping and go-to-sleep | All UEs | 29.44% | 10.7 / 12 |
|  | Satisfied UEs | 29.92% |  |

Based on the results in table 8, XR-PMW with PDCCH skipping and go-to-sleep can achieves pretty high power saving gain, i.e., 24.01%~29.92%. XR-PMW is designed flexibly for XR traffic, so the PDCCH skipping and go-to-sleep mechanism can effectively reduce the unnecessary PDCCH monitoring without degradation of capacity.

Observation 17: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 24.01% and 24.39% mean power saving gain are achieved by XR-dedicated PDCCH monitoring window scheme with go-to-sleep without capacity loss for all UEs and only satisfied UEs, respectively.

Observation 18: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 29.44% and 29.92% mean power saving gain are achieved by XR-dedicated PDCCH monitoring window scheme with PDCCH skipping and go-to-sleep without capacity loss for all UEs and only satisfied UEs, respectively.

Capacity and power enhancement using additional packet delay budget

Packet delay budget has direct impact on XR capacity. According to section 2.2.2, reducing the PDB requirements can boost the UE satisfaction rate within the range of guaranteed reliability. Therefore, for XR service, additional packet delay budget (APDB) increases system capacity. We evaluated the UE satisfaction rate of 45Mbps@60FPS AR/VR DL service with different APDB in Indoor hotspot scenario. The evaluation results are shown in Figure 14.

Figure 14: UE satisfaction rate of 45Mbps@60FPS AR/VR DL service with different APDB in InH.

From Figure 14, we can observe that the UE satisfaction rate and capacity increase with the increase of APDB. When the UE has 5ms APDB, the capacity increased 2 UEs per cell, the UE satisfaction rate increased 3.13%~39.58%. Obviously, the APDB has positive impact on XR capacity.

Observation 19: Additional packet delay budget can improve XR capacity.

gNB scheduling awareness of UE playout buffer

XR capacity will be further improved significantly if the APDB for each UE considered by gNB for scheduling optimization. UE playout buffer is a good example that illustrates the capacity enhancement using additional delay budget. XR traffic arrival is not strictly periodic due to the delay jitter caused by the network transport of XR packets. The XR receiver at the UE will have playout buffers to battle the delay jitter and ensure the quality and continuity of the received XR video/audio signals. The playout buffer is to store a few XR video frames at the application layer before it plays out on the screen. The size of the playout buffer depends on the network delay jitter. If the size of the playout buffer is fed back to the gNB scheduler, gNB could have APDB. APDB can give gNB more time to schedule UE within the delay budget requirements of the XR service and more likely to successfully transmit packets. gNB knowing the size of playout buffer can preferentially schedule UE with packet delay close to deadline and better channel conditions. Besides, gNB also has more retransmission chance due to additional packet delay budget. In our simulation, the UE playout buffer can store 2~4 video frames with 60fps frame rate, and gNB preferentially schedule UE whose packet delay is closed to deadline. In particular, packet must be delivered successfully to UE before the video frame at playout buffer is ran out, otherwise the packet is loss. The retransmission times depends on the PDB only. Figure 15 shows the 30Mbps@60fps XR capacity w/ different size of playout buffer and w/o playout buffer.

Figure 15: UE satisfaction rate w/ and w/o playout buffer

It is observed from Figure 15, XR capacity increases significantly with gNB scheduling awareness of UE playout buffer. XR capacity for Indoor 30Mbps@60fps DL traffic increases from 12 UEs per cell to 20UEs per cell if UE playout buffer can store more than 3 XR video frames. The gNB scheduling awareness of UE playout buffer achieves 67% capacity gain for XR.

Observation 20: The gNB scheduling awareness of UE playout buffer achieves 67% capacity gain for Indoor 30Mbps@60fps DL traffic, if UE playout buffer can store more than 3 XR video frames.

In addition to improving XR capacity, UE playout buffer can also be used for enhancing the performance of some power saving scheme. Table 9 shows the performance of some power saving cases for 30Mbps AR/VR service with gNB scheduling awareness of UE playout buffer.

The details for these cases are:

- Baseline: No power saving scheme is applied, i.e., UE is always in Active Time.

  - C-DRX(16,8,4) with go-to-sleep: UE is configured with C-DRX (16,8,4). UE transforms to sleep state once the all PDSCH of frame is received successfully and all ACKs for PDSCH are reported.

  - C-DRX(16,8,4) with PDCCH skipping and go-to-sleep: UE is configured with C-DRX (16,8,4). UE performs PDCCH skipping when On Duration starts until UE starts receiving PDSCH. UE transforms to sleep state once the all PDSCH of frame is received successfully and all ACKs for PDSCH are reported.

- PDCCH skipping: UE performs PDCCH skipping with certain duration. When the packet arrivals, gNB doesn’t send PDCCH skipping command to the UE until the all PDSCH of frame is received successfully and all ACKs for PDSCH are reported.

Table 9: Evaluation of power saving cases with gNB scheduling awareness of UE playout buffer

| Power saving cases with UE playout buffer | Considered UE set | Mean Power Saving Gain (PSG) compared to Always-on | #satisfied UEs per cell / #UEs per cell |
| --- | --- | --- | --- |
| Baseline | - | - | 11.5 / 12 |
| C-DRX(16,8,4) with go-to-sleep | All UEs | 26.43% | 11.3 / 12 |
|  | Satisfied UEs | 26.78% |  |
| C-DRX(16,8,4) with PDCCH skipping and go-to-sleep | All UEs | 28.51% | 11.2 / 12 |
|  | Satisfied UEs | 28.71% |  |
| PDCCH skipping | All UEs | 30.26% | 11 / 12 |
|  | Satisfied UEs | 30.36% |  |

Based on the results above, with UE playout buffer, the significant UE power saving gain, i.e., 26.43%~30.36%, can be achieved without capacity loss. With UE playout buffer, UE is allowed to easily receive packet with more relaxed packet delay requirements. For power saving schemes that may introduce extra delay, i.e., C-DRX and PDCCH skipping, the configuration can be more radical, e.g., the longer duration of PDCCH skipping and shorter OnDuration of CDRX. Due to the playout buffer provides additional packet delay budget, the UE has more time to sleep if the rest of delay budget is enough.

Observation 21: With gNB scheduling awareness of UE playout buffer, 26.43% and 26.78% mean power saving gain are achieved by C-DRX with go-to-sleep without capacity loss for DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell for all UEs and only satisfied UEs, respectively.

Observation 22: With gNB scheduling awareness of UE playout buffer, 28.51% and 28.71% mean power saving gain are achieved by C-DRX with PDCCH skipping and go-to-sleep without capacity loss for DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell for all UEs and only satisfied UEs, respectively.

Observation 23: With gNB scheduling awareness of UE playout buffer, 30.26% and 30.36% mean power saving gain are achieved by PDCCH skipping without capacity loss for DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell for all UEs and only satisfied UEs, respectively.

Conclusion

In this contribution, the performance evaluation results are presented based on XR and CG traffic model and simulation assumption. Based on these results, we have the following observations:

Observation 1: In Indoor Hotspot scenario, the system capacity without any power saving scheme is 12 and 15 for DL AR/VR service and CG service of 30Mbps data rate, respectively.

Observation 2: In Indoor Hotspot scenario, the system capacity without any power saving scheme is at least 12 and 6 for UL pose/control service of 0.2Mbps data rate and UL AR service of 10 Mbps data rate, respectively.

Observation 3: In Dense Urban scenario, the system capacity without any power saving scheme is 8 and 10 for DL AR/VR service and CG service of 30Mbps data rate, respectively.

Observation 4: The XR capacity decreases as the data rate increases.

Observation 5: XR capacity increases as PER and PDB requirement relaxes.

Observation 6: With the increase of system load, the proportion of PDCCH monitoring power consumption decreases, while that of PDSCH reception power consumption increases.

Observation 7: Higher frame rate, e.g., shorter traffic periodicity, causes more UE power consumption under same data rate.

Observation 8: Higher data rate requires higher UE power consumption.

Observation 9: For XR service in Indoor Hotspot scenario, system capacity with the DRX configuration could be reduced by 4.86%~41.66% compared to system capacity without any power saving scheme.

Observation 10: DRX with larger duty cycle achieves less capacity loss when DRX cycle matches with XR traffic generation.

Observation 11: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 2.39% and 2.4% mean power saving gain are achieved by C-DRX(16,12,4) without capacity loss for all UEs and only satisfied UEs, respectively.

Observation 12: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 6.14% and 6.18% mean power saving gain are achieved by C-DRX(6,4,2) with lightly capacity loss for all UEs and only satisfied UEs, respectively.

Observation 13: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 12.57% and 12.75% mean power saving gain are achieved by C-DRX(16,8,4) with severe capacity loss for all UEs and only satisfied UEs, respectively.

Observation 14: For XR services, existing C-DRX is difficult to have balance between the capacity enhancement and UE power saving performance.

Observation 15: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 66.15% capacity gain can be achieved by XR-dedicated PDCCH monitoring window scheme compared with C-DRX.

Observation 16: The XR-dedicated PDCCH monitoring window scheme does not affect data transmission of other service.

Observation 17: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 24.01% and 24.39% mean power saving gain are achieved by XR-dedicated PDCCH monitoring window scheme with go-to-sleep without capacity loss for all UEs and only satisfied UEs, respectively.

Observation 18: For DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell, 29.44% and 29.92% mean power saving gain are achieved by XR-dedicated PDCCH monitoring window scheme with PDCCH skipping and go-to-sleep without capacity loss for all UEs and only satisfied UEs, respectively.

Observation 19: Additional packet delay budget can improve XR capacity.

Observation 20: The gNB scheduling awareness of UE playout buffer achieves 67% capacity gain for Indoor 30Mbps@60fps DL traffic, if UE playout buffer can store more than 3 XR video frames.

Observation 21: With gNB scheduling awareness of UE playout buffer, 26.43% and 26.78% mean power saving gain are achieved by C-DRX with go-to-sleep without capacity loss for DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell for all UEs and only satisfied UEs, respectively.

Observation 22: With gNB scheduling awareness of UE playout buffer, 28.51% and 28.71% mean power saving gain are achieved by C-DRX with PDCCH skipping and go-to-sleep without capacity loss for DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell for all UEs and only satisfied UEs, respectively.

Observation 23: With gNB scheduling awareness of UE playout buffer, 30.26% and 30.36% mean power saving gain are achieved by PDCCH skipping without capacity loss for DL 30Mbps XR service in indoor Hotspot with 12 UEs per cell for all UEs and only satisfied UEs, respectively.

References

- RAN1#104b-e, Chairman Notes.

- RAN1#105-e, Chairman Notes.

- Appendix

- A1 Traffic model

Traffic are generated from XR/CG sever/device periodically and are transported to RAN with propagation delay and jitter. Each packet size is generated based on the Truncated Gaussian distribution. The additional delay is considered as a variable followed Truncated Gaussian distribution. The detailed traffic parameters for XR/CG evaluations are shown in Table 10. Note that these parameter values are those before the truncation. In SLS, the packet is obtained by capturing the sample from Gaussian distribution within a truncated range.

Table 10: Traffic parameters for XR evaluations

| Gaussian Parameters |  | DL AR/VR/CG |  |  | UL AR | UL pose/control |
| --- | --- | --- | --- | --- | --- | --- |
| Bit rate | --- | 30Mbps | 45Mbps | 60Mbps | 10Mbps | 0.2Mbps |
| FPS | --- | 60fps | 60fps | 60fps | 60fps | 250fps |
| Packet size | Mean packet size | 62500byte | 93800byte | 125000byte | 20800byte | 100byte |
|  | Std. packet size value | 6563byte | 9800byte | 13125byte | 2184byte | 0 |
|  | Min packet size | 31250byte | 46900byte | 62500byte | 10400byte | 100byte |
|  | Max packet size | 93750byte | 140600byte | 187500byte | 31200byte | 100byte |
| Inter-arrival time | Packet generated period | 16.7ms | 16.7ms | 16.7ms | 16.7ms | 4ms |
|  | Mean jitter value | 4 | 4 | 4 | --- |  |
|  | STD. jitter value | 2 | 2 | 2 |  |  |
|  | Max jitter value | 8 | 8 | 8 |  |  |
|  | Min jitter value | 0 | 0 | 0 |  |  |

- A2 Power model

TR 38.840 is reused to assess the UE power consumption evaluation for XR performance evaluation. The power consumption model of FR1 for XR performance evaluation is shown in Table 11 and Table 12.

Table 11: UE power consumption model for XR baseline performance evaluation

| Power state | Relative Power |
| --- | --- |
| Micro Sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |
| PDCCH_only | 100 |
| PDCCH+PDSCH | 300 |

Table 12: UE power consumption during the state transition

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| *Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

- A3 Simulation scenario

In RAN1#103e and RAN1#104e, most simulation parameters for XR/CG performance evaluation were agreed. The simulation assumptions of Indoor Hotspot and Dense Urban are shown in Table 13 and Table 14.

Table 13: Simulation parameters of Indoor Hotspot

| Scenario | Indoor Hotspot |
| --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 |
| Carrier frequency | FR1:4GHz |
| Bandwidth | FR1:100MHz |
| Subcarrier spacing | FR1: 30 kHz |
| BS height | 3m |
| UE height | hUT=1.5 m |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| BS receiver | MMSE-IRC MMSE-IRC |
| UE receiver |  |
| UE speed | 3 km/h |
| Channel estimation | Realistic |
| MCS | Up to 64QAM |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, |
| TX power | gNB: FR1: 24dBm/20MHz; |
| gNB antenna configuration | gNB: - FR1:32TxRU, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| TDD configuration | DDDSU for DL DDDUU for UL |
| MIMO transmission | MU-MIMO for DL SU-MIMO for UL |
| Scheduling | PF, Dynamic scheduled |
| Processing delay | UE PDSCH processing capability 1; DL NACK to retransmission delay: 2ms; UL previous transmission to current: 2ms. |

Table 14: Simulation parameters of Dense Urban

| Scenario | Dense Urban |
| --- | --- |
| Layout | 21 cell with wraparound ISD：200m |
| Carrier frequency | FR1:4GHz |
| Bandwidth | FR1:100MHz |
| Subcarrier spacing | FR1:30kHz |
| BS height | 25m |
| UE height | hUT=1.5 m |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| BS receiver | MMSE-IRC MMSE-IRC |
| UE receiver |  |
| UE speed | 3 km/h |
| Channel estimation | Realistic |
| MCS | Up to 64QAM |
| BS antenna pattern | 3-sector antenna radiation pattern, 8dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, |
| TX power | gNB: FR1:44dBm/20MHz UE: 23dBm |
| gNB antenna configuration | gNB: - FR1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) , (dH, dV) = (0.5λ, 0.5λ) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| TDD configuration | DDDSU for DL DDDUU for UL |
| MIMO transmission | SU-MIMO for DL SU-MIMO for UL |
| Scheduling | PF, Dynamic scheduled |
| Processing delay | UE PDSCH processing capability 1; DL NACK to retransmission delay: 2ms; UL previous transmission to current: 2ms. |