3GPP TSG RAN WG1 #106bis-e		R1-2109566

e-Meeting, October 11th – 19th, 2021

Source: 	MediaTek Inc.

Title:	Discussion on RAN2 LS on broadcast session delivery about MCCH design

Agenda item:	5

Document for:	Discussion & Decision

- Introduction

In RAN2#113bis e-meeting, RAN2 discussed the details of broadcast session delivery about the MCCH design and achieved the following agreements:

- The concept of MCCH transmission window, similar to the one used for LTE SC-PTM, is used for NR MCCH scheduling. The exact parameters to define the window are FFS (discussed in the following proposals). - The MCCH transmission window is defined by MCCH repetition period, MCCH window duration and radio frame/slot offset. - New RNTI is defined for scheduling MCCH. - Common search space is needed for MCCH scheduling. RAN2 should request RAN1 to discuss the details of CSS for MCCH. - R2 assumes PDCCH occasions for MCCH search space are associated with SSBs in a pre-defined manner so that the UE can receive MCCH scheduling on PDCCH occasions according to its detected SSB. - R2 assumes, In case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1. - R2 assumes that If common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331. - Request RAN1 to discuss the details of the configuration of the bandwidth for MCCH reception. - The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary. - The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent. - UE in RRC IDLE/INACTIVE should be able to monitor/read both MCCH channel and SI/Paging without BWP switch. It is up to RAN1 to decide how this is ensured. - It is up to RAN1 to to decide about the RNTI and DCI format used for MCCH change notifications. - FFS whether to support multiple MCCH, e.g. to support different service types. - RAN2 will discuss and down-select from the following two options for the UE to get aware of session stop/modification: - Reading MCCH once per each MCCH modification period when receiving an ongoing broadcast session - DCI used for MCCH notification indicates the change of an ongoing broadcast session - Both idle/inactive UEs and connected mode UEs can receive MBS services transmitted by NR MBS delivery mode 2 (Broadcast service as already agreed, TBD other). The ability for connected mode UEs to receive this may depend on the network provisioning of the service (e.g. which freq), UE connected mode configuration and UE capabilities. - The two-step based approach (i.e. BCCH and MCCH) as adopted by LTE SC-PTM is reused for the transmission of PTM configuration for NR MBS delivery mode 2. - Assume it is possible to reuse LTE SC-PTM mechanism for the CONNECTED UEs to receive the PTM configuration for NR MBS delivery mode 2, i.e. broadcast based manner. - Assume that MCCH change notification mechanism is used to notify the changes of MCCH configuration due to session start for delivery mode 2 of NR MBS (other cases FFS, if any).

Regarding the physical layer design for MCCH, RAN2 send LS [2] as copied following to RAN1 to request RAN1’s detailed discussion in the following meeting.

- Details of Common Search Space design for MCCH channel, e.g. is SS#0 allowed to be configured as a search space for MCCH, is search space other than SS#0 allowed to be configured as a search space for MCCH. - Details of the allowed transmission bandwidth/BWP configurations for MCCH transmission. - Details of the RNTI and DCI design for carrying MCCH change notifications. - NOTE: RAN2 is still discussing some aspects that may have an impact on this issue, e.g. whether or not to support multiple MCCH or whether or not a notification about the modification/stop of an ongoing session is needed, as indicated above. RAN2 will update RAN1 as soon as further agreements are made on these items.

RAN1 discussed the above issue in last meeting and achieved some agreements about the MCCH physical layer design (e.g., search space design). In addition, the RAN2’s assumption were confirmed in [3].

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH. Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, RAN1 confirms the following assumptions made by RAN2 - RAN2 assumes, in case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1. - RAN2 assumes that if common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331.

However, some issues’ discussion were not convergence, it cannot reply RAN2’s LS with a full answer due to limited meeting time unit (e.g., CFR, RNTI and DCI design for MCCH). In this contribution, we will further discuss the remaining issue about MCCH design and refer to the updated RAN2’s LS made by RAN2#114-e meeting [4].

MBS specific SIB is defined to carry MCCH configuration. MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX). L1 parameters that need to be included in MCCH are pending further RAN1 progress and input. Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH. Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any. FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation. At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period. We support single MCCH (in this release) MCCH is mapped to the DL-SCH for NR MBS delivery mode 2.

- Discussion

  - CFR design for MCCH

Regarding the CFR configuration for MCCH, the following conclusion was endorsed in the RAN#93-e meeting [5].

For a configured/defined CFR for GC-PDCCH/PDSCH carrying MCCH and MTCH for broadcast reception with UEs in RRC IDLE/INACTIVE state: - Support Case-C - Support at least one of Case D and Case E. Down-selection to be made at RAN1#106b-e - Note: Case C, D and E are defined in previous agreements.

Regarding the CFR for MCCH and MTCH, we support unified CFR for MCCH and MTCH reception as discussed in our companion paper [6]. Considering the impact to legacy UE, it should be ensured that the SIB-1 configured initial BWP keep the same behaviour for legacy mechanism, e.g., RRC_IDLE/INACTIVE UEs only apply the configuration of the SIB-1 configured initial BWP until after the reception of RRCSetup/RRCResume/RRCReestablishment. For MBS reception, it has been agreed that CFR was supported and UE receive MBS in the range of CFR. Even though it has agreed that CFR can use the same size of CORESET 0, CORESET 0 has limited frequency range and become “crowded”. Besides, DM2 broadcast services is a typical scenario for MBS, it may need some larger resources for broadcast reception. If the bandwidth of initial BWP is changed due to introducing the MBS services, it also will affect the legacy UEs’s capability as discussed in our companion contribution [6]. Therefore, we suggest to discuss the CFR configuration independently.

Proposal 1: CFR can be configured with any size as long as it covers CORESET#0 for MCCH reception.

  - CSS design for broadcast MCCH and MTCH

In the previous RAN1 meeting, it has agreed that CSS is supported for group-common PDCCH for RRC_IDLE RRC_INACTIVE UEs and the same CSS type is supported for MCCH and MTCH. Both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH, the detailed agreements are copied as following:

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH.

However, which CSS type used for both searchspace#0 and search space other than searspace#0 is controversial. The following agreements were achieved in last meeting.

Agreement: Study whether the Type-x CSS supported for multicast in RRC_CONNECTED can be reused as baseline for broadcast in RRC_IDLE/RRC_INACTIVE for GC-PDCCH scheduling MCCH and MTCH.

For broadcast reception, as delivery mode 2 defined by RAN2, it can be received by all RRC states UE, e.g., RRC_CONNECTED/RRC_IDLE/RRC_INACTIVE states, which means some UEs receive broadcast in RRC connected states, some UEs receive broadcast in RRC INACTIVE states, and some UEs receive broadcast in RRC IDLE states. Therefore, we prefer the same CSS type can be used for all three RRC states for broadcast transmission. Besides, the CSS type’s discussion is ongoing for RRC_CONNECTED UEs in AI8.12.1. From our perspective, the same CSS type with corresponding RNTI (e.g., MCCH-RNTI) can be reused for MCCH channel. The network can configure this CSS type with MCCH-RNTI to support group common PDSCH reception associated with MCCH information.

Proposal 2: The CSS type defined in AI 8.12.1 (e.g., a new Type-x CSS) for MBS group scheduling can be used for both searchSpace#0 and search space other than searchSpace#0 for GC-PDCCH scheduling MCCH.

  - DCI design for broadcast MCCH

Regarding the DCI format used for broadcast RRC IDLE/INACTIVE UEs, DCI format 1_0 was agreed as baseline for GC-PDCCH of MCCH and MTCH in previous meeting.

Agreement: The DCI format for GC-PDCCH scheduling a GC-PDSCH carrying MCCH/MTCH at least includes the following fields for broadcast reception with UEs in RRC_IDLE/INACTIVE state: - FDRA field - TDRA field - Modulation and coding scheme - Redundancy version - FFS: - MCCH change notification (if supported and only for MCCH), - RB numbering starts from the lowest RB of the CFR and support of resource allocation with granularity of single or multiple RBs. - HARQ process number and New data indicator - VRB-to-PRB mapping - other fields if needed.

Considering the UE processing complexity, the same DCI format used for MCCH/MTCH can be reused for MCCH change notification no matter which alternative is adopted. Since it has been agreed as baseline for GC-PDCCH of MCCH and MTCH in previous meeting, we propose DCI 1_0 is used for MCCH change notification.

Proposal 3: MBS DCI format 1_0 used for MCCH and MTCH reception is reused for NR MBS MCCH change notification.

In LTE SC-PTM, MCCH change notification is also indicated by PDCCH using DCI format 1C with CRC scrambled by SC-N-RNTI. For Rel-17 NR MBS, the similar MCCH change notification can be reused, e.g., the new RNTI (MCCH-N-RNTI) can be defined for NR MBS MCCH change notification. As for the notification of MCCH configuration changes due to a session start and the notification of MCCH configuration changes of an ongoing session, the two bits also can be included in the DCI format 1_0 with MCCH-N-RNTI. Thus, we suggest that DCI format 1_0 scrambled by a new RNTI (e.g., MCCH-N-RNTI) can be used for MCCH change notification.

Proposal 4: A new RNTI (e.g., MCCH-N-RNTI) can be used for MCCH change notification.

- Conclusion

In this contribution, we discuss the MCCH physical layer design for RRC_IDLE/RRC_INACTIVE UEs, and the following proposals are suggested:

Proposal 1: CFR can be configured with any size as long as it covers CORESET#0 for MCCH reception.

Proposal 2: The CSS type defined in AI 8.12.1 (e.g., a new Type-x CSS) for MBS group scheduling can be used for both searchSpace#0 and search space other than searchSpace#0 for GC-PDCCH scheduling MCCH.

Proposal 3: MBS DCI format 1_0 used for MCCH and MTCH reception is reused for NR MBS MCCH change notification.

Proposal 4: A new RNTI (e.g., MCCH-N-RNTI) can be used for MCCH change notification.

- Reference

- RAN2 Chairman’s Notes, RAN2 #113bis-e, April 2021.

- R2-2104639, “LS on broadcast session delivery and MCCH design”, Huawei, HiSilicon, RAN2#113bis-e, April 12th - 20th 2021.

- RAN1 Chairman’s Notes, RAN1 #105-e, May 2021.

- R2-2106544, “LS on update for MCCH design”, Huawei, HiSilicon, RAN2#114-e, May 19th - 27th 2021.

- RAN Chairman’s Notes, RAN #93-e, September 2021.

- R1-2109569, “Discussion on MBS for RRC_IDLE/INACTIVE UEs”, MediaTek, RAN1#106bis-e, October 11th-19th, 2021